Bay Area Air Quality Management District

939 Ellis Street

San Francisco, CA  94109

(415) 771-6000

- Major Facility Review Permit

- Issued To:

- Shell Martinez Refinery, Shell Oil Products US

- Facility # A0011

- Facility Address:

- 3485 Pacheco Blvd.

Martinez, CA  94553

- Mailing Address:

- P O Box 711

Martinez, CA  94553

Responsible Official 	Facility Contact

Aamir Farid,	T. K. Makarewicz,

General Manager	Manager, Environmental Affairs Department

925-313-3333	925-313-3705

Type of Facility:	Petroleum Refining	BAAQMD Engineering Division Contact:

Primary SIC:	2911	M.K. Carol Lee

Product:	Petroleum Refining	Senior Air Quality Engineer

- ISSUED BY THE BAY AREA AIR QUALITY MANAGEMENT DISTRICT

Jack Broadbent, Executive Office/Air Pollution Control Officer		Date

- TABLE OF CONTENTS

I.		STANDARD CONDITIONS	3

II.		EQUIPMENT	8

iii.		GENERALLY APPLICABLE REQUIREMENTS	54

IV. 		SOURCE-SPECIFIC APPLICABLE REQUIREMENTS	57

V. 		sCHEDULE OF COMPLIANCE	325

VI. 		PERMIT CONDITIONS	325

VII.		APPLICABLE LIMITS & COMPLIANCE MONITORING REQUIREMENTS	439

Viii.		TEST METHODS	561

Ix.		PERMIT SHIELD	567

X.			Glossary	577

XI.		APPLICABLE STATE IMPLEMENTATION PLAN	587

# I.  STANDARD CONDITIONS

## Administrative Requirements

The permit holder shall comply with all applicable requirements in the following regulations:

BAAQMD Regulation 1 - General Provisions and Definitions

(as amended by the District Board on 5/2/01);

SIP Regulation 1 - General Provisions and Definitions

(as approved by EPA through 6/28/99);

BAAQMD Regulation 2, Rule 1 - Permits, General Requirements

(as amended by the District Board on 8/1/01);

SIP Regulation 2, Rule 1 - Permits, General Requirements

(as approved by EPA through 1/26/99);

BAAQMD Regulation 2, Rule 2 - Permits, New Source Review

(as amended by the District Board on 5/17/00);

SIP Regulation 2, Rule 2 - Permits, New Source Review and Prevention of Significant Deterioration

(as approved by EPA through 1/26/99);

BAAQMD Regulation 2, Rule 4 - Permits, Emissions Banking

(as amended by the District Board on 5/17/00);

SIP Regulation 2, Rule 4 - Permits, Emissions Banking

(as approved by EPA through 1/26/99); and

BAAQMD Regulation 2, Rule 6 - Permits, Major Facility Review

(as amended by the District Board on 4/16/03).

## Conditions to Implement Regulation 2, Rule 6, Major Facility Review

- This Major Facility Review Permit was issued on December 1, 2003, and expires on November 30, 2008.  The permit holder shall submit a complete application for renewal of this Major Facility Review Permit no later than May 31, 2008 and no earlier than November 30, 2007.  If a complete application for renewal has not been submitted in accordance with this deadline, the facility may not operate after November 30, 2008.  (Regulation 2-6-307, 404.2, & 409.6; MOP Volume II, Part 3, §4.2)

- The permit holder shall comply with all conditions of this permit.  The permit consists of this document and all appendices.  Any non-compliance with the terms and conditions of this permit will constitute a violation of the law and will be grounds for enforcement action; permit termination, revocation and re-issuance, or modification; or denial of a permit renewal application.  (Regulation 2-6-307; MOP Volume II, Part 3, §4.11)

- In the event any enforcement action is brought as a result of a violation of any term or condition of this permit, the fact that it would have been necessary for the permittee to halt or reduce the permitted activity in order to maintain compliance with such term or condition shall not be a defense to such enforcement action. (MOP Volume II, Part 3, §4.11)

- This permit may be modified, revoked, reopened and reissued, or terminated for cause.  (Regulation 2-6-307, 409.8, 415; MOP Volume II, Part 3, §4.11)

- The filing of a request by the facility for a permit modification, revocation and re-issuance, or termination, or the filing of a notification of planned changes or anticipated non-compliance does not stay the applicability of any permit condition.  (Regulation 2-6-409.7; MOP Volume II, Part 3, §4.11)

- This permit does not convey any property rights of any sort, or any exclusive privilege.  (Regulation 2-6-409.7; MOP Volume II, Part 3, §4.11)

- The permit holder shall supply within 30 days any information that the District requests in writing to determine whether cause exists for modifying, revoking and reissuing, or terminating the permit or to determine compliance with the permit.  (Regulation 1-441, Regulation 2-6-409.4 & 501; MOP Volume II, Part 3, §4.11)

- Any records required to be maintained pursuant to this permit which the permittee considers to contain proprietary or trade secret information shall be prominently designated as such.  Copies of any such proprietary or trade secret information which are provided to the District shall be maintained by the District in a locked confidential file, provided, however, that requests from the public for the review of any such information shall be handled in accordance with the District's procedures set forth in Section 11 of the District’s Administrative Code.  (Regulation 2-6-419; MOP Volume II, Part 3, §4.11)

- Proprietary or trade secret information provided to EPA will be subject to the requirements of 40 CFR Part 2, Subpart B - Public Information, Confidentiality of Business Information.  (40 CFR Part 2)

- The emissions inventory submitted with the application for this Major Facility Review Permit is an estimate of actual emissions or the potential to emit for the time period stated and is included only as one means of determining applicable requirements for emission sources.  It does not establish, or constitute a basis for establishing, any new emission limitations.  (MOP Volume II, Part 3, §4.11)

- The responsible official shall certify all documents submitted by the facility pursuant to the major facility review permit.  The certification shall state that based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  The certifications shall be signed by a responsible official for the facility.

## Requirement to Pay Fees

The permit holder shall pay annual fees in accordance with District Regulation 3, including Schedule P.  (Regulation 2-6-402 & 409.13, Regulation 3; MOP Volume II, Part 3, §4.12)

## Inspection and Entry

Access to Facility:  The permit holder shall provide reasonable access to the facility and equipment which is subject to this permit to the APCO and/or to his or her designee.  (Regulation 1-440, Regulation 2-6-409.3; MOP Volume II, Part 3, §4.14)

## Records

1.	The permit holder must provide any information, records, and reports requested or specified by the APCO.  (Regulation 1-441, Regulation 2-6-409.4)

2.	Notwithstanding the specific wording in any requirement, all records for federally enforceable requirements shall be maintained for at least five years from the date of creation of the record.  (Regulation 2-6-501, Regulation 3; MOP Volume II, Part 3, §4.7)

## Monitoring Reports

Reports of all required monitoring must be submitted to the District at least once every six months, except where an applicable requirement specifies more frequent reporting. The first reporting period for this permit shall be December 1, 2003, to May 31, 2004.  The second reporting period for this permit shall be June 1, 2004, to June 30, 2004.  Subsequent reports shall be for the following periods:  July 1st through December 31st and January 1st through June 30th.  All reports are due on the last day of the month after the end of the reporting period.  All instances of non-compliance shall be clearly identified in these reports.  The reports shall be certified by the responsible official as true, accurate, and complete.  In addition, all instances of non-compliance with the permit shall be reported in writing to the District’s Compliance and Enforcement Division within 10 calendar days of the discovery of the incident.  Within 30 calendar days of the discovery of any incident of non-compliance, the facility shall submit a written report including the probable cause of non-compliance and any corrective or preventative actions. The reports shall be sent to the following address:

Director of Compliance and Enforcement

Bay Area Air Quality Management District

939 Ellis Street

San Francisco, CA 94109

Attn:  Title V Reports

(Regulation 2-6-502, Regulation 3; MOP Volume II, Part 3, §4.7)

## Compliance Certification

Compliance certifications shall be submitted annually by the responsible official of this facility to the Bay Area Air Quality Management District and to the Environmental Protection Agency.  The first certification period shall be December 1, 2003, to November 30, 2004. The second certification period shall be December 1, 2004, to December 31, 2004.  Subsequent certification periods will be January 1st to December 31st.  All compliance certifications are due on the last day of the month after the end of the certification period.  The certification must list each applicable requirement, the compliance status, whether compliance was continuous or intermittent, the method used to determine compliance, and any other specific information required by the permit.  The permit holder may satisfy this requirement through submittal of District-generated Compliance Certification forms.  The certification should be directed to the District’s Compliance and Enforcement Division at the address above, and a copy of the certification shall be sent to the Environmental Protection Agency at the following address:

- Director of the Air Division

USEPA, Region IX

75 Hawthorne Street

San Francisco, CA   94105

Attention:  Air-3

(MOP Volume II, Part 3, §4.5 and 4.15)

## Emergency Provisions

- The permit holder may seek relief from enforcement action in the event of a breakdown, as defined by Regulation 1-208 of the District's Rules and Regulations, by following the procedures contained in Regulations 1-431 and 1-432.  The District will thereafter determine whether breakdown relief will be granted in accordance with Regulation 1-433.  (MOP Volume II, Part 3, §4.8)

- The permit holder may seek relief from enforcement action for a violation of any of the terms and conditions of this permit by applying to the District's Hearing Board for a variance pursuant to Health and Safety Code Section 42350.  The Hearing Board will determine after notice and hearing whether variance relief should be granted in accordance with the procedures and standards set forth in Health and Safety Code Section 42350 et seq.  (MOP Volume II, Part 3, §4.8)

- The granting by the District of breakdown relief or the issuance by the Hearing Board of a variance will not provide relief from federal enforcement.  (MOP Volume II, Part 3, §4.8)

## Severability

In the event that any provision of this permit is invalidated by a court or tribunal of competent jurisdiction, or by the Administrator of the EPA, all remaining portions of the permit shall remain in full force and effect.  (Regulation 2-6-409.5; MOP Volume II, Part 3, §4.10)

J.	Miscellaneous Conditions

For grandfathered sources, the throughput limits as shown in Condition # 18618 are based upon District records at the time of the MFR permit issuance.  The facility must report any exceedance of these limits following the procedures in Section I.F.  This reporting requirement is intended to facilitate a determination of whether a modification has occurred as defined in Regulation 2-1-234.3.  The throughput limits for grandfathered sources are for reporting purposes only.  Exceedance of this limit does not establish a presumption that a modification has occurred, nor does compliance with the limit establish a presumption that a modification has not occurred.

## Accidental Release

This facility is subject to 40 CFR Part 68, Chemical Accident Prevention Provisions.  The permit holder shall submit a risk management plan (RMP) by the date specified in §68.10.  The permit holder shall also certify compliance with the requirements of Part 68 as part of the annual compliance certification, as required by Regulation 2, Rule 6.  (40 CFR Part 68, Regulation 2, Rule 6)

# II.  EQUIPMENT

| Table II A - Permitted Sources Each of the following sources has been issued a permit to operate pursuant to the requirements of BAAQMD Regulation 2, Permits. The capacities in this table are the maximum allowable capacities for each source, pursuant to Standard Condition I.J and Regulation 2-1-301. For sources that burn fuel, only gaseous fuels are used except for the following that also burn fuel oil: S1476, S1477, S1486, S1487, S1488, S1491, S1492, S1493, S1495, S1496, S1497, S1498, S1500, S1504, S1507, S1508, S1509, S1510, S1512, S1511, S1763, S4002, S4003, S4190, S4191, S4192, and S4193. |  |  |  |  |
| --- | --- | --- | --- | --- |
| S# | Description | Make or Type | Model | Capacity |
| 3 | Tank 3 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 4 | Tank 4 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 13 | Tank 13 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 14 | Tank 14 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 19 | Tank 19 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 20 | Tank 20 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 21 | Tank 21 Asphalt Storage | Custom Made, Fixed Roof | NA | 2,314,200 Gallon |
| 22 | Tank 22 Asphalt Storage | Custom Made, Fixed Roof | NA | 2,314,200 Gallon |
| 23 | Tank 23 Asphalt Storage | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 24 | Tank 24 Asphalt Storage | Custom Made, Fixed Roof | NA | 2,310,000 Gallon |
| 26 | Tank 26 Asphalt Storage | Custom Made, Fixed Roof | NA | 1,050,000 Gallon |
| 34 | Tank 34 | Custom Made, Floating Roof | NA | 336,000 Gallon |
| 63 | Tank 63 | Custom Made, Fixed Roof | NA | 19,740 Gallon |
| 129 | Tank 129 | Custom Made, External Floating Roof | NA | 634,477 Gallon |
| 224 | Tank 224 | Custom Made, Fixed Roof | NA | 23,100 Gallon |
| 257 | Tank 257 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 355 | Tank 355 | Custom Made, Fixed Roof | NA | 19,740 Gallon |
| 396 | Tank 396 | Custom Made, Fixed Roof | NA | 362,040 Gallon |
| 397 | Tank 397 | Custom Made, Fixed Roof | NA | 362,040 Gallon |
| 432 | Tank 432 | Custom Made, Fixed Roof | NA | 19,740 Gallon |
| 483 | Tank 483 | Custom Made, Fixed Roof | NA | 3,380,160 Gallon |
| 484 | Tank 484 | Custom Made, Fixed Roof | NA | 3,380,160 Gallon |
| 497 | Tank 497 Asphalt Storage | Custom Made, Fixed Roof | NA | 1,611,960 Gallon |
| 530 | Tank 530 | Custom Made, Fixed Roof | NA | 3,452,232 Gallon |
| 532 | Tank 532 | Custom Made, Fixed Roof | NA | 3,452,190 Gallon |
| 534 | Tank 534 | Custom Made, Fixed Roof | NA | 3,452,190 Gallon |
| 540 | Tank 540 | Custom Made, External Floating Roof | NA | 3,435,180 Gallon |
| 541 | Tank 541 | Custom Made, External Floating Roof | NA | 3,435,180 Gallon |
| 544 | Tank 544 | Custom Made, External Floating Roof | NA | 4,923,576 Gallon |
| 545 | Tank 545 | Custom Made, External Floating Roof | NA | 4,923,576 Gallon |
| 548 | Tank 548 | Custom Made, Fixed Roof | NA | 19,950 Gallon |
| 549 | Tank 549 | Custom Made, Fixed Roof | NA | 19,100 Gallon |
| 552 | Tank 552 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 553 | Tank 553 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 554 | Tank 554 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 555 | Tank 555 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 556 | Tank 556 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 557 | Tank 557 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 558 | Tank 558 Asphalt Storage | Custom Made, Fixed Roof | NA | 69,300 Gallon |
| 559 | Tank 559 Asphalt Storage | Custom Made, Fixed Roof | NA | 71,400 Gallon |
| 560 | Tank 560 Asphalt Storage | Custom Made, Fixed Roof | NA | 625,380 Gallon |
| 561 | Tank 561 Asphalt Storage | Custom Made, Fixed Roof | NA | 625,380 Gallon |
| 567 | Tank 567 Asphalt Storage | Custom Made, Fixed Roof | NA | 22,680 Gallon |
| 568 | Tank 568 | Custom Made, Fixed Roof | NA | 14,280 Gallon |
| 571 | Tank 571 Asphalt Storage | Custom Made, Fixed Roof | NA | 155,820 Gallon |
| 572 | Tank 572 Asphalt Storage | Custom Made, Fixed Roof | NA | 93,660 Gallon |
| 573 | Tank 573 Asphalt Storage | Custom Made, Fixed Roof | NA | 93,660 Gallon |
| 598 | Tank 598 Asphalt Storage | Custom Made, Fixed Roof | NA | 33,600 Gallon |
| 610 | Tank 610 | Custom Made, Fixed Roof | NA | 5,615,400 Gallon |
| 611 | Tank 611 | Custom Made, Fixed Roof | NA | 5,615,400 Gallon |
| 612 | Tank 612 | Custom Made, Fixed Roof | NA | 5,615,400 Gallon |
| 613 | Tank 613 | Custom Made, Fixed Roof | NA | 5,615,400 Gallon |
| 815 | Tank 815 Asphalt Storage | Custom Made, Fixed Roof | NA | 863,058 Gallon |
| 856 | Tank 856 | Custom Made, Fixed Roof | NA | 22,890 Gallon |
| 858 | Tank 858 | Custom Made, Internal Floating Roof | NA | 45,360 Gallon |
| 860 | Tank 860 | Custom Made | NA | 24,000 Gallons |
| 861 | Tank 861 | Custom Made | NA | 24,000 Gallons |
| 867 | Tank 867 Asphalt Storage | Custom Made, Fixed Roof | NA | 93,156 Gallon |
| 868 | Tank 868 Asphalt Storage | Custom Made, Fixed Roof | NA | 93,156 Gallon |
| 876 | Tank 876 Asphalt Storage | Custom Made, Fixed Roof | NA | 121,296 Gallon |
| 952 | Tank 952 | Custom Made, Internal Floating Roof | NA | 75,600 Gallons |
| 961 | Tank 961 Asphalt Storage | Custom Made, Fixed Roof | NA | 130,200 Gallon |
| 967 | Tank 967 | Custom Made, Fixed Roof | NA | 3,399,648 Gallon |
| 985 | Tank 985 Asphalt Storage | Custom Made, Fixed Roof | NA | 841,512 Gallon |
| 992 | Tank 992 | Custom Made, External Floating Roof | NA | 395,808 Gallon |
| 1004 | Tank 1004 | Custom Made | NA | 22,700 Gallon |
| 1005 | Tank 1005 | Custom Made, Fixed Roof | NA | 10,080 Gallon |
| 1006 | Tank 1006 | Custom Made, External Floating Roof | NA | 82,908 Gallon |
| 1017 | Tank 1017 Asphalt Storage | Custom Made, Fixed Roof | NA | 210,210 Gallon |
| 1018 | Tank 1018 Asphalt Storage | Custom Made, Fixed Roof | NA | 210,210 Gallon |
| 1023 | Tank 1023 | Custom Made, Internal Floating Roof | NA | 45,700 Gallon |
| 1031 | Tank 1031 | Custom Made, External Floating Roof | NA | 2,274,720 Gallon |
| 1041 | Tank 1041 Asphalt Storage | Custom Made, Fixed Roof | NA | 339,360 Gallon |
| 1043 | Tank 1043 Asphalt Storage | Custom Made, Fixed Roof | NA | 189,000 Gallon |
| 1044 | Tank 1044 Asphalt Storage | Custom Made, Fixed Roof | NA | 189,000 Gallon |
| 1045 | Tank 1045 Asphalt Storage | Custom Made, Fixed Roof | NA | 189,000 Gallon |
| 1046 | Tank 1046 | Custom Made, External Floating Roof | NA | 2,961,000 Gallon |
| 1048 | Tank 1048 Asphalt Storage | Custom Made, Fixed Roof | NA | 210,000 Gallon |
| 1050 | Tank 1050 | Custom Made, Internal Floating Roof | NA | 34,000 Gallon |
| 1051 | Tank 1051 | Custom Made, External Floating Roof | NA | 3,486,000 Gallon |
| 1063 | Tank 1063 ETP 1 | Custom Made, External Floating Roof | NA | 211,493 Gallon |
| 1067 | Tank 1067 ETP 1 | Custom Made, External Floating Roof | NA | 3,486,000 Gallon |
| 1070 | Tank 1070 | Custom Made, Fixed Roof | NA | 2,335,830 Gallon |
| 1072 | Tank 1072 | Custom Made, External Floating Roof | NA | 6,344,775 Gallon |
| 1075 | Tank 1075 Asphalt Storage | Custom Made, Fixed Roof | NA | 39,480 Gallon |
| 1076 | Tank 1076 | Custom Made, External Floating Roof | NA | 4,060,656 Gallon |
| 1077 | Tank 1411 | Custom Made, External Floating Roof | NA | 497,242 Gallon |
| 1114 | Tank 1114 Spent Acid | Custom Made, Fixed Roof | NA | 51,660 Gallon |
| 1115 | Tank 1115 Spent Acid | Custom Made, Fixed Roof | NA | 51,660 Gallon |
| 1116 | Tank 1116 Fresh Acid | Custom Made, Fixed Roof | NA | 51,660 Gallon |
| 1117 | Tank 1117 Skim | Custom Made, Fixed Roof | NA | 4,512 Gallon |
| 1129 | Tank 1129 | Custom Made, External Floating Roof | NA | 6,344,775 Gallon |
| 1130 | Tank 1130 | Custom Made, External Floating Roof | NA | 6,344,775 Gallon |
| 1131 | Tank 1131 | Custom Made, External Floating Roof | NA | 6,344,775 Gallon |
| 1133 | Tank 1133 | Custom Made, Fixed Roof | NA | 7,224,000 Gallon |
| 1134 | Tank 1134 | Custom Made, Fixed Roof | NA | 7,224,000 Gallon |
| 1139 | Tank 1139 | Custom Made, Fixed Roof | NA | 3,399,648 Gallon |
| 1140 | Tank 1140 | Custom Made, Fixed Roof | NA | 4,219,511 Gallon |
| 1141 | Tank 1141 | Custom Made | NA | 2,310,000 Gallon |
| 1146 | Tank 1146 | Custom Made, Internal Floating Roof | NA | 210,976 Gallon |
| 1147 | Tank 1147 | Custom Made, Internal Floating Roof | NA | 210,976 Gallon |
| 1159 | Tank 1159 | Custom Made, External Floating Roof | NA | 2,274,720 Gallon |
| 1160 | Tank 1160 Asphalt Storage | Custom Made, Fixed Roof | NA | 2,349,900 Gallon |
| 1161 | Tank 1161 | Custom Made, External Floating Roof | NA | 11,279,601 Gallon |
| 1186 | Tank 1186 | Custom Made, Fixed Roof | NA | 7,056 Gallon |
| 1191 | Tank 1256 Crude Oil Storage | Custom Made, External Floating Roof | NA | 15,572,899 Gallon |
| 1192 | Tank 1257 Crude Oil Storage | Custom Made, External Floating Roof | NA | 15,572,899 Gallon |
| 1235 | Tank 739 Chem Storage | Custom Made, Fixed Roof | NA | 32,130 Gallon |
| 1236 | Tank 740 Chem Storage | Custom Made, Fixed Roof | NA | 32,130 Gallon |
| 1408 | LUBS Asphalt Blending and Shipping | Custom Made | NA | NA |
| 1409 | LUBS Sulfonation Plant (SULF) | Custom Made | NA | NA |
| 1411 | LUBS Atmospheric Distillation LDU | Custom Made | NA | NA |
| 1412 | LUBS Vacuum Distillation LDU | Custom Made | NA | NA |
| 1415 | LUBS Furfural Plant (SEP3) | Custom Made | NA | NA |
| 1416 | LUBS -Lube Hydrotreater 1 (LHT1) | Custom Made | NA | NA |
| 1417 | OPCEN Distillate Saturation Unit (DSU) | Custom Made | NA | NA |
| 1420 | DH Crude Unit (CU) | Custom Made | NA | NA |
| 1421 | DH Vacuum Flasher Unit (VFU) | Custom Made | NA | NA |
| 1422 | DH Marine Fuel Oil Blender | Custom Made | NA | NA |
| 1423 | DH Gas Oil Straightrun Hydrotreater (GOHT) | Custom Made | NA | NA |
| 1424 | DH Naphtha Straightrun Hydrotreater (NHT) | Custom Made | NA | NA |
| 1425 | DH Catalytic Reformer Unit (CRU) | Custom Made | NA | NA |
| 1426 | CP Catalytic Cracking Unit (CCU) | Custom Made | NA | NA |
| 1427 | CP Catalytic Gas Plant (CGP) | Custom Made | NA | NA |
| 1428 | CP Catalytic Feed Hydrotreater (CFH) | Custom Made | NA | NA |
| 1429 | CP Catalytic Gasoline Hydrotreater (CGH) | Custom Made | NA | NA |
| 1430 | CP Alkylation Plant (ALKY) | Custom Made | NA | NA |
| 1431 | CP Sulfur Plant 1 (SRU1) | Custom Made | NA | NA |
| 1432 | CP Sulfur Plant 2 (SRU2) | Custom Made | NA | NA |
| 1433 | CP Light CC Gasoline Treater | Custom Made | NA | NA |
| 1434 | CP Unsaturated C3/C4 Treater | Custom Made | NA | NA |
| 1435 | CP Fuel Gas Treater 1 | Custom Made | NA | NA |
| 1436 | CP Fuel Gas Treater 2 | Custom Made | NA | NA |
| 1445 | DH Hydrogen Plant 1 (HP1) | Custom Made | NA | NA |
| 1446 | DH Saturates Gas Plant (SGP) | Custom Made | NA | NA |
| 1447 | DH Saturates Dry Gas Treater | Custom Made | NA | NA |
| 1448 | DH Saturates Gas Plant C3/C4 Treater | Custom Made | NA | NA |
| 1449 | DH Hydrocracking Unit (HCU) | Custom Made | NA | NA |
| 1457 | Cooling Water Tower (CWT-32) | Marley/Fluor | NA | NA |
| 1462 | LOG Distillate Blender (Jet & Diesel Fuel) | Custom Made | NA | NA |
| 1463 | LOG Gasoline Blender | Custom Made | NA | NA |
| 1464 | LOG Thin Fuel Blender (Wharf) | Custom Made | NA | NA |
| 1465 | LOG Light Oil Products Gross Oil Separator | Custom Made | NA | NA |
| 1466 | LOG Wastewater Pond 8 | Custom Made | NA | NA |
| 1467 | LOG BioTreater for wastewater Pond 7 | Custom Made | NA | NA |
| 1468 | LOG Wastewater Pond 6 | Custom Made | NA | NA |
| 1469 | LOG API Separator with inlet Box and Bar Screen (ETP 1) | Custom Made | NA | NA |
| 1470 | LOG LPG Loading Flare | John Zink Co. | STF-U-8, 1 | NA |
| 1471 | LOP Auxiliary Flare | John Zink Co. | STF-SA-36S | NA |
| 1472 | LOP Main Flare | Custom Made | NA | NA |
| 1476 | LUBS F-24 Atmospheric Feed | Peabody | MU-24, 6 | See Condition # 16688 |
| 1477 | LUBS F-25 Vacuum Feed | COEN | B-18, 3; JZ, HP-14, 1 | See Condition # 16688 |
| 1478 | LUBS F-26 Furfural Raff | Nat. Gas Co., 24 | 24 | See Condition # 16688 |
| 1479 | LUBS F-27 Furfural Extr | Nat. Gas Co., 48 | 48 | See Condition # 16688 |
| 1480 | LUBS F-69 Asphalt Circulation | COEN | S-15, 2 | See Condition # 16688 |
| 1481 | OPCEN F-30 DSU | Western Products | UF, 10 | See Condition # 16688 |
| 1483 | LUBS F-32 Asphalt Circulation | COEN | S-13 1/2, 2 | See Condition # 16688 |
| 1484 | LUBS F-34 LHT Charge | NAO | No. 25, 3 | See Condition # 16688 |
| 1486 | DH F-40 CU Feed | John Zink | MA-18, 20 | See Condition # 16688 |
| 1487 | DH F-41B VFU Feed | John Zink | DB-16, 20 | See Condition # 16688 |
| 1488 | DH F-41A VFU Feed | John Zink | DB-16, 20 | See Condition # 16688 |
| 1490 | DH F-43 GOHT Feed | NAO | No.30, 3 | See Condition # 16688 |
| 1491 | DH F-44 NHT Feed | Custom Made | NA | See Condition # 16688 |
| 1492 | DH F-45 Primary Column Reboil | NAO | No. 30, 6 | See Condition # 16688 |
| 1493 | DH F-46 Stabilizer Reboil | NAO | No. 30, 4 | See Condition # 16688 |
| 1494 | DH F-47 Secondary Column Reboil | Custom Made | NA | See Condition # 16688 |
| 1495 | DH F-49 CRU Preheat | NAO | No. 325,16 | See Condition # 16688 |
| 1496 | DH F-50 CRU | NAO | No. 25, 21 | See Condition # 16688 |
| 1497 | DH F-51 CRU | NAO | No. 25, 11 | See Condition # 16688 |
| 1498 | DH F-52 CRU Reboil | NAO | No. 30, 4 | See Condition # 16688 |
| 1499 | DH F-53 CRU Regen | NAO | No. 30, 3 | See Condition # 16688 |
| 1500 | DH F-55 SGP Heat Medium | NAO | No. 30, 15 | See Condition # 16688 |
| 1502 | DH F-57 HCU First Stage Feed | NAO | No. 15, 16 | See Condition # 16688 |
| 1503 | DH F-58 HCU Second Stage Feed | NAO | No. 15,16 | See Condition # 16688 |
| 1504 | DH F-59 HCU Second Stage Reboil | John Zink | MA-20, 12 | See Condition # 16688 |
| 1505 | DH F-60 HP1 Steam Methane Reformer | Custom Made | Custom Made | See Condition # 16688 |
| 1506 | CP F-61 CGP Feed | NAO | No. 15, 4 | See Condition # 16688 |
| 1507 | UTIL CO Boiler 1 | Custom Made | Custom Made | See Condition # 18618, Part 1 |
| 1508 | CP F-63 CFH Feed | Lummus | Lummus | See Condition # 16688 |
| 1509 | UTIL CO Boiler 2 | J2 | YS-24, 4 | See Condition # 18618, Part 1 |
| 1510 | CP F-66 CCU Preheat | NAO | No. 30, 12 | See Condition # 16688 |
| 1511 | CP F-67 CCU LGO Reboil | NAO | #25, 8 | See Condition # 16688 |
| 1512 | UTIL CO Boiler 3 |  |  | See Condition # 18618, Part 1 |
| 1514 | UTIL F-70 Boiler 4 | Formey | MG/TTL-5, 4 | See Condition # 16688 |
| 1515 | DH F-71 HCU First Stage Reboil | NAO | No. 30, 8 | See Condition # 16688 |
| 1523 | LUBS Loading Rack Asphalt Inside T/T | Custom Made | NA | NA |
| 1524 | LUBS Loading Rack Asphalt Outside T/T AND T/C | Custom Made | NA | NA |
| 1525 | LUBS Loading Rack Asphalt Paving T/T | Custom Made | NA | NA |
| 1539 | LUBS LR-25 Loading Rack Acid Sludge and Sulfonation T/T | Custom Made | NA | NA |
| 1540 | LUBS LR-26 Oleum Unloading | Custom Made | NA | NA |
| 1598 | MAINT Gasoline Dispensing Facility | Service Station G7114, 2 gasoline nozzles | NA | NA |
| 1650 | MAINT Sandblasting Sand Hopper | Custom Made | NA | 725 Cu.Ft. |
| 1750 | Tank 1218 Spent Acid | Custom Made | NA | 200 Barrel |
| 1751 | Tank 1330 | Custom Made, Fixed Roof | NA | 6,344,775 Gallon |
| 1752 | Tank 1331 | Custom Made, Fixed Roof | NA | 6,344,775 Gallon |
| 1753 | Tank 1332 Gasoline | Custom Made, Fixed Roof | NA | 9,136,477 Gallon |
| 1754 | Tank 1333 Gasoline | Custom Made, Fixed Roof | NA | 9,136,477 Gallon |
| 1755 | Tank 1334 Gasoline | Custom Made, External Floating Roof | NA | 4,314,447 Gallon |
| 1756 | Tank 1335 Gasoline | Custom Made, External Floating Roof | NA | 4,314,447 Gallon |
| 1757 | Tank 1336 | Custom Made, Fixed Roof | NA | 4,060,656 Gallon |
| 1758 | Tank 1337 | Custom Made, Fixed Roof | NA | 4,060,656 Gallon |
| 1759 | OPCEN Flexicoker (FXU) | Custom Made | NA | NA |
| 1760 | OPCEN F-102 FXU Steam Superheater | Custom Made | NA | See Condition # 16688 |
| 1761 | OPCEN F-104 HP2 Steam Methane Reformer | Custom Made | NA | See Condition # 16688 |
| 1762 | DH F-128 CRU Interheater | Custom Made | NA | See Condition # 16688 |
| 1763 | DH F-126 CU Feed Heater | Custom Made | NA | See Condition # 16688 |
| 1764 | OPCEN Dimersol Plant (DIMER) | Custom Made | NA | NA |
| 1765 | OPCEN Sulfur Plant 3 (SRU3) | Custom Made | NA | See Condition # 18618, Part 1 |
| 1767 | OPCEN V-1019 Coke Silo | CB&I | NA | 1800 tons |
| 1768 | OPCEN V-1020 Coke Silo | CB&I | NA | 1800 tons |
| 1769 | OPCEN V-1021 Dry Fines Silo | CB&I | NA | 750 tons |
| 1770 | OPCEN C3/C4 Splitter | Custom Made | NA | NA |
| 1771 | OPCEN Flexigas Flare | John Zink | STF-V66 | NA |
| 1772 | OPCEN Hydrocarbon Flare | John Zink | STF-S-25C | NA |
| 1774 | OPCEN Hydrogen Plant 2 (HP2) | Custom Made | NA | NA |
| 1778 | Cooling Water Tower (CWT-50) | Marley | NA | NA |
| 1779 | OPCEN CPI Oil/Water Separator | Custom Made | NA | NA |
| 1800 | UTIL F-88 Boiler 5 | Erie City, 23M Keystone | NA | See Condition # 16688 |
| 1802 | LOGV-1533 Odorant Storage Tank | Custom Made | NA | 2,000 Gallon |
| 1803 | OPCEN Coke Corral | Custom Made | NA | NA |
| 1804 | MAINT Paint Spray Booth and Facility Coating | Custom Made | NA | NA |
| 1805 | Tank 12038 | Custom Made, Fixed Roof | NA | 47,750 Gallon |
| 1900 | MAINT Machine Shop Parts Cleaner | Custom Made | NA | NA |
| 1902 | MAINT Seal Room Parts Cleaner | Custom Made | NA | NA |
| 1903 | MAINT Paint Shop Solvent Tub | Custom Made | NA | NA |
| 2000 | OPCEN Corrosion Inhibitor Injection | Custom Made | NA | NA |
| 2001 | LOG Marine Loading Berth 1 | Custom Made | NA | NA |
| 2002 | LOG Marine Loading Berth 2 | Custom Made | NA | NA |
| 2003 | LOG Marine Loading Berth 3 | Custom Made | NA | NA |
| 2004 | LOG Marine Loading Berth 4 | Custom Made | NA | NA |
| 2007 | LOG Dissolved Nitrogen Floatation Unit North ETP 1 (DNF) | Custom Made | NA | NA |
| 2008 | LOG Dissolved Nitrogen Floatation Unit South ETP 1 (DNF) | Custom Made | NA | NA |
| 2009 | LUBS Wastewater Separator Dubbs Box | Custom Made | NA | NA |
| 2010 | LOG Wastewater Junction Boxes | Custom Made | NA | NA |
| 2011 | LOG Wastewater Collection Sumps (4) | Custom Made | NA | NA |
| 2012 | DH V-12378 Perchloroethylene Storage System | Custom Made | NA | 3,217 Gallon |
| 2013 | Tank 12467 | Custom Made, External Floating Roof | NA | 6,299,393 Gallon |
| 2014 | LOG Final ETP 1&2 Holding Ponds 5C and 5D | Custom Made | NA | 8,000,000 Gallon |
| 2445 | Tank 12445 | Custom Made, Internal Floating Roof | NA | 8,590,994 Gallon |
| 2446 | Tank 12446 | Custom Made, Internal Floating Roof | NA | 8,590,994 Gallon |
| 3000 | Portable Vacuum Distillation Unit (CCR Technologies Inc.) | Custom Made | NA | See Condition # 18618, Part 1 |
| 4001 | DC Delayed Coking Unit (DCU) | Custom Made | NA | NA |
| 4002 | DC F-13425-A DCU | Custom Made | NA | See Condition # 16688 |
| 4003 | DC F-13425-B DCU | Custom Made | Custom Made | See Condition # 16688 |
| 4005 | DC Coke Handling Facility | Custom Made | Custom Made | NA |
| 4020 | DC Distillate Hydrotreater (DHT) | Custom Made | NA | NA |
| 4021 | DC -F-13909 DHT Recycle | Custom Made | NA | See Condition # 16688 |
| 4031 | DC F-14012 HGHT Reboil | Custom Made | NA | See Condition # 16688 |
| 4050 | DC Catalytic Gas Depentanizer (CGDP) | Custom Made | NA | NA |
| 4080 | DC Isomerization Unit (ISOM) | Custom Made | NA | NA |
| 4140 | DC Heavy Cracked Gasoline Hydrotreater (HGHT) | Custom Made | NA | NA |
| 4141 | DC F-14011 HGHT Feed | Custom Made | NA | See Condition # 16688 |
| 4160 | DC Hydrogen Plant 3 (HP3) | Custom Made | NA | NA |
| 4161 | DC H-101 HP3 Steam Methane Reformer | KTI 90 MMSCFD H2 Reformer | NA | See Condition # 16688 |
| 4170 | LUBS Lube Hydrotreater 2 (LHT2) | Custom Made | NA | NA |
| 4171 | LUBS F-13000 LHT2 Feed | Custom Made | NA | See Condition # 16688 |
| 4180 | OPCEN Sulfur Plant 4 (SRU4) | Custom Made | NA | NA |
| 4190 | UTIL Boiler 6 Gas Turbine 1 | GE | Frame 6 | See Condition # 18618, Part 1 |
| 4191 | UTIL Boiler 6 Supplemental Steam Generator 1 | Custom-Made | NA | See Condition # 18618, Part 1 |
| 4192 | UTIL Boiler 6 Gas Turbine 2 | GE | Frame 6 | See Condition # 18618, Part 1 |
| 4193 | UTIL Boiler 6 Supplemental Steam Generator 2 | Custom-Made | NA | See Condition # 18618, Part 1 |
| 4201 | DC Clean Fuels Flare | John Zink | NA | NA |
| 4210 | Cooling Water Tower (CWT-13278) | Custom Made | NA | NA |
| 4211 | DC V-13222 ISOM Maintenance Drop Out Vessel | Custom Made | NA | NA |
| 4212 | DC V-13441 ISOM Maintenance Drop Out Vessel | Custom Made | NA | NA |
| 4307 | Tank 14687 MDEA Make-up | Custom Made, Fixed Roof | NA | 6200 Gallon |
| 4309 | Tank 14517 DEA | Custom Made, Fixed Roof | NA | 17000 Gallon |
| 4310 | Tank –13285 Sour Water | Custom Made, External Floating Roof | NA | 4,200,000 Gallon |
| 4311 | DC V-12555 ISOM Perchloroethylene Vessel | Custom Made, Pressure Vessel | NA | 1,785 Gallon |
| 4319 | Tank15096 Recovered Oil | Custom Made, Fixed Roof | NA | 1054878 Gallon |
| 4322 | Tank14571 Sour Water | Custom Made, Internal Floating Roof | NA | 710,896 Gallon |
| 4329 | Tank13260 Pentane | Custom Made, Pressure Vessel | NA | 2,146,308 Gallon |
| 4330 | Tank13261 Pentane | Custom Made, Pressure Vessel | NA | 2,146,308 Gallon |
| 4334 | Tank13276 Alkylate | Custom Made, Fixed Roof | NA | 4,231,613 Gallon |
| 4338 | LOG Pentane Loading Facility | Custom Made | NA | NA |
| 4347 | OPCEN Pit for Sulfur Plant 4 | Custom Made | NA | NA |
| 4349 | Tank –13262 Pentane | Custom Made, Pressure Vessel | NA | 2,146,308 Gallon |
| 4350 | LOG Tank 13187 Process Wastewater | Custom Made, Fixed Roof | NA | 1,096,377 Gallon |
| 4356 | LOG Tank 13188 Process Wastewater | Custom Made, Fixed Roof | NA | 1,096,377 Gallon |
| 5112 | LOG V-15112 Main Proto Vessel with Nitrogen Blanket | Custom Made, Fixed Roof | NA | 1,300 Gallon |
| 5113 | LOG V-15113 Main Proto Vessel with Nitrogen Blanket | Custom Made, Fixed Roof | NA | 1,300 Gallon |
| 5114 | LOG V-15114 Main Proto Vessel with Nitrogen Blanket | Custom Made, Fixed Roof | NA | 1,300 Gallon |
| 5115 | LOG Dissolved Nitrogen Flotation Unit North ETP 2 (DNF) | Custom Made | NA | NA |
| 5116 | LOG Dissolved Nitrogen Flotation Unit South ETP 2 (DNF) | Custom Made | NA | NA |
| 5117 | LOG Biotreater Tank SPM-14121 ETP 2 | Custom Made | NA | NA |
| 5118 | LOG SPM-14135 Bioclarifier ETP 2 | Custom Made | NA | NA |
| 5119 | LOG SPM-14137 Bioclarifier ETP 2 | Custom Made | NA | NA |
| 5121 | LOG SPM-14111 DNF Float Tank ETP 2 | Custom Made, Fixed Roof | NA | 12,000 Gallon |
| 5125 | LOG V-15117 Standby Proto Vessel | Custom Made, Fixed Roof | NA | 217 Gallon |
| 5140 | Diesel Engine | Volkswagen | SV45-4025BB | 53 HP |
| 6051 | Diesel Engine | Caterpillar | Cat 3406B | 470 HP |
| 6052 | Diesel Engine | Detroit Diesel | DDFF-T6VT | 302 HP |
| 6053 | Diesel Engine | Detroit Diesel | 9123700 Class2 | 1050 HP |
| 6054 | Diesel Engine | Cummins | N855-F | 240 HP |
| 6055 | Diesel Engine | Caterpillar | 3408B | 597 HP |
| 6056 | Diesel Engine | Caterpillar | 3406BD1 | 519 HP |
| 6057 | Diesel Engine | Caterpillar | 3406B | 519 HP |
| 6058 | Diesel Engine | VW |  | 80 HP |
| 6059 | Diesel Engine | Caterpillar | 1W6784 | 145 HP |
| 6060 | Diesel Engine | Ingersol | Cummings Diesel | 450 HP |
| 12490 | LOG Tank 12519 Wastewater ETP 1&2 | Custom Made, External Floating Roof | NA | 7,402,238 Gallon |
| 12491 | LOG Tank 12520 Wastewater ETP 1&2 | Custom Made, External Floating Roof | NA | 7,402,238 Gallon |
| 17095 | Tank 17095 | Custom Made, External Floating Roof | NA | 90,648 bbl |

| Table II B – Abatement Devices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Abatement Device | Description | Source(s) Controlled | Applicable Requirement | Operating Parameters | Limit or Efficiency |
| A1 | Mist Eliminator | S24 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A2 | Mist Eliminator | S860, S861, S1004 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A3 | Mist Eliminator | S552, S553, S554, S555, S556, S557, S558, S559, S1523 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A4 | Mist Eliminator | S560, S561, S815, S985 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A5 | Mist Eliminator – | S567 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A6 | Mist Eliminator | S1043, S1044, S1045 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A7 | Mist Eliminator | S1048, S1525 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A10 | Mist Eliminator – | S1160 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A11 | Mist Eliminator | S1411 | None | None | None |
| A12 | Electrostatic Precipitator 1 | S1426, S1507 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A13 | Electrostatic Precipitator 2 | S1426, S1509 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A14 | Electrostatic Precipitator 3 | S1426, S1512 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A22 | Vent Gas Compressors (3) for Vacuum Flasher Unit | S1421 | None | None | None |
| A23 | Vapor Control for Gasoline Dispensing Facility | S1598 | Regulation 87-301 & 302 | None | None |
| A25 | Vapor Recovery System J-149 for Vine Hill Storage Tanks | S610, S611, S612, S613, S1133, S1134, S1751, S1752, S1753, S1754, S1757, S1758, S4334 | Regulation 8-5-306; 40CFR60.112 a(a)(3); 40CFR60.112b(a)(3)(i); and 40 CFR 60.112(b)(3) (ii) | None | Abatement efficiency of at least 95% by weight |
| A26 | Vapor Recovery System J-150 for Interim Gasoline Storage Tanks | S13, S14, S19, S20, S534, S1070, S1139, S1140, S1141, S5112, S5113, S5114, S5125 | Regulation 8-5-306 | None | Abatement efficiency of at least 95% by weight |
| A27 | –SO2 Absorber for Sulfonation Plant | S1409, S1539, | None | None | None |
| A33 | LOP Vapor Recovery System | S1114, S1115, S1416, S1417, S1420, S1421, S1423, S1424, S1425, S1426, S1427, S1428, S1429, S1430 S1433, S1434, S1435, S1436, S1445, S1446, S1447, S1448, S1449, S2012, S4170 | Regulation 8-5-306, 8-28-303, 40CFR60.112b(a)(3)(i); and 40 CFR 60.112(b)(3) (ii) | None | Abatement efficiency of at least 95% by weight |
| A52 | SCOTNo. 1 | S1431 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| A53 | Mist Eliminator | S571, S572, S573, S1524 | Regulation 6-301 | None | Ringelmann No. 1 for 3 minutes in any hour |
| A54 | Mist Eliminator | S867, S868 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A56 | Vapor Recovery System for Crude String Storage Tanks | S483, S484, S530, S532, S4319, S4350, S4356 | Regulation 8-5-306, 40CFR60.112b(a)(3)(i); and 40 CFR 60.112(b)(3) (ii) | None | Abatement efficiency of at least 95% by weight |
| A57 | Mist Eliminator | S23, S26, S497 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A75 | Stretford Unit | S1759 | Condition # 7618 Part E2 | None | H2S limits in Flexigas |
| A76 | SCOT No. 3 | S1765 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| A77 | Baghouse for Flexicoker Silo | S1767, S1768, S1769 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A100 | ThermalOxidizer for Marine Vapor Recovery System | S2001, S2002, S2003, S2004 | Regulation 8-44-301 and Condtion 4288 Part 6 | Temperature > 1400 oF | destruction efficiency > 95 weight% |
| A101 | Flare for Vine Hill Vapor Recovery System | S610, S611, S612, S613, S1133, S1134, S1751, S1752, S1753, S1754, S1757, S1758, S4334 | Condition # 7761 Part 6 | None | None |
| A102 | Flare for Crude String Vapor Recovery System | S483, S484, S530, S532, S4319, S4350, S4356 | Condition # 7761 Part 11 | None | None |
| A103 | Flare for Interim Gasoline Vapor Recovery System | S1070, S13, S14, S534, S1139, S1140, S1141, S19, S20, S5112, S5113, S5114, S5125 | Condition # 7761 Part 1 | None | None |
| A1114 | Caustic Scrubber for Alky Spent Acid Tanks | S1114, S1115 | None | None | None |
| A1401 | API Headworks Water Scrubber (ETP 1) | S1469 | None | None | None |
| A1402 | API Headworks Carbon Adsorption for Water Scrubber (ETP 1) | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A1426 | Baghouse for CCU Spent Catalyst Hopper (Portable) | S1426 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A1427 | Baghouse for Catalyst Additive Storage and Injection System for CCU | S1426 | Condition 12911, Part 2 | None | Ringelmann No. 0.5 |
| A1431 | SCOT No. 2 | S1432 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| A1465 | Water Scrubber for LOP Gross Oil Separator | S1465 | None | None | None |
| A1467 | Carbon Adsorption Vessel for LOP Gross Oil Separator | S1465 | None | None | None |
| A1469 | Wastewater Separator Scrubber 1 (ETP 1) | S1469 | None | None | None |
| A1470 | Wastewater Separator Scrubber 2 (ETP 1) | S1469 | None | None | None |
| S1470 | LOG LPG Loading Flare | S4338 | Condition 12271 Part 74 | None | Overall capture and destruction efficiency of 98.5% by weight. |
| A1471 | Wastewater Separator Scrubber 3 | S1469 | None | None | None |
| S1471 | LOP Auxiliary Flare | S1114, S1115, S1416, S1417, S1420, S1421, S1423, S1424, S1425, S1426, S1427, S1428, S1429, S1430, S1433, S1434, S1435, S1436, S1445, S1446, S1447, S1448, S1449, S2012, S4170 | None | None | None |
| A1472 | Wastewater Separator Scrubber 4 (ETP 1) | S1469 | None | None | None |
| S1472 | LOP Main Flare | S1114, S1115, S1416, S1417, S1420, S1421, S1423, S1424, S1425, S1426, S1427, S1428, S1429, S1430, S1433, S1434, S1435, S1436, S1445, S1446, S1447, S1448, S1449, S2012, S4170 | None | None | None |
| A1473 | Carbon Adsorption Vessel for wwastewater Scrubber 1 (ETP 1) | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A1474 | Carbon Adsorption Vessel for Wastewater Scrubber 2 (ETP 1) | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A1475 | Carbon Adsorption Vessel for Wastewater Scrubber 3 (ETP 1) | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A1476 | Carbon Adsorption Vessel for Wastewater Scrubber 4 (ETP 1) | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A1501 | F-56 Backup Thermal Oxidizer for Sulfur Plants 1 and 2 | S1431, S1432 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| S1507 | UTIL CO Boiler 1 | S1426 | None | None | None |
| S1509 | UTIL CO Boiler 2 | S1426 | None | None | None |
| S1512 | UTIL CO Boiler 3 | S1426 | None | None | None |
| A1517 | F-77 Primary Thermal Oxidizer for Sulfur Plants 1 and 2 | S1431, S1432 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| A1518 | F-109 Catalytic Oxidizer for SCOT No. 3 | S1765 | Regulation 9-1-307 | None | 250 ppmv SO2 dry, at 0% oxygen |
| A1541 | Mist Eliminator | S1540, S860, S861, S1004 | Regulation 6-301 | None | Ringelmann No. 1 for < 3 minutes in any hour |
| A1765 | Scrubber for Sulfur Loading Rack Vent | S1765 S4347 S4180 | Condition 12271 Part 72 | None | Overall capture/ removal efficiency of 95% by weight and H2S shall not exceed 5 ppm |
| S1771 | OPCEN Flexigas Flare | S1759 | None | None | None |
| S1772 | OPCEN Hydrocarbon Flare | S1759, S1764, S1770, S1774 | None | None | None |
| A1779 | Scrubber No. 1 for CPI Separator Water | S1779 | None | None | None |
| A1780 | Scrubber No. 2 for CPI Separator Water | S1779 | None | None | None |
| A1781 | Carbon Adsorber w/ Water Scrubber No. 1 for CPI Separator | S1779 | None | None | None |
| A1782 | Carbon Adsorber w/ Water Scrubber No. 2 for CPI Separator | S1779 | None | None | None |
| A1803 | Water Spray Sprinklers for OPCEN Coke Corral | S1803 | Regulation 6-301&Condition # 4041 Parts 3 and 4 | None | Ringelmann No. 0.5 |
| A1805 | Carbon Adsorption (2 Canisters) | S1805 | Condition # 4298 Parts 5 and 6 and 40CFR61.354 (d) and 40CFR61.349 (a)(2)(ii) |  | Change out with unspent carbon upon breakthrough defined as detection at its outlet of 50 ppm VOC, measured as methane and Abatement efficiency of at least 95% by weight |
| A2007 | Water Scrubber for DNF Unit | S2007 | None | None | None |
| A2008 | Carbon Adsorber (2 Canisters) for Water Scrubber at DNF Unit | S2007 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A2009 | Water Scrubber for DNF Unit | S2008 | None | None | None |
| A2012 | Carbon Adsorber (2 Canisters) for Water Scrubber at DNF | S2008 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| 2017 | DNF Outlet Sump Water Scrubber (ETP 1) | S2007, S2008 | None |  |  |
| A2020 | DNF Outlet Sump Carbon Adsorption for Water Scrubber (ETP 1) | S2007, S2008 | 40CFR61.349 (a)(2)(ii) |  | Abatement efficiency of at least 95% by weight |
| A4002 | SCR No. 1 for Delayed Coking Unit | S4002 | Condition # 12271 Part 35 | None | NOx < 10 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4003 | SCR No. 2 for Delayed Coking Unit | S4003 | Condition # 12271 Part 35 | None | NOx < 10 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4005 | Particulate Collection Device for Delayed Coking Unit for Coke Corral | S4005 | Condition # 12271 Part 79 | None | 0.01 grain/DSCF |
| A4006 | Particulate Collection Device for Delayed Coking Unit for Hopper Loading | S4005 | Condition # 12271 Part 81 | None | 0.01 grain/DSCF |
| A4141 | SCR for HGHT Heaters | S4031, S4141 | Condition # 12271 Part 35 | None | NOx < 10 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4161 | SCR for Hydrogen Plant 3 Steam Methane Reformer | S4161 | Condition # 12271 Part 29 | None | NOx < 10ppmv, dry @ 15% O2, averaged over 3 hr |
| A4180 | SCOT No. 4 | S4180 | Condition # 12271 Part 66 and 67 | None | Total reduced sulfur prior to A4181 shall not exceed 100 ppm, dry, at 0% oxygen, averaged over 8 hours. |
| A4181 | Thermal Oxidizer for Sulfur Plant 4 | S4180, S4347 | Condition # 12271 Part 68 | None | Conversion Efficiency < 95 weight% |
| A4190 | SCR No. 1 for Boiler 6 | S4191, S4190 | Condition # 12271 Part 24c | None | NOx < 5 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4191 | Catalytic Oxidation No. 1 for Boiler 6 | S4190, S4191 | Condition # 12271 Part 24c | None | NOx < 5 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4192 | SCR No. 2 for Boiler 6 | S4193, S4192 | Condition # 12271 Part 24c | None | NOx < 5 ppmv, dry @ 15% O2, averaged over 3 hr |
| A4193 | Catalytic Oxidation No. 2 for Boiler 6 | S4192, S4193 | Condition # 12271 Part 24c | None | NOx < 5 ppmv, dry @ 15% O2, averaged over 3 hr |
| S4201 | DC Clean Fuels Flare | S4211, S4212, S4080, S4140, S4160, S4001, S4020, S4050 | Condition 12271 Part 57, 60 and 61 | None | Hydrocarbon destruction efficiency of 98.5% |
| A5115 | Carbon Adsorption Vessel (ETP 2) | S5115, S5116 | 40CFR61.349 (a)(2)(ii); Condition 11313 Part 2 | 1000 ppmv change out of second to last carbon bed | Abatement efficiency of at least 95% by weight; 1000 ppmv change out of second to last carbon bed |
| A5116 | Carbon Adsorption Vessel (ETP 2) | S5115, S5116 | 40CFR61.349 (a)(2)(ii); Condition # 11313 4 Part 3 | 500 ppmv change out of second to last carbon bed | Abatement efficiency of at least 95% by weight; 500 ppmv change out of second to last carbon bed |
| A20070 | Scrubber for ETP 1&2 API Flume – East | S1469 | None | None | None |
| A20080 | Carbon Adsorption Vessel on ETP 1&2 API Flume – East | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |
| A20090 | Scrubber System on ETP 1&2 API Flume – West | S1469 | None | None | None |
| A20120 | Carbon Adsorption Vessel on ETP 1&2 API Flume – West | S1469 | 40CFR61.349 (a)(2)(ii) | None | Abatement efficiency of at least 95% by weight |

The following table reflects tank sources which have been determine to be exempt from permitting requirements but have applicable requirements:

Table IIC - Exempt Source List of Tanks

| Source Number | Source Description | Permit Exemption |
| --- | --- | --- |
| 1 | Tank 1 | 2-1-123.3.2 2-1-123.3.3 |
| 2 | Tank 2 | 2-1-123.3.2 2-1-123.3.3 |
| 8 | Tank 8 | 2-1-123.3.2 2-1-123.3.3 |
| 10 | Tank 10 | 2-1-123.3.2 2-1-123.3.3 |
| 25 | Tank 25 | 2-1-123.3.2 2-1-123.3.3 |
| 28 | Tank 28 | 2-1-123.3.2 2-1-123.3.3 |
| 30 | Tank 30 | 2-1-123.3.2 2-1-123.3.3 |
| 31 | Tank 31 | 2-1-123.3.2 2-1-123.3.3 |
| 32 | Tank 32 | 2-1-123.3.2 2-1-123.3.3 |
| 33 | Tank 33 | 2-1-123.3.2 2-1-123.3.3 |
| 35 | Tank 35 | 2-1-123.3.2 2-1-123.3.3 |
| 36 | Tank 36 | 2-1-123.3.2 2-1-123.3.3 |
| 38 | Tank 38 | 2-1-123.3.2 2-1-123.3.3 |
| 49 | Tank 49 | 2-1-123.3.2 2-1-123.3.3 |
| 50 | Tank 50 | 2-1-123.3.2 2-1-123.3.3 |
| 53 | Tank 53 | 2-1-123.3.2 2-1-123.3.3 |
| 56 | Tank 56 | 2-1-103 |
| 84 | Tank 84 | 2-1-123.3.2 2-1-123.3.3 |
| 90 | Tank 90 | 2-1-123.3.2 2-1-123.3.3 |
| 109 | Tank 109 | 2-1-123.3.2 2-1-123.3.3 |
| 130 | Tank 130 | 2-1-123.3.2 |
| 145 | Tank 145 | 2-1-123.3.2 2-1-123.3.3 |
| 146 | Tank 146 | 2-1-123.3.2 |
| 147 | Tank 147 | 2-1-123.3.2 |
| 148 | Tank 148 | 2-1-123.3.2 |
| 149 | Tank 149 | 2-1-123.3.2 |
| 222 | Tank 222 | 2-1-123.3.2 2-1-123.3.3 |
| 258 | Tank 258 | 2-1-123.3.2 |
| 259 | Tank 259 | 2-1-123.3.2 2-1-123.3.3 |
| 260 | Tank 260 | 2-1-123.3.2 2-1-123.3.3 |
| 261 | Tank 261 | 2-1-123.3.2 2-1-123.3.3 |
| 262 | Tank 262 | 2-1-123.3.2 2-1-123.3.3 |
| 263 | Tank 263 | 2-1-123.3.2 2-1-123.3.3 |
| 267 | Tank 267 | 2-1-123.3.2 2-1-123.3.3 2-1-123.10 |
| 332 | Tank 332 | 2-1-123.3.2 2-1-123.3.3 |
| 334 | Tank 334 | 2-1-123.3.2 |
| 335 | Tank 335 | 2-1-123.3.2 |
| 336 | Tank 336 | 2-1-123.3.2 |
| 337 | Tank 337 | 2-1-123.3.2 |
| 343 | Tank 343 | 2-1-123.3.2 2-1-123.3.3 |
| 344 | Tank 344 | 2-1-123.3.2 2-1-123.3.3 |
| 346 | Tank 346 | 2-1-123.3.2 2-1-123.3.3 |
| 356 | Tank 356 | 2-1-123.3.2 2-1-123.3.3 |
| 368 | Tank 368 | 2-1-123.3.2 2-1-123.3.3 |
| 393 | Tank 393 | 2-1-123.3.2 2-1-123.3.3 |
| 394 | Tank 394 | 2-1-123.3.2 2-1-123.3.3 |
| 398 | Tank 398 | 2-1-123.3.2 2-1-123.3.3 |
| 400 | Tank 400 | 2-1-123.3.2 2-1-123.3.3 |
| 401 | Tank 401 | 2-1-123.3.2 |
| 418 | Tank 418 | 2-1-123.3.3 |
| 419 | Tank 419 | 2-1-123.3.3 |
| 422 | Tank 422 | 2-1-123.3.2 2-1-123.3.3 |
| 423 | Tank 423 | 2-1-123.3.2 2-1-123.3.3 |
| 424 | Tank 424 | 2-1-123.3.2 2-1-123.3.3 |
| 426 | Tank 426 | 2-1-123.3.2 2-1-123.3.3 |
| 427 | Tank 427 | 2-1-123.3.2 2-1-123.3.3 |
| 428 | Tank 428 | 2-1-123.3.2 2-1-123.3.3 |
| 467 | Tank 467 | 2-1-123.3.3 |
| 468 | Tank 468 | 2-1-123.3.3 |
| 475 | Tank 475 | 2-1-123.3.2 |
| 476 | Tank 476 | 2-1-123.3.2 |
| 477 | Tank 477 | 2-1-123.3.2 2-1-123.3.3 |
| 478 | Tank 478 | 2-1-123.3.2 2-1-123.3.3 |
| 485 | Tank 485 | 2-1-123.3.2 |
| 514 | Tank 514 | 2-1-123.3.2 2-1-123.3.3 |
| 523 | Tank 523 | 2-1-123.3.2 2-1-123.3.3 |
| 524 | Tank 524 | 2-1-123.3.2 2-1-123.3.3 |
| 525 | Tank 525 | 2-1-123.3.2 |
| 526 | Tank 526 | 2-1-123.3.2 |
| 536 | Tank 536 | 2-1-123.3.2 |
| 537 | Tank 537 | 2-1-123.3.2 |
| 597 | Tank 597 | 2-1-123.3.2 2-1-123.3.3 |
| 608 | Tank 608 | 2-1-123.3.2 |
| 609 | Tank 609 | 2-1-123.3.2 |
| 713 | Tank 713 | 2-1-123.3.2 |
| 714 | Tank 714 | 2-1-123.3.2 |
| 786 | Tank 786 | 2-1-123.3.2 2-1-123.3.3 |
| 787 | Tank 787 | 2-1-123.3.2 2-1-123.3.3 |
| 804 | Tank 804 | 2-1-123.3.2 2-1-123.3.3 |
| 806 | Tank 806 | 2-1-123.3.2 2-1-123.3.3 |
| 814 | Tank 814 | 2-1-123.3.2 2-1-123.3.3 |
| 822 | Tank 822 | 2-1-123.3.2 2-1-123.3.3 |
| 833 | Tank 833 | 2-1-123.3.2 2-1-123.3.3 |
| 834 | Tank 834 | 2-1-123.3.2 2-1-123.3.3 |
| 835 | Tank 835 | 2-1-123.3.2 |
| 837 | Tank 837 | 2-1-123.3.2 2-1-123.3.3 |
| 839 | Tank 839 | 2-1-123.3.2 2-1-123.3.3 |
| 850 | Tank 850 | 2-1-123.3.3 |
| 851 | Tank 851 | 2-1-123.3.2 2-1-123.3.3 |
| 869 | Tank 869 | 2-1-123.3.2 2-1-123.3.3 |
| 877 | Tank 877 | 2-1-123.3.2 2-1-123.3.3 |
| 879 | Tank 879 | 2-1-123.3.2 2-1-123.3.3 |
| 880 | Tank 880 | 2-1-123.3.2 2-1-123.3.3 |
| 888 | Tank 888 | 2-1-103 |
| 890 | Tank 890 | 2-1-123.3.2 2-1-123.3.3 |
| 902 | Tank 902 | 2-1-123.3.2 2-1-123.3.3 |
| 903 | Tank 903 | 2-1-123.3.2 2-1-123.3.3 |
| 926 | Tank 926 | 2-1-123.3.2 2-1-123.3.3 |
| 927 | Tank 927 | 2-1-123.3.2 2-1-123.3.3 |
| 928 | Tank 928 | 2-1-123.3.2 2-1-123.3.3 |
| 929 | Tank 929 | 2-1-123.3.2 2-1-123.3.3 |
| 932 | Tank 932 | 2-1-123.3.2 |
| 933 | Tank 933 | 2-1-123.3.2 |
| 934 | Tank 934 | 2-1-123.3.2 2-1-123.3.3 |
| 935 | Tank 935 | 2-1-123.3.11 |
| 942 | Tank 942 | 2-1-123.3.2 2-1-123.3.3 |
| 953 | Tank 953 | 2-1-123.3.2 2-1-123.3.3 |
| 954 | Tank 954 | 2-1-123.3.2 2-1-123.3.3 |
| 955 | Tank 955 | 2-1-123.3.2 2-1-123.3.3 |
| 956 | Tank 956 | 2-1-123.3.2 2-1-123.3.3 |
| 957 | Tank 957 | 2-1-123.3.2 2-1-123.3.3 |
| 958 | Tank 958 | 2-1-123.3.2 2-1-123.3.3 |
| 959 | Tank 959 | 2-1-123.3.2 2-1-123.3.3 |
| 962 | Tank 962 | 2-1-123.3.2 |
| 965 | Tank 965 | 2-1-123.3.2 |
| 966 | Tank 966 | 2-1-123.3.2 |
| 968 | Tank 968 | 2-1-123.3.2 2-1-123.3.3 |
| 980 | Tank 980 (Butane Sphere) | 2-1-123.3.1 |
| 981 | Tank 981 | 2-1-123.3.2 2-1-123.3.3 |
| 987 | Tank 987 | 2-1-123.3.2 2-1-123.3.3 |
| 988 | Tank 988 | 2-1-123.3.2 2-1-123.3.3 |
| 993 | Tank 993 | 2-1-123.3.2 2-1-123.3.3 |
| 1000 | Tank 1000 | 2-1-123.3.2 2-1-123.3.3 |
| 1001 | Tank 1001 | 2-1-123.3.3 |
| 1003 | Tank 1003 | 2-1-123.3.3 |
| 1009 | Tank 1009 | 2-1-123.3.2 2-1-123.3.3 |
| 1012 | Tank 1012 | 2-1-123.3.2 2-1-123.3.3 |
| 1013 | Tank 1013 | 2-1-123.3.2 2-1-123.3.3 |
| 1020 | Tank 1020 | 2-1-123.2 |
| 1024 | Tank 1024 | 2-1-123.3.3 |
| 1026 | Tank 1026 | 2-1-123.3.2 |
| 1027 | Tank 1027 | 2-1-123.3.2 |
| 1032 | Tank 1032 | 2-1-123.3.2 2-1-123.3.3 |
| 1034 | Tank 1034 | 2-1-123.3.3 |
| 1040 | Tank 1040 | 2-1-123.3.2 2-1-123.3.3 |
| 1047 | Tank 1047 (Butane Sphere) | 2-1-123.3.1 |
| 1049 | Tank 1049 | 2-1-123.3.2 2-1-123.3.3 |
| 1052 | Tank 1052 | 2-1-123.3.2 2-1-123.3.3 |
| 1053 | Tank 1053 | 2-1-123.3.2 2-1-123.3.3 |
| 1054 | Tank 1054 | 2-1-123.3.2 2-1-123.3.3 |
| 1055 | Tank 1055 | 2-1-123.3.2 2-1-123.3.3 |
| 1056 | Tank 1056 | 2-1-123.3.2 2-1-123.3.3 |
| 1057 | Tank 1057 | 2-1-123.3.2 2-1-123.3.3 |
| 1058 | Tank 1058 | 2-1-123.3.2 2-1-123.3.3 |
| 1059 | Tank 1059 | 2-1-123.3.2 2-1-123.3.3 |
| 1060 | Tank 1060 | 2-1-123.3.2 2-1-123.3.3 |
| 1061 | Tank 1061 | 2-1-123.3.2 2-1-123.3.3 |
| 1062 | Tank 1062 | 2-1-123.3.2 2-1-123.3.3 |
| 1066 | Tank 1066 | 2-1-123.3.2 2-1-123.3.3 |
| 1068 | Tank 1068 | 2-1-123.2 |
| 1069 | Tank 1069 | 2-1-123.3.2 2-1-123.3.3 |
| 1071 | Tank 1071 | 2-1-123.3.2 2-1-123.3.3 |
| 1073 | Tank 1073 | 2-1-123.3.2 2-1-123.3.3 |
| 1074 | Tank 1074 | 2-1-123.3.2 2-1-123.3.3 |
| 1081 | Tank 1081 | 2-1-123.3.2 2-1-123.3.10 |
| 1084 | Tank 1084 | 2-1-123.3.2 2-1-123.3.3 |
| 1112 | Tank 1112 | 2-1-123.3.2 2-1-123.3.3 |
| 1125 | Tank 1125 | 2-1-123.3.2 2-1-123.3.3 |
| 1128 | Tank 1128 | 2-1-123.3.2 |
| 1132 | Tank 1132 | 2-1-123.3.2 |
| 1142 | Tank 1142 | 2-1-123.3.2 |
| 1143 | Tank 1143 | 2-1-123.3.2 |
| 1144 | Tank 1144 | 2-1-123.3.2 |
| 1149 | V 1049 Butane Sphere | 2-1-123.3.1 |
| 1150 | V 1150 Butane Sphere | 2-1-123.3.1 |
| 1151 | V 1151 Butane Sphere | 2-1-123.3.1 |
| 1152 | V 1152 Propane Bullet | 2-1-123.3.1 |
| 1153 | V 1153 Propane Bullet | 2-1-123.3.1 |
| 1154 | V 1154 Propane Bullet | 2-1-123.3.1 |
| 1165 | Tank 1165 | 2-1-123.3.3 |
| 1173 | Tank 1173 | 2-1-123.3.2 2-1-123.3.3 |
| 1174 | Tank 1174 | 2-1-123.3.3 |
| 1175 | Tank 1175 | 2-1-123.3.3 |
| 1185 | Tank 1185 | 2-1-103 |
| 1190 | Tank 1190 | 2-1-123.3.3 |
| 1231 | Tank 1231 Fresh DEA | 2-1-123.3.2 2-1-123.3.3 |
| 1347 | Tank 1347 | 2-1-123.3.2 |
| 1348 | Tank 1348 | 2-1-123.3.2 |
| 1564 | Tank 1564 | 2-1-123.3.2 |
| 1780 | Tank 1338 Butane Sphere | 2-1-123.3.1 |
| 1781 | Tank 1341 Propane Bullet | 2-1-123.3.1 |
| 2006 | Tank 906 Caustic Service | 2-1-123.2 |
| 5123 | LOG V-15115 Standby Proto Vessel | 2-1-123.1 |
| 5124 | LOG V-15116 Standby Proto Vessel | 2-1-123.1 |
| 12166 | Tank 12166 | 2-1-123.2 |

# III.  GENERALLY APPLICABLE REQUIREMENTS

The permit holder shall comply with all applicable requirements, including those specified in the BAAQMD and SIP Rules and Regulations and other federal requirements cited below.  These requirements apply in a general manner to the facility and/or to sources exempt from the requirement to obtain a District Permit to Operate.  The District has determined that these requirements will not be violated under normal, routine operations, and that no additional periodic monitoring or reporting to demonstrate compliance is warranted.  In cases where a requirement, in addition to being generally applicable, is also specifically applicable to one or more sources, the requirement and the source are also included in Section IV, Source-Specific Applicable Requirements, of this permit.

The dates in parenthesis in the Title column identify the versions of the regulations being cited and are, as applicable:

- BAAQMD regulation(s):  The date(s) of adoption or most recent amendment of the regulation by the District Board of Directors

- Any federal requirement, including a version of a District regulation that has been approved into the SIP:  The most recent date of EPA approval of any portion of the rule, encompassing all actions on the rule through that date

The full language of SIP requirements is on EPA Region 9’s website.  The address is included at the end of this permit.

NOTE:

There are differences between the current BAAQMD rules and the versions of the rules in the SIP. All sources must comply with both versions of the rule until US EPA has reviewed and approved the District’s revision of the regulation.

| Table III Generally Applicable Requirements |  |  |
| --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/2/01) | N |
| SIP Regulation 1 | General Provisions and Definitions (6/28/99) | Y |
| BAAQMD Regulation 2, Rule 1 | General Requirements (8/1/01) | N |
| BAAQMD 2-1-429 | Federal Emissions Statement (6/7/95) | Y |
| SIP Regulation 2, Rule 1 | General Requirements (6/7/95) | Y |
| BAAQMD Regulation 4 | Air Pollution Episode Plan (3/20/91) | N |
| SIP Regulation 4 | Air Pollution Episode Plan (8/06/90) | Y |
| BAAQMD Regulation 5 | Open Burning (3/6/02) | N |
| SIP Regulation 5 | Open Burning (9/4/98) | Y |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) | Y |
| BAAQMD Regulation 7 | Odorous Substances (3/17/82) | N |
| BAAQMD Regulation 8, Rule 1 | Organic Compounds - General Provisions (6/15/94) | Y |
| BAAQMD Regulation 8, Rule 2 | Organic Compounds – Miscellaneous Operations (06/15/94) | Y |
| BAAQMD Regulation 8, Rule 3 | Organic Compounds - Architectural Coatings (11/21/01) | N |
| SIP Regulation 8, Rule 3 | Organic Compounds - Architectural Coatings (2/18/98) | Y |
| BAAQMD Regulation 8, Rule 4 | Organic compounds - General Solvent and Surface Coating Operations (10/16/02) | N |
| SIP Regulation 8, Rule 4 | Organic compounds - General Solvent and Surface Coating Operations (12/23/97) | Y |
| BAAQMD Regulation 8, Rule 15 | Organic Compounds – Emulsified and Liquid Asphalts (6/1/94) | Y |
| BAAQMD Regulation 8, Rule 16, Section 302.1 | Organic Compounds – Solvent Cleaning Operations, Conveyorized Solvent Cleaner Requirements, General Requirements (10/16/2002) | Y |
| BAAQMD Regulation 8, Rule 16, Section 302.2 | Organic Compounds – Solvent Cleaning Operations, Conveyorized Solvent Cleaner Requirements, General Equipment Requirements (10/16/2002) | Y |
| BAAQMD Regulation 8, Rule 16, Section 302.3 | Organic Compounds – Solvent Cleaning Operations, Conveyorized Solvent Cleaner Requirements, Requirements when using a volatile solvent (10/16/2002) | Y |
| BAAQMD Regulation 8, Rule 19, Section 307 | Organic Compounds – Surface Coating of Miscellaneous Metal Parts and Products, Prohibition of Specification (10/16/2002) | Y |
| BAAQMD Regulation 8, Rule 19, Section 320 | Organic Compounds – Surface Coating of Miscellaneous Metal Parts and Products, Solvent Evaporative Loss Minimization (10/16/2002) | Y |
| BAAQMD Regulation 8, Rule 40 | Organic Compounds – Aeration of Contaminated Soil and Removal of Underground Storage Tanks (04/19/2001) | Y |
| BAAQMD Regulation 8, Rule 47 | Organic Compounds - Air Stripping and Soil Vapor Extraction Operations (6/15/94) | Y |
| BAAQMD Regulation 8, Rule 49 | Organic Compounds - Aerosol Paint Products (12/20/95) | N |
| SIP Regulation 8, Rule 49 | Organic Compounds - Aerosol Paint Products (3/22/95) | Y |
| BAAQMD Regulation 8, Rule 51 | Organic Compounds - Adhesive and Sealant Products (7/17/02) | N |
| SIP Regulation 8, Rule 51 | Organic Compounds - Adhesive and Sealant Products (2/26/02) | Y |
| BAAQMD Regulation 9, Rule 1-110 | Conditional Exemption for facilities performing SO2 Area Monitoring (GLM). (3/15/95) | Y |
| BAAQMD Regulation 9, Rule 2 | H2S Limitation and Area Monitoring (10/6/99) | N |
| BAAQMD Regulation 11, Rule 2 | Hazardous Pollutants - Asbestos Demolition, Renovation and Manufacturing (10/7/98) | Y |
| BAAQMD Regulation 11, Rule 10 | Hazardous Pollutants – Hexavalent Chromium Emissions from Cooling Towers (11/15/99) | Y |
| BAAQMD Regulation 12, Rule 4 | Miscellaneous Standards of Performance - Sandblasting (7/11/90) | N |
| SIP Regulation 12, Rule 4 | Miscellaneous Standards of Performance - Sandblasting (9/2/81) | N |
| EPA Regulation 40 CFR 82 | Protection of Stratospheric Ozone (2/21/95) | Y |
| Subpart F, 40 CFR 82.156 | Leak Repair | Y |
| Subpart F, 40 CFR 82.161 | Certification of Technicians | Y |
| Subpart F, 40 CFR 82.166 | Records of Refrigerant | Y |
| Subpart M, 40 CFR 61 | Asbestos Demolition and Renovation | Y |
| California Health and Safety Code Section 44300 et seq. | Air Toxics “Hot Spots” Information and Assessment Act of 1987 | N |
| 40 CFR Part 61, Subpart M | National Emission Standards for Hazardous Air Pollutants – National Emission Standard for Asbestos (6/19/95) | Y |

# IV. SOURCE-SPECIFIC APPLICABLE REQUIREMENTS

The permit holder shall comply with all applicable requirements, including those specified in the BAAQMD and SIP Rules and Regulations and other federal requirements cited below.  The requirements cited in the following tables apply in a specific manner to the indicated source(s).

The dates in parenthesis in the Title column identify the versions of the regulations being cited and are, as applicable:

1.	BAAQMD regulation(s):  The date(s) of adoption or most recent amendment of the regulation by the District Board

2.	Any federal requirement, including a version of a District regulation that has been approved into the SIP:  The most recent date of EPA approval of any portion of the rule, encompassing all actions on the rule through that date

The full text of each permit condition cited is included in Section VI, Permit Conditions, of this permit.  The full language of SIP requirements is on EPA Region 9’s website.  The address is included at the end of this permit.  All other text may be found in the regulations themselves.

| Table IV – A Source-specific Applicable Requirements S3 - Tank 3, S4 - Tank 4, S34 - Tank 34, S224 – Tank 224, S257 - Tank 257, S396 – Tank 396, S397 -Tank 397, S548 - Tank 548, S549 - Tank 549, S856 - Tank 856, S967 - Tank 967, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| - BAAQMD · Regulation 8, Rule 5 | - Organic Compounds, Storage of Organic Liquids (06/05/03) |  |  |  |
| 8-5-117 | Exemption, Low Vapor Pressure |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR RECORDKEEPING ONLY – GROUP 2 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(i)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for non-exempt tanks? | 63.654(i)(1) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV – B Source-specific Applicable Requirements S3 - Tank 3, S4 - Tank 4, S13 - Tank 13, S14 - Tank 14, S20 - Tank 20, S21 - Tank 21 Asphalt Storage, S22 - Tank 22 Asphalt Storage, S23 - Tank 23 Asphalt Storage, S24 - Tank 24 Asphalt Storage, S26 - Tank 26 Asphalt Storage, S129 - Tank 129, S257 - Tank 257, S483 - Tank 483, S484 - Tank 484, S497 - Tank 497 Asphalt Storage, S530 - Tank 530, S532 - Tank 532, S541 - Tank 541, S545 - Tank 545, S548 - Tank 548, S552 - Tank 552 Asphalt Storage, S553 - Tank 553 Asphalt Storage, S554 - Tank 554 Asphalt Storage, S555 - Tank 555 Asphalt Storage, S556 - Tank 556 Asphalt Storage, S557 - Tank 557 Asphalt Storage, S558 - Tank 558 Asphalt Storage, S559 - Tank 559 Asphalt Storage, S560 - Tank 560 Asphalt Storage, S561 - Tank 561 Asphalt Storage, S567 - Tank 567 Asphalt Storage, S568 - Tank 568, S572 - Tank 572 Asphalt Storage, S573 - Tank 573 Asphalt Storage, S598 - Tank 598 Asphalt Storage, S610 - Tank 610, S611 - Tank 611, S612 - Tank 612, S613 - Tank 613, S815 - Tank 815 Asphalt Storage, S860 - Tank 860, S861 - Tank 861, S967 - Tank 967, S985 - Tank 985 Asphalt Storage, S1004 - Tank 1004, S1006 - Tank 1006, S1031 - Tank 1031, S1043 - Tank 1043 Asphalt Storage, S1044 - Tank 1044 Asphalt Storage, S1045 - Tank 1045 Asphalt Storage, S1046 - Tank 1046, S1051 - Tank 1051, S1076 - Tank 1076, S1129 - Tank 1129, S1130 - Tank 1130, S1131 - Tank 1131, S1134 - Tank 1134, S1140 - Tank 1140, , S1146 - Tank 1146, S1147 - Tank 1147, S1159 - Tank 1159, S1160 - Tank 1160 Asphalt Storage, S1161 - Tank 1161, S1235 - Tank 739 Chem Storage, S1236 - Tank 740 Chem Storage, S1133 - Tank 1133, S1409 - LUBS Sulfonation Plant (SULF), S1411 - LUBS Atmospheric Distillation LDU, S1415 - LUBS Furfural Plant (SEP3), S1416 - LUBS Lube Hydrotreater 1 (LHT1),, S1417 - OPCEN Distillate Saturation Unit (DSU), S1420 - DH Crude Unit (CU), S1423 - DH Gas Oil Straightrun Hydrotreater (GOHT), S1424 - DH Naphtha Straightrun Hydrotreater (NHT), S1425 - DH Catalytic Reformer Unit (CRU), S1426 - CP Catalytic Cracking Unit (CCU), S1428 - CP Catalytic Feed Hydrotreater (CFH), S1429 - CP Catalytic Gasoline Hydrotreater (CGH), S1430 - CP Alkylation Plant (ALKY), S1431 - CP Sulfur Plant 1 (SRU1), S1432 - CP Sulfur Plant 2 (SRU2), S1445 - DH Hydrogen Plant 1 (HP1), S1449 - DH Hydrocracking Unit (HCU), S1507 - UTIL CO Boiler 1, S1509 - UTIL CO Boiler 2, S1512 - UTIL CO Boiler 3, S1751 - Tank 1330, S1752 - Tank 1331, S1753 - Tank 1332 Gasoline, S1754 - Tank 1333 Gasoline, S1755 - Tank 1334 Gasoline, S1756 - Tank 1335 Gasoline, S1757 - Tank 1336, S1758 - Tank 1337, S1759 - OPCEN Flexicoker (FXU), S1764 - OPCEN Dimersol Plant (DIMER), S1765 - OPCEN Sulfur Plant 3 (SRU3), S1774 - OPCEN Hydrogen Plant 2 (HP2), S1900 - MAINT_Machine Shop Parts Cleaner, S1903 - MAINT Paint Shop Solvent Tub , S3000 - Portable Vacuum Distillation Unit (CCR Technologies Inc.), S4001 - DC Delayed Coking Unit (DCU), S4020 - DC Distillate Hydrotreater (DHT), S4080 - DC Isomerization Unit (ISOM), S4140 - DC Heavy Cracked Gasoline Hydrotreater (HGHT), S4160 - DC Hydrogen Plant –3 (HP3), S4170 - LUBS Lube Hydrotreater 2 (LHT2), S4180 - OPCEN Sulfur Plant 4 (SRU4), SS4334 - Tank13276 Alkylate |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Condition # 18618 |  |  |  |
| Part 1 | Throughput limit (basis: Regulation 2-1-234.3) | N |  |
| Part 2 | Recordkeeping (basis: Regulation 2-1-234.3) | N |  |

| Table IV – Ca Source-specific Applicable Requirements S4307 Tank 14687 MDEA Make-up |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| - BAAQMD · Regulation 8, Rule 5 | - Organic Compounds, Storage of Organic Liquids (06/05/03) |  |  |  |
| 8-5-117 | Exemption, Low Vapor Pressure |  | Y |  |
| NSPS Subpart Kb; BAAQMD 10-17 | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR RECORDKEEPING ONLY |  |  |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for non-exempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| BAAQMD Condition # 12271 |  |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] |  | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part O | Offsets requirements [basis: Offsets] |  | Y |  |
| Part 51 | Degassing abatement requirements [basis: BACT] |  | Y |  |

| Table IV – Cb Source-specific Applicable Requirements S4309 – Tank 14517 DEA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| - BAAQMD · Regulation 8, Rule 5 | - Organic Compounds, Storage of Organic Liquids (06/05/03) |  |  |  |
| 8-5-117 | Exemption, Low Vapor Pressure |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR TANKS ALSO SUBJECT TO NSPS Kb REQUIREMENTS FOR RECORDKEEPING ONLY |  |  |  |
| 63.640(n) | Which rule governs for storage vessels subject to both Refinery MACT and NSPS subpart Kb? | 63.640(n)(1) NSPS subpart Kb | Y |  |
| NSPS Subpart Kb; BAAQMD 10-17 | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR RECORDKEEPING ONLY |  |  |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for non-exempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| BAAQMD Condition # 12271 |  |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] |  | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] |  | Y |  |
| Part O | Offsets requirements [basis: Offsets] |  | Y |  |
| Part 51 | Degassing abatement requirements [basis: BACT] |  | Y |  |

| Table IV – D Source-specific Applicable Requirements S13 - Tank 13 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AEc for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part 45 | Equipment specification [basis: BACT] | Y |  |

| Table IV – Ea Source-specific Applicable Requirements S14 - Tank 14, S19 - Tank 19, S20 – Tank 20, S483 - Tank 483, S484 - Tank 484, S530 - Tank 530, S-534 – Tank 534, S610 - Tank 610, S611 - Tank 611, S612 - Tank 612, S613 – Tank 613, S1070 - Tank 1070, S1133 - Tank 1133, S1134 - Tank 1134, S1139 – Tank 1139, S1140 - Tank 1140, S1141 – Tank 1141 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-501.1 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |

| Table IV – Eb Source-specific Applicable Requirements S-534 – Tank 534, S1141 – Tank 1141 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ea for additional requirements. |  |  |
| BAAQMD Condition # 20398 |  |  |  |
| Part 1 | Throughput limit (basis: Cumulative Increase) | Y |  |
| Part 2 | RVP limit (basis: Cumulative Increase) | Y |  |
| Part 3 | Abatement requirement (basis: BACT, Cumulative Increase) | Y |  |
| Part 4 | Recordkeeping (basis: Cumulative Increase) | Y |  |

| Table IV – Ec Source-specific Applicable Requirements S1751 – Tank 1330, S1752 – Tank 1331, S1753 - Tank 1332 Gasoline, S1754 – Tank 1333 Gasoline, S1757 - Tank 1336, S1758 - Tank 1337 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes |  | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve |  | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-501.1 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| NSPS Subpart Ka; BAAQMD 10-16 | Storage Vessels for Petrolum Liquid REQUIREMENTS FOR FIXED ROOF TANK-CONTROL DEVICE |  |  |  |
| 60.112a(a) | Control device Performance requirements: | 60.112a(a)(3) at least 95% efficient | Y |  |

| Table IV – F Source-specific Applicable Requirements S14 - Tank 14, S20 – Tank 20, S483 - Tank 483, S484 - Tank 484 S530 – Tank 530, S532 – Tank 532, S1139 - Tank 1139, S1140 - Tank 1140, S1141 – Tank 1141, S1751 - Tank 1330, S1752 - Tank 1331, S1753 - Tank 1332 Gasoline, S1754 - Tank 1333 Gasoline, S1757 -Tank 1336, S1758 - Tank 1337 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ea, Eb and M for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part D.1.a | Liquid with a vapor pressure of 1.5 psig or greater shall not be stored in more than 12 tanks at any one time.[basis: Cumulative Increase] | Y |  |
| Part D.1.b | Abatement Requirement. [basis: Cumulative Increase] | Y |  |
| Part G | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – G Source-specific Applicable Requirements S19 - Tank 19, S1139 - Tank 1139 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ea for additional requirements. |  |  |
| BAAQMD Condition # 18646 |  |  |  |
| Part 1 | Throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2a | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2bi | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2bii | Material storage limit [basis: Cumulative Increase; Toxics] | N |  |
| Part 3 | Abatement requirement [basis: BACT/TBACT] | Y |  |
| Part 4 | Storage conditions [basis: Cumulative Increase] | Y |  |

| Table IV - Ha Source-specific Applicable Requirements S21 –Tank 21 Asphalt Storage, S22 –Tank 22 Asphalt Storage, S23 –Tank 23 Asphalt Storage, S24 –Tank 24 Asphalt Storage, S26 –Tank 26 Asphalt Storage, S567 - Tank 567 Asphalt Storage, S598 - Tank 598 Asphalt Storage S497 –Tank 497 Asphalt Storage, S552 –Tank 552 Asphalt Storage, S553 –Tank 553 Asphalt Storage, S554 –Tank 554 Asphalt Storage, S555 –Tank 555 Asphalt Storage, S556 –Tank 556 Asphalt Storage, S557 –Tank 557 Asphalt Storage, S558 –Tank 558 Asphalt Storage, S559 –Tank 559 Asphalt Storage, S560 –Tank 560 Asphalt Storage, S561 –Tank 561 Asphalt Storage, S571 –Tank 571 Asphalt Storage, S572 –Tank 572 Asphalt Storage, S573 –Tank 573 Asphalt Storage, S815 –Tank 815 Asphalt Storage, S867–Tank 867 Asphalt Storage, S868–Tank 868 Asphalt Storage, S876–Tank 876 Asphalt Storage, S961–Tank 961 Asphalt Storage, S985–Tank 985 Asphalt Storage, S1017–Tank 1017 Asphalt Storage, S1018–Tank 1018 Asphalt Storage, S1041–1041 Asphalt Storage Tank, S1043–Tank 1043 Asphalt Storage, S1044–Tank 1044 Asphalt Storage, S1045–Tank 1045 Asphalt Storage, S1048 –Tank 1048 Asphalt Storage, S1075 - Tank 1075 Asphalt Storage, S1160 –Tank 1160 Asphalt Storage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |  |
| 6-301 | Ringelmann No. 1 Limitation |  | Y |  |
| 6-305 | Visible Particles |  | Y |  |
| 6-311 | General Operations (process weight rate limitation) |  | Y |  |
| 6-401 | Appearance of Emissions |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR RECORDKEEPING ONLY – GROUP 2 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(i)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for non-exempt tanks? | 63.654(i)(1) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV - Hb Source-specific Applicable Requirements S1408 – LUBS Asphalt Blending and Shipping, S1523 – LUBS Loading Rack Asphalt Inside T/T, S1524 – LUBS Loading Rack Asphalt Outside T/T and T/C, S1525 – LUBS Loading Rack Asphalt Paving T/T S1539 – LUBS LR-25 Loading Rack Acid Sludge and Sulfonation T/T |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |

| Table IV – I Source-specific Applicable Requirements S63 – Tank 63 S355 – Tank 355 S432 – Tank 432 S568 – Storage Tank 568 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes |  | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve |  | Y |  |
| 8-5-304 | Requirements for External Floating Roofs |  | Y |  |
| 8-5-305 | Requirements for Internal Floating Roofs |  | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems |  | Y |  |
| 8-5-307 | Requirements for Pressure Tanks and Blanketed Tanks |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof |  | Y |  |
| 8-5-402 | Inspection Requirements for Internal Floating Roof |  | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR RECORDKEEPING ONLY – GROUP 2 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records for 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(i)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 63.654(i)(1) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV – Ja Source-specific Applicable Requirements S129 - Tank 129, S540 – Tank 540, S541 - Tank 541, S544 - Tank 544, S545 - Tank 545, S1031 - Tank 1031, S1046 - Tank 1046, S1051 – Tank 1051, S1072 - Tank 1072, S1129 – Tank 1129, S1159 - Tank 1159, S1161 - Tank 1161, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-304 | Requirements for External Floating Roofs |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS – GROUP 1 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.646(a) | EFR Rim Seals: vapor-mounted primary seal: liquid-mounted primary seal: mechanical-shoe primary seal: | 63.646(a) 63.119(c)(1)(I) - (1)(iii) Not Allowed OK with rim-mounted secondary OK with rim-mounted secondary | Y |  |
|  | Must vapor-mounted rim seals be continuous on EFRs? | 63.646(a) 63.119(c)(1)(iii) YES | Y |  |
|  | Are EFR rim seals allowed to be pulled back or temporarily removed during inspection? | 63.646(a) 63.119(c)(1)(iii) 63.120(b)(4) YES | Y |  |
|  | EFRT operating requirements: When landing the floating roof on its support legs, is the tank to be emptied & either refilled or degassed as soon as possible? | 63.646(a) 63.119(c)(3) & (c)(4) YES | Y |  |
|  | Temporary exemption from operating requirements while the external floating roof is landed on its support legs? * | 63.646(a) 63.119(c)(3) EXEMPT | Y |  |
|  | EFR Internal Inspections: up-close visual inspection of the floating roof, seals, & fittings: | 63.646(a) & 63.120(b) each time the tank is emptied & degassed | Y |  |
|  | EXTENSIONS OF TIME: If EFRT is unsafe to inspect & cannot be emptied within 45 days? | 63.646(a) & 63.120(b) up to 2 extensions of 30 days each, if needed | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR seal gap measurements: | 63.646(a) 63.120(b)(1) & (9) Required- Notifications&reports per Ongoing Reports | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Primary Seal: | 63.646(a) 63.120(b)(1)(i) every 5 years | Y |  |
|  | Seal Gap Measurements: For existing EFRTs in compliance by the compliance date: | 63.646(a) 63.120(b)(1)(i) & (iii) measure gaps of both seals prior to the compliance date | Y |  |
|  | Seal Gap Measurements: For new EFRTs: | 63.646(a) 63.120(b)(1)(i) & (iii) measure gaps of both seals prior to initial fill | Y |  |
|  | Seal Gap Measurements: For affected EFRTs with a mechanical-shoe or liquid-mounted primary-only rim seal, prior to installing a secondary seal; PRIOR TO COMPLIANCE: UPON COMPLIANCE: | 63.646(a) 63.120(b)(1)(ii) annual primary seal gap measurements * 63.646(a) 63.120(b)(1)(ii) measure gaps of both seals within 90 days | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Secondary Seal: | 63.646(a) 63.120(b)(1)(iii) annually | Y |  |
|  | Seal Gap Measurements: For EFRTs returned to affected service after 1 year or more of exempt service: | 63.646(a) 63.120(b)(1)(iv) measure gaps of both seals within 90 days | Y |  |
|  | MEASUREMENT COND'S: Are EFR seal gap measurements to be made with the roof floating? | 63.646(a) 63.120(b)(2)(i) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Presence of a gap determined by inserting a 1/8 in. probe? | 63.646(a) 63.120(b)(2)(ii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Use probes of various widths to determine the gap area? | 63.646(a) 63.120(b)(2)(iii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Sum the gap areas & divide by the diameter of the tank? | 63.646(a) 63.120(b)(3) & (4) YES | Y |  |
|  | EFR Primary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 63.646(a) 63.120(b)(3) 10 in2 per foot of vessel diameter. 1.5 in. | Y |  |
|  | EFR Secondary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 63.646(a) 63.120(b)(4) 1 in2 per foot of vessel diameter. 0.5 in. | Y |  |
|  | Is the metallic shoe of an EFR mechanical-shoe seal required to have its bottom in the liquid and extend at least 24 in. above the liquid? | 63.646(a) 63.120(b)(5)(i) YES | Y |  |
|  | Shall there be no holes, tears, or openings in the EFR seals? | 63.646(a) 63.120(b)(5)(ii) & (6)(ii) YES | Y |  |
|  | UNSAFE CONDITIONS: Delay of EFR seal gap measurements allowed for unsafe conditions? If unable to make safe to measure, must the EFRT be emptied? | 63.646(a) 63.120(b)(7)(i) up to 30 additional days 63.120(b)(7)(ii) YES, within 45 days of determining unsafe | Y |  |
|  | EFRT REPAIRS: Time allowed for repair of defects found during in-service inspections of EFRs: If unable to repair, empty the EFRT & remove from service? | 63.646(a) 63.120(b)(8) make repairs within 45 days 63.120(b)(8) YES, within 45 days | Y |  |
|  | EXTENSIONS OF TIME: If EFRT defects cannot be repaired & the tank cannot be emptied within 45 days? | 63.646(a) 63.120(b)(8) up to 2 extensions of 30 days each, if needed | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR internal inspections: | 63.646(a) 63.120(b)(10) internal inspection not required for initial compl | Y |  |
|  | EFRT REPAIRS: Repair of defects if the tank is empty? | 63.646(a) 63.120(b)(10)(i) prior to refilling | Y |  |
| 63.646(f) | Deck openings (wells) other than for vents, drains, or legs to have covers that are kept closed except for access? | 63.646(f)(1) REQUIRED | Y |  |
|  | EFR rim space vents to remain closed except when the pressure setting is exceeded? | 63.646(f)(2) REQUIRED | Y |  |
|  | EFR automatic bleeder vent (vacuum breaker) to be closed except when the deck is landed? | 63.646(f)(3) REQUIRED | Y |  |
| 63.646(l) | Notification of Inspections: Is the State or local authority allowed to waive the notification requirements? | 63.646(l) 63.654(h)(2)(I)(C)&(ii) YES | Y |  |
| 63.654(g) | Report of periodic inspections, etc. AFTER documenting initial compliance? | 63.654(g) begin Sept 13, 1999 then semiannual | Y |  |
|  | Periodic Reports: Report of EFR inspection failures to include: | 63.654(g)(2) - (4) date of inspection, identification of tank, description of failure, & date of repair or emptying | Y |  |
|  | Periodic Reports: EFR report to include a prior request for 30-day extension, with documentation of need? | 63.654(g)(2) - (4) prior request is not required | Y |  |
|  | Periodic Reports: Additional information to be included if an extension is utilized for an EFR: | 63.654(g)(2)(i) 63.654(g)(3)(ii) document the reason for the extension | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections if there was no out-of-compliance? | 63.654(g)(3)(i) Not required | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections when there is out-of-compliance? | 63.654(g)(3)(i) Required within 60 days after each semiannual period | Y |  |
| 63.654(h) | Notification of Inspections: Is 30-day notice required for internal inspections of EFRTs (i.e., prior to filling or refilling); but a 7-day verbal notice acceptable if the event is unplanned? | 63.654(h)(2)(i) 63.646(a) 63.120(b)(10) REQUIRED | Y |  |
|  | Notification of Inspections: Is 30-day notice required prior to EFR seal gap measurements? | 63.654(h)(2)(ii) 63.646(a) 63.120(b)(9) REQUIRED | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(I)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 63.654(I)(1) 63.646(a)&63.119(a)(3) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Recordkeeping for inspections: Keep inspection reports as specified. | 63.654(I)(1) 63.123(c) - (e) all inspections | Y |  |
|  | Records of EFR inspection reports: | 63.654(I)(1) 63.123(d) all inspections | Y |  |
|  | Recordkeeping for delayed repairs: When utilizing a delay of repair provision, keep documentation of the reason for the delay. | 63.654(I)(1) 63.123 (g) required | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV – Jb Source-specific Applicable Requirements S992 - Tank 992, S1076 - Tank 1076, S1130 - Tank 1130, S1131 - Tank 1131 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-304 | Requirements for External Floating Roofs |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR RECORDKEEPING ONLY – GROUP 2 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(i)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for non-exempt tanks? | 63.654(i)(1) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV – Jc Source-specific Applicable Requirements S1755 – Tank 1334 Gasoline, S1756 – Tank 1335 Gasoline S1191 - Tank -1256 Crude Oil Storage, S1192 - Tank –1257 Crude Oil Storage, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-304 | Requirements for External Floating Roofs |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| NSPS Subpart Ka; BAAQMD 10-16 | Storage Vessels for Petrolum Liquid REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS |  |  |  |
| 60.112a | Standards for Volatile Organic Compounds |  | Y |  |
| 60.112a(a)(1) | Standards for external floating-roof tanks |  | Y |  |
| 60.113a | Testing and Procedures |  | Y |  |
| 60.113a(a)(1) | Testing requirements for external floating-roof tanks |  | Y |  |
| 60.115a | Monitoring of operation |  | Y |  |
| 60.115a(a) | Recordkeeping requirements |  | Y |  |
| 60.115a(b)-(c) | Determining vapor pressure |  | Y |  |
| NSPS Subpart A | New Source Performance Standards GENERAL PROVISIONS |  |  |  |
| 60.7(f) | General recordkeeping requirements |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS – GROUP 1 TANKS |  |  |  |
| 63.642(e) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 63.642(e) & 63.654(i)(4) keep all other records 5 years, retrievable within 24 hr | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 63.642(e) & 63.654(i)(4) required | Y |  |
| 63.646(a) | EFR Rim Seals: vapor-mounted primary seal: liquid-mounted primary seal: mechanical-shoe primary seal: | 63.646(a) 63.119(c)(1)(i) - (1)(iii) Not Allowed OK with rim-mounted secondary OK with rim-mounted secondary | Y |  |
|  | Must vapor-mounted rim seals be continuous on EFRs? | 63.646(a) 63.119(c)(1)(iii) YES | Y |  |
|  | Are EFR rim seals allowed to be pulled back or temporarily removed during inspection? | 63.646(a) 63.119(c)(1)(iii) 63.120(b)(4) YES | Y |  |
|  | EFRT operating requirements: When landing the floating roof on its support legs, is the tank to be emptied & either refilled or degassed as soon as possible? | 63.646(a) 63.119(c)(3) & (c)(4) YES | Y |  |
|  | Temporary exemption from operating requirements while the external floating roof is landed on its support legs? * | 63.646(a) 63.119(c)(3) EXEMPT | Y |  |
|  | EFR Internal Inspections: up-close visual inspection of the floating roof, seals, & fittings: | 63.646(a) & 63.120(b) each time the tank is emptied & degassed | Y |  |
|  | EXTENSIONS OF TIME: If EFRT is unsafe to inspect & cannot be emptied within 45 days? | 63.646(a) & 63.120(b) up to 2 extensions of 30 days each, if needed | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR seal gap measurements: | 63.646(a) 63.120(b)(1) & (9) Required- Notifications&reports per Ongoing Reports | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Primary Seal: | 63.646(a) 63.120(b)(1)(i) every 5 years | Y |  |
|  | Seal Gap Measurements: For existing EFRTs in compliance by the compliance date: | 63.646(a) 63.120(b)(1)(i) & (iii) measure gaps of both seals prior to the compliance date | Y |  |
|  | Seal Gap Measurements: For new EFRTs: | 63.646(a) 63.120(b)(1)(i) & (iii) measure gaps of both seals prior to initial fill | Y |  |
|  | Seal Gap Measurements: For affected EFRTs with a mechanical-shoe or liquid-mounted primary-only rim seal, prior to installing a secondary seal; PRIOR TO COMPLIANCE: UPON COMPLIANCE: | 63.646(a) 63.120(b)(1)(ii) annual primary seal gap measurements * 63.646(a) 63.120(b)(1)(ii) measure gaps of both seals within 90 days | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Secondary Seal: | 63.646(a) 63.120(b)(1)(iii) annually | Y |  |
|  | Seal Gap Measurements: For EFRTs returned to affected service after 1 year or more of exempt service: | 63.646(a) 63.120(b)(1)(iv) measure gaps of both seals within 90 days | Y |  |
|  | MEASUREMENT COND'S: Are EFR seal gap measurements to be made with the roof floating? | 63.646(a) 63.120(b)(2)(i) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Presence of a gap determined by inserting a 1/8 in. probe? | 63.646(a) 63.120(b)(2)(ii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Use probes of various widths to determine the gap area? | 63.646(a) 63.120(b)(2)(iii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Sum the gap areas & divide by the diameter of the tank? | 63.646(a) 63.120(b)(3) & (4) YES | Y |  |
|  | EFR Primary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 63.646(a) 63.120(b)(3) 10 in2 per foot of vessel diameter. 1.5 in. | Y |  |
|  | EFR Secondary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 63.646(a) 63.120(b)(4) 1 in 2 per foot of vessel diameter. 0.5 in. | Y |  |
|  | Is the metallic shoe of an EFR mechanical-shoe seal required to have its bottom in the liquid and extend at least 24 in. above the liquid? | 63.646(a) 63.120(b)(5)(i) YES | Y |  |
|  | Shall there be no holes, tears, or openings in the EFR seals? | 63.646(a) 63.120(b)(5)(ii) & (6)(ii) YES | Y |  |
|  | UNSAFE CONDITIONS: Delay of EFR seal gap measurements allowed for unsafe conditions? If unable to make safe to measure, must the EFRT be emptied? | 63.646(a) 63.120(b)(7)(i) up to 30 additional days 63.120(b)(7)(ii) YES, within 45 days of determining unsafe | Y |  |
|  | EFRT REPAIRS: Time allowed for repair of defects found during in-service inspections of EFRs: If unable to repair, empty the EFRT & remove from service? | 63.646(a) 63.120(b)(8) make repairs within 45 days 63.120(b)(8) YES, within 45 days | Y |  |
|  | EXTENSIONS OF TIME: If EFRT defects cannot be repaired & the tank cannot be emptied within 45 days? | 63.646(a) 63.120(b)(8) up to 2 extensions of 30 days each, if needed | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR internal inspections: | 63.646(a) 63.120(b)(10) internal inspection not required for initial compl | Y |  |
|  | EFRT REPAIRS: Repair of defects if the tank is empty? | 63.646(a) 63.120(b)(10)(i) prior to refilling | Y |  |
| 63.646(f) | Deck openings (wells) other than for vents, drains, or legs to have covers that are kept closed except for access? | 63.646(f)(1) REQUIRED | Y |  |
|  | EFR rim space vents to remain closed except when the pressure setting is exceeded? | 63.646(f)(2) REQUIRED | Y |  |
|  | EFR automatic bleeder vent (vacuum breaker) to be closed except when the deck is landed? | 63.646(f)(3) REQUIRED | Y |  |
| 63.646(l) | Notification of Inspections: Is the State or local authority allowed to waive the notification requirements? | 63.646(l) 63.654(h)(2)(i)(C)&(ii) YES | Y |  |
| 63.654(g) | Report of periodic inspections, etc. AFTER documenting initial compliance? | 63.654(g) begin Sept 13, 1999 then semiannual | Y |  |
|  | Periodic Reports: Report of EFR inspection failures to include: | 63.654(g)(2) - (4) date of inspection, identification of tank, description of failure, & date of repair or emptying | Y |  |
|  | Periodic Reports: EFR report to include a prior request for 30-day extension, with documentation of need? | 63.654(g)(2) - (4) prior request is not required | Y |  |
|  | Periodic Reports: Additional information to be included if an extension is utilized for an EFR: | 63.654(g)(2)(i) 63.654(g)(3)(ii) document the reason for the extension | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections if there was no out-of-compliance? | 63.654(g)(3)(i) Not required | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections when there is out-of-compliance? | 63.654(g)(3)(i) Required within 60 days after each semiannual period | Y |  |
| 63.654(h) | Notification of Inspections: Is 30-day notice required for internal inspections of EFRTs (i.e., prior to filling or refilling); but a 7-day verbal notice acceptable if the event is unplanned? | 63.654(h)(2)(i) 63.646(a) 63.120(b)(10) REQUIRED | Y |  |
|  | Notification of Inspections: Is 30-day notice required prior to EFR seal gap measurements? | 63.654(h)(2)(ii) 63.646(a) 63.120(b)(9) REQUIRED | Y |  |
| 63.654(i) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 63.654(i)(1) 63.123(a) Keep record readily accessible for the service life of the tank | Y |  |
|  | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 63.654(i)(1) 63.646(a)&63.119(a)(3) 63.123(a) Required Keep record readily accessible for service life of the tank * | Y |  |
|  | Recordkeeping for inspections: Keep inspection reports as specified. | 63.654(i)(1) 63.123(c) - (e) all inspections | Y |  |
|  | Records of EFR inspection reports: | 63.654(i)(1) 63.123(d) all inspections | Y |  |
|  | Recordkeeping for delayed repairs: When utilizing a delay of repair provision, keep documentation of the reason for the delay. | 63.654(i)(1) 63.123 (g) required | Y |  |
|  | Applicability records: Additional recordkeeping requirements for certain tanks. | 63.654(i)(1)(iv) determination of HAP content Keep record readily accessible for service life of the tank | Y |  |

| Table IV – Kb Source-specific Applicable Requirements S1076 - Tank 1076 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – J and Ka for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part D.2 | Liquid vapor pressure limit of less than 1.5 psia (basis: Cumulative Increase) | Y |  |
| Part G | Recordkeeping (basis: Cumulative Increase) | Y |  |

| Table IV – M Source-specific Applicable Requirements S532 - Tank 532 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-501.1 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.640 | Emission points at petroleum refinery | Y |  |
| 63.640(d)(5) | No testing, monitoring, recordkeeping,or reporting is required for refinery fuel gas systems or emission points routed to refinery fuel gas systems | Y |  |
| 63.647 | Wastewater provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with section 61.340 to 61.355 of 40 CFR 61, subpart FF for each stream that meets the definition of 63.641 | Y |  |
| 63.654 | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) | Owners/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements of 61.356 and 61.357 of 40 CFR 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |
| NESHAP 40 CFR 61 Subpart A | National Emission Standards for Hazardous Air Pollutants - General Provisions (3/16/95) |  |  |
| 61.01 | Lists of pollutants and applicability of part 61 | Y |  |
| 61.04 | Address | Y |  |
| 61.05 | Prohibited activities | Y |  |
| 61.07 | Application for approval of construction or modification | Y |  |
| 61.09 | Notification of startup | Y |  |
| 61.10 | Source reporting and waiver request | Y |  |
| 61.12 | Compliance with standards and maintenance requirements | Y |  |
| 61.13 | Emission tests and waiver of emission tests | Y |  |
| 61.14 | Monitoring requirements | Y |  |
| 61.19 | Circumvention | Y |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.343 | Standards: Tanks | Y |  |
| 61.343(a)(1) | Owner/operator shall install, operate, and maintain a fixed-roof and closed-vent system that routes all organic vapors vented from the tank to a control device. | Y |  |
| 61.343(a)(1) (i) | Fixed Roof leak requirements - < 500 ppmv above background, determined initially then annually by method in 61.355(h) of this subpart. | Y |  |
| 61.343(a)(1) (ii) | Closed-vent system and control device designed and operated in accordance with 61.349 of this subpart | Y |  |
| 61.343(b) | For a tank that meets all of the conditions of paragraph (b)(1), the owner/operator may elect to comply with paragraph (b)(2) as an alternative to the requirements of (a)(1). | Y |  |
| 61.343(b)(1) | The waste managed in the tank complying with (b)(2) shall meet all of the following conditions: wastestream managed in the tank must have a flow-weighted annual average water content less than or equal to 10% water, on a volume basis and the waste managed in the tank either: has a maximum organic vapor pressure less than 0.75 psi, or has a maximum organic vapor pressure less than 4.0 psi and is managed in a tank less than 40,000 gallons, or has a maximum organic vapor pressure less than 11.1 psi and is managed in a tank having a design capacity less than 20,000 gallons. | Y |  |
| 61.343(b)(2) | Owner/operator shall install, operate, and maintain a fixed roof as specified in paragraph (a)(1)(i). | Y |  |
| 61.343(b)(3) | may use devices that vent directly to atmosphere provided the devices are closed, sealed during normal operations except when the device needs to open to prevent damage or deformation of the tank or cover. | Y |  |
| 61.343(c) | Inspect all fixed-roof, seal, access doors, and all other openings visually initially and quarterly for cracks and gaps and that doors and openings are closed and gasketed properly. | Y |  |
| 61.343(d) | When a broken seal or gasket is identified, or when detectable emissions are measured, first efforts at repair shall be made as soon as practicable, but not later than 45 calendar days after identification. | Y |  |
| 61.349 | Standards: Closed vent systems and control devices | Y |  |
| 61.349(a) | Closed vent system and control device | Y |  |
| 61.349(a)(1) | Closed vent system requirements | Y |  |
| 61.350 | Delay of Repair | Y |  |
| 61.355 | Test methods, procedures and compliance provisions | Y |  |
| 61.355(h) | Test equipment for compliance with no detectable emissions as required in 61.343 through 61.347 and 61.349 | Y |  |
| 61.356 | Recordkeeping requirements | Y |  |
| 61.356(h) | Reporting requirements | Y |  |
| 61.357 | Reporting requirements | Y |  |
| 61.357(d) | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste | Y |  |
| 61.357(d)(8) | Reporting requirements | Y |  |

| Table IV – N Source-specific Applicable Requirements S540 - Tank 540 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ja for additional requirements. |  |  |
| BAAQMD Condition # 11951 |  |  |  |
| Part 1 | Annual throughput limit with recordkeeping [basis: Cumulative Increase] | Y |  |
| Part 3 | Equipment specification requirement [basis: BACT/TBACT] | Y |  |

| Table IV – O Source-specific Applicable Requirements S544 - Tank 544 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ja for additional requirements. |  |  |
| BAAQMD Condition # 11850 |  |  |  |
| Part 1 | Equipment specification requirement [basis: Cumulative Increase] | Y |  |

| Table IV – P Source-specific Applicable Requirements S545 - Tank 545 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ja for additional requirements. |  |  |
| BAAQMD Condition # 12174 |  |  |  |
| Part 1 | Equipment specification requirement [basis: Cumulative Increase] | Y |  |

| Table IV – Q Source-specific Applicable Requirements S549 - Tank 549 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – A & B for additional requirements. |  |  |
| BAAQMD Condition # 6111 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2 | Vapor pressure limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Benzene content limit [basis: Toxics] | N |  |
| Part 4 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – R Source-specific Applicable Requirements S858 - Tank 858, S952 - Tank 952, S1023 - Tank 1023, S1050 Tank 1050, S2445 - Tank 12445, S2446 - Tank 12446 S4322 - Tank 14571 Sour Water , |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-305 | Requirements for Internal Floating Roofs |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-402 | Inspection Requirements for Internal Floating Roof |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR INTERNAL FLOATING ROOF TANKS ALSO SUBJECT TO NSPS Kb |  |  |  |
| 63.640(n) | Which rule governs for storage vessels subject to both Refinery MACT and NSPS subpart Kb? | 63.640(n)(1) NSPS subpart Kb | Y |  |
|  | Does Refinery MACT provide for delay of NSPS Kb seal gap measurements due to unsafe conditions? | 63.640(n)(8)(ii) YES – up to 30 days, or empty the tank within 45 days | Y |  |
|  | Does Refinery MACT provide for extensions of time to perform NSPS Kb inspections of unsafe tanks? | 63.640(n)(8)(iii) YES – up to 2 extensions of 30 days each | Y |  |
|  | Does Refinery MACT provide for extensions of time to repair defects found during NSPS Kb inspections? | 63.640(n)(8)(iii) YES – up to 2 extensions of 30 days each | Y |  |
|  | Does Refinery MACT provide for waiving the NSPS Kb prior-request requirement for extensions of time? | 63.640(n)(8)(iii) YES | Y |  |
|  | Does Refinery MACT provide for submitting NSPS Kb documentation of the need for an extension with the next semi-annual periodic report? | 63.640(n)(8)(iv) YES | Y |  |
|  | Does Refinery MACT provide for submitting reports of NSPS Kb inspection failures on the semi-annual periodic report schedule? | 63.640(n)(8)(v) YES | Y |  |
| NSPS Subpart Kb | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR INTERNAL FLOATING ROOF TANKS |  | Y |  |
| 60.112b(a) | IFRT operating requirements: When landing the floating roof on its support legs, is the tank to be emptied & either refilled or degassed as soon as possible? | 60.112b(a)(1)(i) YES | Y |  |
|  | Temporary exemption from operating requirements while the internal floating roof is landed on its support legs? * | 60.112b(a)(1)(i) EXEMPT | Y |  |
|  | IFR Rim Seals: vapor-mounted primary seal: liquid-mounted primary seal: mechanical-shoe primary seal: | 60.112b(a)(1)(ii) OK with rim-mounted secondary OK alone OK alone | Y |  |
|  | Must a liquid-mounted seal be in contact with the liquid between the wall of the storage vessel and the floating roof continuously around the circumference of the tank? | 60.112b(a)(1)(ii)(A) REQUIRED | Y |  |
|  | Must IFR vapor-mounted rim seals be continuous? | 60.112b(a)(1)(ii)(B) REQUIRED | Y |  |
|  | Must a flexible coated fabric span the annualar space between the metal sheet and the floating roof? | 60.112b(a)(1)(ii)(C) REQUIRED | Y |  |
|  | IFR deck openings other than for vents to project into liquid? | 60.112b(a)(1)(iii) REQUIRED | Y |  |
|  | Deck openings (wells) other than for vents, drains, or legs to have covers that are kept closed except for access? | 60.112b(a)(1)(iv) REQUIRED | Y |  |
|  | IFR access hatch & gauge float well covers to be bolted closed? | 60.112b(a)(1)(iv) REQUIRED | Y |  |
|  | IFR well covers to be gasketed? | 60.112b(a)(1)(iv) & (ix) REQUIRED | Y |  |
|  | IFRT unslotted guidepoles to have a gasketed cap at the top of the pole? | 60.112b(a)(1)(iv) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) | Y |  |
|  | IFRT slotted guidepoles to have a deck cover gasket and pole wiper, and either an internal float or a pole sleeve? | 60.112b(a)(1)(iv) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) | Y |  |
|  | IFR automatic bleeder vent (vacuum breaker) to be closed except when the deck is landed? | 60.112b(a)(1)(v) REQUIRED | Y |  |
|  | IFR vents to be gasketed? | 60.112b(a)(1)(v) & (vi) REQUIRED | Y |  |
|  | IFR rim space vents to remain closed except when the pressure setting is exceeded? | 60.112b(a)(1)(vi) REQUIRED | Y |  |
|  | IFR sample penetration to be a sample well with a slit-fabric seal over 90% of the opening? | 60.112b(a)(1)(vii) REQUIRED | Y |  |
|  | IFR guidepole & column wells allowed a flexible-fabric sleeve seal or a gasketed cover? | 60.112b(a)(1)(viii) OK for columns | Y |  |
| 60.113b(a) | IFR/CFR Internal Inspections: (up close visual inspection of the floating roof, seals, & fittings): | 60.113b(a)(1) & (4) prior to initial fill, then every 10 years, including each emptying/degassing | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For IFR/CFR internal inspections: | 60.113b(a)(1) & (5) Required- notifications&reports per Ongoing Reports | Y |  |
|  | Shall there be no holes, tears, or openings in the IFR seals? | 60.113b(a)(1), (2), &(4) REQUIRED | Y |  |
|  | Is there to be no liquid on the internal floating roof? | 60.113b(a)(2) REQUIRED | Y |  |
|  | Tank Top Visual Inspections (of IFR/CFR from manways and hatches of the fixed roof): | 60.113b(a)(2) annually after initial fill | Y |  |
|  | IFRT REPAIRS: Time allowed for repair of defects found during in-service inspections: | 60.113b(a)(2) make repairs within 45 days | Y |  |
|  | IFRT REPAIRS: If unable to repair, empty the tank & remove from service? | 60.113b(a)(2) YES, within 45 days | Y |  |
|  | EXTENSIONS OF TIME: If defects cannot be repaired & the IFRT cannot be emptied within 45 days? | 60.113b(a)(2) 1 extension of 30 days, if needed * | Y |  |
|  | Periodic Reports: IFR/CFR report to include prior request for 30-day extension, with documentation of need? | 60.113b(a)(2) required * | Y |  |
|  | Periodic Reports: Additional information to be included if an extension is utilized for an IFR/CFR: | 60.113b(a)(2) document the reason for the extension * | Y |  |
|  | OPTION: Does this rule allow an internal inspection every 5 years to replace both inspections noted above, if the IFR/CFR is equipped with a secondary seal? | 60.113b(a)(3) & (4) YES | Y |  |
|  | IFRT REPAIRS: Repair of defects if the tank is empty? | 60.113b(a)(4) prior to refilling | Y |  |
|  | Notification of Inspections: Is 30-day notice required for internal inspections of IFRTs & CFRTs (i.e., prior to filling or refilling); but a 7-day verbal notice acceptable if the event is unplanned? | 60.113b(a)(5) REQUIRED | Y |  |
| 60.115b | Recordkeeping for inspections: Keep inspection reports as specified. | 60.115b Keep for 2 years | Y |  |
| 60.115b(a) | IFRT report to include: | 60.115b(a)(1) description of control equipment | Y |  |
|  | Records of IFR & CFR inspection reports: | 60.115b(a)(2) all IFR inspections | Y |  |
|  | Periodic Reports: Report of IFR/CFR inspections that find out-of-compliance? | 60.115b(a)(3) & (4) Required within 30 days for in-service inspections * (not required for out-of-service inspections) | Y |  |
|  | Periodic Reports: Report of IFR/CFR inspection failures to include: | 60.115b(a)(3) & (4) date of inspection, identification of tank, description of failure, & date of repair or emptying | Y |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| 60.116b(c) | Applicability records: Additional recordkeeping requirements for certain tanks. | 60.116b(c) identification & TVP of the stored product, if capacity > 20,000 gallons and TVP > 2.2, OR capacity > 40,000 gallons and TVP > 0.51 Keep record as long as the tank is in that service | Y |  |
| 60.116b(e) | True vapor pressure (TVP) determination for applicability: | 60.116b(e) maximum TVP of the stored liquid, based on highest calendar month average storage temperature | Y |  |
| NSPS Subpart A | New Source Performance Standards GENERAL PROVISIONS |  |  |  |
| 60.7(a) | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days after begin construction | Y |  |
|  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill | Y |  |
|  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup | Y |  |
|  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change | Y |  |
| 60.7(f) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required | Y |  |
| 60.14(g) | Achieve compliance for: New Tanks (or tanks that become affected as a result of a change or modification)? | 60.14(g) up to 180 days after modifications (otherwise prior to fill) | Y |  |

| Table IV – S Source-specific Applicable Requirements S858 - Tank 858 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – R and T for additional requirements. |  |  |
| BAAQMD Condition # 4977 |  |  |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Records [basis: Cumulative Increase] | Y |  |

| Table IV – U Source-specific Applicable Requirements S1006 - Tank 1006, S2013 - Tank 12467, S4310 - Tank13285 Sour Water, S17095 – Tank 17095 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-304 | Requirements for External Floating Roofs |  | Y |  |
| 8-5-320 | Tank Fitting Requirements |  | Y |  |
| 8-5-321 | Primary Seal Requirements |  | Y |  |
| 8-5-322 | Secondary Seal Requirements |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-405 | Information Required |  | Y |  |
| 8-5-501 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS ALSO SUBJECT TO NSPS Kb |  |  |  |
| 63.640(n) | Which rule governs for storage vessels subject to both Refinery MACT and NSPS subpart Kb? | 63.640(n)(1) NSPS subpart Kb | Y |  |
|  | Does Refinery MACT provide for EFR secondary seals to be pulled back or temporarily removed during NSPS Kb inspections of the primary seal? | 63.640(n)(8)(i) YES | Y |  |
|  | Does Refinery MACT provide for delay of NSPS Kb seal gap measurements due to unsafe conditions? | 63.640(n)(8)(ii) YES – up to 30 days, or empty the tank within 45 days | Y |  |
|  | Does Refinery MACT provide for extensions of time to perform NSPS Kb inspections of unsafe tanks? | 63.640(n)(8)(iii) YES – up to 2 extensions of 30 days each | Y |  |
|  | Does Refinery MACT provide for extensions of time to repair defects found during NSPS Kb inspections? | 63.640(n)(8)(iii) YES – up to 2 extensions of 30 days each | Y |  |
|  | Does Refinery MACT provide for waiving the NSPS Kb prior-request requirement for extensions of time? | 63.640(n)(8)(iii) YES | Y |  |
|  | Does Refinery MACT provide for submitting NSPS Kb documentation of the need for an extension with the next semi-annual periodic report? | 63.640(n)(8)(iv) YES | Y |  |
|  | Does Refinery MACT provide for submitting reports of NSPS Kb inspection failures on the semi-annual periodic report schedule? | 63.640(n)(8)(v) YES | Y |  |
|  | Does Refinery MACT provide for not reporting the results of NSPS Kb inspections when there was no out-of-compliance (i.e., recordkeeping only)? | 63.640(n)(8)(vi) YES | Y |  |
| NSPS Subpart Kb | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS |  |  |  |
| 60.112b(a) | EFR Rim Seals: vapor-mounted primary seal: liquid-mounted primary seal: mechanical-shoe primary seal: | 60.112b(a)(2)(i) Not Allowed OK with rim-mounted secondary OK with rim-mounted secondary | Y |  |
|  | Must the completely cover the annular space between the edge of the floating roof and tank wall? | 60.112b(a)(2)(i)(A) YES | Y |  |
|  | Must vapor-mounted rim seals be continuous on EFRs? | 60.112b(a)(2)(i)(B) YES | Y |  |
|  | Deck openings (wells) other than for vents, drains, or legs to have covers that are kept closed except for access? | 60.112b(a)(2)(ii) REQUIRED * | Y |  |
|  | EFR well covers to be gasketed? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR vents to be gasketed? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR deck openings other than for vents to project into liquid? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR rim space vents to remain closed except when the pressure setting is exceeded? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR automatic bleeder vent (vacuum breaker) to be closed except when the deck is landed? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR emergency roof drains to have seals covering at least 90% of the opening? | 60.112b(a)(2)(ii) REQUIRED | Y |  |
|  | EFR guidepole wells to have a deck cover gasket and a pole wiper? | 60.112b(a)(2)(ii) guidepole requirements are specified in FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) | Y |  |
|  | EFRT unslotted guidepoles to have a gasketed cap at the top of the pole? | 60.112b(a)(2)(ii) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) | Y |  |
|  | EFRT slotted guidepoles to have either an internal float or a pole sleeve? | 60.112b(a)(2)(ii) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) | Y |  |
|  | EFRT operating requirements: When landing the floating roof on its support legs, is the tank to be emptied & either refilled or degassed as soon as possible? | 60.112b(a)(2)(iii) YES | Y |  |
|  | Temporary exemption from operating requirements while the external floating roof is landed on its support legs? * | 60.112b(a)(2)(iii) EXEMPT | Y |  |
| 60.113b(b) | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR seal gap measurements: | 60.113b(b)(1) & (5) Required- notifications&reports per Ongoing Reports | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Primary Seal: | 60.113b(b)(1)(i) every 5 years | Y |  |
|  | Seal Gap Measurements: For new EFRTs: | 60.113b(b)(1)(i) &(ii) measure gaps of both seals within 60 days after initial fill | Y |  |
|  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Secondary Seal: | 60.113b(b)(1)(ii) annually | Y |  |
|  | Seal Gap Measurements: For EFRTs returned to affected service after 1 year or more of exempt service: | 60.113b(b)(1)(iii) measure gaps of both seals within 60 days | Y |  |
|  | MEASUREMENT COND'S: Are EFR seal gap measurements to be made with the roof floating? | 60.113b(b)(2)(i) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Presence of a gap determined by inserting a 1/8 in. probe? | 60.113b(b)(2)(ii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Use probes of various widths to determine the gap area? | 60.113b(b)(2)(iii) YES | Y |  |
|  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Sum the gap areas & divide by the diameter of the tank? | 60.113b(b)(3) YES | Y |  |
|  | EFRT REPAIRS: Time allowed for repair of defects found during in-service inspections of EFRs: If unable to repair, empty the EFRT & remove from service? | 60.113b(b)(4) make repairs within 45 days 60.113b(b)(4) YES, within 45 days | Y |  |
|  | EFR Primary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 60.113b(b)(4)(i) 10 in2 per foot of vessel diameter. 1.5 in. | Y |  |
|  | Shall there be no holes, tears, or openings in the EFR seals? | 60.113b(b)(4)(i) & (ii) YES | Y |  |
|  | Is the metallic shoe of an EFR mechanical-shoe seal required to have its bottom in the liquid and extend at least 24 in. above the liquid? | 60.113b(b)(4)(i)(A) YES | Y |  |
|  | Must there be no hole, tears, or other openings in the shoe, seal fabric or seal envelope? | 60.113b(b)(4)(i)(B) YES | Y |  |
|  | Must the secondary seal be installed above the primary seal so that it completely covers the space between the roof edge and the tank wall? | 60.113b(b)(4)(ii)(A) YES except as provided in (b)(2)(iii) | Y |  |
|  | EFR Secondary Seal Gap Inspection Criteria: maximum area: maximum gap width: | 60.113b(b)(4)(ii)(B) 1 in2 per foot of vessel diameter. 0.5 in. | Y |  |
|  | Must there be no hole, tears, or other openings in the seal or seal fabric? | 60.113b(b)(4)(ii)(C) YES | Y |  |
|  | Are EFR rim seals allowed to be pulled back or temporarily removed during inspection? | 60.113b(b)(4)(ii)(B) not addressed * | Y |  |
|  | EXTENSIONS OF TIME: If EFRT defects cannot be repaired & the tank cannot be emptied within 45 days? | 60.113b(b)(4)(iii) 1 extension of 30 days, if needed * | Y |  |
|  | Periodic Reports: EFR report to include a prior request for 30-day extension, with documentation of need? | 60.113b(b)(4)(iii) required * | Y |  |
|  | Periodic Reports: Additional information to be included if an extension is utilized for an EFR: | 60.113b(b)(4)(iii) document the reason for the extension * | Y |  |
|  | Notification of Inspections: Is 30-day notice required prior to EFR seal gap measurements? | 60.113b(b)(5) REQUIRED | Y |  |
|  | EFR Internal Inspections: up-close visual inspection of the floating roof, seals, & fittings: | 60.113b(b)(6) each time the tank is emptied & degassed | Y |  |
|  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR internal inspections: | 60.113b(b)(6) internal inspection not required for initial compl | Y |  |
|  | EFRT REPAIRS: Repair of defects if the tank is empty? | 60.113b(b)(6)(i) prior to refilling | Y |  |
|  | Notification of Inspections: Is 30-day notice required for internal inspections of EFRTs (i.e., prior to filling or refilling); but a 7-day verbal notice acceptable if the event is unplanned? | 60.113b(b)(6)(ii) REQUIRED | Y |  |
| 60.115b | Recordkeeping for inspections: Keep inspection reports as specified. | 60.115b Keep for 2 years | Y |  |
| 60.115b(b) | EFRT report to include: | 60.115b(b)(1) description of control equipment | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections if there was no out-of-compliance? | 60.115b(b)(2) Required within 60 days of inspection * | Y |  |
|  | Records of EFR inspection reports: | 60.115b(b)(3) EFR seal gap measurements | Y |  |
|  | Periodic Reports: Report EFR seal gap inspections when there is out-of-compliance? | 60.115b(b)(4) Required within 30 days of inspection * | Y |  |
|  | Periodic Reports: Report of EFR inspection failures to include: | 60.115b(b)(4) date of inspection, identification of tank, description of failure, & date of repair or emptying | Y |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| 60.116b(c) | Applicability records: Additional recordkeeping requirements for certain tanks. | 60.116b(c) identification & TVP of the stored product, if capacity > 20,000 gallons and TVP > 2.2, OR capacity > 40,000 gallons and TVP > 0.51 Keep record as long as the tank is in that service | Y |  |
| 60.116b(e) | True vapor pressure (TVP) determination for applicability: | 60.116b(e) maximum TVP of the stored liquid, based on highest calendar month average storage temperature | Y |  |
| NSPS Subpart A | New Source Performance Standards GENERAL PROVISIONS |  |  |  |
| 60.7(a) | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days. after begin construction | Y |  |
|  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill | Y |  |
|  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup | Y |  |
|  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change | Y |  |
| 60.7(f) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required | Y |  |
| 60.14(g) | Achieve compliance for: New Tanks (or tanks that become affected as a result of a change or modification)? | 60.14(g) up to 180 d. after modifications (otherwise prior to fill) | Y |  |

| Table IV – W Source-specific Applicable Requirements S1006 - Tank 1006, S2013 - Tank 12467, S2445 – Tank 12445, S2446 – Tank 12446, - S4322 – Tank 14571 Sour Water |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | For S1006 & S2013, See Table IV – U for additional requirements. For S2445 & S2446, See Table IV – R &CN for additional requirements. For S4322, See Table IV – R & DB for additional requirements. |  |  |
| BAAQMD Condition # 17648 |  |  |  |
| Part 1 | Slotted guidepole equipment requirements (basis: STERPP) | Y |  |
| Part 2 | Slotted guidepole operating requirements (basis: STERPP) | Y |  |
| Part 3 | Visual inspection requirements (basis: STERPP) | Y |  |
| Part 4 | Out of service (basis: STERPP) | Y |  |

| Table IV – X Source-specific Applicable Requirements S1023 - Tank 1023 S1050 - Tank 1050 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – R & T for additional requirements. |  |  |
| BAAQMD Condition # 7133 |  |  |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – Y Source-specific Applicable Requirements S1063 - Tank 1063 ETP 1, S1067 – Tank 1067 ETP 1 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-304 | Requirements for External Floating Roofs | Y |  |
| 8-5-320 | Tank Fitting Requirements | Y |  |
| 8-5-321 | Primary Seal Requirements | Y |  |
| 8-5-322 | Secondary Seal Requirements | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-401 | Inspection Requirements for External Floating Roof | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-405 | Information Required | Y |  |
| 8-5-501 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |
| NESHAP 40 CFR 61 Subpart A | National Emission Standards for Hazardous Air Pollutants - General Provisions (3/16/95) |  |  |
| 61.01 | Lists of pollutants and applicability of part 61 | Y |  |
| 61.04 | Address | Y |  |
| 61.05 | Prohibited activities | Y |  |
| 61.07 | Application for approval of construction or modification | Y |  |
| 61.09 | Notification of startup | Y |  |
| 61.10 | Source reporting and waiver request | Y |  |
| 61.12 | Compliance with standards and maintenance requirements | Y |  |
| 61.13 | Emission tests and waiver of emission tests | Y |  |
| 61.14 | Monitoring requirements | Y |  |
| 61.19 | Circumvention | Y |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.350 | Delay of Repair | Y |  |
| 61.351 | Alternative standards for tanks | Y |  |
| 61. 351(a) | Alternate to the standards for tanks specified in 61.343 | Y |  |
| 61.351(a)(2) | An external floating roof meeting the requirements of 40CFR60.112b(a)(2) | Y |  |
| 61.356 | Recordkeeping requirements | Y |  |
| 61.356(k) | Comply with recordkeeping requirements in 40CFR60.115b | Y |  |
| 61.357 | Reporting requirements | Y |  |
| 61.357(f) | Comply with reporting requirements in 40CFR60.115b | Y |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) | Y |  |
| 63.647 | Wastewater provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with section 61.340 to 61.355 of 40 CFR 61, subpart FF for each stream that meets the definition of 63.641 | Y |  |
| 63.654 | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) | Owners/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements of 61.356 and 61.357 of 40 CFR 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| Table IV – AA Source-specific Applicable Requirements S1070 - Tank 1070 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – Ea for additional requirements. |  |  |
| BAAQMD Condition # 18153 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2a | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2b | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2ci | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2cii | Material storage limit [basis: Cumulative Increase, Toxics] | N |  |
| Part 3 | Equipment requirement [basis: BACT, Cumulative Increase] | Y |  |
| Part 4 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – AB Source-specific Applicable Requirements S1072 - Tank 1072 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – J & K for additional requirements. |  |  |
| BAAQMD Condition # 7382 |  |  |  |
| Part 4 | Material storage requirement [basis: Cumulative Increase] | Y |  |
| Part 5 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 6 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – AC Source-specific Applicable Requirements S1077 - Tank 1411 S12490 – LOG Tank 12519 Wastewater ETP 1& 2 S12491 - LOG Tank 12520 Wastewater ETP 1&2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Applicable Requirement |  | Regulation Title or Description of Requirement |  |  | Federally Enforceable (Y/N) |  | Future Effective Date |
| BAAQMD Reg 8 Rule 5 |  | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |  |  |
| 8-5-111 |  | Tank Removal From and Return to Service |  |  | Y |  |  |
| 8-5-112 |  | Tanks in Operation – maintenance and inspection |  |  | Y |  |  |
| 8-5-301 |  | Storage Tank Control Requirements |  |  | Y |  |  |
| 8-5-304 |  | Requirements for External Floating Roofs |  |  | Y |  |  |
| 8-5-320 |  | Tank Fitting Requirements |  |  | Y |  |  |
| 8-5-321 |  | Primary Seal Requirements |  |  | Y |  |  |
| 8-5-322 |  | Secondary Seal Requirements |  |  | Y |  |  |
| 8-5-328 |  | Tank Degassing Requirements |  |  | Y |  |  |
| 8-5-401 |  | Inspection Requirements for External Floating Roof |  |  | Y |  |  |
| 8-5-404 |  | Certification |  |  | Y |  |  |
| 8-5-405 |  | Information Required |  |  | Y |  |  |
| 8-5-501 |  | Records |  |  | Y |  |  |
| 8-5-502 |  | Tank Degassing Annual Source Test Requirement |  |  | Y |  |  |
| 8-5-503 |  | Portable Hydrocarbon Detector |  |  | Y |  |  |
| NSPS Subpart Kb |  | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR EXTERNAL FLOATING ROOF TANKS |  |  | Y |  |  |
| 60.112b(a) |  | EFR Rim Seals: vapor-mounted primary seal: liquid-mounted primary seal: mechanical-shoe primary seal: | 60.112b(a)(2)(i) Not Allowed OK with rim-mounted secondary OK with rim-mounted secondary |  | Y |  |  |
|  |  | Must the completely cover the annular space between the edge of the floating roof and tank wall? | 60.112b(a)(2)(i)(A) YES |  | Y |  |  |
|  |  | Must vapor-mounted rim seals be continuous on EFRs? | 60.112b(a)(2)(i)(B) YES |  | Y |  |  |
|  |  | Deck openings (wells) other than for vents, drains, or legs to have covers that are kept closed except for access? | 60.112b(a)(2)(ii) REQUIRED * |  | Y |  |  |
|  |  | EFR well covers to be gasketed? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR vents to be gasketed? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR deck openings other than for vents to project into liquid? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR rim space vents to remain closed except when the pressure setting is exceeded? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR automatic bleeder vent (vacuum breaker) to be closed except when the deck is landed? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR emergency roof drains to have seals covering at least 90% of the opening? | 60.112b(a)(2)(ii) REQUIRED |  | Y |  |  |
|  |  | EFR guidepole wells to have a deck cover gasket and a pole wiper? | 60.112b(a)(2)(ii) guidepole requirements are specified in FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) |  | Y |  |  |
|  |  | EFRT unslotted guidepoles to have a gasketed cap at the top of the pole? | 60.112b(a)(2)(ii) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) |  | Y |  |  |
|  |  | EFRT slotted guidepoles to have either an internal float or a pole sleeve? | 60.112b(a)(2)(ii) Required per FR notices 65 FR 2336 (01/14/00) 65 FR 19891(04/13/00) |  | Y |  |  |
|  |  | EFRT operating requirements: When landing the floating roof on its support legs, is the tank to be emptied & either refilled or degassed as soon as possible? | 60.112b(a)(2)(iii) YES |  | Y |  |  |
|  |  | Temporary exemption from operating requirements while the external floating roof is landed on its support legs? * | 60.112b(a)(2)(iii) EXEMPT |  | Y |  |  |
| 60.113b(b) |  | Notification of Inspections: Are notifications of Inspections to demonstrate initial compliance required, For EFR seal gap measurements: | 60.113b(b)(1) & (5) Required- notifications&reports per Ongoing Reports |  | Y |  |  |
|  |  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Primary Seal: | 60.113b(b)(1)(I) every 5 years |  | Y |  |  |
|  |  | Seal Gap Measurements: For new EFRTs: | 60.113b(b)(1)(i) &(ii) measure gaps of both seals within 60 days after initial fill |  | Y |  |  |
|  |  | Seal Gap Measurements: FREQUENCY AFTER INITIAL COMPLIANCE, For the EFR Secondary Seal: | 60.113b(b)(1)(ii) annually |  | Y |  |  |
|  |  | Seal Gap Measurements: For EFRTs returned to affected service after 1 year or more of exempt service: | 60.113b(b)(1)(iii) measure gaps of both seals within 60 days |  | Y |  |  |
|  |  | MEASUREMENT COND'S: Are EFR seal gap measurements to be made with the roof floating? | 60.113b(b)(2)(I) YES |  | Y |  |  |
|  |  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Presence of a gap determined by inserting a 1/8 in. probe? | 60.113b(b)(2)(ii) YES |  | Y |  |  |
|  |  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Use probes of various widths to determine the gap area? | 60.113b(b)(2)(iii) YES |  | Y |  |  |
|  |  | DETERMINATION OF EFR RIM-SEAL GAP AREAS: Sum the gap areas & divide by the diameter of the tank? | 60.113b(b)(3) YES |  | Y |  |  |
|  |  | EFRT REPAIRS: Time allowed for repair of defects found during in-service inspections of EFRs: If unable to repair, empty the EFRT & remove from service? | 60.113b(b)(4) make repairs within 45 days 60.113b(b)(4) YES, within 45 days |  | Y |  |  |
|  |  | EFR Primary Seal Gap Inspection Criteria: Maximum area: Maximum gap width: | 60.113b(b)(4)(I) 10 in2 per foot of vessel diameter. 1.5 in. |  | Y |  |  |
|  |  | Shall there be no holes, tears, or openings in the EFR seals? | 60.113b(b)(4)(i) & (ii) YES |  | Y |  |  |
|  |  | Is the metallic shoe of an EFR mechanical-shoe seal required to have its bottom in the liquid and extend at least 24 in. above the liquid? | 60.113b(b)(4)(i)(A) YES |  | Y |  |  |
|  |  | Must there be no hole, tears, or other openings in the shoe, seal fabric or seal envelope? | 60.113b(b)(4)(i)(B) YES |  | Y |  |  |
|  |  | Must the secondary seal be installed above the primary seal so that it completely covers the space between the roof edge and the tank wall? | 60.113b(b)(4)(ii)(A) YES except as provided in (b)(2)(iii) |  | Y |  |  |
|  |  | EFR Secondary Seal Gap Inspection Criteria: Maximum area: Maximum gap width: | 60.113b(b)(4)(ii)(B) 1 in2 per foot of vessel diameter. 0.5 in. |  | Y |  |  |
|  |  | Must there be no hole, tears, or other openings in the seal or seal fabric? | 60.113b(b)(4)(ii)(C) YES |  | Y |  |  |
|  |  | Are EFR rim seals allowed to be pulled back or temporarily removed during inspection? | 60.113b(b)(4)(ii)(B) not addressed * |  | Y |  |  |
|  |  | EXTENSIONS OF TIME: If EFRT defects cannot be repaired & the tank cannot be emptied within 45 days? | 60.113b(b)(4)(iii) 1 extension of 30 days, if needed * |  | Y |  |  |
|  |  | Periodic Reports: EFR report to include a prior request for 30-day extension, with documentation of need? | 60.113b(b)(4)(iii) required * |  | Y |  |  |
|  |  | Periodic Reports: Additional information to be included if an extension is utilized for an EFR: | 60.113b(b)(4)(iii) document the reason for the extension * |  | Y |  |  |
|  |  | Notification of Inspections: Is 30-day notice required prior to EFR seal gap measurements? | 60.113b(b)(5) REQUIRED |  | Y |  |  |
|  |  | EFR Internal Inspections: up-close visual inspection of the floating roof, seals, & fittings: | 60.113b(b)(6) each time the tank is emptied & degassed |  | Y |  |  |
|  |  | Notification of Inspections: Are notifications of inspections to demonstrate initial compliance required, For EFR internal inspections: | 60.113b(b)(6) internal inspection not required for initial compl |  | Y |  |  |
|  |  | EFRT REPAIRS: Repair of defects if the tank is empty? | 60.113b(b)(6)(i) prior to refilling |  | Y |  |  |
|  |  | Notification of Inspections: Is 30-day notice required for internal inspections of EFRTs (i.e., prior to filling or refilling); but a 7-day verbal notice acceptable if the event is unplanned? | 60.113b(b)(6)(ii) REQUIRED |  | Y |  |  |
| 60.115b |  | Recordkeeping for inspections: Keep inspection reports as specified. | 60.115b Keep for 2 years |  | Y |  |  |
| 60.115b(b) |  | EFRT report to include: | 60.115b(b)(1) description of control equipment |  | Y |  |  |
|  |  | Periodic Reports: Report EFR seal gap inspections if there was no out-of-compliance? | 60.115b(b)(2) Required within 60 days of inspection * |  | Y |  |  |
|  |  | Records of EFR inspection reports: | 60.115b(b)(3) EFR seal gap measurements |  | Y |  |  |
|  |  | Periodic Reports: Report EFR seal gap inspections when there is out-of-compliance? | 60.115b(b)(4) Required within 30 days of inspection * |  | Y |  |  |
|  |  | Periodic Reports: Report of EFR inspection failures to include: | 60.115b(b)(4) date of inspection, identification of tank, description of failure, & date of repair or emptying |  | Y |  |  |
| 60.116b(a) |  | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years |  | Y |  |  |
| 60.116b(b) |  | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank |  | Y |  |  |
| 60.116b(c) |  | Applicability records: Additional recordkeeping requirements for certain tanks. | 60.116b(c) identification & TVP of the stored product, if capacity > 20,000 gallons and TVP > 2.2, OR capacity > 40,000 gallons and TVP > 0.51 Keep record as long as the tank is in that service |  | Y |  |  |
| 60.116b(e) |  | True vapor pressure (TVP) determination for applicability: | 60.116b(e) maximum TVP of the stored liquid, based on highest calendar month average storage temperature |  | Y |  |  |
| NSPS Subpart A |  | New Source Performance Standards GENERAL PROVISIONS |  |  | Y |  |  |
| 60.7(a) |  | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days after begin construction |  | Y |  |  |
|  |  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill |  | Y |  |  |
|  |  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup |  | Y |  |  |
|  |  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change |  | Y |  |  |
| 60.7(f) |  | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years |  | Y |  |  |
|  |  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required |  | Y |  |  |
| 60.14(g) |  | Achieve compliance for: New Tanks (or tanks that become affected as a result of a change or modification)? | 60.14(g) up to 180 days after modifications (otherwise prior to fill) |  | Y |  |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  | Y |  |  |  |
| 61.350 | Delay of repair |  |  | Y |  |  |  |
| 61.351 | Alternative standards for tanks |  |  | Y |  |  |  |
| 61.351(a) | Alternate to the standards for tanks specified in 61.343 |  |  | Y |  |  |  |
| 61.351(a)(2) | An external floating roof meeting the requirements of 40CFR60.112b(a)(2) |  |  | Y |  |  |  |
| 61.356 | Recordkeeping requirements |  |  | Y |  |  |  |
| 61.356(k) | Comply with recordkeeping requirements in 40CFR60.115b |  |  | Y |  |  |  |
| 61.357 | Reporting requirements |  |  | Y |  |  |  |
| 61.357(f) | Comply with reporting requirements in 40CFR60.115b |  |  | Y |  |  |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |  |  |  |  |
| 63.647 | Wastewater provisions |  |  | Y |  |  |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR part 61, subpart FF for each stream that meets the definition of 63.641. |  |  | Y |  |  |  |
| 63.654 | Reporting and recordkeeping requirements |  |  | Y |  |  |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR part 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. |  |  | Y |  |  |  |
| 63.654(i)(4) | All information required to be reported under 63.654 (a) through (h) shall be retained for 5 years. |  |  | Y |  |  |  |

| Table IV – AEa Source-specific Applicable Requirements S1114 - Tank 1114 Spent Acid, S1115 - Tank 1115 Spent Acid, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes |  | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve |  | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-501.1 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR FIX ROOF TANKS-CONTROL DEVICE ALSO SUBJECT TO NSPS Kb |  |  |  |
| 63.640(n) | Which rule governs for storage vessels subject to both Refinery MACT and NSPS subpart Kb? | 63.640(n)(1) NSPS subpart Kb | Y |  |
| NSPS Subpart Kb | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR FIXED ROOF TANK-CONTROL DEVICE |  | Y |  |
| 60.112b(a) | Closed vent system Performance requirements: | 60.112b(a)(3)(i) no detectable emissions (i.e., < 500 ppm) | Y |  |
| BAAQMD Condition # 7215 |  |  |  |  |
| Part 1 | Abatement requirement [basis: Cumulative Increase] |  | Y |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] |  | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] |  | Y |  |

| Table IV – AEb Source-specific Applicable Requirements S1805 Tank 12038 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes |  | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve |  | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-501.1 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| Refinery MACT | NESHAP for Petroleum Refineries REQUIREMENTS FOR FIX ROOF TANKS-CONTROL DEVICE ALSO SUBJECT TO NSPS Kb |  |  |  |
| 63.647 | Wastewater provisions |  | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR part 61, subpart FF for each stream that meets the definition of 63.641. |  | Y |  |
| 63.654 | Reporting and recordkeeping requirements |  | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR part 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. |  | Y |  |
| 63.654(i) | Recordkeeping |  | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. |  | Y |  |
| NSPS Subpart Kb | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR FIXED ROOF TANK-CONTROL DEVICE |  |  |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| NSPS Subpart A | New Source Performance Standards GENERAL PROVISIONS |  | Y |  |
| 60.7(a) | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days after begin construction | Y |  |
|  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill | Y |  |
|  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup | Y |  |
|  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change | Y |  |
| 60.7(f) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required | Y |  |
| 60.14(g) | Achieve compliance for: New Tanks (or tanks that become affected as a result of a change or modification)? | 60.14(g) up to 180 d. after modifications (otherwise prior to fill) | Y |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |  |
| 61.343 | Standards: Tanks |  | Y |  |
| 61.343(a)(1) | Owner/operator shall install, operate, and maintain a fixed-roof and closed-vent system that routes all organic vapors vented from the tank to a control device. |  | Y |  |
| 61.343(a)(1)(i) | Fixed Roof leak requirements - < 500 ppmv above background, determined initially then annually by method in 61.355(h) of this subpart. |  | Y |  |
| 61.343(b) | For a tank that meets all of the conditions of paragraph (b)(1), the owner/operator may elect to comply with paragraph (b)(2) as an alternative to the requirements of (a)(1). |  | Y |  |
| 61.343(c) | Inspect all fixed-roof, seal, access doors, and all other openings visually initially and quarterly for cracks and gaps and that doors and openings are closed and gasketed properly. |  | Y |  |
| 61.343(d) | When a broken seal or gasket is identified, or when detectable emissions are measured, first efforts at repair shall be made as soon as practicable, but not later than 45 calendar days after identification. |  | Y |  |
| 61.349 | Standards: Closed vent systems and control devices |  | Y |  |
| 61.349(a) | Closed vent system and control device |  | Y |  |
| 61.349(a)(1) | Closed vent system requirements |  | Y |  |
| 61.349(a)(2)(ii) | Vapor recovery system (e.g. carbon adsorption system) recovers or controls organic emissions with an efficiency of 95% by weight |  | Y |  |
| 61.349(b) | Operated at all times except maintenance or repair. |  | Y |  |
| 61.349(f) | Visible inspection and repair |  | Y |  |
| 61.349(g) | Visually inspected quarterly |  | Y |  |
| 61.349(h) | Monitor control device in accordance with 61.354(c) |  | Y |  |
| 61.350 | Delay of Repair |  | Y |  |
| 61.354 | Monitoring of operations |  | Y |  |
| 61.354(c) | Monitoring of operations for control devices |  | Y |  |
| 61.354(c)(8) | Monitoring for carbon adsorption |  | Y |  |
| 61.354(d) | Replace carbon immediately when carbon breakthrough is indicated. |  | Y |  |
| 61.355 | Test methods, procedures and compliance provisions |  | Y |  |
| 61.355(h) | Leak inspection procedures |  | Y |  |
| 61.356 | Recordkeeping requirements |  | Y |  |
| 61.356(d) | recordkeeping and retention requirements |  | Y |  |
| 61.356(h) | waste stream records |  | Y |  |
| 61.357 | Reporting requirements |  | Y |  |
| 61.357(d) | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste |  | Y |  |
| 61.357(d)(8) | Reporting requirements |  | Y |  |
| BAAQMD Condition # 7215 |  |  |  |  |
| Part 1 | Abatement requirement [basis: Cumulative Increase] |  | Y |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] |  | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] |  | Y |  |

| Table IV – AEc Source-specific Applicable Requirements S13 - Tank 13, S4334 – Tank 13276 Alkylate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |  |
| 8-5-111 | Tank Removal From and Return to Service |  | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection |  | Y |  |
| 8-5-301 | Storage Tank Control Requirements |  | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes |  | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve |  | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems |  | Y |  |
| 8-5-328 | Tank Degassing Requirements |  | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves |  | Y |  |
| 8-5-404 | Certification |  | Y |  |
| 8-5-501.1 | Records |  | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement |  | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector |  | Y |  |
| NSPS Subpart Kb | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR FIXED ROOF TANK-CONTROL DEVICE |  |  |  |
| 60.112b(a) | Closed vent system Performance requirements: | 60.112b(a)(3)(i) no detectable emissions (i.e., < 500 ppm) | Y |  |
|  | Control device Performance requirements: | 60.112b(a)(3)(ii) at least 95% efficient | Y |  |
| 60.113b(c) | Control device (other than flare) Compliance demonstration: | 60.113b(c)(1) operating plan, efficiency demo, & parameter(s) to be monitored | Y |  |
|  | Other (initial) Reports: For control device Other-than flare? | 60.113b(c)(1) submit operating plan for approval, with the initial notification | Y |  |
|  | Control device (other than flare) Operating requirements: | 60.113b(c)(2) operate and monitor per the plan | Y |  |
| 60.115b | Recordkeeping for inspections: Keep inspection reports as specified. | 60.115b Keep for 2 years | Y |  |
| 60.115b(c) | Recordkeeping for tanks Routed to a control device other than a flare: | 60.115b(c) operating plan & records of parametric monitoring data | Y |  |
| 60.116b(a) | Applicability records: Time period for keeping records of applicability determination, Unless specified otherwise. | 60.116b(a) Keep for 2 years | Y |  |
| 60.116b(b) | Applicability records: Records of dimensions & capacity required for Nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank | Y |  |
| 60.116b(e) | True vapor pressure (TVP) determination for applicability: | 60.116b(e) maximum TVP of the stored liquid, based on highest calendar month average storage temperature | Y |  |
| 60.116b(g) | Applicability determination: Miscellaneous recordkeeping exemptions: | 60.116b(g) keeping record of TVP is not required if tank is routed to a compliant control device | Y |  |
| NSPS Subpart A | New Source Performance Standards GENERAL PROVISIONS |  | Y |  |
| 60.7(a) | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days after begin construction | Y |  |
|  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill | Y |  |
|  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup | Y |  |
|  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change | Y |  |
| 60.7(f) | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years | Y |  |
|  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required | Y |  |
| 60.14(g) | Achieve compliance for: New Tanks (or tanks that Become affected as a result of a change or modification)? | 60.14(g) up to 180 d. after modifications (otherwise prior to fill) | Y |  |
| BAAQMD Condition # 7215 |  |  |  |  |
| Part 1 | Abatement requirement [basis: Cumulative Increase] |  | Y |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] |  | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] |  | Y |  |

| Table IV – AF Source-specific Applicable Requirements S1114 - Tank 1114 Spent Acid, S1115 - Tank 1115 Spent Acid, |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AE for additional requirements. |  |  |
| BAAQMD Condition # 7215 |  |  |  |
| Part 1 | Abatement requirement [basis: Cumulative Increase] | Y |  |
| Part 2 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – AG Source-specific Applicable Requirements S1116 - Tank 1116 Fresh Acid |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| BAAQMD Condition # 10275 |  |  |  |
| Part 1 | Material storage requirement [basis: Cumulative Increase] | Y |  |

| Table IV – AH Source-specific Applicable Requirements S1117 - Tank 1117 Skim, S1186 - Tank 1186 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-501.1 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |

| Table IV – AI Source-specific Applicable Requirements S1117 – Tank 1117 Skim |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AH for additional requirements. |  |  |
| BAAQMD Condition # 12190 |  |  |  |
| Part 1 | Annual throughput limit and recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV – AJ Source-specific Applicable Requirements S1129 - Tank 1129, S1130 - Tank 1130, S1131 - Tank 1131, S4310 - Tank 13285 Sour Water |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | For S1129, See Table IV – Ja for additional requirements. For S1130 & S1131, See Table IV – Jb for additional requirements. For S4310, See Table IV – U for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 47 | Equipment specification [basis: BACT] | Y |  |
| Part 51 | Degassing abatement requirements [basis: BACT] | Y |  |

| Table IV – AK Source-specific Applicable Requirements S1146 - Tank 1146, S1147 - Tank 1147 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-305 | Requirements for Internal Floating Roofs | Y |  |
| 8-5-320 | Tank Fitting Requirements | Y |  |
| 8-5-321 | Primary Seal Requirements | Y |  |
| 8-5-322 | Secondary Seal Requirements | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-402 | Inspection Requirements for Internal Floating Roof | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-405 | Information Required | Y |  |
| 8-5-501 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.350 | Delay of Repair | Y |  |
| 61.351 | Alternative standards for tanks | Y |  |
| 61.351(a) | Alternate to the standards for tanks specified in 61.343 |  |  |
| 61.351(a)(2) | An internal floating roof meeting the requirements of 40CFR60.112b(a)(1) | Y |  |
| 61.356 | Recordkeeping requirements | Y |  |
| 61.356(k) | Comply with recordkeeping and retention requirements in 40CFR60.115b | Y |  |
| 61.357 | Reporting requirements | Y |  |
| 61.357(f) | Comply with reporting requirements in 40CFR60.115b | Y |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 | Wastewater provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with section 61.340 to 61.355 of 40 CFR 61, subpart FF for each stream that meets the definition of 63.641 | Y |  |
| 63.654 | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) | Owners/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements of 61.356 and 61.357 of 40 CFR 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| Table IV - AL Source-specific Applicable Requirements S1409 - LUBS Sulfonation Plant (SULF), S1411 - LUBS Atmospheric Distillation LDU, S1412 - LUBS Vacuum Distillation LDU, S1415 - LUBS Furfural Plant (SEP3), S1416 – LUBS Lube Hydrotreater 1 (LHT1), S1417 – OPCEN Distillate Saturation Unit (DSU), S1420 – DH Crude Unit (CU), S1421 – DH Vacuum Flasher Unit (VFU), S1422 – DH Marine Fuel Oil Blender, S1423 – DH Gas Oil Straightrun Hydrotreater (GOHT), S1424 – DH Naphtha Straightrun Hydrotreater (NHT), S1425 – DH Catalytic Reformer Unit (CRU), S1427 – CP Catalytic Gas Plant (CGP), S1428 – CP Catalytic Feed Hydrotreater (CFH), S1429 – CP Catalytic Gasoline Hydrotreater (CGH), S1430 – CP Alkylation Plant (ALKY), S1433 – CP Light CC Gasoline Treater, S1434 – CP Unsaturated C3/C4 Treater, S1435 – CP Fuel Gas Treater 1, S1436 – CP Fuel Gas Treater 2, S1445 – DH Hydrogen Plant 1 (HP1), S1446 – DH Saturates Gas Plant (SGP), S1447 – DH Saturates Dry Gas Treater, S1448 – DH Saturates Gas Plant C3/C4 Treater, S1449 – DH Hydrocracking Unit (HCU), S1462 – LOG Distillate Blender (Jet & Diesel Fuel), S1463 – LOG Gasoline Blender, S1464 – LOG Thin Fuel Blender (Wharf), S1759 – OPCEN Flexicoker (FXU), S1764 – OPCEN Dimersol Plant (DIMER), S1770 – OPCEN C3/C4 Splitter, S1774 – OPCEN Hydrogen Plant 2 (HP2), S4001 – DC Delayed Coking Unit (DCU) , S4020 – DC Distillate Hydrotreater (DHT), S4050 - DC Catalytic Gas Depentanizer (CGDP), S4080 - DC isomerization Unit (ISOM), S4140 – DC Heavy Cracked Gasoline Hydrotreater (HGHT), S4160 – DC Hydrogen Plant 3 (HP3), S4170 – LUBS Lube Hydrotreater –2 (LHT2) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 9 | Vacuum Producing Systems (10/03/84) |  |  |
| 8-9-301 | Vacuum Producing Systems | Y |  |
| BAAQMD Regulation 8, Rule 10 | Organic Compounds – Process Vessel Depressurization (10/03/84) |  |  |
| 8-10-301 | Process Vessel Depressurizing | Y |  |
| 8-10-401 | Turnaround Records | Y |  |
| BAAQMD Regulation 8, Rule 18 | Organic Compounds – Equipment Leaks (06/05/03) |  |  |
| 8-18-301 | General | Y |  |
| 8-18-302 | Valves | Y |  |
| 8-18-303 | Pumps and Compressors | Y |  |
| 8-18-304 | Connections | Y |  |
| 8-18-305 | Pressure Relief Devices | Y |  |
| 8-18-306 | Non-repairable Equipment | Y |  |
| 8-18-307 | Liquid Leak | Y |  |
| 8-18-308 | Alternative Compliance | Y |  |
| 8-18-401 | Inspection | Y |  |
| 8-18-402 | Indentification | Y |  |
| 8-18-403 | Visual Inspection Schedule | Y |  |
| 8-18-404 | Alternative Inspection Schedule | Y |  |
| 8-18-405 | Alternative Emission Reduction Plan | Y |  |
| 8-18-406 | Interim Compliance | Y |  |
| 8-18-501 | Portable Hydrocarbon Detector | Y |  |
| 8-18-502 | Records | Y |  |
| BAAQMD Regulation 8, Rule 28 | Organic Compounds – Episodic Releases from Pressure Relief Devices at Petroleum Refineries and Chemical Plants (03/18/98) |  |  |
| 8-28-302 | Pressure Relief Devices at New or Modified Sources at Petroleum Refineries | N |  |
| 8-28-303 | Pressure Relief Devices at Existing Sources at Petroleum Refineries | N |  |
| 8-28-304 | Repeat Release Pressure Relief Devices at Petroleum Refineries | N |  |
| 8-28-401 | Reporting at Petroleum Refineries and Chemical Plants | N |  |
| 8-28-402 | Inspection | N |  |
| 8-28-403 | Records | N |  |
| 8-28-404 | Identification | N |  |
| 8-28-405 | Prevention Measures Procedures | N |  |
| SIP Regulation 8, Rule 28 | Organic Compounds – Episodic Releases from Pressure Relief Devices at Petroleum Refineries and Chemical Plants (12/9/94) |  |  |
| 8-28-401 | Reporting at Petroleum Refineries and Chemical Plants | Y |  |
| 8-28-402 | Inspection | Y |  |
| 8-28-403 | Records | Y |  |
| 8-28-404 | Identification | Y |  |

| Table IV - AN Source-specific Applicable Requirements S1426 – CP Catalytic Cracking Unit (CCU), S1429 – CP Catalytic Gasoline Hydrotreater (CGH), S1430 – CP Alkylation Plant (ALKY), S1449 – DH Hydrocracking Unit (HCU), S1764 - OPCEN Dimersol Plant (DIMER), , S4080 - DC Isomerization Unit (ISOM), S4140 - DC Heavy Cracked Gasoline Hydrotreater (HGHT): |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AL or AP (for S1426) for additional requirements. |  |  |
| BAAQMD Condition # 18643 |  |  |  |
| Part 2 | New pump in light liquid requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 3 | New valves in POC gas and light liquid requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 4 | New flanges requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 5 | New centrifugal compressors requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 6 | New pressure relief valve requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |

| Table IV - ANb Source-specific Applicable Requirements S1417 – OPCEN Distillate Saturation Unit (DSU), S4050 - DC Catalytic Gas Depentanizer (CGDP) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AL for additional requirements. |  |  |
| BAAQMD Condition # 18643 |  |  |  |
| Part 2 | New pump in light liquid requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 3 | New valves in POC gas and light liquid requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 4 | New flanges requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 5 | New centrifugal compressors requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 6 | New pressure relief valve requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 7 | Process sample systems requirement [basis: BACT/TBACT; Cumulative Increase] | Y |  |

| Table IV - AO Source-specific Applicable Requirements S1420 – DH Crude Unit (CU) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AL & AM for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.a | Crude oil process throughput limit [basis: Regulation 2-2-302] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV - AOa Source-specific Applicable Requirements S1425 – DH Catalytic Reformer Unit (CRU) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AL for additional requirements. |  |  |
| 40 CFR Part 63 Subpart A | MACT General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y | 4/11/05 |
| 63.6 | Compliance with Standards and Maintenance Requirements | Y | 4/11/05 |
| 63.6(e) | Operation and Maintenance Requirements | Y | 4/11/05 |
| 63.6(f) | Compliance with Nonopacity Emission Standards | Y | 4/11/05 |
| 63.6(g) | Use of Alternative Nonopacity Emission Standard (optional | Y | 4/11/05 |
| 63.7 | Performance Tests | Y | 9/8/05 |
| 63.8 | Monitoring | Y | 4/11/05 |
| 63.9 | Notifications | Y | 4/11/05 |
| 63.9(e) | Notification of Performance Test | Y | 30 days before test |
| 63.9(g) | Notification Requirements for sources with Continuous Monitoring Systems | Y | Simultaneous with notice of performance test |
| 63.9(h) | Notification of Compliance Status | Y | 5/11/05 and Subsequent |
| 63.9(j) | Change in information already provided | Y | 4/11/05 |
| 63.10 | Recordkeeping and Reporting Requirements | Y | 4/11/05 |
| 63.10(a) | General Information | Y | 4/11/05 |
| 63.10(b) | General Recordkeeping Requirements | Y | 4/11/05 |
| 63.10(b)(2) | Records to be maintained | Y | 4/11/05 |
| 63.10(c) | Recordkeeping requirements for Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(d) | General Reporting Requirements | Y | 4/11/05 |
| 63.10(e) | Additional reports for sources with Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(e)(2) | Reporting results of Continuous Monitoring System performance evaluation | Y | 9/8/05 |
| 63.10(e)(3) | Excess Emissions and Continuous Monitoring System Performance Report and Summary Report | Y | 4/11/05 |
| 40 CFR 63 Subpart UUU | NESHAP for Petroleum Refineries: Catalytic Reforming Units |  |  |
| 63.1567 | Requirements for Inorganic HAP Emissions from Catalytic Reforming Units | Y | 4/11/05 |
| 63.1567(a) | Emission Limitations and Work Practice Standards | Y | 4/11/05 |
| 63.1567(a)(1) | Sulfur Emission Limitations for Hydrogen Chloride (HCl) during coke burn-off and catalyst rejuvenation using wet scrubber: Reduce uncontrolled HCl emissions by 97% or to a concentration of 10 ppmvd corrected to 3%O2 (Table 22 Item 2) | Y | 4/11/05 |
| 63.1567(a)(2) | Operating limits for daily average pH of water and average liquid-to-gas ratio exiting wet scrubber during coke burn-off and catalyst rejuvenation: daily average pH of scrubbing liquid not fall below the limit established during performance test; daily average liquid-to-gas ratio not to fall below the limit established during performance test (Table 23 Item 1.a) | Y | 9/8/05 |
| 63.1567(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate in compliance with the plan | Y | 4/11/05 |
| 63.1567(b) | Initial Compliance Demonstration | Y | 4/11/05 |
| 63.1567(b)(1) | Install Continuous Parameter Monitoring System to record pH of water and liquid and gas flow rate to scrubber (Table 24, Item 1) | Y | 4/11/05 |
| 63.1567(b)(2) | Performance Test: measure HCl concentration at the outlet (for the concentration standard) or at the inlet and outlet (for the percent reduction standard) of the scrubber (Table 25, Item 1.a) | Y | 9/8/05 |
| 63.1567(b)(3) | Establish Operating Limit: measure and record pH of scrubbing liquid and gas and liquid flow rate every 15 minutes during the performance test. Determine hourly average. (Table 25, Items 1.b and 1.c) | Y | 9/8/05 |
| 63.1567(b)(4) | Demonstrate Initial Compliance with Emission Limitations: reduce HCl concentration by 97% or to 10 ppmv (Table 26, Item 2) | Y | 9/8/05 |
| 63.1567(b)(5) | Demonstrate Initial Compliance with Work Practice Standard by submitting Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1567(b)(6) | Submit Notice of Initial Compliance Status | Y | 5/11/05 |
| 63.1567(c) | Continuous Compliance Demonstration | Y | 4/11/05 |
| 63.1567(c)(1) | Demonstrate Continuous Compliance with Emission Limitation: maintain 97% control efficiency or 10 ppmv HCl concentration (Table 27, Item 2) and collect hourly and daily pH monitoring data and hourly average liquid-to-gas ratio, and maintain both above the operating limit established during performance test (Table 28, Items 1.a and 1.b) | Y | 9/8/05 |
| 63.1567(c)(2) | Demonstrate Continuous Compliance with Work Practice Standard through maintaining records to document conformance with the Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1570 | General Compliance Requirements | Y | 4/11/05 |
| 63.1570(a) | Operate in compliance with non-opacity standards at all times except during periods of startup, shutdown, and malfunction, as specified in 63.6(f)(1) | Y | 4/11/05 |
| 63.1570(c) | Operate and maintain source including pollution control and monitoring equipment in accordance with 63.6(e)(1). Between 4/11/05 and the date continuous monitoring systems are installed and validated and operating limits have been set, maintain a log detailing operation and maintenance of process and equipment. | Y | 4/11/05 |
| 63.1570(d) | Develop and implement startup, shutdown, and malfunction plan (SSMP) in accordance with 63.6(e)(3) | Y | 4/11/05 |
| 63.1570(e) | Operate in accordance with SSMP during periods of startup, shutdown, and malfunction | Y | 4/11/05 |
| 63.1570(f) | Report deviations from compliance with this subpart according to the requirements of 63.1575 | Y | 4/11/05 |
| 63.1570(g) | Deviations that occur during startup, shutdown, or malfunction are not violations if operating in accordance with SSMP | Y | 4/11/05 |
| 63.1571 | Performance Tests | Y | 4/11/05 |
| 63.1571(a) | Conduct Performance Test and submit results no later than 150 days after compliance date | Y | 9/8/05 |
| 63.1571(a)(1) | For emission limitation or work practice standard where compliance not demonstrated using performance test, opacity observation, or visible emission observation, conduct initial compliance demonstration within 30 days after compliance date | Y | 5/11/05 |
| 63.1571(b) | Requirements for Performance Tests | Y | 4/11/05 |
| 63.1571(b)(1) | Conduct performance tests in accordance with the requirements of 63.7(e)(1) | Y | 9/8/05 |
| 63.1571(b)(2) | Except for opacity and visual emissions observations, conduct three separate test runs of at least an hour for each performance test | Y | 9/8/05 |
| 63.1571(b)(3 | Conduct each performance evaluation in accordance with the requirements of 63.8(e) | Y | 9/8/05 |
| 63.1571(b)(4) | Performance tests not conducted during periods of startup, shutdown, or malfunction | Y | 9/8/05 |
| 63.1571(b)(5) | Arithmetic average of emission rates | Y | 9/8/05 |
| 63.1571(c) | Procedures for an Engineering Assessment (optional in lieu of performance test) | Y | 5/11/05 |
| 63.1571(d)(4) | Adjust process or control device measured values when establishing operating limit (optional) | Y | 9/8/05 |
| 63.1571(e) | Changes to Operating limits (optional) | Y | 9/8/05 |
| 63.1572 | Monitoring installation, operation, and maintenance requirements | Y | 4/11/05 |
| 63.1572(c) | Continuous parameter monitoring requirements | Y | 4/11/05 |
| 63.1572(c)(1) | Locate the air flow and liquid flow sensors and other necessary equipment that provides representative flow; use flow rate sensor with 5% accuracy; reduce abnormal conditions due to up/down stream disturbances; conduct semiannual calibration (Table 41, Item 3); and locate pH sensor in a position that provides a representative measurement; ensure the sample is properly mixed and representative; check calibration every 8 hours; inspect all components; record inspection results (Table 41, Item 5) | Y | 4/11/05 |
| 63.1572(c)(2) | Complete a minimum of one cycle for each 15-minute period; four cycles of operation for a valid hour of data | Y | 4/11/05 |
| 63.1572(c)(3) | Valid hourly data at least 75% of process operating hours | Y | 4/11/05 |
| 63.1572(c)(4) | Determine and record hourly and daily average of all recorded readings | Y | 4/11/05 |
| 63.1572(c)(5) | Record results of inspection, calibration, and validation check | Y | 4/11/05 |
| 63.1572(d) | Data monitoring and collection requirements | Y | 4/11/05 |
| 63.1572(d)(1) | Conduct monitoring at all times source is operating except for monitoring malfunctions, repairs, and QA/QC activities | Y | 4/11/05 |
| 63.1572(d)(2) | Not use data recorded during monitoring malfunctions, repairs, and QA/QC activities | Y | 4/11/05 |
| 63.1573 | Monitoring Alternatives | Y | 4/11/05 |
| 63.1573(b) | Alternatives for monitoring for pH (Table 41, Item 5) (optional) | Y | 4/11/05 |
| 63.1573(c) | Automated data compression system (optional) | Y | 4/11/05 |
| 63.1573(d) | Monitoring for alternative parameters (optional) | Y | 4/11/05 |
| 63.1573(e) | Alternative Monitoring Requests (optional) | Y | 4/11/05 |
| 63.1574 | Notification Requirements | Y | 4/11/05 |
| 63.1574(a) | Notifications Required by Subpart A | Y | 5/11/05 and subsequent |
| 63.1574(a)(2) | Submit notification of intent to conduct performance test 30 days before scheduled (instead of 60 days) | Y | 30 days before test |
| 63.1574(a)(3) | Notification of Compliance Status | Y | 5/11/05 |
| 63.1574(a)(3)(i) | Submit Notification of Compliance Status for initial compliance demonstration that does not include a performance test, no later than 30 days following completion of initial compliance demonstration | Y | 5/11/05 |
| 63.1574(a)(3)(ii) | Submit Notification of Compliance Status for initial compliance demonstration that includes a performance test, no later than 150 days after source compliance date | Y | 9/8/05 |
| 63.1574(d) | Information to be Submitted in Notice of Compliance Status (Table 42): identification of affected sources and emission points (Item 1); initial compliance demonstration (Item 2); continuous compliance (Item 3) | Y | 5/11/05 |
| 63.1574(f) | Requirement to prepare Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1574(f)(1) | Submit plan to permitting authority for review and approval along with NOCS. Include duty to prepare and implement plan into Part 70 or 71 permit. | Y | 5/11/05 |
| 63.1574(f)(2) | Minimum contents of Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1575 | Reports | Y | 7/31/05 |
| 63.1575(a) | Required reports: Statement that there were no deviations or report including information in 1575(d) or (e) (Table 43, Item 1) | Y | 7/31/05 |
| 63.1575(b) | Specified semiannual report submittal dates | Y | 7/31/05 |
| 63.1575(c) | Information required in compliance report | Y | 7/31/05 |
| 63.1575(d) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is not used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(f) | Additional information for compliance reports | Y | 7/31/05 |
| 63.1575(f)(1) | Requirement to submit performance test reports | Y | 1/31/06 |
| 63.1575(f)(2) | Submittal of requested change in the applicability of an emission standard | Y | 7/31/05 |
| 63.1575(g) | Submittal of reports required by other regulations in place of or as part of compliance report if they contain the required information | Y | 7/31/05 |
| 63.1575(h) | Reporting requirements for startups, shutdowns, and malfunctions | Y | 7/31/05 |
| 63.1576 | Recordkeeping | Y | 4/11/05 |
| 63.1576(a) | Required Records – General | Y | 4/11/05 |
| 63.1576(d) | Records required by Tables 20, 21, 27, and 28 of Subpart UUU | Y | 4/11/05 |
| 63.1576(e) | Maintain copy of Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1576(f) | Records of changes that affect emission control system performance | Y | 4/11/05 |
| 63.1576(g) | Records in a form suitable and readily available for review | Y | 4/11/05 |
| 63.1576(h) | Maintain records for 5 years | Y | 4/11/05 |
| 63.1576(i) | Records onsite for two years; may be maintained offsite for remaining 3 years | Y | 4/11/05 |

| Table IV – AP Source-specific Applicable Requirements S1426 – CP Catalytic Cracking Unit (CCU) |  |  |  |
| --- | --- | --- | --- |
|  | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) |  |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.5 | SO2 and opacity monitors at catalyst regenerators of CCUs | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures |  |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-302 | Opacity Limitation (where opacity monitor is required by the District) | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| 6-501 | Sampling Facilities and Instruments Required (where opacity monitor is required by the District) | Y |  |
| 6-502 | Data, Records and Reporting (where opacity monitor is required by the District) | Y |  |
| BAAQMD Regulation 8, Rule 9 | Vacuum Producing Systems (10/03/84) |  |  |
| 8-9-301 | Vacuum Producing Systems | Y |  |
| BAAQMD Regulation 8, Rule 10 | Organic Compounds – Process Vessel Depressurization (10/03/84) |  |  |
| 8-10-301 | Process Vessel Depressurizing | Y |  |
| 8-10-401 | Turnaround Records | Y |  |
| BAAQMD Regulation 8, Rule 18 | Organic Compounds – Equipment Leaks (06/05/03) |  |  |
| 8-18-301 | General | Y |  |
| 8-18-302 | Valves | Y |  |
| 8-18-303 | Pumps and Compressors | Y |  |
| 8-18-304 | Connections | Y |  |
| 8-18-305 | Pressure Relief Devices | Y |  |
| 8-18-306 | Non-repairable Equipment | Y |  |
| 8-18-307 | Liquid Leak | Y |  |
| 8-18-308 | Alternative Compliance | Y |  |
| 8-18-401 | Inspection | Y |  |
| 8-18-402 | Indentification | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-310 | Emission Limitations for Fluid Catalytic Cracking Units, Fluid Cokers, and Coke Calcining Kilns | Y |  |
| 9-1-310.1 | Sulfur dioxide limit | Y |  |
| 9-1-310.3 | Compliance with 9-1-110.1 and 110.2 | Y |  |
| 9-1-502 | Emission Monitoring Requirements (Regulations 1-520, 1-522) | Y |  |
| 40 CFR 60 Subpart A | General Provisions (7/1/2000) |  |  |
| 60.7 | Notification and record keeping. | Y |  |
| 60.8 | Performance tests. | Y |  |
| 60.11 | Compliance with standards and maintenance requirements. | Y |  |
| 60.12 | Circumvention. | Y |  |
| 60.13 | Monitoring requirements. | Y |  |
| 60.19 | General notification and reporting requirements. | Y |  |
| 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (8/17/89) |  |  |
| 60.102 | Standard for Particulate Matter | Y |  |
| 60.102(a)(1) | Limit on particulate matter from catalyst regenerator | Y |  |
| 60.102(a)(2) | Limit on opacity of gases from catalyst regenerator | Y |  |
| 60.102(b) | Limit on particulate matter from catalyst regenerator where gases pass through an incinerator or waste heat boiler in which auxiliary or supplemental fuel is burned | Y |  |
| 60.103 | Standard for Carbon Monoxide | Y |  |
| 60.103(a) | Limit on carbon monoxide emissions from catalyst regenerator | Y |  |
| 60.104 | Standard for Sulfur Oxides | Y |  |
| 60.104(b)(2) | Limit on sulfur oxide emissions from catalyst regenerator without add-on control device | Y |  |
| 60.104(c) | 7-day rolling average | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(1) | Continuous opacity monitoring requirement for catalyst regenerator emissions to atmosphere | Y |  |
| 60.105(a)(2) | Continuous CO concentration monitoring requirement for catalyst regenerator emissions to atmosphere | Y |  |
| 60.105(c) | Average coke burn-off rate (Mg (tons) per hour) and hours of operation | Y |  |
| 60.105(d) | Rate of combustion of liquid or solid fossil-fuels and the hours of operation | Y |  |
| 60.105(e) | Periods of excess emissions | Y |  |
| 60.105(e)(1) | Opacity | Y |  |
| 60.105(e)(2) | Carbon monoxide | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(a) | Use procedures and test methods in appendix A | Y |  |
| 60.106(b) | Particulate matter test method and compliance | Y |  |
| 60.106(c) | Determination of particulate matter if auxiliary liquid or solid fossil-fuels are burned in an incinerator-waste heat boiler | Y |  |
| 60.106(d) | CO test method and compliance | Y |  |
| 60.106(g) | SO2 test method and compliance | Y |  |
| 60.106(i)(12) | Alternative test methods and procedures | Y |  |
| 60.107 | Reporting and recordkeeping requirements. | Y |  |
| 60.108 | Performance test and compliance provisions. | Y |  |
| 60.108(a) | Performance tests conducted for compliance under 60.104(b) | Y |  |
| 60.108(d) | Daily performance tests | Y |  |
| 60.108(e) | Written notification and daily tests if change of compliance method under 60.104(b) | Y |  |
| 40 CFR Part 63 Subpart A | MACT General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y | 4/11/05 |
| 63.6 | Compliance with Standards and Maintenance Requirements | Y | 4/11/05 |
| 63.6(e) | Operation and Maintenance Requirements | Y | 4/11/05 |
| 63.6(f) | Compliance with Nonopacity Emission Standards | Y | 4/11/05 |
| 63.6(g) | Use of Alternative Nonopacity Emission Standard (optional | Y | 4/11/05 |
| 63.6(h) | Compliance with Opacity and Visible Emission Standards | Y | 4/11/05 |
| 63.7 | Performance Tests | Y | 9/8/05 |
| 63.8 | Monitoring | Y | 4/11/05 |
| 63.9 | Notifications |  |  |
| 63.9(e) | Notification of Performance Test | Y | 30 days before test |
| 63.9(g) | Notification Requirements for sources with Continuous Monitoring Systems | Y | Simultaneous with notice of performance test |
| 63.9(h) | Notification of Compliance Status | Y | 5/11/05 and Subsequent |
| 63.9(j) | Change in information already provided | Y | 4/11/05 |
| 63.10 | Recordkeeping and Reporting Requirements | Y | 4/11/05 |
| 63.10(a) | General Information | Y | 4/11/05 |
| 63.10(b) | General Recordkeeping Requirements | Y | 4/11/05 |
| 63.10(b)(2) | Records to be maintained | Y | 4/11/05 |
| 63.10(c) | Recordkeeping requirements for Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(d) | General Reporting Requirements | Y | 4/11/05 |
| 63.10(e) | Additional reports for sources with Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(e)(2) | Reporting results of Continuous Monitoring System performance evaluation | Y | 9/8/05 |
| 63.10(e)(3) | Excess Emissions and Continuous Monitoring System Performance Report and Summary Report | Y | 4/11/05 |
| 63.10(e)(4) | Reporting COMS data produced during performance test | Y | 9/8/05 |
| 40 CFR 63 Subpart UUU | NESHAP for Petroleum Refineries: Catalytic Cracking Units |  |  |
| 63.1564 | Requirements for metal HAP emissions from catalytic cracking units | Y | 4/11/05 |
| 63.1564(a) | Emission Limitations and Work Practice Standards | Y | 4/11/05 |
| 63.1564(a)(1)(i) | PM Emission Limitation for catalytic cracking units electing to meet NSPS limit (Option 1): PM emissions must not exceed 1 lb per 1,000 lb of coke burn-off, plus 0.1 lb/MMBtu of heat input to the CO boiler; and opacity must not exceed 30% except for one 6-minute opacity reading in any one hour (Table 1, Item 2) | Y | 4/11/05 |
| 63.1564(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times according to the procedures in the plan | Y | 4/11/05 |
| 63.1564(b) | Initial Compliance Demonstration | Y | 4/11/05 |
| 63.1564(b)(1) | Install Continuous Opacity Monitoring System (Table 3, Item 2) | Y | 4/11/05 |
| 63.1564(b)(2) | Performance Test (Table 4, Items 1.a through 1.e, 2.a through 2.c) | Y | 9/8/05 |
| 63.1564(b)(4)(i) | Compute PM emission rate (lb/1,000 lb coke burn-off rate) for each Performance Test run | Y | 9/8/05 |
| 63.1564(b)(5) | Initial Compliance Demonstration: Calculate PM emission rate using Equations 1, 2 and 3 of 63.1564(b)(4)(i); Opacity limit met; Continuous Opacity Monitoring System meets applicable requirements (Table 5, Item 2) | Y | 9/8/05 |
| 63.1564(b)(6) | Demonstrate initial compliance with work practice standard by submitting Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1564(b)(7) | Submit Notification of Compliance Status | Y | 5/11/05 |
| 63.1564(c) | Continuous Compliance Demonstration | Y | 4/11/05 |
| 63.1564(c)(1) | Demonstration of continuous compliance with emission limitation: Record daily the average coke burn-off rate; maintain PM emission rate below limit; record daily the rate of fuel combustion and hours of fuel combustion; maintain PM emissions from fuel combustion below limits; collect continuous opacity monitoring data and maintain opacity within limit (Table 6, Item 2) | Y | 4/11/05 |
| 63.1564(c)(2) | Demonstration of continuous compliance with work practice standard by maintaining records to document conformance with Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1565 | Requirements for organic HAP emissions from catalytic cracking units | Y | 4/11/05 |
| 63.1565(a) | Emission Limitations and Work Practice Standards | Y | 4/11/05 |
| 63.1565(a)(1)(i) | Emission limitation for catalytic cracking units electing to meet NSPS standard (Option 1): CO emissions from CO boiler must not exceed 500 ppmvd (Table 8, Item 2.a) | Y | 4/11/05 |
| 63.1565(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times in accordance with the plan | Y | 4/11/05 |
| 63.1565(b) | Initial Compliance Demonstration | Y | 4/11/05 |
| 63.1565(b)(1) | Install, operate and maintain Continuous Emission Monitoring System to measure and record the concentration of CO emissions (Table 10, Item 2.b) | Y | 4/11/05 |
| 63.1565(b)(2) | Performance Test (Table 11, Items 1.a through 1.d, 2) | Y | 9/8/05 |
| 63.1565(b)(4) | Demonstrate initial compliance with CO limit: hourly CO emissions over 24-hour period of performance test not more than 500 ppmvd; performance evaluation shows CEMS meets applicable requirements (Table 12, Item 2.a.ii) | Y | 9/8/05 |
| 63.1565(b)(5) | Demonstrate initial compliance with work practice standard by submitting Operation, Maintenance, and Monitoring Plan as part of NOCS. | Y | 5/11/05 |
| 63.1565(b)(6) | Submit Notice of Compliance Status. | Y | 5/11/05 |
| 63.1565(c) | Continuous Compliance Demonstration | Y | 4/11/05 |
| 63.1565(c)(1) | Demonstrate continuous compliance with emission limitation by collecting hourly average CO monitoring data and maintaining hourly average CO concentrations within limit (Table 13, Item 2.i) | Y | 4/11/05 |
| 63.1565(c)(2) | Demonstrate continuous compliance with work practice standard by maintaining records to document conformance with Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1569 | Bypass Lines | Y | 4/11/05 |
| 63.1569(a) | Work Practice Standards for Bypass Lines | Y | 4/11/05 |
| 63.1569(a)(1) (i) | Work Practice Standard (Option 1): Install and operate a device (Level Indicator) to continuously detect, at least every hour, whether flow is present in bypass line. (Table 36, Item 1) | Y | 4/11/05 |
| 63.1569(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times according to the procedures in the plan | Y | 4/11/05 |
| 63.1569(b) | Initial Compliance Demonstration | Y | 4/11/05 |
| 63.1569(b)(1) | Performance Test for level monitor: record during the performance test whether the level recorder was operating and whether flow was detected at any time (Table 37, Item 1) | Y | 9/8/05 |
| 63.1569(b)(2) | Demonstrate Initial Compliance with work practice standard during performance test (Option 1): equipment operates properly during each performance test run (Table 38, Item 1.a) | Y | 9/8/05 |
| 63.1569(b)(3) | Submit Operation, Maintenance, and Monitoring Plan as part of NOCS | Y | 5/11/05 |
| 63.1569(b)(4) | Submit Notice of Compliance Status | Y | 5/11/05 |
| 63.1569(c) | Continuous Compliance Demonstration | Y | 4/11/05 |
| 63.1569(c)(1) | Demonstrate continuous compliance with (Option 1): Continuously monitor and record whether flow is present in bypass line; record whether device is operating properly (Table 39, Item 1) | Y | 4/11/05 |
| 63.1569(c)(2) | Demonstrate continuous compliance with Work Practice Standard by complying with procedures in Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1570 | General Compliance Requirements | Y | 4/11/05 |
| 63.1570(a) | Operate in compliance with non-opacity standards at all times except during periods of startup, shutdown, and malfunction, as specified in 63.6(f)(1) | Y | 4/11/05 |
| 63.1570(b) | Operate in compliance with opacity and visible emissions standards at all times except during periods of startup, shutdown, and malfunction, as specified in 63.6(h)(1) | Y | 4/11/05 |
| 63.1570(c) | Operate and maintain source including pollution control and monitoring equipment in accordance with 63.6(e)(1). Between 4/11/05 and the date continuous monitoring systems are installed and validated and operating limits have been set, maintain a log detailing operation and maintenance of process and equipment. | Y | 4/11/05 |
| 63.1570(d) | Develop and implement startup, shutdown, and malfunction plan (SSMP) in accordance with 63.6(e)(3) | Y | 4/11/05 |
| 63.1570(e) | Operate in accordance with SSMPP during periods of startup, shutdown, and malfunction | Y | 4/11/05 |
| 63.1570(f) | Report deviations from compliance with this subpart according to the requirements of 63.1575 | Y | 4/11/05 |
| 63.1570(g) | Deviations that occur during startup, shutdown, or malfunction are not violations if operating in accordance with SSMP | Y | 4/11/05 |
| 63.1571 | Performance Tests |  |  |
| 63.1571(a) | Conduct Performance Test and submit results no later than 150 days after compliance date | Y | 9/8/05 |
| 63.1571(a)(1) | For emission limitation or work practice standard where compliance not demonstrated using performance test, opacity observation, or visible emission observation, conduct initial compliance demonstration within 30 days after compliance date | Y | 5/11/05 |
| 63.1571(b) | Requirements for Performance Tests | Y | 9/8/05 |
| 63.1571(b)(1) | Conduct performance tests in accordance with the requirements of 63.7(e)(1) | Y | 9/8/05 |
| 63.1571(b)(2) | Except for opacity and visual emissions observations, conduct three separate test runs of at least an hour for each performance test | Y | 9/8/05 |
| 63.1571(b)(3 | Conduct each performance evaluation in accordance with the requirements of 63.8(e) | Y | 9/8/05 |
| 63.1571(b)(4) | Performance tests not conducted during periods of startup, shutdown, or malfunction | Y | 9/8/05 |
| 63.1571(b)(5) | Arithmetic average of emission rates | Y | 9/8/05 |
| 63.1572 | Monitoring installation, operation, and maintenance requirements | Y | 4/11/05 |
| 63.1572(a) | Requirements for installation, operation, and maintenance of continuous emission monitoring system | Y | 4/11/05 |
| 63.1572(a)(1) | CO CEMS must meet requirements of Performance Specification 4 (40 CFR Part 60, App B) (Table 40, Item 2) | Y | 4/11/05 |
| 63.1572(a)(2) | Conduct performance evaluation for CO CEMS according to Performance Specification 4 (Table 40, Item 2) | Y | 9/8/05 |
| 63.1572(a)(3) | CEMS complete one cycle of operation for each 15-minute period | Y | 4/11/05 |
| 63.1572(a)(4) | Data reduction per 63.8(g)(2) | Y | 4/11/05 |
| 63.1572(b) | Requirements for installation, operation, and maintenance of continuous opacity monitoring system | Y | 4/11/05 |
| 63.1572(b)(1) | COMS must meet requirements of Performance Specification 1 (40 CFR Part 60, App B) (Table 40, Item 1) | Y | 4/11/05 |
| 63.1572(b)(2) | Conduct performance evaluation for CO CEMS according to Performance Specification 1 | Y | 9/8/05 |
| 63.1572(b)(3) | COMS complete one cycle of sampling and analyzing for each 10-second period and one cycle of data recording for each 6-minute period | Y | 4/11/05 |
| 63.1572(d) | Data monitoring and collection requirements | Y | 4/11/05 |
| 63.1572(d)(1) | Conduct monitoring at all times, except for monitoring malfunctions, repairs, and QA/QC activities | Y | 4/11/05 |
| 63.1572(d)(2) | Data recorded during monitoring malfunctions, repairs, and QA/QC activities not used for compliance purposes | Y | 4/11/05 |
| 63.1573 | Monitoring Alternatives | Y | 4/11/05 |
| 63.1573(d) | Monitoring for alternative parameters (optional) | Y | 4/11/05 |
| 63.1573(e) | Alternative Monitoring Requests (optional) | Y | 4/11/05 |
| 63.1574 | Notification Requirements | Y | 4/11/05 |
| 63.1574(a) | Notifications Required by Subpart A | Y | 5/11/05 and subsequent |
| 63.1574(a)(2) | Submit notification of intent to conduct performance test 30 days before scheduled (instead of 60 days) | Y | 30 days before test |
| 63.1574(a)(3) | Notification of Compliance Status | Y | 5/11/05 |
| 63.1574(a)(3)(i) | Submit Notification of Compliance Status for initial compliance demonstration that does not include a performance test, no later than 30 days following completion of initial compliance demonstration | Y | 5/11/05 |
| 63.1574(a)(3)(ii) | Submit Notification of Compliance Status for initial compliance demonstration that includes a performance test, no later than 150 days after source compliance date | Y | 9/8/05 |
| 63.1574(d) | Information to be Submitted in Notice of Compliance Status (Table 42): identification of affected sources and emission points (Item 1); initial compliance demonstration (Item 2); continuous compliance (Item 3) | Y | 5/11/05 |
| 63.1574(f) | Requirement to prepare Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1574(f)(1) | Submit plan to permitting authority for review and approval along with NOCS. Include duty to prepare and implement plan into Part 70 or 71 permit. | Y | 5/11/05 |
| 63.1574(f)(2) | Minimum contents of Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1575 | Reports | Y | 4/11/05 |
| 63.1575(a) | Required reports: Statement that there were no deviations or report including information in 1575(d) or (e) (Table 43, Item 1) | Y | 7/31/05 |
| 63.1575(b) | Specified semiannual report submittal dates | Y | 7/31/05 |
| 63.1575(c) | Information required in compliance report | Y | 7/31/05 |
| 63.1575(d) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is not used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(e) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(f) | Additional information for compliance reports | Y | 7/31/05 |
| 63.1575(f)(1) | Requirement to submit performance test reports | Y | 1/31/06 |
| 63.1575(f)(2) | Submittal of requested change in the applicability of an emission standard | Y | 7/31/05 |
| 63.1575(g) | Submittal of reports required by other regulations in place of or as part of compliance report if they contain the required information | Y | 4/11/05 |
| 63.1575(h) | Reporting requirements for startups, shutdowns, and malfunctions | Y | 4/11/05 |
| 63.1576 | Recordkeeping | Y | 4/11/05 |
| 63.1576(a) | Required Records – General | Y | 4/11/05 |
| 63.1576(b) | Records for CEMS and COMS | Y | 4/11/05 |
| 63.1576(c) | Records of visible emissions observations | Y | 4/11/05 |
| 63.1576(d) | Records required by Tables 6, 7, 13, and 14 of Subpart UUU | Y | 4/11/05 |
| 63.1576(e) | Maintain copy of Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1576(f) | Records of changes that affect emission control system performance | Y | 4/11/05 |
| 63.1576(g) | Records in a form suitable and readily available for review | Y | 4/11/05 |
| 63.1576(h) | Maintain records for 5 years | Y | 4/11/05 |
| 63.1576(i) | Records onsite for two years; may be maintained offsite for remaining 3 years | Y | 4/11/05 |
| Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.4 | Annual review of CCU emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulates, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.c | Catalytic Cracker feed throughput limit [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| Condition # 12911 | Fugitive requirements for pressure relief valves [basis: BACT] | Y |  |
| Part 1 | Catalyst Storage and Injection System Baghouse (A1427) in proper working order [basis: BACT] | Y |  |
| Part 2 | Ringelmann 0.5 requirement [basis: BACT] | Y |  |
| Part 3 | vent to S1507, S1509, and/or S1512 [basis: Regulation 2-6-409.2] | Y |  |
| Part 4 | Water seal of the dump stack [basis: Regulation 6-301] | Y |  |
| Part 5 | Monitoring and recordkeeping of water seal [basis: Regulation 6-301] | Y |  |
| BAAQMD Condition # 18407 |  |  |  |
| Part 1 | Abated by A1426 Baghouse for CCU Spent Catalyst Hopper (Portable) (basis: Cumulative Increase) | Y |  |
| Part 2 | Throughput loading limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV - AQ Source-specific Applicable Requirements S1431 – CP Sulfur Plant 1 (SRU1) S1432 – CP Sulfur Plant 2 (SRU2) S1765– OPCEN Sulfur Plant3 (SRU3) S4180 – OPCEN Sulfur PLant 4 (SRU4) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/2/01) |  |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.4 | SO2 monitor at sulfur recovery plants emitting more than 100 lb/day SO2 | Y |  |
| 1-520.8 | Monitors required by BAAQMD Regulation 10 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures |  |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-330 | Sulfur Recovery Units (SO3, H2SO4 emission limitations) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (3/15/95) |  |  |
| 9-1-307 | Emission Limitations for Sulfur Recovery Plants | Y |  |
| 9-1-313 | Sulfur Removal Operations at Petroleum Refineries (processing more than 20,000 bbl/day of crude oil) | N |  |
| 9-1-313.2 | Operation of a sulfur removal and recovery system that removes andrecovers: 95% of H2S from refinery fuel gas, and 95% of H2S and ammonia from process water streams | N |  |
| 9-1-502 | Emission Monitoring Requirements (Regulations 1-520, 1-522) | Y |  |
| SIP Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (06/08/99) |  |  |
| 9-1-313.2 | operation of a sulfur removal and recovery system that removes and recovers: 95% of H2S from refinery fuel gas, 95% of H2S and ammonia from process water streams | Y |  |
| 40 CFR 60 Subpart A | General Provisions (2/12/98) |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.13(a) | Continuous monitoring systems subject to Appendix B, and Appendix F(if used to demonstrate compliance with continuous emission limits), ofPart 60 | Y |  |
| 60.13(b) | Continuous monitoring systems and devices operational prior toperformance tests required by 60.8 | Y |  |
| 60.13(d) | Continuous monitoring system zero and span calibration requirements | Y |  |
| 60.13(e) | Continuous monitoring system minimum frequency of operation | Y |  |
| 60.13(f) | Continuous monitoring system installation location requirement | Y |  |
| 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (8/17/89) |  |  |
| 60.104 | Standards for Sulfur Dioxide | Y |  |
| 60.104(a)(2) (i) | Claus sulfur recovery plant emissions shall not exceed 250 ppmv SO2 at 0% excess air (for oxidation or reduction control systems followed by incineration) | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(5) | Claus sulfur recovery plants with oxidation or reduction control systems followed by incineration shall continuously monitor and record the concentration (dry basis, 0% excess air) of SO2 emissions; a continuousO2 monitor shall also be used to provide excess air correction | Y |  |
| 60.105(e)(4) | Definition of excess emissions | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(a) | Use procedures and test methods in appendix A | Y |  |
| 60.106(f)(1) | SO2 test method and compliance | Y |  |
| 60.106(f)(3) | Oxygen concentration for excess air | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 8 | Source test to verify compliance with Regulation 6-330 (basis: Regulation 2-6-409.2) | Y |  |
| Part 10 | Source test to verify compliance with Regulation 9-1-313.2 (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - AQa Source-specific Applicable Requirements S1431 – CP Sulfur Plant 1 (SRU1) S1432 – CP Sulfur Plant 2 (SRU2) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AQ for additional requirements. |  |  |
| 40 CFR Part 63 Subpart A | MACT General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y | 4/11/05 |
| 63.6 | Compliance with Standards and Maintenance Requirements | Y | 4/11/05 |
| 63.6(e) | Operation and Maintenance Requirements | Y | 4/11/05 |
| 63.6(f) | Compliance with Nonopacity Emission Standards | Y | 4/11/05 |
| 63.6(g) | Use of Alternative Nonopacity Emission Standard (optional | Y | 4/11/05 |
| 63.7 | Performance Tests | Y | 9/8/05 |
| 63.8 | Monitoring | Y | 4/11/05 |
| 63.9 | Notifications | Y | 4/11/05 |
| 63.9(e) | Notification of Performance Test | Y | 30 days before test |
| 63.9(g) | Notification Requirements for sources with Continuous Monitoring Systems | Y | Simultaneous with notice of performance test |
| 63.9(h) | Notification of Compliance Status | Y | 5/11/05 and Subsequent |
| 63.9(j) | Change in information already provided | Y | 4/11/05 |
| 63.10 | Recordkeeping and Reporting Requirements | Y | 4/11/05 |
| 63.10(a) | General Information | Y | 4/11/05 |
| 63.10(b) | General Recordkeeping Requirements | Y | 4/11/05 |
| 63.10(b)(2) | Records to be maintained | Y | 4/11/05 |
| 63.10(c) | Recordkeeping requirements for Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(d) | General Reporting Requirements | Y | 4/11/05 |
| 63.10(e) | Additional reports for sources with Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(e)(2) | Reporting results of Continuous Monitoring System performance evaluation | Y | 9/8/05 |
| 63.10(e)(3) | Excess Emissions and Continuous Monitoring System Performance Report and Summary Report | Y | 4/11/05 |
| 40 CFR 63 Subpart UUU | NESHAP for Petroleum Refineries: Sulfur Recovery Units |  |  |
| 63.1568 | Requirements for Sulfur Recovery Units | Y | 4/11/05 |
| 63.1568(a) | Emission Limitations and Work Practice Standards for Sulfur Recovery Units | Y | 4/11/05 |
| 63.1568(a)(1) | Emission Limitations | Y | 4/11/05 |
| 63.1568(a)(1)(i) | Sulfur Emission Limitation from Claus sulfur recovery units electing to meet NSPS Limits: 250 ppmvd SO2 at 0% excess air. (Table 29, Item 2.a) | Y | 4/11/05 |
| 63.1568(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times according to the procedures in the plan | Y | 4/11/05 |
| 63.1568(b) | Demonstrate Initial Compliance with Emission Limitation and Work Practice Standard | Y | 4/11/05 |
| 63.1568(b)(1) | Continuous Emission Monitoring System to measure and record hourly average SO2 concentration, with O2 monitor to correct excess air concentration (Table 31, Item 2.a) | Y | 4/11/05 |
| 63.1568(b)(2) | Performance Test: measure SO2 concentration using CEMS every 15 minutes for 24 hours and reduce the data to 1-hr averages (Table 32, Item1) | Y | 4/11/05 |
| 63.1568(b)(5) | Demonstrate Initial Compliance with Emission Limitation: Average SO2 emissions measured by CEMS in initial performance test not greater than 250 ppmvd at 0% excess O2, and monitoring system meets applicable requirements (Table 33, Item 2.a) | Y | 9/8/05 |
| 63.1568(b)(6) | Demonstrate initial compliance by submitting Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1568(b)(7) | Submit Notice of Compliance Status | Y | 5/11/05 |
| 63.1568(c) | Demonstrate Continuous Compliance with Emission Limitation and Work Practice Standards | Y | 4/11/05 |
| 63.1568(c)(1) | Demonstrate Continuous Compliance with Emission Limitation: collect hourly average SO2 monitoring data; maintain hourly average below applicable limit; determine and record each 12-hour concentration; report 12-hour concentration greater than applicable limitation (Table 34, Item 2.a) | Y | 4/11/05 |
| 63.1568(c)(2) | Demonstrate Continuous Compliance with Work Practice Standards by complying with the procedures in Operation, Maintenance, and Monitoring Plan. | Y | 4/11/05 |
| 63.1569 | Bypass Lines | Y | 4/11/05 |
| 63.1569(a) | Work Practice Standards for Bypass Lines | Y | 4/11/05 |
| 63.1569(a)(1) (i) | Work Practice Standard: Option 3 – Install solid blind between flanges (Table 36, Item 3) | Y | 4/11/05 |
| 63.1569(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times according to the procedures in the plan | Y | 4/11/05 |
| 63.1569(b) | Initial Compliance Demonstration | Y | 5/11/05 |
| 63.1569(b)(2) | Initial Compliance Demonstration: as part of Notification of Compliance Status, certify that equipment is installed, the equipment operational by compliance date, and identify the type of equipment installed. (Table 38, Item 1.c) | Y | 5/11/05 |
| 63.1569(b)(3) | Submit Operation, Maintenance, and Monitoring Plan as part of NOCS | Y | 5/11/05 |
| 63.1569(b)(4) | Submit Notification of Compliance Status | Y | 4/11/05 |
| 63.1569(c) | Continuous Compliance Demonstration | Y | 4/11/05 |
| 63.1569(c)(1) | Visually inspect the blind at least once a month and record whether the blind is maintained in the correct position (Table 39, Item 3) | Y | 4/11/05 |
| 63.1569(c)(2) | Comply with procedures in Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1570 | General Compliance Requirements | Y | 4/11/05 |
| 63.1570(a) | Operate in compliance with non-opacity standards at all times except during periods of startup, shutdown, and malfunction, as specified in 63.6(f)(1) | Y | 4/11/05 |
| 63.1570(c) | Operate and maintain source including pollution control and monitoring equipment in accordance with 63.6(e)(1). Between 4/11/05 and the date continuous monitoring systems are installed and validated and operating limits have been set, maintain a log detailing operation and maintenance of process and equipment. | Y | 4/11/05 |
| 63.1570(d) | Develop and implement startup, shutdown, and malfunction plan (SSMP) in accordance with 63.6(e)(3) | Y | 4/11/05 |
| 63.1570(e) | Operate in accordance with SSMPP during periods of startup, shutdown, and malfunction | Y | 4/11/05 |
| 63.1570(f) | Report deviations from compliance with this subpart according to the requirements of 63.1575 | Y | 4/11/05 |
| 63.1570(g) | Deviations that occur during startup, shutdown, or malfunction are not violations if operating in accordance with SSMP | Y | 4/11/05 |
| 63.1571 | Performance Tests | Y | 5/11/05 |
| 63.1571(a) | Conduct Performance Test and submit results no later than 150 days after compliance date | Y | 9/8/05 |
| 63.1571(a)(1) | For emission limitation or work practice standard where compliance not demonstrated using performance test, opacity observation, or visible emission observation, conduct initial compliance demonstration within 30 days after compliance date | Y | 5/11/05 |
| 63.1571(b) | Requirements for Performance Tests | Y | 9/8/05 |
| 63.1571(b)(1) | Conduct performance tests in accordance with the requirements of 63.7(e)(1) | Y | 9/8/05 |
| 63.1571(b)(2) | Conduct three separate test runs of at least an hour for each performance test | Y | 9/8/05 |
| 63.1571(b)(3 | Conduct each performance evaluation in accordance with the requirements of 63.8(e) | Y | 9/8/05 |
| 63.1571(b)(4) | Performance tests not conducted during periods of startup, shutdown, or malfunction | Y | 9/8/05 |
| 63.1571(b)(5) | Arithmetic average of emission rates | Y | 9/8/05 |
| 63.1572 | Monitoring installation, operation, and maintenance requirements | Y | 4/11/05 |
| 63.1572(a) | Requirements for installation, operation, and maintenance of continuous emission monitoring system | Y | 4/11/05 |
| 63.1572(a)(1) | SO2 CEMS must meet requirements of Performance Specification 2 (40 CFR Part 60, App B) (Table 40, Item 4) | Y | 4/11/05 |
| 63.1572(a)(2) | Conduct performance evaluation for SO2 CEMS according to Performance Specification 2 (Table 40, Item 4) | Y | 9/8/05 |
| 63.1572(a)(3) | CEMS complete one cycle of operation for each 15-minute period | Y | 4/11/05 |
| 63.1572(a)(4) | Data reduction per 63.8(g)(2) | Y | 4/11/05 |
| 63.1572(d) | Data monitoring and collection requirements | Y | 4/11/05 |
| 63.1572(d)(1) | Conduct monitoring at all times, except for monitoring malfunctions, repairs, and QA/QC activities | Y | 4/11/05 |
| 63.1572(d)(2) | Data recorded during monitoring malfunctions, repairs, and QA/QC activities not used for compliance purposes | Y | 4/11/05 |
| 63.1573 | Monitoring Alternatives | Y | 4/11/05 |
| 63.1573(d) | Monitoring for alternative parameters (optional) | Y | 4/11/05 |
| 63.1573(e) | Alternative Monitoring Requests (optional) | Y | 4/11/05 |
| 63.1574 | Notification Requirements | Y | 5/11/05 |
| 63.1574(a) | Notifications Required by Subpart A | Y | 5/11/05 and subsequent |
| 63.1574(a)(2) | Submit notification of intent to conduct performance test 30 days before scheduled (instead of 60 days) | Y | 30 days before test |
| 63.1574(a)(3) | Notification of Compliance Status | Y | 5/11/05 |
| 63.1574(a)(3)(i) | Submit Notification of Compliance Status for initial compliance demonstration that does not include a performance test, no later than 30 days following completion of initial compliance demonstration | Y | 5/11/05 |
| 63.1574(a)(3)(ii) | Submit Notification of Compliance Status for initial compliance demonstration that includes a performance test, no later than 150 days after source compliance date | Y | 9/8/05 |
| 63.1574(d) | Information to be Submitted in Notice of Compliance Status (Table 42): identification of affected sources and emission points (Item 1); initial compliance demonstration (Item 2); continuous compliance (Item 3) | Y | 5/11/05 |
| 63.1574(f) | Requirement to prepare Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1574(f)(1) | Submit plan to permitting authority for review and approval along with NOCS. Include duty to prepare and implement plan into Part 70 or 71 permit. | Y | 5/11/05 |
| 63.1574(f)(2) | Minimum contents of Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1575 | Reports | Y | 7/31/05 |
| 63.1575(a) | Required reports: Statement that there were no deviations or report including information in 1575(d) or (e) (Table 43, Item 1) | Y | 7/31/05 |
| 63.1575(b) | Specified semiannual report submittal dates | Y | 7/31/05 |
| 63.1575(c) | Information required in compliance report | Y | 7/31/05 |
| 63.1575(d) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is not used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(e) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(f) | Additional information for compliance reports | Y | 7/31/05 |
| 63.1575(f)(1) | Requirement to submit performance test reports | Y | 1/31/06 |
| 63.1575(f)(2) | Submittal of requested change in the applicability of an emission standard | Y | 7/31/05 |
| 63.1575(g) | Submittal of reports required by other regulations in place of or as part of compliance report if they contain the required information | Y | 4/11/05 |
| 63.1575(h) | Reporting requirements for startups, shutdowns, and malfunctions | Y | 7/31/05 |
| 63.1576 | Recordkeeping | Y | 4/11/05 |
| 63.1576(a) | Required Records – General | Y | 4/11/05 |
| 63.1576(b) | Records for CEMS | Y | 4/11/05 |
| 63.1576(d) | Records required by Tables 34 and 35 of Subpart UUU | Y | 4/11/05 |
| 63.1576(e) | Maintain copy of Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1576(f) | Records of changes that affect emission control system performance | Y | 4/11/05 |
| 63.1576(g) | Records in a form suitable and readily available for review | Y | 4/11/05 |
| 63.1576(h) | Maintain records for 5 years | Y | 4/11/05 |
| 63.1576(i) | Records onsite for two years; may be maintained offsite for remaining 3 years | Y | 4/11/05 |

| Table IV – AQb Source-specific Applicable Requirements S1765– OPCEN Sulfur Plant3 (SRU3) S4180 – OPCEN Sulfur Plant4 (SRU4) |  |  |  |
| --- | --- | --- | --- |
|  | See Table IV – AQ for additional requirements. |  |  |
| 40 CFR Part 63 Subpart A | MACT General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y | 4/11/05 |
| 63.6 | Compliance with Standards and Maintenance Requirements | Y | 4/11/05 |
| 63.6(e) | Operation and Maintenance Requirements | Y | 4/11/05 |
| 63.6(f) | Compliance with Nonopacity Emission Standards | Y | 4/11/05 |
| 63.6(g) | Use of Alternative Nonopacity Emission Standard (optional | Y | 4/11/05 |
| 63.8 | Monitoring | Y | 4/11/05 |
| 63.9 | Notifications | Y | 4/11/05 |
| 63.9(h) | Notification of Compliance Status | Y | 5/11/05 and Subsequent |
| 63.9(j) | Change in information already provided | Y | 4/11/05 |
| 63.10 | Recordkeeping and Reporting Requirements | Y | 4/11/05 |
| 63.10(a) | General Information | Y | 4/11/05 |
| 63.10(b) | General Recordkeeping Requirements | Y | 4/11/05 |
| 63.10(b)(2) | Records to be maintained | Y | 4/11/05 |
| 63.10(c) | Recordkeeping requirements for Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(d) | General Reporting Requirements | Y | 4/11/05 |
| 63.10(e) | Additional reports for sources with Continuous Monitoring Systems | Y | 4/11/05 |
| 63.10(e)(2) | Reporting results of Continuous Monitoring System performance evaluation | Y | 9/8/05 |
| 63.10(e)(3) | Excess Emissions and Continuous Monitoring System Performance Report and Summary Report | Y | 4/11/05 |
| 40 CFR 63 Subpart UUU | NESHAP for Petroleum Refineries: Sulfur Recovery Units |  |  |
| 63.1568 | Requirements for Sulfur Recovery Units | Y | 4/11/05 |
| 63.1568(a) | Emission Limitations and Work Practice Standards for Sulfur Recovery Units | Y | 4/11/05 |
| 63.1568(a)(1) | Sulfur Emission Limitation from Claus sulfur recovery units subject to NSPS: 250 ppmvd SO2 at 0% excess air (Table 29, Item 1.a) | Y | 4/11/05 |
| 63.1568(a)(3) | Prepare Operation, Maintenance, and Monitoring Plan and operate at all times according to the procedures in the plan | Y | 4/11/05 |
| 63.1568(b) | Demonstrate Initial Compliance with Emission Limitation and Work Practice Standard | Y | 4/11/05 |
| 63.1568(b)(1) | Continuous Emission Monitoring System to measure and record hourly average SO2 concentration, with O2 monitor to correct excess air concentration (Table 31, Item 1.a) | Y | 4/11/05 |
| 63.1568(b)(5) | Demonstrate Initial Compliance with Emission Limitation : Certification that vent meets NSPS SO2 limit; certification that CEMS meets applicable requirements of 63.1572. (Table 33, Item 1.a) | Y | 5/11/05 |
| 63.1568(b)(6) | Demonstrate initial compliance by submitting Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1568(b)(7) | Submit Notice of Compliance Status | Y | 5/11/05 |
| 63.1568(c) | Demonstrate Continuous Compliance with Emission Limitation and Work Practice Standards | Y | 4/11/05 |
| 63.1568(c)(1) | Demonstrate Continuous Compliance with Emission Limitation: collect hourly average SO2 monitoring data; maintain hourly average below applicable limit; determine and record each 12-hour concentration; report 12-hour concentration greater than applicable limitation (Table 34, Item 1.a) | Y | 4/11/05 |
| 63.1568(c)(2) | Demonstrate Continuous Compliance with Work Practice Standards by complying with the procedures in Operation, Maintenance, and Monitoring Plan. | Y | 4/11/05 |
| 63.1570 | General Compliance Requirements | Y | 4/11/05 |
| 63.1570(a) | Operate in compliance with non-opacity standards at all times except during periods of startup, shutdown, and malfunction, as specified in 63.6(f)(1) | Y | 4/11/05 |
| 63.1570(c) | Operate and maintain source including pollution control and monitoring equipment in accordance with 63.6(e)(1). Between 4/11/05 and the date continuous monitoring systems are installed and validated and operating limits have been set, maintain a log detailing operation and maintenance of process and equipment. | Y | 4/11/05 |
| 63.1570(d) | Develop and implement startup, shutdown, and malfunction plan (SSMP) in accordance with 63.6(e)(3) | Y | 4/11/05 |
| 63.1570(e) | Operate in accordance with SSMPP during periods of startup, shutdown, and malfunction | Y | 4/11/05 |
| 63.1570(f) | Report deviations from compliance with this subpart according to the requirements of 63.1575 | Y | 4/11/05 |
| 63.1570(g) | Deviations that occur during startup, shutdown, or malfunction are not violations if operating in accordance with SSMP | Y | 4/11/05 |
| 63.1572 | Monitoring installation, operation, and maintenance requirements | Y | 4/11/05 |
| 63.1572(a) | Requirements for installation, operation, and maintenance of continuous emission monitoring system | Y | 4/11/05 |
| 63.1572(a)(1) | SO2 CEMS must meet requirements of Performance Specification 2 (40 CFR Part 60, App B) (Table 40, Item 4) | Y | 4/11/05 |
| 63.1572(a)(3) | CEMS complete one cycle of operation for each 15-minute period | Y | 4/11/05 |
| 63.1572(a)(4) | Data reduction per 63.8(g)(2) | Y | 4/11/05 |
| 63.1572(d) | Data monitoring and collection requirements | Y | 4/11/05 |
| 63.1572(d)(1) | Conduct monitoring at all times, except for monitoring malfunctions, repairs, and QA/QC activities | Y | 4/11/05 |
| 63.1572(d)(2) | Data recorded during monitoring malfunctions, repairs, and QA/QC activities not used for compliance purposes | Y | 4/11/05 |
| 63.1573 | Monitoring Alternatives | Y | 4/11/05 |
| 63.1573(d) | Monitoring for alternative parameters (optional) | Y | 4/11/05 |
| 63.1573(e) | Alternative Monitoring Requests (optional) | Y | 4/11/05 |
| 63.1574 | Notification Requirements | Y | 4/11/05 |
| 63.1574(a) | Notifications Required by Subpart A | Y | 5/11/05 and subsequent |
| 63.1574(a)(3) | Notification of Compliance Status | Y | 5/11/05 |
| 63.1574(d) | Information to be Submitted in Notice of Compliance Status (Table 42): identification of affected sources and emission points (Item 1); initial compliance demonstration (Item 2); continuous compliance (Item 3) | Y | 5/11/05 |
| 63.1574(f) | Requirement to prepare Operation, Maintenance, and Monitoring Manual | Y | 4/11/05 |
| 63.1574(f)(1) | Submit plan to permitting authority for review and approval along with NOCS. Include duty to prepare and implement plan into Part 70 or 71 permit. | Y | 5/11/05 |
| 63.1574(f)(2) | Minimum contents of Operation, Maintenance, and Monitoring Plan | Y | 5/11/05 |
| 63.1575 | Reports | Y | 4/11/05 |
| 63.1575(a) | Required reports: Statement that there were no deviations or report including information in 1575(d) or (e) (Table 43, Item 1) | Y | 7/31/05 |
| 63.1575(b) | Specified semiannual report submittal dates | Y | 7/31/05 |
| 63.1575(c) | Information required in compliance report | Y | 7/31/05 |
| 63.1575(d) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is not used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(e) | Information required for deviations from emission limitations and work practice standards where CEMS or COMS is used to comply with emission limitation or work practice standard | Y | 7/31/05 |
| 63.1575(f) | Additional information for compliance reports | Y | 7/31/05 |
| 63.1575(f)(2) | Submittal of requested change in the applicability of an emission standard | Y | 7/31/05 |
| 63.1575(g) | Submittal of reports required by other regulations in place of or as part of compliance report if they contain the required information | Y | 4/11/05 |
| 63.1575(h) | Reporting requirements for startups, shutdowns, and malfunctions | Y | 7/31/05 |
| 63.1576 | Recordkeeping | Y | 4/11/05 |
| 63.1576(a) | Required Records – General | Y | 4/11/05 |
| 63.1576(b) | Records for CEMS | Y | 4/11/05 |
| 63.1576(d) | Records required by Tables 34 and 35 of Subpart UUU | Y | 4/11/05 |
| 63.1576(e) | Maintain copy of Operation, Maintenance, and Monitoring Plan | Y | 4/11/05 |
| 63.1576(f) | Records of changes that affect emission control system performance | Y | 4/11/05 |
| 63.1576(g) | Records in a form suitable and readily available for review | Y | 4/11/05 |
| 63.1576(h) | Maintain records for 5 years | Y | 4/11/05 |
| 63.1576(i) | Records onsite for two years; may be maintained offsite for remaining 3 years | Y | 4/11/05 |

| Table IV - AR Source-specific Applicable Requirements S1765– OPCEN Sulfur Plant 3 (SRU3) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AQ for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.5 | Baseline for sulfur plants [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV – AS Source-specific Applicable Requirements S1457 – Cooling Water Tower (CWT-32) S1778 – Cooling Water Tower (CWT-50) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 |  | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 |  | Ringelmann No. 1 Limitation | Y |  |
| 6-305 |  | Visible Particles | Y |  |
| 6-310 |  | Particle Weight Limitation | Y |  |
| 6-311 |  | General Operations (process weight rate limitation) | Y |  |
| 6-401 |  | Appearance of Emissions | Y |  |
| BAAQMD Regulation 8 Rule 2 |  | Organic Compounds – Miscellaneous Operations (03/22/95) |  |  |
| 8-2-301 |  | Miscellaneous Operations | Y |  |

| Table IV – AT Source-specific Applicable Requirements S1465 – LOG Light Oil Products Gross Oil Separator S1779 - OPCEN CPI Oil/Water Separator S2009 - LUBS Wastewater Separator Dubbs Box |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 8 | Wastewater (Oil-Water) Separator (08/29/94) |  |  |
| 8-8-302 | Wastewater separators rated capacity larger than or equal to 18.9 liters per seconds (300 gal/min), must be equipped with one of the following: | Y |  |
| 8-8-302.4 | a solid, sealed, gasketed, fixed cover which totally encloses the separator tank, chamber, or basin liquid contents, with all cover openings closed and sealed, except when the opening is being used for inspection, maintenance, or wastewater sampling. The cover may include a pressure-vacuum valve. Also includes leak inspection frequency and leak criteria. | Y |  |
| 8-8-303 | Gauging and Sampling Devices | Y |  |
| 8-8-501 | API Separator or Air Flotation Bypassed Wastewater Records | Y |  |
| 8-8-503 | Inspection and Repair Records | Y |  |

| Table IV – AU Source-specific Applicable Requirements S1465 – LOG Light Oil Products Gross Oil Separator |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AT for additional requirements. |  |  |
| Condition # 5077 |  |  |  |
| Part 4 | Design Rated Capacity Limit [basis: Cumulative Increase] | Y |  |
| Part 6 | Reporting of Emission Leaks [basis: Regulation 8-8-302.1] | Y |  |

| Table IV – AV Source-specific Applicable Requirements S1469 – LOG API Separator (ETP 1) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 8 | Wastewater (Oil-Water) Separator (08/29/94) |  |  |
| 8-8-302 | Wastewater separators rated capacity larger than or equal to 18.9 liters per seconds (300 gal/min), must be equipped with one of the following: | Y |  |
| 8-8-302.4 | a solid, sealed, gasketed, fixed cover which totally encloses the separator tank, chamber, or basin liquid contents, with all cover openings closed and sealed, except when the opening is being used for inspection, maintenance, or wastewater sampling. The cover may include a pressure-vacuum valve. Also includes leak inspection frequency and leak criteria. | Y |  |
| 8-8-303 | Gauging and Sampling Devices | Y |  |
| 8-8-501 | API Separator or Air Flotation Bypassed Wastewater Records | Y |  |
| 8-8-503 | Inspection and Repair Records | Y |  |
| District Regulation 11 Rule 12 | Hazardous Pollutants - National Emission Standards for Benzene Emissions from Benzene Transfer Operations and Benzene Waste Operations (7/18/90, Refer to NESHAP Subpart FF below) | Y |  |
| NESHAP 40 CFR 61 Subpart FF | National Emission Standard for Benzene Waste Operations (3/7/90) | Y |  |
| 61.347 | Standards: Oil-water separators | Y |  |
| 61.347(a) | Except as provided in 61.352 of this subpart, each oil-water separator shall meet the following standards: | Y |  |
| 61.347(a)(1) | install, operate and maintain a fixed-roof and closed-vent system that routes all organic vapors to a control device | Y |  |
| 61.347(b) | cover seals, access hatches, and all other openings shall be checked visually initially and quarterly thereafter to ensure no cracks, gaps occur between the cover and wall and that access hatches are closed and gasketed properly | Y |  |
| 61.347(c) | except for delay or repair, when a broken seal or gasket or other problem is identified, or when detectable emissions are measured, first efforts repairs shall be made as soon as possible, but not later than 15 calendar days after identification | Y |  |
| 61.349 | Standards: Closed vent systems and control devices | Y |  |
| 61.349(a) | Closed vent system and control device | Y |  |
| 61.349(a)(1) | Closed vent system requirements | Y |  |
| 61.349(a)(2)(ii) | Vapor recovery system (e.g. carbon adsorption system) recovers or controls organic emissions with an efficiency of 95% by weight | Y |  |
| 61.349(b) | Operated at all times except maintenance or repair. | Y |  |
| 61.349(f) | Visible inspection and repair | Y |  |
| 61.349(g) | Visually inspected quarterly | Y |  |
| 61.349(h) | Monitor control device in accordance with 61.354(c) | Y |  |
| 61.350 | Delay of Repair | Y |  |
| 61.354 | Monitoring of operations | Y |  |
| 61.354(c) | Monitoring of operations for control devices | Y |  |
| 61.354(c)(8) | Monitoring for carbon adsorption | Y |  |
| 61.354(d) | Replace carbon immediately when carbon breakthrough is indicated. | Y |  |
| 61.355 | Test methods, procedures and compliance provisions | Y |  |
| 61.356 | Recordkeeping requirements | Y |  |
| 61.356(d) | recordkeeping and retention requirements | Y |  |
| 61.356(h) | waste stream records | Y |  |
| 61.357 | Reporting requirements | Y |  |
| 61.357(d) | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste | Y |  |
| - 61.357(d)(8) | Reporting requirements | Y |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 | Wastewater provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR part 61, subpart FF for each stream that meets the definition of 63.641. | Y |  |
| 63.654 | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR part 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i) | Recordkeeping | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |
| Condition # 5077 |  |  |  |
| Part 1 | Design Rated Capacity Limit [basis: Cumulative Increase] | Y |  |
| Part 3 | Reporting of Emission Leaks [basis: Regulation 8-8-302.1] | Y |  |

| Table IV – AW Source-specific Applicable Requirements S1470 – LOG LPG Loading Flare |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 40 CFR Part 60 Subpart A | General Provisions |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.7(a) | Notification | Y |  |
| 60.7(g) | Alternative notification | Y |  |
| 60.7(h) | Specific provisions | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.13(i) | Alternatives to any monitoring procedures or requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part 74 | Vapors from S4338 shall be collected and controlled by S1470, LOG LPG Loading Flare, with an overall capture and destruction of 98.5% by weight. [basis: BACT] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 12 | Flarethroughput limit (basis: Regulations 8-1-110.3; 2-1-403) | Y | 06/01/04 |
| Part 13 | Flaring recordkeeping requirements (basis: Regulations 8-1-110.3; 2-6-409.2; 2-6-501) | Y | 06/01/04 |
| Part 14 | Inspection of flaring requirements (basis: Regulation 2-6-4092.) | Y | 06/01/04 |
| Part 15 | Inspection procedures (basis: Regulation 6-301; 2-1-403) | Y | 06/01/04 |
| Part 16 | Visual inspection requirements (basis: Regulation 2-6-403) | Y | 06/01/04 |
| Part 17 | Recordkeeping requirements for flaring events (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |

| Table IV – AXa Source-specific Applicable Requirements A101 – Flare for Vine Hill Vapor Recovery System A102 – Flare for Crude String Vapor Recovery System A103 – Flare for Interim Gasoline Vapor Recovery System |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Exempt from fuel gas H2S limit | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 19 | Upset gas limitation (basis: 60.104(a)(1); Regulation 2-1-403) | Y | 06/01/04 |
| BAAQMD Condition # 20747 |  |  |  |
| Part 1 | Exemption for Process Upset Gases | Y |  |
| Part 2 | Recordkeeping (basis: Regulation 2-6-401) | Y |  |

| Table IV – AXb Source-specific Applicable Requirements S1471 - LOP Auxiliary Flare S1472 - LOP Main Flare |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| BAAQMD Regulation 12-11 | Flare Monitoring at Petroleum Refineries (06/04/03) |  |  |
| 12-11-401 | Flare Data Reporting Requirements | N |  |
| 12-11-402 | Flow Verification Report | N | 6/4/04 |
| 12-11-501 | Vent Gas Flow Monitoring | N | 12/4/04 |
| 12-11-502 | Vent Gas Composition Monitoring | N |  |
| 12-11-502.3 | Vent Gas Composition Monitoring | N | 03/4/04 |
| 12-11-503 | Pilot Monitoring | N |  |
| 12-11-504 | Pilot and Purge Gas Monitoring | N |  |
| 12-11-505 | Recordkeeping Requirements | N |  |
| 12-11-506 | General Monitoring Requirements | N |  |
| 12-11-506.1 | Periods of Inoperation of Vent Gas Monitoring | N | 09/4/04 |
| 12-11-507 | Video Monitoring | N | 12/4/03 |
| BAAQMD Condition # 18618 |  |  |  |
| Part 12 | Flarethroughput limit (basis: Regulations 8-1-110.3; 2-1-403) | Y | 06/01/04 |
| Part 13 | Flaring recordkeeping requirements (basis: Regulations 8-1-110.3; 2-6-409.2; 2-6-501) | Y | 06/01/04 |
| Part 14 | Inspection of flaring requirements (basis: Regulation 2-6-4092.) | Y | 06/01/04 |
| Part 15 | Inspection procedures (basis: Regulation 6-301; 2-1-403) | Y | 06/01/04 |
| Part 16 | Visual inspection requirements (basis: Regulation 2-6-403) | Y | 06/01/04 |
| Part 17 | Recordkeeping requirements for flaring events (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |
| Part 18 | Flexigas flare requirements (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |
| Part 19 | Upset gas limitation (basis: 60.104(a)(1); Regulation 2-1-403) | Y | 06/01/04 |

| Table IV - AY Source-specific Applicable Requirements S1476 – LUBS F-24 Atmosphric Feed S1477 – LUBS F-25 Vacuum Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (05/02/2001) |  |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emission Monitoring If Required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-301.3 | ...Test-firing on Non-gaseous fuel Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| 40 CFR Part 60 Subpart A | General Provisions |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | Monitoring requirement for H2S (dry basis) in fuel gas prior to Combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Fuel usage limit resulting from non-compliance [basis: Regulation 2-2-302] |  |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions | Y |  |
| Part E.1 | Sulfur limit of liquid fuel [Regulation 9-1-304] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502 | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 6 | Visible emission inspection during tube cleaning (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - AZ Source-specific Applicable Requirements S1478 - LUBS F-26 Furfural Raff, S1479 - LUBS F-27 Furfural Extr, S1480 - LUBS F-69 Asphalt Circulation, S1481 - OPCEN F-30 DSU, S1483 - LUBS F-32 Asphalt Circulation, S1484 – LUBS F-34 LHT Charge, S1506 - CP F-61 CGP Feed, S1760 - OPCEN F-102 FXU Steam Superheater |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (05/02/2001) |  |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures |  |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| 40 CFR Part 60 Subpart A | General Provisions |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |

| Table IV - AZa Source-specific Applicable Requirements S1480 - LUBS F-69 Asphalt Circulation, S1484 – LUBS F-34 LHT Charge, S1506 - CP F-61 CGP Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| 40 CFR Part 60 Subpart A | General Provisions |  |  |
| 60.13(i) | Alternatives to any monitoring procedures or requirements | Y |  |

| Table IV - BA Source-specific Applicable Requirements S1486 - DH F-40 CU Feed, S1487 - DH F-41B VFU Feed, S1488 – DH F-41A VFU Feed, S1491 – DH F-44 NHT Feed, S1492 – DH F-45 Primary Column Reboil, S1493 – DH F-46 Stabilizer Reboil, S1495 – DH F-49 CRU Preheat, S1496 – DH F-50 CRU, S1497 – DH F-51 CRU, S1498 – DH F-52 CRU Reboil S1508 – CP F-63 CFH Feed, S1510 - CP F-66 CCU Preheat, S1511 - CP F-67 CCU LGO Reboil |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) |  |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emission Monitoring If Required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-301.3 | ...Test-firing on Non-gaseous fuel Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions [basis: Regulation 2-2-302] | Y |  |
| Part E.1 | Sulfur limit of liquid fuel [Regulation 9-1-304] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 6 | Visible emission inspection during tube cleaning (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - BB Source-specific Applicable Requirements S1486 – DH F-40 CU Feed S1487 - DH F-41B VFU Feed S1488 – DH F-41A VFU Feed S1495 – DH F-49 CRU Preheat S1496 – DH F-50 CRU S1497 – DH F-51 CRU S1508 - CP F-63 CFH Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BA for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |

| Table IV - BC Source-specific Applicable Requirements S1490 – DH F-43 GOHT Feed S1499 – DH F-53 CRU Regen S1762 – DH F-128 CRU Interheater |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |

| Table IV - BD Source-specific Applicable Requirements S1494 – DH F-47 Secondary Column Reboil S1502 – DH F-57 HCU First Stage Feed S1503 – DH F-58 HCU Second Stage Feed S1505 - DH F-60 HP1 Steam Methane Reformer S1515 – DH F-71 HCU First Stage Reboil S1761 - OPCEN F-104 HP2 Steam Methane Reformer S4031 – DC F-14012 HGHT Reboil S4141 – DC F-14011 HGHT Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) |  |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |

| Table IV - BE Source-specific Applicable Requirements S1494 – DH F-47 Secondary Column Reboil S1502 – DH F-57 HCU First Stage Feed S1503 – DH F-58 HCU Second Stage Feed S1505 - DH F-60 HP1 Steam Methane Reformer S1515 – DH F-71 HCU First Stage Reboil S1761 - OPCEN F-104 HP2 Steam Methane Reformer |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BD for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV - BF Source-specific Applicable Requirements S1494 – DH F-47 Secondary Column Reboil |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV –BD for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |

| Table IV – BG Source-specific Applicable Requirements S1500 DH F-55 SGP Heat Medium S1504 - DH F-59 HCU Second Stage Reboil S1763 - DH F-126 CU Feed S4002 – DC F-13425-A DCU S4003 – DC F13425-B DCU |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emission Monitoring If Required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-301.3 | ...Test-firing on Non-gaseous fuel Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 2 | Recordkeeping (basis: Regulation 2-1-234.3) | N |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 6 | Visible emission inspection during tube cleaning (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - BH Source-specific Applicable Requirements S1500 DH F-55 SGP Heat Medium S1504 - DH F-59 HCU Second Stage Reboil S1763 - DH F-126 CU Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BG for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions | Y |  |
| Part E.1 | Sulfur limit of liquid fuel [Regulation 9-1-304] | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV - BI Source-specific Applicable Requirements S1504 - DH F-59 HCU Second Stage Reboil S1763 - DH F-126 CU Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV –BG & BH for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |

| Table IV - BJ Source-specific Applicable Requirements S1505 - DH F-60 HP1 Steam Methane Reformer S1515 – DH F-71 HCU First Stage Reboil S1761 - OPCEN F-104 HP2 Steam Methane Reformer |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV –BD & BE for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |

| Table IV - BK Source-specific Applicable Requirements S1507 - UTIL CO Boiler .1 S1509 - UTIL CO Boiler 2 S1512 - UTIL CO Boiler 3 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emission Monitoring If Required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-303.1 | Federal Interim Facility-wide NOx emission limit for CO Boilers | Y |  |
| 9-10-304 | NOx emission limit for CO Boilers | N |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions | Y |  |
| Part E.1 | Sulfur limit of liquid fuel [Regulation 9-1-304] | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 85 | CO Boilers NOx limit [basis: Offsets] | Y |  |
| Part 86 | NOx CEM [basis: Offsets] | Y |  |
| Part 87 | 365-day average NOx emission [basis: Offsets] | Y |  |
| Part 88 | Reporting requirements [basis: Offsets] | Y |  |
| Part 90 | CO Boilers SO2 limit [basis: Offsets] | Y |  |
| Part 91 | SO2 CEM [basis: Offsets] | Y |  |
| Part 92 | 365-day average SO2 emission [basis: Offsets] | Y |  |
| Part 93 | Reporting requirements [basis: Offsets] | Y |  |
| BAAQMD Condition # 17533 |  |  |  |
| Part 2 | Ammonia emission limit [basis: Toxics] | N |  |
| Part 3 | If 3 separate violations of Part 2 [basis: Toxics] | N |  |
| Part 5 | Automatic controls of urea injection system [basis: Offsets] | Y |  |
| Part 8 | Annual source test of ammonia [basis: Toxics] | N |  |
| Part 9 | Daily records [basis: Offsets] | Y |  |
| Part 10 | Temperature indicator and recorder [basis: Offsets] | Y |  |
| Part 11 | Curtailment if excess opacity [basis: Regulation 6-301] | Y |  |
| Part 12 | Additional source test results availability [basis: Offsets] | Y |  |
| Part 13 | NOx and O2 CEM and recording system [basis: Offsets] | Y |  |
| Part 14 | Field accuracy test of CEM [basis: Offsets] | Y |  |
| Part 15 | If CEM inoperable [basis: Offsets] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 9 | Source Test for grain loading rate (basis: Regulation 2-6-409.2) | Y |  |

| Table IV – BL Source-specific Applicable Requirements S1514 - UTIL F-70 Boiler 4 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| 6-501 | Sampling Facilities and Instruments Required | Y |  |
| 6-502 | Data, Records and Reporting | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-301.3 | ...Test-firing on Non-gaseous fuel Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) |  |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions | Y |  |
| Part E.1 | Sulfur limit of liquid fuel [Regulation 9-1-304] | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 17532 |  |  |  |
| Part 1 | Gaseous fuel only. [basis: Regulation 1-520.1] | Y |  |
| Part 3 | Emissions limit [basis: Shell-EPA Consent Decree] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 6 | Visible emission inspection during tube cleaning (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - BM Source-specific Applicable Requirements S1523 – LUBS Loading Rack Asphalt Inside T/T |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | - See Table IV – Hb for additional requirements. |  |  |
| - BAAQMD - Condition # - 4101 |  |  |  |
| - Part 1 | - Abatement Requirement [basis: Cumulative Increase] | - Y |  |

| Table IV - BN Source-specific Applicable Requirements S1540 – LUBS LR-26 Oleum Unloading |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 12, Rule 10 | Oleum Transfer Operations |  |  |
| 12-10-301 | Operating Requirements | N |  |
| 12-10-302 | Secondary Container Requirements | N |  |
| 12-10-401 | Oleum Transfer Procedure Requirements | N |  |
| 12-10-501 | Records | N |  |

| Table IV – BO Source-specific Applicable Requirements S1598 – MAINT Gasoline Dispensing Facility |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 7 | Organic Compounds - Gasoline Dispensing Facilities (3/24/03) |  |  |
| 8-7-113 | Tank Gauging and Inspection Exemption | Y |  |
| 8-7-301 | Phase I Requirements | Y |  |
| 8-7-301.1 | Requirement for CARB Phase I System | Y |  |
| 8-7-301.2 | Installation of Phase I Equipment per CARB Requirements | Y |  |
| 8-7-301.3 | Submerged Fill Pipes | Y |  |
| 8-7-301.5 | Maintenance of Phase I Equipment per Manufacturers Guidelines or CARB Executive Order | Y |  |
| 8-7-301.6 | Leak-Free, Vapor-Tight | Y |  |
| 8-7-301.7 | Poppetted Drybreaks | Y |  |
| 8-7-301.8 | No Coaxial Phase 1 Systems on New and Modified Tanks | Y |  |
| 8-7-301.9 | CARB-Certified Anti-Rotational Coupler or Swivel Adapter | Y |  |
| 8-7-301.10 | System Vapor Recovery Rate | Y |  |
| 8-7-301.11 | CARB-Certified Spill Box | Y |  |
| 8-7-301.12 | Drain Valve Permanently Plugged | Y |  |
| 8-7-301.13 | Vapor Tightness and Testing | Y |  |
| 8-7-302 | Phase II Requirements | Y |  |
| 8-7-302.1 | Requirement for CARB Certified Phase II System | Y |  |
| 8-7-302.2 | Maintenance of Phase II System per CARB Requirements | Y |  |
| 8-7-302.3 | Maintenance of All Equipment as Specified by Manufacturer | Y |  |
| 8-7-302.4 | Repair of Defective Parts Within 7 Days | Y |  |
| 8-7-302.5 | Leak-Free, Vapor-Tight | Y |  |
| 8-7-302.6 | Insertion Interlocks | Y |  |
| 8-7-302.7 | Built-In Vapor Check Valve | Y |  |
| 8-7-302.8 | Minimum Liquid Removal Rate | Y |  |
| 8-7-302.9 | Coaxial Hose | Y |  |
| 8-7-302.10 | Galvanized Piping or Flexible Tubing | Y |  |
| 8-7-302.12 | Liquid Retainment Limit | Y |  |
| 8-7-302.13 | Spitting Limit | Y |  |
| 8-7-302.14 | Back Pressure for Vapor Balance |  |  |
| 8-7-303 | Topping Off | Y |  |
| 8-7-304 | Certification Requirements | Y |  |
| 8-7-306 | Prohibition of Use | Y |  |
| 8-7-307 | Posting of Operating Instructions | Y |  |
| 8-7-308 | Operating Practices | Y |  |
| 8-7-309 | Contingent Vapor Recovery Requirements | Y |  |
| 8-7-311 | Exempt Tank Requirements | Y |  |
| 8-7-313 | Requirements for New or Modified Phase II Installations | Y |  |
| 8-7-315 | Pressure Vacuum Valve Requirement, Underground Storage Tank | Y |  |
| 8-7-401 | Permit Requirements, New and Modified Installations | Y |  |
| 8-7-406 | Testing Requirements, New and Modified Installations | Y |  |
| 8-7-407 | Periodic Testing | Y |  |
| 8-7-408 | Test Notification | Y |  |
| 8-7-501 | Burden of Proof | Y |  |
| 8-7-502 | Right of Access | Y |  |
| 8-7-503 | Record Keeping Requirements | Y |  |
| 8-7-503.1 | Gasoline Dispensed Records | Y |  |
| 8-7-503.2 | Dispensing Facility Maintenance Records | Y |  |
| 8-7-503.3 | Dispensing Records Retention | Y |  |
| BAAQMD Condition # 14098 |  |  |  |
| Part 1 | Annual gasoline throughput limit [basis: Cumulative Increase, Toxics] | N |  |
| Part 2 | Recordkeeping [basis: Toxics, Cumulative Increase, Toxics] | N |  |

| Table IV - BP Source-specific Applicable Requirements S1650 – MAINT Sandblasting Sand Hopper |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |

| Table IV - BQ Source-specific Applicable Requirements S1759 – OPCENFlexicoker (FXU) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AL & AM for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.b | Flexicoker feed process limit [basis: Regulation 2-2-302] | Y |  |
| Part C.e | Sulfur content limit of liquid fuel [basis: Regulation 2-2-302] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV - BR Source-specific Applicable Requirements S1760 - OPCEN F-102 FXU Steam Superheater |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AZ for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |

| Table IV – BS Source-specific Applicable Requirements S1762 – DH F-128 CRU Interheater |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BC for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] |  |  |

| Table IV - BU Source-specific Applicable Requirements S1767 – OPCEN V 1019 Coke Silo, S1768– OPCEN V1020 Coke Silo S1769– OPCEN V 1021 Dry Fines Silo |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part D.3 | Abatement Requirement [basis: BACT, Cumulative Increase] | Y |  |

| Table IV - BW Source-specific Applicable Requirements S1771 - OPCEN Flexigas Flare |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| BAAQMD Regulation 12-11 | Flare Monitoring at Petroleum Refineries (06/04/03) |  |  |
| 12-11-114 | Limited Exemption, Total Hydrocarbon and Methane Composition Monitoring and Reporting | N |  |
| 12-11-401 | Flare Data Reporting Requirements | N |  |
| 12-11-401.1 | Total Volumetric Flow. | N |  |
| 12-11-401.4 | If molecular weight measured, report average molecular weight for each hour of the month. | N |  |
| 12-11-401.6 | Report any 24-hour period during which more than 1 million standard cubic feet of vent gas was flared. | N |  |
| 12-11-401.7 | Report flare monitoring downtimes. | N |  |
| 12-11-401.8 | Report archive of images recorded for month pursuant to Section 12-11-507. | N |  |
| 12-11-401.9 | For each day and for the month provide calculated methane, non-methane and sulfur dioxide emissions. A flare control efficiency of 93 percent shall be used for flexi-gas flares. | N |  |
| 12-11-402 | Flow Verification Report | N | 6/4/04 |
| 12-11-501 | Vent Gas Flow Monitoring | N | 12/4/04 |
| 12-11-502 | Vent Gas Composition Monitoring | N |  |
| 12-11-502.3 | Vent Gas Composition Monitoring | N | 03/4/04 |
| 12-11-503 | Pilot Monitoring | N |  |
| 12-11-504 | Pilot and Purge Gas Monitoring | N |  |
| 12-11-505 | Recordkeeping Requirements | N |  |
| 12-11-506 | General Monitoring Requirements | N |  |
| 12-11-506.1 | Periods of Inoperation of Vent Gas Monitoring | N | 09/4/04 |
| 12-11-507 | Video Monitoring | N | 12/4/03 |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Standard for SO2 | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | Monitoring requirement for H2S (dry basis) in fuel gas prior tocombustion (in lieu of separate combustion device exhaust SO2monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 12 | Flarethroughput limit (basis: Regulations 8-1-110.3; 2-1-403) | Y | 06/01/04 |
| Part 13 | Flaring recordkeeping requirements (basis: Regulations 8-1-110.3; 2-6-409.2; 2-6-501) | Y | 06/01/04 |
| Part 14 | Inspection of flaring requirements (basis: Regulation 2-6-4092.) | Y | 06/01/04 |
| Part 15 | Inspection procedures (basis: Regulation 6-301; 2-1-403) | Y | 06/01/04 |
| Part 16 | Visual inspection requirements (basis: Regulation 2-6-403) | Y | 06/01/04 |
| Part 17 | Recordkeeping requirements for flaring events (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |

| Table IV - BX Source-specific Applicable Requirements S1772 - OPCEN Hydrocarbon Flare |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| BAAQMD Regulation 12-11 | Flare Monitoring at Petroleum Refineries (06/04/03) |  |  |
| 12-11-401 | Flare Data Reporting Requirements | N |  |
| 12-11-402 | Flow Verification Report | N | 6/4/04 |
| 12-11-501 | Vent Gas Flow Monitoring | N | 12/4/04 |
| 12-11-502 | Vent Gas Composition Monitoring | N |  |
| 12-11-502.3 | Vent Gas Composition Monitoring | N | 03/4/04 |
| 12-11-503 | Pilot Monitoring | N |  |
| 12-11-504 | Pilot and Purge Gas Monitoring | N |  |
| 12-11-505 | Recordkeeping Requirements | N |  |
| 12-11-506 | General Monitoring Requirements | N |  |
| 12-11-506.1 | Periods of Inoperation of Vent Gas Monitoring | N | 09/4/04 |
| 12-11-507 | Video Monitoring | N | 12/4/03 |
| BAAQMD Condition # 18618 |  |  |  |
| Part 12 | Flarethroughput limit (basis: Regulations 8-1-110.3; 2-1-403) | Y | 06/01/04 |
| Part 13 | Flaring recordkeeping requirements (basis: Regulations 8-1-110.3; 2-6-409.2; 2-6-501) | Y | 06/01/04 |
| Part 14 | Inspection of flaring requirements (basis: Regulation 2-6-4092.) | Y | 06/01/04 |
| Part 15 | Inspection procedures (basis: Regulation 6-301; 2-1-403) | Y | 06/01/04 |
| Part 16 | Visual inspection requirements (basis: Regulation 2-6-403) | Y | 06/01/04 |
| Part 17 | Recordkeeping requirements for flaring events (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |

| Table IV – BY Source-specific Applicable Requirements S1779 - OPCEN CPI Oil/Water Separator |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AT for additional requirements. |  |  |
| Condition # 5077 |  |  |  |
| Part 7 | Design Rated Capacity Limit [basis: Cumulative Increase] | Y |  |
| Part 9 | Reporting of Emission Leaks [basis: Regulation 8-8-302.1] | Y |  |

| Table IV – BZ Source-specific Applicable Requirements S1800 - UTIL F-88 Boiler 5 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) |  |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.1 | Baseline profiles [basis: Regulation 2-2-302] | Y |  |
| Part B.3 | Emission factors used for combustion [basis: Regulation 2-2-302] | Y |  |
| Part B.6 | Modification of baseline profiles for particulate, NOx, SO2, and CO [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.d | Total fuel usage [basis: Regulation 2-2-302] | Y |  |
| Part E | Fuel Conditions | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |

| Table IV – CA Source-specific Applicable Requirements S1803– OPCEN Coke Corral |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Condition # 4041 |  |  |  |
| Part 1 | Material limitation [basis: Cumulative Increase] | Y |  |
| Part 2 | Vehicle tire and wheels washing [basis: Cumulative Increase, Public Nuisances] | Y |  |
| Part 3 | Visible particulate emission limit [basis: Cumulative Increase] | Y |  |
| Part 4 | Water spray requirement [basis: Cumulative Increase] | Y |  |
| Part 5 | Covering of stockpiles and/or containers [basis: Cumulative Increase,] | Y |  |
| Part 6 | Stockpile height limitation [basis: BACT] | Y |  |
| Part 8 | Storage of Petroleum Coke-Oil Mixture [basis: Cumulative Increase] | Y |  |
| Part 10 | Visible emissions monitoring [basis: Regulation 2-6-409.2] | Y |  |
| Part 11 | Recordkeeping [basis: Regulation 2-6-409.2] | Y |  |
| Part 12 | Sampling and analysis [basis: Regulation 2-6-409.2] | Y |  |

| Table IV – CB Source-specific Applicable Requirements S1804 – MAINT Paint Spray Booth and Facility Coating |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| BAAQMD Regulation 8, Rule 1 | Organic Compounds – General Provisions (11/4/98) |  |  |
| 8-1-320 | Storage and Disposal of Solvent Impregnated Cloth or Paper | Y |  |
| 8-1-321 | Closed Containers for Spent or Fresh Organic Solvents | Y |  |
| 8-1-322 | Spray Equipment Cleanup Limitation | Y |  |
| BAAQMD Regulation 8, Rule 4 | Organic Compounds – General Solvent and Surface Coating Operations (10/16/2002) |  |  |
| 8-4-302 | Solvents and Surface Coating Requirements | N |  |
| 8-4-312 | Solvent Evaporation Loss Minimization | N |  |
| 8-4-312.1 | Storage and Disposal of Solvent Impregnated Cloth or Paper | N |  |
| 8-4-312.2 | No Organic Compounds for Cleanup of Spray Equipment Unless Controls are Used | N |  |
| 8-4-312.3 | Closed Containers for Spent or Fresh Organic Solvents | N |  |
| 8-4-313 | Surface Preparation Standards | N |  |
| 8-4-501 | Recordkeeping | N |  |
| 8-4-501.1 | Maintain Data Necessary to Evaluate Compliance | Y |  |
| 8-4-501.2 | Annual Records of Coating Applied and Solvent Used | Y |  |
| 8-4-501.3 | Daily Recording of Key System Operating Parameters | Y |  |
| 8-4-501.4 | Monthly Usage Records | N |  |
| 8-4-501.5 | Records Retention | N |  |
| SIP Regulation 8, Rule 4 | Organic Compounds – General Solvent and Surface Coating Operations (12/20/95) |  |  |
| 8-4-302 | Limitation on Solvents and Surface Coatings | Y |  |
| 8-4-312 | Solvent Evaporation Loss Minimization | Y |  |
| 8-4-501 | Recordkeeping | Y |  |
| 8-4-501.4 | Records Retention | Y |  |
| BAAQMD Regulation 8, Rule 19 | Organic Compounds – Surface Coating of Miscellaneous Metal Parts and Products (10/16/2002) |  |  |
| 8-19-134 | Limited Exemption, Coating Records | Y |  |
| 8-19-302 | Limits | Y |  |
| 8-19-307 | Prohibition of Specification | N |  |
| 8-19-312 | Specialty Coating Limitations | Y |  |
| 8-19-313 | Spray Application Equipment Limitations | Y |  |
| 8-19-320 | Solvent Evaporative Loss Minimization | N |  |
| 8-19-321 | Surface Preparation Solvent < 0.42 lb/gal | N |  |
| 8-19-501 | Records | N |  |
| SIP Regulation 8, Rule 19 | Organic Compounds – Surface Coating of Miscellaneous Metal Parts and Products (12/20/95) |  |  |
| 8-19-307 | Prohibition of Specification | Y |  |
| 8-19-320 | Solvent Evaporative Loss Minimization | Y |  |
| 8-19-501 | Records | Y |  |
| BAAQMD Regulation 8, Rule 31 | Organic Compounds – Surface Coating of Plastic Parts and Products (10/16/2002) |  |  |
| 8-31-123 | Exemption, Small User | Y |  |
| 8-31-124 | Limited Exemption, Coating Records | Y |  |
| 8-31-302 | Limits | Y |  |
| 8-31-306 | Flexible Coatings | Y |  |
| 8-31-307 | Prohibition of Specification | N |  |
| 8-31-309 | Specialty Coating Limitations | Y |  |
| 8-31-310 | Spray Application Equipment Limitations | Y |  |
| 8-31-320 | Solvent Evaporative Loss Minimization | N |  |
| 8-31-321 | Surface Preparation Solvent < 0.42 lb/gal | N |  |
| 8-31-501 | Records | N |  |
| SIP Regulation 8, Rule 31 | Organic Compounds – Surface Coating of Plastic Parts and Products (12/20/95) |  |  |
| 8-31-307 | Prohibition of Specification | Y |  |
| 8-31-320 | Solvent Evaporative Loss Minimization | Y |  |
| 8-31-501 | Records | Y |  |
| BAAQMD Regulation 8, Rule 32 | Organic Compounds – Wood Products Coating (06/19/96) |  |  |
| 8-32-111 | Exemption, Small Coating Operations | Y |  |
| BAAQMD Regulation 8, Rule 49 | Organic Compounds – Aerosol Paint Products (12/20/95) |  |  |
| 8-49-301 | Limits | N |  |
| 8-49-302 | Prohibitation of Non-Intended Use | Y |  |
| 8-49-303 | Multi-Component Applications | N |  |
| SIP Regulation 8, Rule 49 | Organic Compounds – Aerosol Paint Products (03/22/95) |  |  |
| 8-49-301 | Limits | Y |  |
| 8-49-303 | Multi-Component Applications | Y |  |
| BAAQMD Regulation 8, Rule 51 | Organic Compounds – Adhesive and Sealant Products (7/17/02) |  |  |
| 8-51-114 | Exemption, Low Usage | Y |  |
| 8-51-115 | Exemption, Low VOC Adhesive or Sealant Products | Y |  |
| 8-51-125 | Limited Exemption, Low Usage of Contact Bond Adhesive | Y |  |
| 8-51-126 | Limited Exemption, Facilities Using contact Bond Adhesive Primaily for Special Substrates | Y |  |
| 8-51-306 | Prohibition of Specification | Y |  |
| 8-51-320 | Solvent Evaporative Loss Minimization | Y |  |
| 8-51-504 | Burden of Proof | Y |  |
| BAAQMD Condition # 4303 |  |  |  |
| Part 1 | Annual cleanup solvent usage limit [basis: Cumulative Increase] | Y |  |
| Part 2 | Annual coating usage limit. [basis: Cumulative Increase] | Y |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |
| Part 4 | Operational requirements [basis: Cumulative Increase, Regulation 6-301] | Y |  |

| Table IV – CC Source-specific Applicable Requirements S1805 - Tank 12038 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AE for additional requirements. |  |  |
| BAAQMD Condition # 4298 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2 | Vented to A1805 [basis: 40CFR61, Section 61.343(a) and (b)] | Y |  |
| Part 3 | Monitoring of A1805 [basis: 40CFR61, Section 61.343(a) and (b)] | Y |  |
| Part 4 | Recordkeeping of Monitoring for A1805 [basis: 40CFR61, Section 61.343(a) and (b)] | Y |  |
| Part 5 | Changeout of primary carbon vessel [basis: 40CFR61, Section 61.343(a) and (b) & Equilon Consent Decree, paragraphs 55 & 59] | Y |  |
| Part 6 | Changeout of secondary carbon vessel [basis: 40CFR61, Section 61.343(a) and (b) & Equilon Consent Decree, paragraphs 55 & 59] | Y |  |
| Part 7 | Recordkeeping of changeout [basis: 40CFR61, Section 61.343(a)] | Y |  |
| Part 8 | Recordkeeping of throughput limit [basis: Cumulative Increase] | Y |  |

| Table IV – CD Source-specific Applicable Requirements S1900 – MAINT Machine Shop Parts Cleaner S1902 - MAINT Seal Room Parts Cleaner S1903 – MAINT Paint Shop Solvent Tub |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 1 | Organic Compounds – General Provisions (6/15/94) |  |  |
| 8-1-320 | Surface Preparation, Clean-up, Coating, Ink, Paint Removal | Y |  |
| 8-1-321 | Closed Containers for Spent or Fresh Organic Solvents | Y |  |
| 8-1-322 | Spray Equipment Cleanup Limitation | Y |  |
| BAAQMD Regulation 8, Rule 16 | Organic Compounds – Solvent Cleaning Operations (10/16/2002) |  |  |
| 8-16-303 | Cold Cleaner Requirements | N |  |
| 8-16-303.1 | General Operating Requirements | N |  |
| 8-16-303.1.1 | Maintain equipment in good working order. | Y |  |
| 8-16-303.1.2 | Leak Repair Requirement | Y |  |
| 8-16-303.1.3 | Solvent Storage or Disposal – Evaporation Prevention | N |  |
| 8-16-303.1.4 | Waste Solvent Disposal | N |  |
| 8-16-303.1.4(a) | Covered Containers for Waste Solvent Awaiting Pick-up | N |  |
| 8-16-303.1.4(b) | On-site Waste Treatment | N |  |
| 8-16-303.1.5 | Solvent Evaporation Minimization Devices shall not be Removed | N |  |
| 8-16-303.1.6 | Solvent Spray Requirements | N |  |
| 8-16-303.2 | Cold Cleaner Operating Requirements | Y |  |
| 8-16-303.2.1 | Solvent shall be Drained from Cleaned Parts | Y |  |
| 8-16-303.2.2 | Solvent Agitation | Y |  |
| 8-16-303.2.3 | Solvent Cleaning of Porous or Absorbent Materials is Prohibited | Y |  |
| 8-16-303.3 | Cold Cleaner General Equipment Requirements | N |  |
| 8-16-303.3.1 | Container | Y |  |
| 8-16-303.3.2 | Solvent Evaporation Reduction for Idle Equipment | N |  |
| 8-16-303.3.3 | Used Solvent Returned to Container | N |  |
| 8-16-303.3.4 | Label Stating Operating Requirements | N |  |
| 8-16-303.4 | Control Device (one of the following) | N |  |
| 8-16-303.4.1 | Freeboard Ratio > 0.75 | N |  |
| 8-16-303.5 | VOC content < 0.42 pounds per gallon or comply with 8-16-303.4.1 and other options | N |  |
| 8-16-501 | Solvent Records | N |  |
| 8-16-501.2 | Facility-wide Monthly Solvent Usage Records | N |  |
| 8-16-501.3 | Annual Records of Type and Amount of Solvent Used for Wipe Cleaning | N |  |
| 8-16-501.5 | Records Retained | N |  |
| SIP Regulation 8, Rule 16 | Organic Compounds – Solvent Cleaning Operations (12/9/94) |  |  |
| 8-16-303.1.4 | Waste Solvent Disposal | Y |  |
| 8-16-303.1.4(a) | Covered Containers for Waste Solvent Awaiting Pick-up | Y |  |
| 8-16-303.1.4(b) | On-site Waste Treatment | Y |  |
| 8-16-303.1.5 | Solvent Evaporation Minimization Devices shall not be Removed | Y |  |
| 8-16-303.1.6 | Solvent Spray Requirements | Y |  |
| 8-16-303.3 | Cold Cleaner General Equipment Requirements | Y |  |
| 8-16-303.3.2 | Solvent Evaporation Reduction for Idle Equipment | Y |  |
| 8-16-303.3.3 | Used Solvent Returned to Container | Y |  |
| 8-16-303.3.4 | Label Stating Operating Requirements | Y |  |
| 8-16-303.4 | Control Device (one of the following) | Y |  |
| 8-16-303.4.1 | Freeboard Ratio > 0.75 | Y |  |
| 8-16-501 | Solvent Records | Y |  |
| 8-16-501.2 | Facility-wide Quarterly Solvent Usage Records | Y |  |

| Table IV - CE Source-specific Applicable Requirements S2000 – OPCEN Corrosion Inhibitor Injection |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| Condition # 4364 |  |  |  |
| Part 2 | Corrosion Inhibitor Usage Limit [basis: Cumulative Increase] | Y |  |
| Part 4 | Records [basis: Cumulative Increase] | Y |  |

| - Table IV – CF - Source-specific Applicable Requirements - S2001, 2002, 2003 and 2004 - - LOG Marine Loading Berths 1, 2, 3 and 4 |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Future Effective - Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (05/02/2001) |  |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| - BAAQMD Regulation 8, Rule 44 | - Organic Compounds-Marine Vessel Loading Terminals (1/4/89) | - Y |  |
| - 8-44-301 | - Marine Terminal Loading Limit | - Y |  |
| - 8-44-301.1 | - Limited to 5.7 gram per cubic meter (2 lb per 1000 bbls) of organic liquid loaded, or | - Y |  |
| - 8-44-301.2 | - POC emissions reduced 95% by weight from uncontrolled conditions | - Y |  |
| - 8-44-302 | - Emission control equipment | - Y |  |
| - 8-44-303 | - Operating practice | - Y |  |
| - 8-44-304 | - Equipment Maintenance | - Y |  |
| - 8-44-304.1 | - Certified leak free, gas tight and in good working order | - Y |  |
| - 8-44-304.2 | - Loading ceases any time gas or liquid leaks are discovered | - Y |  |
| - 8-44-402 | - Safety/Emergency Operations | - Y |  |
| - 8-44-402.1 | - Rule does not require act/omission in violation of Coast Guard/other rules | - Y |  |
| - 8-44-402.2 | - Rule does not prevent act/omission for vessel safety or saving life at sea | - Y |  |
| - 8-44-501 | - Record keeping | - Y |  |
| - 8-44-501.1 | - Name and location | - Y |  |
| - 8-44-501.2 | - Responsible company | - Y |  |
| - 8-44-501.3 | - Dates and times | - Y |  |
| - 8-44-501.4 | - Name, registry of the vessel loaded and legal owner | - Y |  |
| - 8-44-501.5 | - Prior cargo carried | - Y |  |
| - 8-44-501.6 | - Type, amount of liquid cargo loaded | - Y |  |
| - 8-44-501.7 | - Condition of tanks | - Y |  |
| - 8-44-502 | - Burden of proof | - Y |  |
| - NESHAPS Part 63 Subpart Y | - National Emission Standards for Marine Tank Vessel Loading Operations | - Y |  |
| - 63.560(a)(3) | - Record keeping in 63.567(j)(4) and emission estimation in 63.565(l) apply to existing sources < 10 and 25 tons | - Y |  |
| - 63.565(l) | - Emission estimation procedures | - Y |  |
| - 63.567(j)(4) | - Retain records of emission estimates per 63.565(l), and actual throughputs, by commodity, for 5 years | - Y |  |
| - BAAQMD - Condition # - 4288 |  |  |  |
| - Part 3a | - Install pressure and temperature monitors and recorders [basis: Regulation 8-44-301] | - Y |  |
| - Part 3b | - CAP adjustment [basis: BARCT-Adjustment] | - Y |  |
| - Part 3c | - Tracking wharf emissions [basis: Regulation 8-44-301] | - Y |  |
| - Part 5 | - Operation of Marine Vapor Recovery System [basis: Regulation 8-44-301] | - Y |  |
| - Part 6 | - Minimum temperature requirement [basis: Regulation 8-44-301] | - Y |  |
| - Part 9 | - Leak test requirement [basis: Regulation 8-44-304] | - Y |  |
| - Part 10 | - Loading pressure requirement [basis: Regulation 8-44-303] | - Y |  |
| - Part 11 | - Maintenance records [basis: Regulation 8-44-501] | - Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part A | Emissions limitations [basis: Regulation 2-2-302] | Y |  |
| Part B | Emissions profiles and emission factors [basis: Regulation 2-2-302] | Y |  |
| Part B.2 | Emission factors for marine activity [basis: Regulation 2-2-302] | Y |  |
| Part B.7 | Modification of baseline profile [basis: Regulation 2-2-302] | Y |  |
| Part C | Actions resulting from non-compliance [basis: Regulation 2-2-302] | Y |  |
| Part C.f | Large cargo vessel limitation [basis: Regulation 2-2-302] | Y |  |
| Part F | Reporting [basis: Cumulative Increase] | Y |  |
| Part G | Records [basis: Cumulative Increase] | Y |  |

| Table IV – CG Source-specific Applicable Requirements S2007 - LOG Dissolved Nitrogen Floatation Unit North ETP 1 (DNF), S2008 - LOG Dissolved Nitrogen Floatation Unit South ETP 1 (DNF), S5115 – LOG Dissolved Nitrogen Flotation Unit North ETP 2 (DNF), S5116 – LOG Dissolved Nitrogen Flotation Unit South ETP 2 (DNF) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 8 | Wastewater (Oil-Water) Separator (08/29/94) |  |  |
| 8-8-303 | Gauging and Sampling Devices | Y |  |
| 8-8-307 | Air Flotation Unit: any air flotation unit and/or pre-air flotation unit flocculation sump, basin, chamber or tank with a maximum allowable capacity greater than 400 gals/min unless is equipped with one of the following: | Y |  |
| 8-8-307.1 | a solid, gasketed, fixed cover totally enclosing the vessel liquid contents, with all cover openings closed, except for inspection, maintenance, or wastewater sampling. The cover may include an atmospheric vent or a pressure/vacuum valve. Also includes gap inspection frequency and limits. | Y |  |
| 8-8-501 | API Separator or Air Flotation Bypassed Wastewater Records | Y |  |
| 8-8-503 | Inspection and Repair Records | Y |  |
| District Regulation 11 Rule 12 | Hazardous Pollutants - National Emission Standards for Benzene Emissions from Benzene Transfer Operations and Benzene Waste Operations (7/18/90, Refer to NESHAP Subpart FF below) | Y |  |
| NESHAP 40 CFR 61 Subpart FF | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.343 | Standards: Tanks | Y |  |
| 61.343(a)(1) | Owner/operator shall install, operate, and maintain a fixed-roof and closed-vent system that routes all organic vapors vented from the tank to a control device. | Y |  |
| 61.343(a)(1)(i) | Fixed Roof leak requirements - < 500 ppmv above background, determined initially then annually by method in 61.355(h) of this subpart. | Y |  |
| 61.343(b) | For a tank that meets all of the conditions of paragraph (b)(1), the owner/operator may elect to comply with paragraph (b)(2) as an alternative to the requirements of (a)(1). | Y |  |
| 61.343(c) | Inspect all fixed-roof, seal, access doors, and all other openings visually initially and quarterly for cracks and gaps and that doors and openings are closed and gasketed properly. | Y |  |
| 61.343(d) | When a broken seal or gasket is identified, or when detectable emissions are measured, first efforts at repair shall be made as soon as practicable, but not later than 45 calendar days after identification. | Y |  |
| 61.349 | Standards: Closed vent systems and control devices | Y |  |
| 61.349(a) | Closed vent system and control device | Y |  |
| 61.349(a)(1) | Closed vent system requirements | Y |  |
| 61.349(a)(2)(ii) | Vapor recovery system (e.g. carbon adsorption system) recovers or controls organic emissions with an efficiency of 95% by weight | Y |  |
| 61.349(b) | Operated at all times except maintenance or repair. | Y |  |
| 61.349(f) | Visible inspection and repair | Y |  |
| 61.349(g) | Visually inspected quarterly | Y |  |
| 61.349(h) | Monitor control device in accordance with 61.354(c) | Y |  |
| 61.350 | Delay of Repair | Y |  |
| 61.354 | Monitoring of operations | Y |  |
| 61.354(c) | Monitoring of operations for control devices | Y |  |
| 61.354(c)(8) | Monitoring for carbon adsorption | Y |  |
| 61.354(d) | Replace carbon immediately when carbon breakthrough is indicated. | Y |  |
| 61.355 | Test methods, procedures and compliance provisions | Y |  |
| 61.355(h) | Leak inspection procedures | Y |  |
| 61.356 | Recordkeeping requirements | Y |  |
| 61.356(d) | recordkeeping and retention requirements | Y |  |
| 61.356(h) | waste stream records | Y |  |
| 61.357 | Reporting requirements | Y |  |
| 61.357(d) | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste | Y |  |
| - 61.357(d)(8) | Reporting requirements | Y |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 | Wastewater provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR part 61, subpart FF for each stream that meets the definition of 63.641. | Y |  |
| 63.654 | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR part 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i) | Recordkeeping | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| Table IV – CH Source-specific Applicable Requirements S2007 - LOG Dissolved Nitrogen Floatation Unit North ETP 1 (DNF), S2008 - LOG Dissolved Nitrogen Floatation Unit South ETP 1 (DNF) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – CG for additional requirements. |  |  |
| Condition # 5077 |  |  |  |
| Part 1 | Design Rated Capacity Limit [basis: Cumulative Increase] | Y |  |

| Table IV – CI Source-specific Applicable Requirements S2009 - LUBS Wastewater Separator Dubbs Box |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AT for additional requirements. |  |  |
| Condition # 5077 |  |  |  |
| Part 10 | Design Rated Capacity Limit [basis: Cumulative Increase] | Y |  |
| Part 12 | Reporting of Emission Leaks [basis: Regulation 8-8-302.1] | Y |  |

| Table IV – CJ Source-specific Applicable Requirements S2010 - LOG Wastewater Junction Boxes S2011 - LOG Wastewater Collection Sumps (4) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 8 | Wastewater (Oil-Water) Separator |  |  |
| 8-8-303 | Gauging and Sampling Devices | Y |  |
| 8-8-308 | Junction Box: equipped with either a solid, gasketed, fixed cover totally enclosing the junction box or a solid manhole cover. May include openings in the covers and vent pipes if the total open area of the junction box does not exceed 12.6 square inches and all vent pipes are at least 3 feet in length. | Y |  |

| Table IV – CK Source-specific Applicable Requirements S2012 – DH V-12378 Perchloroethylene Storage System |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV –B for additional requirements. |  |  |
| Condition # 6110 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2 | Material storage limitation [basis: Cumulative Increase; Toxics] | N |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] |  |  |

| Table IV – CL Source-specific Applicable Requirements S2013 - Tank 12467 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – U & V & W for additional requirements. |  |  |
| BAAQMD Condition # 6503 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2 | Material storage limit [basis: Cumulative Increase] | N |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |
| Part 5 | Specification requirements [basis: Cumulative Increase, BACT/TBACT] | Y |  |

| Table IV – CN Source-specific Applicable Requirements S2445 - Tank 12445, S2446 - Tank 12446 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – R & T & W for additional requirements. |  |  |
| BAAQMD Condition # 6707 |  |  |  |
| Part 1 | Annual throughput limit [basis: Cumulative Increase] | Y |  |
| Part 2a | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2bi | Material storage limit [basis: Cumulative Increase] | Y |  |
| Part 2bii | Material storage limit [basis: Cumulative Increase, Toxics] | N |  |
| Part 3 | Equipment specifications [basis: BACT/TBACT; Cumulative Increase] | Y |  |
| Part 4 | Recordkeeping [basis: Cumulative Increase] | Y |  |
| Part 5 | Installation of primary seal [basis: BACT/TBACT] | Y |  |

| Table IV - CO Source-specific Applicable Requirements S3000 – Portable Vacuum Distillation Unit (CCR Technologies Inc.) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (12/19/90) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Condition # 11806 |  |  |  |
| Part 1 | Operation limitation [basis: Cumulative Increase] |  |  |
| Part 2 | Hard-pipe connections [basis: Cumulative Increase] | Y |  |
| Part 3 | Non-condensable gases from condenser vent combusted in heater burner [basis: Cumulative Increase]] | Y |  |
| Part 4 | Only lean amine solution distilled [basis: Cumulative Increase] | Y |  |
| Part 5 | Neutralize acid gases and react residual H2S [basis: Cumulative Increase] | Y |  |
| Part 6 | Natural gas fired only [basis: Cumulative Increase] | Y |  |
| Part 7 | Recordkeeping [basis: Cumulative Increase] | Y |  |

| Table IV - CP Source-specific Applicable Requirements S4002 – DC F-13425-A DCU S4003 – DC F-13425-B DCU |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BG for additional requirements. |  |  |
| 40 CFR 60 Subpart A | General Provisions (7/1/2000) |  |  |
| 60.13(i) | Alternatives to any monitoring procedures or requirement | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 35 | Furnaces with SCR NOx emission limit [basis: BACT] | Y |  |
| Part 36 | Furnaces with SCR CO emission limit [basis: BACT] | Y |  |
| Part 37 | Furnaces with SCR Ammonia emission limit [basis: BACT] | N |  |
| Part 38 | Furnaces with SCR CEM recorder for NOx and O2 [basis: BACT] | Y |  |
| Part 105 | Source Test for CO emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 111 | Source Test for NH3 emissions [basis: Regulation 2-6-409.2] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |

| Table IV - CQ Source-specific Applicable Requirements S4005 – DC Coke Handling Facility |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 75 | Visible emissions limit [basis: BACT] | Y |  |
| Part 76 | Washing of Pad areas [basis: BACT] | Y |  |
| Part 77 | Water spray requirement [basis: BACT] | Y |  |
| Part 78 | Enclosed conveying system [basis: BACT] | Y |  |
| Part 79 | Water spray and abatement requirements of Coke Corral [basis: BACT] | Y |  |
| Part 80 | Enclosing and washing of Coke Corral [basis: BACT] | Y |  |
| Part 81 | Enclosed Loading Hopper and abatement requirements [basis: BACT] | Y |  |
| Part 82a | Wind shield to shroud discharge port [basis: BACT] | Y |  |
| Part 82b | Water Wash System [basis: BACT] | Y |  |
| Part 82c | Street sweeper [basis: BACT] | Y |  |
| Part 82d | Unpaved area limitation [basis: BACT] | Y |  |
| Part 83 | Loadout limit [basis: Offsets, Cumulative Increase] | Y |  |
| Part 84 | Collect water and handpipe to tanks [basis: BACT] | Y |  |

| Table IV – CR Source-specific Applicable Requirements S4001 – DC Delayed Coking Unit (DCU) S4020 – DC Distillate Hydrotreater (DHT) S4050 - DC Catalytic Gas Depentanizer (CGDP) S4080 - DC isomerization Unit (ISOM) S4140 – DC Heavy Cracked Gasoline Hydrotreater (HGHT) S4160 – DC Hydrogen Plant –3 (HP3) S4170 – LUBS Lube Hydrotreater –2 (LHT2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
|  |  | See Table IV – AL & AM for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |  |
| Part A |  | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B |  | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C |  | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L |  | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M |  | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part N |  | Process vessel depressurization requirements [basis: BACT] | Y |  |
| Part O |  | Offsets requirements [basis: Offsets] | Y |  |
| Part 1 |  | Fugitive requirements for new pumps in light liquid or toxic service [basis: BACT] | Y |  |
| Part 3 |  | Definitions [basis: BACT] | Y |  |
| Part 4 |  | Fugitive requirements for valves in gaseous hydrocarbon service [basis: BACT] | Y |  |
| Part 5 |  | Fugitive requirements for other valves in POC gas, light liquid and toxic service [basis: BACT] | Y |  |
| Part 6 |  | Fugitive requirements for flow control valves [basis: BACT] | Y |  |
| Part 9 |  | Fugitive requirements for flanges [basis: BACT] | Y |  |
| Part 10 |  | Fugitive requirements for centrifugal compressors [basis: BACT] | Y |  |
| Part 11 |  | Total fugitive emissions limit [basis: Offsets] | Y |  |
| Part 12 |  | Fugitive requirements for pressure relief valves [basis: BACT] | Y |  |
| Part 13 |  | Fugitive requirements for process sample systems [basis: BACT] | Y |  |
| Part 14 |  | Fugitive requirements for process drains [basis: BACT] | Y |  |

| Table IV - CS Source-specific Applicable Requirements S4021 – DC F-13909 DHT Recycle S4171 – LUBS F-13000 LHT2 Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 40 | Natural draft furnaces with low NOx burners NOx emission limit [basis: BACT] | Y |  |
| Part 41 | Forced draft furnaces with low NOx burners NOx emission limit [basis: BACT] | Y |  |
| Part 42 | Furnaces with low NOx burner CO emission limit [basis: BACT] | Y |  |
| Part 43 | Low NOx burner requirement [basis: BACT] | Y |  |
| Part 104 | Source Test for NOx emission [basis: Regulation 2-6-409.2] | Y |  |
| Part 106 | Source Test for CO emission [basis: Regulation 2-6-409.2] | Y |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |

| Table IV - CSa Source-specific Applicable Requirements S4021 – DC F-13909 DHT Recycle S4171 – LUBS F-13000 LHT2 Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.13(i) | Alternatives to any monitoring procedures or requirements | Y |  |

| Table IV - CT Source-specific Applicable Requirements S4031 – DC F-14012 HGHT Reboil S4141 – DC F-14011 HGHT Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BD for additional requirements. |  |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 35 | Furnaces with SCR NOx emission limit [basis: BACT] | Y |  |
| Part 36 | Furnaces with SCR CO emission limit [basis: BACT] | Y |  |
| Part 37 | Furnaces with SCR Ammonia emission limit [basis: BACT] | N |  |
| Part 38 | Furnaces with SCR CEM recorder for NOx and O2 [basis: BACT] | Y |  |
| Part 105 | Source Test for CO emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 111 | Source Test for NH3 emissions [basis: Regulation 2-6-409.2] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |

| Table IV - CTa Source-specific Applicable Requirements S4141 – DC F-14011 HGHT Feed |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – BD & CT for additional requirements. |  |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) |  |  |
| 60.13(i) | Alternatives to any monitoring procedures or requirements | Y |  |

| Table IV – CU Source-specific Applicable Requirements S4161 – DC H-101 HP3 Steam Methane Reformer |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 2, Rule 9 | Interchangeable Emission Reduction Credits (04/07/99) |  |  |
| 2-9-302 | Use of IERC’s | N |  |
| 2-9-303 | Alternative Compliance Plan Using IERC’s | N |  |
| 2-9-304 | Restrictions on the Use of IERC’s | N |  |
| 2-9-306 | Environmental Benefit Surcharge | N |  |
| 2-9-501 | Monitoring and Record Keeping | N |  |
| 2-9-502 | Alternative Compliance Plan Record Keeping and Reporting | N |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| BAAQMD Regulation 9, Rule 3 | Inorganic Gaseous Pollutants - Nitrogen Oxides from Heat Transfer Operations (3/17/82) |  |  |
| 9-3-303 | New or Modified Heat Transfer Operation Limits | N |  |
| BAAQMD Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (7/17/02) |  |  |
| 9-10-301 | Emission Limit for Facility, NOx: 0.033 lb NOx/MMBTU | N |  |
| 9-10-301.1 | ...Start-up/Shutdown Contribution | N |  |
| 9-10-301.2 | ...Out-of-Service Units Contribution | N |  |
| 9-10-303 | Federal Interim Facility-wide NOx emission rate limit | Y |  |
| 9-10-305 | CO emission limit | N |  |
| 9-10-502 | Monitoring | N |  |
| 9-10-502.1 | CEMS for NOx, CO, and O2 | N |  |
| 9-10-502.2 | Fuel flowmeters | N |  |
| 9-10-504 | Recordkeeping | N |  |
| 9-10-504.1 | For All Sources Subject to 9-10-301, 303, 305 | N |  |
| 9-10-505 | Reporting | N |  |
| SIP Regulation 9, Rule 10 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Boilers, Steam Generators, and Process Heaters in Petroleum Refineries (3/29/01) |  |  |
| 9-10-502 | Monitoring | Y |  |
| 40 CFR 60 Subpart A | General Provisions (7/1/2000) |  |  |
| 60.7 | Notification and record keeping. | Y |  |
| 60.8 | Performance tests. | Y |  |
| 60.11 | Compliance with standards and maintenance requirements. | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention. | Y |  |
| 60.13 | Monitoring requirements. | Y |  |
| 60.13(a) | Continuous monitoring systems subject to Appendix B, and Appendix F(if used to demonstrate compliance with continuous emission limits), ofPart 60 | Y |  |
| 60.13(b) | Continuous monitoring systems and devices operational prior toperformance tests required by 60.8 | Y |  |
| 60.13(d) | Continuous monitoring system zero and span calibration requirements | Y |  |
| 60.13(e) | Continuous monitoring system minimum frequency of operation | Y |  |
| 60.13(f) | Continuous monitoring system installation location requirement | Y |  |
| 60.13(i) | Alternatives to any monitoring procedures or rquirements | Y |  |
| 60.19 | General notification and reporting requirements. | Y |  |
| 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (8/17/89) |  |  |
| 60.104 | Standard for Sulfur Oxides | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 29 | Hydrogen plant NOx limit [basis: BACT] | Y |  |
| Part 30 | Hydrogen plant CO limit [basis: BACT] | Y |  |
| Part 31 | Hydrogen plant Abated by A4161 & Ammonia emission limit [basis: BACT] | N |  |
| Part 32 | CEM for NOx and O2 [basis: BACT] | Y |  |
| Part 33 | Hydrogen plant deaerator vented to S4161 [basis: Regulation 8-2] | Y |  |
| Part 34 | Hydrogen plant heat input limit [basis: BACT] | Y |  |
| Part 107 | Source Test for Co emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 112 | Source Test for NH3 emissions [basis: Regulation 2-6-409.2] | N |  |
| BAAQMD Condition # 16688 |  |  |  |
| Part 1 | Maximum firing rate [basis: Regulation 2-1-234] | N |  |
| BAAQMD Condition # 18265 |  |  |  |
| Part 1 | Sources subject to Regulation 9-10 (basis: Regulation 9-10-301 & 305) | N | 06/01/04 |
| Part 2 | O2 monitor and recorder requirement (basis: Regulation 9-10-502) | N | 09/01/04 |
| Part 3 | Operating conditions requirements for those sources without CEM (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 4 | NOx box establishment requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 5 | NOx box ranges (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 6 | NOx Box Deviations (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 7 | Source test requirements (basis: Regulation 9-10-502) | N | 06/01/04 |
| Part 8 | CO source test (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 9 | CO results requires CEM (basis: Regulation 9-10-502, 1-522) | N | 06/01/04 |
| Part 10 | Source test records (basis: recordkeeping; Regulation 9-10-504) | N | 06/01/04 |
| Part 11 | CEM for NOx and O2 monitoring [basis: Regulation 9-10-502] | N |  |
| Part 12 | Equivalent verification system [basis: Regulation 9-10-502] | N |  |
| Part 13 | IERC required to comply with Regulation 9-10 [Regulation 2-9-303] | N |  |
| Part 14 | Monthly summary of daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU) [basis: Regulation 9-10-504] | N |  |
| Part 15 | Monthly sum of daily totals [basis: Regulation 9-10-504] | N |  |
| Part 16 | Source test of S1800 if it operates [basis: Regulation 9-10-501] | N |  |
| Part 17 | Alternative compliance plan [basis: Regulation 9-10-502] | N |  |
| Part 18 | operating range limits for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 19 | Allowable operating ranges for S-1476, S-1477, S-1478, S-1479, S-1508, S-1760 and S-4021 [basis: Regulation 9-10-502] | N |  |
| Part 20 | Maintain fuel flow meter [basis: Regulation 9-10-503] | N |  |
| Part 21 | Recordkeeping [basis: Regulation 9-10-503] | N |  |

| Table IV - CV Source-specific Applicable Requirements S4190 – UTIL Boiler 6 Gas Turbine 1 S4192 – UTIL Boiler 6 Gas Turbine 2 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emissions Monitoring if required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 9 | Inorganic Gaseous Pollutants - Nitrogen Oxides from Stationary Gas Turbines (9/21/94) |  |  |
| 9-9-113 | Exemption - Inspection/Maintenance | Y |  |
| 9-9-114 | Exemption - Startup/Shutdown | Y |  |
| 9-9-301 | Emission Limits - General | Y |  |
| 9-9-301.3 | Emission Limits | Y |  |
| 9-9-401 | Efficiency Certification | Y |  |
| 9-9-501 | Continuous Emission Monitoring (CEM) | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(3) | SO2 CEMs to demonstrate compliance | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| NSPS 40 CFR 60 Subpart GG; BAAQMD 10-40 | Standards of Performance for Stationary Gas Turbines (1/27/82) |  |  |
| 60.333 | Performance Standards, SO2 | Y |  |
| 60.333 (b) | Fuel Sulfur Limit | Y |  |
| 60.334 | Monitoring Requirements | Y |  |
| 60.334 (b) | Fuel Sulfur and Nitrogen Content | Y |  |
| 60.334 (c) | Excess Emissions | Y |  |
| 60.335 | Test Methods and Procedures | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E | Fuel Conditions | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 24a | Cogeneration Power Plant fuel usage limit & install SO2 CEM [basis: Offsets, Cumulative Increase] | Y |  |
| Part 24b | Cogeneration Power Plant Emission Limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part 24c | NOx limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 25a | CO limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 25b | POC emission limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 26 | Ammonia emission limits for gas turbines and HRG boiler pairs [basis: BACT] | N |  |
| Part 27 | CEMs for NOx and O2 and water to fuel ratio monitor [basis: BACT] | Y |  |
| Part 108 | Source Test for CO emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 113 | Source Test for NH3 emissions [basis: Regulation 2-6-409.2] | N |  |
| Part 114 | Source Test for VOC emissions [basis: Regulation 2-6-409.2] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV – CW Source-specific Applicable Requirements S4191 – UTIL Boiler 6 Supplemental Steam Generator 1 S4193 – UTIL Boiler 6 Supplemental Steam Generator 2 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) | Y |  |
| 1-520 | Continuous Emission Monitoring | Y |  |
| 1-520.8 | Monitors as required by Regulation 10, 12 and Section 2-1-403 | Y |  |
| 1-522 | Continuous Emission Monitoring and Recordkeeping Procedures | Y |  |
| 1-522.1 | Approval of Plans and Specifications | Y |  |
| 1-522.2 | Installation Scheduling | Y |  |
| 1-522.3 | CEM Performance Testing | Y |  |
| 1-522.4 | Periods of Inoperation | Y |  |
| 1-522.5 | Daily Monitor Calibration | Y |  |
| 1-522.6 | CEM Maintenance | Y |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, and Cause) | N |  |
| 1-522.8 | Monitoring Data Submittal | Y |  |
| 1-522.9 | Records | Y |  |
| 1-522.10 | Monitors Required By 1-521 or 2-1-403 | Y |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures | Y |  |
| 1-523.1 | Periods of Inoperation Reported | Y |  |
| 1-523.2 | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 | Records | Y |  |
| 1-523.5 | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-522.7 | Excess Emissions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| 1-523.3 | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-304 | Tube Cleaning | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particle Weight Limitation | Y |  |
| 6-310.3 | 0.15 grain per dscf at 6% O2 | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| 9-1-502 | Continuous Emissions Monitoring if required by APCO | Y |  |
| BAAQMD Regulation 9, Rule 9 | Inorganic Gaseous Pollutants - Nitrogen Oxides from Stationary Gas Turbines (9/21/94) |  |  |
| 9-9-113 | Exemption - Inspection/Maintenance | Y |  |
| 9-9-114 | Exemption - Startup/Shutdown | Y |  |
| 9-9-301 | Emission Limits - General | Y |  |
| 9-9-301.3 | Emission Limits | Y |  |
| 9-9-401 | Efficiency Certification | Y |  |
| 9-9-501 | Continuous Emission Monitoring (CEM) | Y |  |
| NSPS 40 CFR 60 Subpart A | Standards of Performance for New Stationary Sources (12/23/71) | Y |  |
| 60.7 | Notification and record keeping | Y |  |
| 60.8 | Performance tests | Y |  |
| 60.11 | Compliance with standards and maintenance requirements | Y |  |
| 60.11(a) | Performance test | Y |  |
| 60.11(d) | Good air pollution control practice for minimizing emissions | Y |  |
| 60.11(f) | Special provisions | Y |  |
| 60.11(g) | Any credible evidence or information | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13 | Monitoring requirements | Y |  |
| 60.19 | General notification and reporting requirements | Y |  |
| NSPS 40 CFR 60 Subpart Db; BAAQMD 10-4 | Standards of Performance for Industrial-Commercial-Institutional Steam Generating Units (3/13/00) |  |  |
| 60.43b | PM Standard | Y |  |
| 60.43b(e) | Capacity Calculation | Y |  |
| 60.43b(f) | Oil-fired: 20% opacity and 27% opacity for 6 min/hr | Y |  |
| 60.43b(g) | PM and Opacity standards applicable at all times, except during startup, shutdown, or malfunction | Y |  |
| 60.44b(a) | NOx Standard | Y |  |
| 60.44b(a)(4) | NOx standard for duct burner used in combined cycle system | Y |  |
| 60.44b(f) | NOx standard for refinery-produced byproduct with oil or natural gas combustion may be determined on a case-by-case basis. | Y |  |
| 60.44b(h) | NOx standard applicable at all times | Y |  |
| 60.44b(i) | 24-hour rolling average | Y |  |
| 60.46b | Compliance/Performance test Methods for PM and NOx | Y |  |
| 60.48b | Emission Monitoring for Particulate Matter and NOx | Y |  |
| 60.48b(a) | Opacity CEM if subject to 60.43b | Y |  |
| 60.48b(h) | NOx CEM not required if subject to §60.44b(a)(4) | Y |  |
| 60.49b | Reporting and Recordkeeping | Y |  |
| 60.49b(a) | Submit notification of date of initial startup | Y |  |
| 60.49b(b) | Initial performance test data and CEMS performance evaluation | Y |  |
| 60.49b(d) | Recordkeeping | Y |  |
| 60.49b(f) | Opacity records | Y |  |
| 60.49b(g) | NOx emission records | Y |  |
| 60.49b(h) | Excess Opacity reports | Y |  |
| 60.49b(o) | Recordkeeping | Y |  |
| 60.49b(v) | Electronic reports of opacity | Y |  |
| 60.49b(w) | Reporting period for reports | Y |  |
| NSPS 40 CFR 60 Subpart J | Standards of Performance for Petroleum Refineries (7/1/00) |  |  |
| 60.104 | Standards for Sulfur Oxides: Compliance Schedule | Y |  |
| 60.104(a)(1) | Fuel gas H2S limit | Y |  |
| 60.105 | Monitoring of Emissions and Operations | Y |  |
| 60.105(a)(3) | SO2 CEMs to demonstrate compliance | Y |  |
| 60.105(a)(4) | monitoring requirement for H2S (dry basis) in fuel gas prior to combustion (in lieu of separate combustion device exhaust SO2 monitors as required by 60.105(a)(3)) | Y |  |
| 60.105(e)(3) | Excess SO2 emission definitions for 60.7(c) | Y |  |
| 60.106 | Test methods and procedures | Y |  |
| 60.106(e) | Method 11 shall be used to verify compliance with 60.104(a)(1) | Y |  |
| BAAQMD Condition # 7618 |  |  |  |
| Part E | Fuel Conditions | Y |  |
| Part E.2 | Processing limit of San Joaquin Valley Crude [basis: Regulation 2-2-302] | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 15 | Refinery fuel gas limits [basis: BACT] | Y |  |
| Part 16 | Continuos gaseous fuel monitor/recorder for H2S [basis: BACT] | Y |  |
| Part 16A | Natural gas line connection upstream of CO Boilers [basis: Offsets] | Y |  |
| Part 17 | Recordkeeping requirements [basis; Cumulative Emissions] | Y |  |
| Part 18 | Fuel usage limitation [basis: Offsets, Cumulative Increase] | Y |  |
| Part 19 | ULSD defined [basis: Offsets, Cumulative Increase] | Y |  |
| Part 20 | Continuous Fuel Flow Monitor [basis: Offsets, Cumulative Increase] | Y |  |
| Part 21 | Calculate and Record Emissions Monthly [basis: Offsets, Cumulative Increase] | Y |  |
| Part 22 | Startup and shutdown limits [basis: BACT] | Y |  |
| Part 23 | HHV basis [basis: Cumulative Emissions] | Y |  |
| Part 24a | Cogeneration Power Plant fuel usage limit & install SO2 CEM [basis: Offsets, Cumulative Increase] | Y |  |
| Part 24b | Cogeneration Power Plant Emission Limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part 24c | NOx limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 25a | CO limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 25b | POC emission limits for gas turbines and HRG boiler pairs [basis: BACT] | Y |  |
| Part 26 | Ammonia emission limits for gas turbines and HRG boiler pairs [basis: BACT] | N |  |
| Part 27 | CEMs for NOx and O2 and water to fuel ratio monitor [basis: BACT] | Y |  |
| Part 108 | Source Test for CO emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 113 | Source Test for NH3 emissions [basis: Regulation 2-6-409.2] | N |  |
| Part 114 | Source Test for VOC emissions [basis: Regulation 2-6-409.2] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 3 | Visible emissions inspection (basis: Regulation 2-6-409.2) | Y |  |
| Part 4 | Fuel certification (basis: Regulation 2-6-409.2) | Y |  |
| Part 6 | Visible emission inspection during tube cleaning (basis: Regulation 2-6-409.2) | Y |  |
| Part 7 | Recordkeeping (basis: Regulation 2-6-409.2) | Y |  |

| Table IV - CX Source-specific Applicable Requirements S4201 – DC Clean Fuels Flare |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| BAAQMD Regulation 12-11 | Flare Monitoring at Petroleum Refineries (06/04/03) |  |  |
| 12-11-401 | Flare Data Reporting Requirements | N |  |
| 12-11-402 | Flow Verification Report | N | 6/4/04 |
| 12-11-501 | Vent Gas Flow Monitoring | N | 12/4/04 |
| 12-11-502 | Vent Gas Composition Monitoring | N |  |
| 12-11-502.3 | Vent Gas Composition Monitoring | N | 03/4/04 |
| 12-11-503 | Pilot Monitoring | N |  |
| 12-11-504 | Pilot and Purge Gas Monitoring | N |  |
| 12-11-505 | Recordkeeping Requirements | N |  |
| 12-11-506 | General Monitoring Requirements | N |  |
| 12-11-506.1 | Periods of Inoperation of Vent Gas Monitoring | N | 09/4/04 |
| 12-11-507 | Video Monitoring | N | 12/4/03 |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part 57 | Prevention of smoke [basis: BACT] | Y |  |
| Part 60 | Natural gas or LPG fueled flare pilots [basis: BACT] | Y |  |
| Part 61 | Hydrocarbon destruction efficiency [basis: BACT] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 12 | Flarethroughput limit (basis: Regulations 8-1-110.3; 2-1-403) | Y | 06/01/04 |
| Part 13 | Flaring recordkeeping requirements (basis: Regulations 8-1-110.3; 2-6-409.2; 2-6-501) | Y | 06/01/04 |
| Part 14 | Inspection of flaring requirements (basis: Regulation 2-6-4092.) | Y | 06/01/04 |
| Part 15 | Inspection procedures (basis: Regulation 6-301; 2-1-403) | Y | 06/01/04 |
| Part 16 | Visual inspection requirements (basis: Regulation 2-6-403) | Y | 06/01/04 |
| Part 17 | Recordkeeping requirements for flaring events (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |
| Part 18 | Flexigas flare requirements (basis: Regulation 2-6-501; 2-6-409.2) | Y | 06/01/04 |
| Part 19 | Upset gas limitation (basis: 60.104(a)(1); Regulation 2-1-403) | Y | 06/01/04 |

| Table IV – CY Source-specific Applicable Requirements S4210 – Cooling Water Tower (CWT-13278) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (05/02/2001) |  |  |  |
| 1-523 | Parametric Monitoring and Recordkeeping Procedures |  | Y |  |
| 1-523.1 |  | Periods of Inoperation Reported | Y |  |
| 1-523.2 |  | Periods of Inoperation Per Incident | Y |  |
| 1-523.3 |  | Violations of Permit Conditions Reported (Nature, Extent, and Cause) | N |  |
| 1-523.4 |  | Records | Y |  |
| 1-523.5 |  | Maintenace and Calibration Required | N |  |
| SIP Regulation 1 |  | PROVISIONS NO LONGER IN CURRENT RULE General Provisions and Definitions (6/28/99) |  |  |
| 1-523.3 |  | Violations of Permit Conditions Reported (Nature, Extent, Cause, and Corrective Action) | Y |  |
| BAAQMD Regulation 6 |  | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 |  | Ringelmann No. 1 Limitation | Y |  |
| 6-305 |  | Visible Particles | Y |  |
| 6-310 |  | Particle Weight Limitation | Y |  |
| 6-311 |  | General Operations (process weight rate limitation) | Y |  |
| 6-401 |  | Appearance of Emissions | Y |  |
| BAAQMD Regulation 8 Rule 2 |  | Organic Compounds – Miscellaneous Operations (03/22/95) |  |  |
| 8-2-301 |  | Miscellaneous Operations | Y |  |
| BAAQMD Condition # 12271 |  |  |  |  |
| Part A |  | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B |  | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C |  | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L |  | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M |  | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part 55 |  | Continuous hydrocarbon analyzer/recorder [basis: BACT] | Y |  |

| Table IV – CZ Source-specific Applicable Requirements S4211 - DC V-13222 ISOM Maintenance Drop Out Vessel S4212 - DC V-13441 ISOM Maintenance Drop Out Vessel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 10 |  | Organic Compounds – Process Vessel Depressurization (10/03/84) |  |  |
| 8-10-301 |  | Process Vessel Depressurizing | Y |  |
| 8-10-401 |  | Turnaround Records | Y |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 |  | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.343 |  | Standards: Tanks | Y |  |
| 61.343(a)(1) |  | Owner/operator shall install, operate, and maintain a fixed-roof and closed-vent system that routes all organic vapors vented from the tank to a control device. | Y |  |
| 61.343(a)(1) (i) |  | Fixed Roof leak requirements - < 500 ppmv above background, determined initially then annually by method in 61.355(h) of this subpart. | Y |  |
| 61.343(a)(1) (ii) |  | Closed-vent system and control device designed and operated in accordance with 61.349 of this subpart | Y |  |
| 61.343(b) |  | For a tank that meets all of the conditions of paragraph (b)(1), the owner/operator may elect to comply with paragraph (b)(2) as an alternative to the requirements of (a)(1). | Y |  |
| 61.343(b)(1) |  | The waste managed in the tank complying with (b)(2) shall meet all of the following conditions: wastestream managed in the tank must have a flow-weighted annual average water content less than or equal to 10% water, on a volume basis and the waste managed in the tank either: has a maximum organic vapor pressure less than 0.75 psi, or has a maximum organic vapor pressure less than 4.0 psi and is managed in a tank less than 40,000 gallons, or has a maximum organic vapor pressure less than 11.1 psi and is managed in a tank having a design capacity less than 20,000 gallons. | Y |  |
| 61.343(b)(2) |  | Owner/operator shall install, operate, and maintain a fixed roof as specified in paragraph (a)(1)(i). | Y |  |
| 61.343(b)(3) |  | may use devices that vent directly to atmosphere provided the devices are closed, sealed during normal operations except when the device needs to open to prevent damage or deformation of the tank or cover. | Y |  |
| 61.343(c) |  | Inspect all fixed-roof, seal, access doors, and all other openings visually initially and quarterly for cracks and gaps and that doors and openings are closed and gasketed properly. | Y |  |
| 61.343(d) |  | When a broken seal or gasket is identified, or when detectable emissions are measured, first efforts at repair shall be made as soon as practicable, but not later than 45 calendar days after identification. | Y |  |
| 61.350 |  | Delay of Repair | Y |  |
| 61.355 |  | Test methods, procedures and compliance provisions | Y |  |
| 61.356 |  | Recordkeeping requirements | Y |  |
| 61.357 |  | Reporting requirements | Y |  |
| 61.357(d) |  | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste | Y |  |
| 61.357(d)(8) |  | Reporting requirements | Y |  |
| MACT 40 CFR 63 Subpart CC |  | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 |  | Wastewater provisions | Y |  |
| 63.647(a) |  | Owners/operators of Group 1 wastewater streams shall comply with section 61.340 to 61.355 of 40 CFR 61, subpart FF for each stream that meets the definition of 63.641 | Y |  |
| 63.654 |  | Reporting and recordkeeping requirements | Y |  |
| 63.654(a) |  | Owners/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements of 61.356 and 61.357 of 40 CFR 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| BAAQMD Condition # 12271 |  |  |  |  |
| Part A |  | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B |  | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C |  | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L |  | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M |  | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part N |  | Process vessel depressurization requirements [basis: BACT] | Y |  |
| Part O |  | Offsets requirements [basis: Offsets] | Y |  |
| Part 52 |  | Enclosed and vapor-tight and vented to S4201 [basis: BACT] | Y |  |
| Part 53 |  | MDO Vessel Requirements [basis: BACT] | Y |  |

| - Table IV – DAa - Source-specific Applicable Requirements S1802 - LOGV-1533 Odorant Storage Tank S4311 - DC V-12555 ISOM Perchloroethylene Vessel, S4329 - Tank 13260 Pentane, S4330 – Tank 13261 Pentane, S4349 - Tank 13262 Pentane |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Notes |
| BAAQMD Regulation 8, Rule 10 | Organic Compounds – Process Vessel Depressurization (10/03/84) |  |  |
| 8-10-301 | Process Vessel Depressurizing | Y |  |
| 8-10-401 | Turnaround Records | Y |  |

| - Table IV – DAb - Source-specific Applicable Requirements S4311 - DC V-12555 ISOM Perchloroethylene Vessel, S4329 - Tank 13260 Pentane, S4330 – Tank 13261 Pentane, S4349 - Tank 13262 Pentane |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Notes |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part N | Process vessel depressurization requirements [basis: BACT] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 46 | Pressure tank requirement [basis: BACT] | Y |  |
| Part 51 | Degassing abatement requirement [basis: BACT] | Y |  |

| - Table IV – DB - Source-specific Applicable Requirements - S4322 - Tank 14571 Sour Water |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Notes |
|  | See Table IV – R & W for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 45 | Seal requirements [basis: BACT] | Y |  |
| Part 51 | Degassing abatement requirement [basis: BACT] | Y |  |

| - Table IV – DD - Source-specific Applicable Requirements S4338 – LOG Pentane Loading Facility |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Notes |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 74 | Loading requirements [basis: BACT] | Y |  |

| Table IV - DE Source-specific Applicable Requirements S4347 – OPCEN Pit for Sulfur Plant 4 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann Number 1 Limitation | Y |  |
| 6-305 | Visible Particles | Y |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| 6-311 | General Operations (process weight rate limitation) | Y |  |
| 6-330 | Sulfur Recovery Units (SO3, H2SO4 emission limitations) | Y |  |
| 6-401 | Appearance of Emissions | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part 63 | Sulfur pit and sulfur storage tank requirements [basis: BACT] | Y |  |

| Table IV - DF Source-specific Applicable Requirements S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AQ for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part D | Applicable source test methods [basis: Manual of Procedures, Volume IV] | Y |  |
| Part E | Notification of the Source Test Section [basis: Manual of Procedures, Volume IV] | Y |  |
| Part G | Source test results [basis: Manual of Procedures Volume IV] | Y |  |
| Part J | CEM requirements [basis: Cumulative Increase, Offsets] | Y |  |
| Part K | CEM data for compliance verification [basis: BACT, Offsets] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 64 | Abatement efficiency of sour water strippers [basis: BACT] | Y |  |
| Part 65 | Sulfur recovery efficiency requirements[basis: BACT] | Y |  |
| Part 66 | Concentration limit of total reduced sulfur [basis: BACT] | Y |  |
| Part 67 | Use of gas chromatograph [basis: BACT] | Y |  |
| Part 68 | SCOT thermal oxidizer efficiency requirements [basis: BACT] | Y |  |
| Part 69 | SO2 CEM requirement [basis: BACT] | Y |  |
| Part 70 | CO concentration limit [basis: BACT, good combustion] | Y |  |
| Part 71 | Shutdown requirements [basis: BACT, offsets] | Y |  |
| Part 72 | Abatement by scrubber of loading of elemental sulfur or sulfuric acid [basis: BACT, odors] | Y |  |
| Part 109 | Source Test for CO emissions [basis: Regulation 2-6-409.2] | Y |  |
| Part 110 | Source Test for H2S emissions [basis: Regulation 2-6-409.2] | Y |  |

| Table IV – DG Source-specific Applicable Requirements S4319 – Tank 15096 Recovered Oil S4350 – LOG Tank 13187 Process Wastewater S4356 – LOG Tank 13188 Process Wastewater |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement |  |  | Federally Enforceable (Y/N) |  | Future Effective Date |  |
| BAAQMD Reg 8 Rule 5 |  | Organic Compounds - STORAGE OF ORGANIC LIQUIDS DEVICE (6/5/03) |  |  |  |  |  |
| 8-5-111 |  | Tank Removal From and Return to Service |  |  | Y |  |  |
| 8-5-112 |  | Tanks in Operation – maintenance and inspection |  |  | Y |  |  |
| 8-5-301 |  | Storage Tank Control Requirements |  |  | Y |  |  |
| 8-5-302 |  | Requirements for Submerged Fill Pipes |  |  | Y |  |  |
| 8-5-303 |  | Requirements for Pressure Vacuum Valve |  |  | Y |  |  |
| 8-5-306 |  | Requirements for Approved Emission Control Systems |  |  | Y |  |  |
| 8-5-328 |  | Tank Degassing Requirements |  |  | Y |  |  |
| 8-5-403 |  | Inspection Requirements for Pressure Vacuum Valves |  |  | Y |  |  |
| 8-5-404 |  | Certification |  |  | Y |  |  |
| 8-5-501.1 |  | Records |  |  | Y |  |  |
| 8-5-502 |  | Tank Degassing Annual Source Test Requirement |  |  | Y |  |  |
| 8-5-503 |  | Portable Hydrocarbon Detector |  |  | Y |  |  |
| NSPS Subpart Kb |  | Volatile Organic Liquid Storage Vessels REQUIREMENTS FOR FIXED ROOF TANK-CONTROL DEVICE |  |  |  |  |  |
| 60.112b(a) |  | Closed vent system Performance requirements: | 60.112b(a)(3)(i) no detectable emissions (i.e., < 500 ppm) |  | Y |  |  |
|  |  | Control device Performance requirements: | 60.112b(a)(3)(ii) at least 95% efficient |  | Y |  |  |
| 60.113b(c) |  | Control device (other than flare) Compliance demonstration: | 60.113b(c)(1) operating plan, efficiency demo, & parameter(s) to be monitored |  | Y |  |  |
|  |  | Other (initial) Reports: For control device other-than flare? | 60.113b(c)(1) submit operating plan for approval, with the initial notification |  | Y |  |  |
|  |  | Control device (other than flare) Operating requirements: | 60.113b(c)(2) operate and monitor per the plan |  | Y |  |  |
| 60.115b |  | Recordkeeping for inspections: Keep inspection reports as specified. | 60.115b Keep for 2 years |  | Y |  |  |
| 60.115b(c) |  | Recordkeeping for tanks routed to a control device other than a flare: | 60.115b(c) operating plan & records of parametric monitoring data |  | Y |  |  |
| 60.116b(a) |  | Applicability records: Time period for keeping records of applicability determination, unless specified otherwise. | 60.116b(a) Keep for 2 years |  | Y |  |  |
| 60.116b(b) |  | Applicability records: Records of dimensions & capacity required for nonexempt tanks? | 60.116b(b) Required Keep record readily accessible for the life of the tank |  | Y |  |  |
| 60.116b(e) |  | True vapor pressure (TVP) determination for applicability: | 60.116b(e) maximum TVP of the stored liquid, based on highest calendar month average storage temperature |  | Y |  |  |
| 60.116b(g) |  | Applicability determination: Miscellaneous recordkeeping exemptions: | 60.116b(g) keeping record of TVP is not required if tank is routed to a compliant control device |  | Y |  |  |
| NSPS Subpart A |  | New Source Performance Standards GENERAL PROVISIONS |  |  | Y |  |  |
| 60.7(a) |  | Initial Notification: Is initial notification of the source’s existence required? | 60.7(a)(1) notification within 30 days after begin construction |  | Y |  |  |
|  |  | Report (document) having initially achieved compliance? | 60.7(a)(3) 60.115b(a)(1) & (b)(1) within 15 days after initial fill |  | Y |  |  |
|  |  | Notification of Compliance Status report: | 60.7(a)(3) [cf. 60.115b(a)(1)&(b)(1)] notification within 15 days after startup |  | Y |  |  |
|  |  | Initial Notification: Is initial notification required if tank becomes affected only as a result of a modification? | 60.7(a)(4) notification 60 days or as soon as practicable before the change |  | Y |  |  |
| 60.7(f) |  | General recordkeeping requirements: Time period for keeping records, unless specified otherwise. | 60.7(f) Keep all reports & notifications for 2 years |  | Y |  |  |
|  |  | General recordkeeping requirements: Keep all reports and notification for the specified period of time. | 60.7(f) required |  | Y |  |  |
| 60.14(g) |  | Achieve compliance for: New Tanks (or tanks that become affected as a result of a change or modification)? | 60.14(g) up to 180 days after modifications (otherwise prior to fill) |  | Y |  |  |
| NESHAP 40 CFR 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |  |  |  |  |
| 61.343 | Standards: Tanks |  |  | Y |  |  |  |
| 61.343(a)(1) | Owner/operator shall install, operate, and maintain a fixed-roof and closed-vent system that routes all organic vapors vented from the tank to a control device. |  |  | Y |  |  |  |
| 61.343(a)(1)(i) | Fixed Roof leak requirements - < 500 ppmv above background, determined initially then annually by method in 61.355(h) of this subpart. |  |  | Y |  |  |  |
| 61.343(a)(1)(ii) | Closed-vent system and control device designed and operated in accordance with 61.349 of this subpart |  |  | Y |  |  |  |
| 61.343(b) | For a tank that meets all of the conditions of paragraph (b)(1), the owner/operator may elect to comply with paragraph (b)(2) as an alternative to the requirements of (a)(1). |  |  | Y |  |  |  |
| 61.343(b)(1) | The waste managed in the tank complying with (b)(2) shall meet all of the following conditions: wastestream managed in the tank must have a flow-weighted annual average water content less than or equal to 10% water, on a volume basis and the waste managed in the tank either: has a maximum organic vapor pressure less than 0.75 psi, or has a maximum organic vapor pressure less than 4.0 psi and is managed in a tank less than 40,000 gallons, or has a maximum organic vapor pressure less than 11.1 psi and is managed in a tank having a design capacity less than 20,000 gallons. |  |  | Y |  |  |  |
| 61.343(b)(2) | Owner/operator shall install, operate, and maintain a fixed roof as specified in paragraph (a)(1)(i). |  |  | Y |  |  |  |
| 61.343(b)(3) | may use devices that vent directly to atmosphere provided the devices are closed, sealed during normal operations except when the device needs to open to prevent damage or deformation of the tank or cover. |  |  | Y |  |  |  |
| 61.343(c) | Inspect all fixed-roof, seal, access doors, and all other openings visually initially and quarterly for cracks and gaps and that doors and openings are closed and gasketed properly. |  |  | Y |  |  |  |
| 61.343(d) | When a broken seal or gasket is identified, or when detectable emissions are measured, first efforts at repair shall be made as soon as practicable, but not later than 45 calendar days after identification. |  |  | Y |  |  |  |
| 61.349 | Standards: Closed vent systems and control devices |  |  | Y |  |  |  |
| 61.349(a) | Closed vent system and control device |  |  | Y |  |  |  |
| 61.349(a)(1) | Closed vent system requirements |  |  | Y |  |  |  |
| 61.350 | Delay of Repair |  |  | Y |  |  |  |
| 61.355 | Test methods, procedures and compliance provisions |  |  | Y |  |  |  |
| 61.355(h) | Test equipment for compliance with no detectable emissions as required in 61.343 through 61.347 and 61.349 |  |  | Y |  |  |  |
| 61.356 | Recordkeeping requirements |  |  | Y |  |  |  |
| 61.356(h) | Reporting requirements |  |  | Y |  |  |  |
| 61.357 | Reporting requirements |  |  | Y |  |  |  |
| 61.357(d) | reporting requirements for facilities with 10 Mg/yr or more total benzene in waste |  |  | Y |  |  |  |
| 61.357(d)(8) |  | Reporting requirements |  | Y |  |  |  |
| MACT 40 CFR 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  | Y |  |  |  |
| 63.640 | Emission points at petroleum refinery |  |  | Y |  |  |  |
| 63.640(d)(5) | No testing, monitoring, recordkeeping,or reporting is required for refinery fuel gas systems or emission points routed to refinery fuel gas systems |  |  | Y |  |  |  |
| 63.647 | Wastewater provisions |  |  | Y |  |  |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR part 61, subpart FF for each stream that meets the definition of 63.641. |  |  | Y |  |  |  |
| 63.654 | Reporting and recordkeeping requirements |  |  | Y |  |  |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR part 61, subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. |  |  | Y |  |  |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. |  |  | Y |  |  |  |
| BAAQMD Condition # 12271 |  |  |  |  |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] |  |  | Y |  |  |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] |  |  | Y |  |  |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] |  |  | Y |  |  |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] |  |  | Y |  |  |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] |  |  | Y |  |  |  |
| Part O | Offsets requirements [basis: Offsets] |  |  | Y |  |  |  |
| Part 45 | Vapor Recovery System [basis: BACT] |  |  | Y |  |  |  |
| Part 51 | Degassing abatement requirements [basis: BACT] |  |  | Y |  |  |  |

| Table IV – DI Source-specific Applicable Requirements S4334 - Tank 13276 Alkylate |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AE for additional requirements. |  |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part A | Monthly and annual emission limits [basis: Offsets, Cumulative Increase] | Y |  |
| Part B | Applicable sources [basis: Offsets, Cumulative Increase] | Y |  |
| Part C | In the case of a process upset or other irregularity [basis: Offsets, Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| Part O | Offsets requirements [basis: Offsets] | Y |  |
| Part 45 | Equipment specification [basis: BACT] | Y |  |
| Part 51 | Degassing abatement requirements [basis: BACT] | Y |  |

| Table IV – DJ Source-specific Applicable Requirements S5112 – LOG V-15112 Main Proto Vessel with Nitrogen Blanket S5113 – LOG V-15113 Main Proto Vessel with Nitrogen Blanket S5114 – LOG V-15114 Main Proto Vessel with Nitrogen Blanket S5125 LOG V-15117 Standby Proto Vessel |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Reg 8 Rule 5 | Organic Compounds - STORAGE OF ORGANIC LIQUIDS (6/5/03) |  |  |
| 8-5-111 | Tank Removal From and Return to Service | Y |  |
| 8-5-112 | Tanks in Operation – maintenance and inspection | Y |  |
| 8-5-301 | Storage Tank Control Requirements | Y |  |
| 8-5-302 | Requirements for Submerged Fill Pipes | Y |  |
| 8-5-303 | Requirements for Pressure Vacuum Valve | Y |  |
| 8-5-306 | Requirements for Approved Emission Control Systems | Y |  |
| 8-5-328 | Tank Degassing Requirements | Y |  |
| 8-5-403 | Inspection Requirements for Pressure Vacuum Valves | Y |  |
| 8-5-404 | Certification | Y |  |
| 8-5-501.1 | Records | Y |  |
| 8-5-502 | Tank Degassing Annual Source Test Requirement | Y |  |
| 8-5-503 | Portable Hydrocarbon Detector | Y |  |

| Table IV – DK Source-specific Applicable Requirements S5112 – LOG V-15112 Main Proto Vessel with Nitrogen Blanket S5113 – LOG V-15113 Main Proto Vessel with Nitrogen Blanket S5114 – LOG V-15114 Main Proto Vessel with Nitrogen Blanket |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – DJ for additional requirements |  |  |
| Condition # 11504 |  |  |  |
| Part 1 | Controlled by A26 [basis: Cumulative Increase, Regulation 8-5-311] | Y |  |

| Table IV – DL Source-specific Applicable Requirements S5115 – LOG Dissolved Nitrogen Flotation Unit North ETP 2 (DNF) S5116 – LOG Dissolved Nitrogen Flotation Unit South ETP 2 (DNF) |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV - CG for additional requirements. |  |  |
| Condition # 11313 |  |  |  |
| Part 1 | Two carbon vessels arranged in series [basis: Regulation 8-8-307] | Y |  |
| Part 2 | Second to last carbon bed change out [basis: Regulation 8-8-307] | Y |  |
| Part 3 | Last carbon bed change out [basis: Regulation 8-8-307] | Y |  |
| Part 4 | Non-Methane Hydrocarbon Emission limits [basis: Regulation 8-8-307] | Y |  |
| Part 5 | Monitor Carbon Beds [basis: Regulation 8-8-504] | Y |  |
| Part 6 | Record monitor readings [basis: Regulation 8-8-307] | Y |  |
| Part 7 | Record monitor readings [basis: Regulation 8-8-307] | Y |  |
| Part 8 | Records on file [basis: Regulation 8-8-307] | Y |  |

| Table IV – DM Source-specific Applicable Requirements S5121 – LOG SPM-14111 DNF Float Tank ETP 2 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 8, Rule 8 | Wastewater (Oil-Water) Separator |  |  |
| 8-8-303 | Gauging and Sampling Devices | Y |  |
| 8-8-305 | Oil-Water Separator And/Or Air Flotation Unit Slop Oil Vessels | Y |  |

| Table IV – DN Source-specific Applicable Requirements S12490 – LOG Tank 12519 Wastewater ETP 1&2 S12491 - LOG Tank 12520 Wastewater ETP 1&2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) |  | Future Effective Date |
|  | See Table IV – AC for additional requirements. |  |  |  |
| Condition # 8502 |  |  |  |  |
| Part 1 | Level indicating device [basis: Cumulative Increase] | Y |  |  |

| Table IV – DNa Source-specific Applicable Requirements S5140 – Diesel Engine, S6051 through S6060 Diesel Engines |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 6 | Particulate Matter and Visible Emissions (09/04/98) |  |  |
| 6-301 | Ringelmann No. 1 Limitation | Y |  |
| 6-305 | Visible Particles |  |  |
| 6-310 | Particulate Weight Limitation | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants - Sulfur Dioxide (3/15/95; SIP approved 6/8/99) |  |  |
| 9-1-301 | Limitations on Ground Level Concentrations | Y |  |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Y |  |
| BAAQMD Regulation 9, Rule 8 | Inorganic Gaseous Pollutants - Nitrogen Oxides and Carbon Monoxide from Stationary Internal Combustion Engines (1/20/93) |  |  |
| 9-8-331 | Essential Public Service, Hours of Operation | N |  |
| 9-8-530 | Emergency Standby Engines, Monitoring and Recordkeeping | N |  |
| BAAQMD Condition # 19097 |  |  |  |
| Part 1 | Hours of operation limit for reliability-related activities [basis: Regulation 9-8-330] | N |  |
| Part 2 | Emergency use [basis: Regulation 9-8-231] | N |  |
| Part 3 | Reliability-related activities [basis: Regulation 9-8-232] | N |  |
| Part 4 | Monitoring [basis: Regulation 9-8-530] | N |  |
| Part 5 | Recordkeeping [basis: Regulation 9-8-530, 1-441] | N |  |
| Part 6 | Fuel oil certification [basis: Regulation 2-6-409.2] | Y |  |
| Part 7 | Visibile emission monitoring [basis: Regulation 2-6-409.2] | Y |  |

| Table IV – DNb Source-specific Applicable Requirements S17095 – Tank 17095 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) |  | Future Effective Date |
|  | See Table IV – U for additional requirements. |  |  |  |
| Condition # 20042 |  |  |  |  |
| Part 1 | Throughput limit [basis: Cumulative Increase] | Y |  |  |
| Part 2 | Design Requirements [basis: BACT, Cumulative Increase] | Y |  |  |
| Part 3 | Recordkeeping [basis: Cumulative Increase] | Y |  |  |

| Table IV – DO Source-specific Applicable Requirements A101 – Flare for Vine Hill Vapor Recovery System A102 – Flare for Crude String Vapor Recovery System A103 – Flare for Interim Gasoline Vapor Recovery System |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
|  | See Table IV – AXa for additional requirements. |  |  |
| BAAQMD Condition # 7761 |  |  |  |
| Part 1 | A103 operating parameters [basis: Regulation 8-5-301] | Y |  |
| Part 2 | Recordkeeping [basis: Regulation 8-5-301] | Y |  |
| Part 3 | Pilot indicator [basis: Regulation 8-5-301] | Y |  |
| Part 4 | Flare failure alarm [basis: Regulation 8-5-301] | Y |  |
| Part 5 | Tanks abated by flare [basis: Regulation 8-5-301] | Y |  |
| Part 6 | A101 operating parameters [basis: Regulation 8-5-301] | Y |  |
| Part 7 | Recordkeeping [basis: Regulation 8-5-301] | Y |  |
| Part 8 | Pilot indicator [basis: Regulation 8-5-301] | Y |  |
| Part 9 | Flare failure alarm [basis: Regulation 8-5-301] | Y |  |
| Part 10 | Tanks abated by flare [basis: Regulation 8-5-301] | Y |  |
| Part 11 | A102 operating parameters [basis: Regulation 8-5-301] | Y |  |
| Part 12 | Recordkeeping [basis: Regulation 8-5-301] | Y |  |
| Part 13 | Pilot indicator [basis: Regulation 8-5-301] | Y |  |
| Part 14 | Flare failure alarm [basis: Regulation 8-5-301] | Y |  |
| Part 15 | Tanks abated by flare [basis: Regulation 8-5-301] | Y |  |

| - Table IV – DP - Applicable Requirements - Subpart GGG Equipment and Components |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Future Effective - Date |
| NSPS Part 60 Subpart A; BAAQMD 10-1 | General Provisions |  |  |
| 60.7(a), (c), (e), (f), (g), and (h) | Notification and record keeping | Y |  |
| 60.8(a) through (f) | Performance tests | Y |  |
| 60.11(a) through (g) | Compliance with standards and maintenance requirements | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13(a) through (f) and (h) | Monitoring requirements | Y |  |
| 60.19(a) through (f) | General notification and reporting requirements | Y |  |
| NSPS Part 60 Subpart GGG; BAAQMD Regulation 10-59 | Standards of Performance for Equipment Leaks (Fugitive Emission Sources) (5/30/84); BAAQMD Standards of Performance for New Stationary Sources (4/19/89) |  |  |
| 60.592 | Subject to provisions of Part 60, Subpart VV | Y |  |
| 60.593 | Exceptions | Y |  |
| BAAQMD Regulation 10-59 | Incorporates by reference 40 CFR 60 Subpart GGG | Y |  |
| NSPS Part 60 Subpart VV; BAAQMD Regulation 10-52 | Standards of Performance for Equipment Leaks (Fugitive Emission Sources) (8/18/95); BAAQMD Standards of Performance for New Stationary Sources (12/20/95) |  |  |
| 60.480 | Applicability and designation of affected facility | Y |  |
| 60.480(d) | An affected facility that qualifies for one or more exemption from 60.482 shall maintain records as required in 60.486(i). | Y |  |
| 60.482-1 | Standards: General | Y |  |
| 60.482-1(b) | Compliance with 60.482-1 to 60.482-10 will be determined…. | Y |  |
| 60.482-1(d) | Equipment that is in vacuum service is excluded from the requirements of 60.482-2 to 60.482-10 if it is identified as required in 60.486(e)(5). | Y |  |
| 60.482-2 | Standards: Pumps in light liquid service | Y |  |
| 60.482-2(a)(1) | Monthly monitoring of each pump, except for 60.482-2(d). | Y |  |
| 60.482-2(a)(2) | Weekly visual inspection of each pump. | Y |  |
| 60.482-2(b)(1) | Air measurement instrument reading >10,000 ppm indicates leak | Y |  |
| 60.482-2(b)(2) | Dripping liquid from pump seal indicates leak | Y |  |
| 60.482-2(c)(1) | Leak repaired within 15 calendar days, except as provided in 60.482-9. | Y |  |
| 60.482-2(c)(2) | First attempt at leak repair made within 5 calendar days. | Y |  |
| 60.482-2(d) | Pump with dual-mechanical seal system that includes barrier fluid system and meets specified requirements is exempt from 60.482-2(a). | Y |  |
| 60.482-2(g) | - Pump designated, per 60.486(f)(1), as unsafe-to-monitor pump is exempt from 60.482-2(a) and (d)(4) through (d)(6) if hazard documented and written monitoring plan is followed. | - Y |  |
| 60.482-2(h) | - Any pump located in an unmanned plant site is exempt from the requirements of 60.482-2(a)(2), (d)(4) and (d)(5) provided each pump is visually inspected as often as practicable and at least monthly. | - Y |  |
| 60.482-3 | - Standards: Compressor | - Y |  |
| 60.482-3(a) | - Each compressor equipped with seal system that includes a barrier fluid system and prevents leakage of VOC to atmosphere. | - Y |  |
| 60.482-3(b) | - Each compressor seal system operated with barrier fluid at pressure greater than compressor stuffing box pressure; or equipped with system that purges barrier fluid into process stream with zero emissions to atmosphere. | - Y |  |
| 60.482-3(c) | Barrier fluid system shall be in heavy liquid service. | - Y |  |
| 60.482-3(d) | Each barrier fluid system equipped with sensor that detects failure of seal system, barrier fluid system or both. | - Y |  |
| 60.482-3(e)(1) | - Each sensor shall be checked daily or shall be equipped with an audible alarm. | - Y |  |
| 60.482-3(e)(2) | - Owner shall determine a criterion that indicates failure of seal system, barrier fluid system, or both. | - Y |  |
| 60.482-3(f) | - If sensor indicates failure based on criterion established in 60.482-3(e)(2), a leak is detected. | - Y |  |
| 60.482-3(g)(1) | - Leak shall be repaired within 15 calendar days, except as provided in 60.482-9. | - Y |  |
| 60.482-3(g)(2) | - First attempt at repair shall be made within 5 calendar days. | - Y |  |
| 60.482-3(j) | - Existing reciprocating compressor in a process unit that becomes an affected facility is exempt from 60.482-3(a) through (e) and (h) if recasting distance piece or replacing compressor are only options for compliance. | - Y |  |
| 60.482-4 | - Standards: Pressure relief devices in gas/vapor service | - Y |  |
| 60.482-4(a) | - Except during pressure releases, pressure relief device shall be operated with no detectable emissions (< 500 ppm). | - Y |  |
| 60.482-4(b)(1) | After each pressure release, pressure release device shall be returned to a condition of no detectable emissions within 5 calendar days after pressure release, except as provided in 60.482-9. | - Y |  |
| 60.482-4(b)(2) | - No later than 5 calendar days after pressure release, the pressure relief device shall be monitored to confirm no detectable emissions. | - Y |  |
| 60.482-4(c) | - Any pressure relief device that is routed to a process or fuel gas system is exempt from 60.482-4(a) and (b). | - Y |  |
| 60.482-4(d)(1) | - Any pressure relief devise that is equipped with a rupture disk upstream of the pressure relief device is exempt from 60.482-4(a) and (b) provided complies with 60.482-4(d)(2). | - Y |  |
| 60.482-4(d)(2) | - After each pressure release, a new rupture disk shall be installed upstream of the pressure relief device as soon as practicable, but no later than 5 calendar days after each pressure release, except as provided in 60.482-9. | - Y |  |
| 60.482-5 | - Standards: Sampling connecting systems | - Y |  |
| 60.482-6 | - Standards: Open-ended valves or lines | - Y |  |
| 60.482-7 | - Standards: Valves in gas/vapor service and in light liquid service | - Y |  |
| 60.482-7(a) | - Monitor monthly to detect leaks, except as provided in 60.482-7(g) and (h) and 60.483-2. | - Y |  |
| 60.482-7(b) | - Instrument reading >10,000 ppm indicates leak. | - Y |  |
| 60.482-7(c) | - Valve that does not have a detectable leak for 2 successive months, can be monitored the first month of every quarter. | - Y |  |
| 60.482-7(d)(1) | - Leak shall be repaired within 15 calendar days, except as provided in 60.482-9. | - Y |  |
| 60.482-7(d)(2) | - First attempt at leak repair shall be made within 5 calendar days. | - Y |  |
| 60.482-7(e) | - Methods for first attempt at repair. | - Y |  |
| 60.482-7(g) | - Valve designated, per 60.486(f)(1), as unsafe-to-monitor valve is exempt from 60.482-7(a) if hazard documented and written monitoring plan is followed. | - Y |  |
| 60.482-7(h) | - Valve designated, per 60.486(f)(1), as difficult-to-monitor valve is exempt from 60.482-7(a) if hazard documented, less than 3% of facility valves are designated and written plan with is followed that requires monitoring at least once per year. | - Y |  |
| 60.482-8 | - Standards: Pumps and valves in heavy liquid service, pressure relief devices in light liquid or heavy liquid service, and flanges and other connectors. | - Y |  |
| 60.482-8(a) | - Monitor within 5 days if evidence of potential leak is found. | - Y |  |
| 60.482-8(b) | - Instrument reading >10,000 ppm indicates leak. | - Y |  |
| 60.482-8(c)(1) | - Leak shall be repaired within 15 calendar days, except as provided in 60.482-9. | - Y |  |
| 60.482-8(c)(2) | - First attempt at leak repair shall be made within 5 calendar days. | - Y |  |
| 60.482-8(d) | - Minimum requirements for first attempt at repair. | - Y |  |
| 60.482-9 | - Standards: Delay of Repair |  |  |
| 60.482-9(a) | - Delay allowed if repair is technically infeasible without a process unit shutdown and repair occurs before end of next process unit shutdown. | - Y |  |
| 60.482-9(b) | - Repair may be delayed for isolated equipment. | - Y |  |
| 60.482-9(c) | - Delay of repair for valves only allowed under certain circumstances. | - Y |  |
| 60.482-9(d)(1) | - Only dual-mechanical seal pumps qualify for delay of repair | - Y |  |
| 60.482-9(d)(2) | - Pump leaks must be repaired within 6 months. | - Y |  |
| 60.482-9(e) | - Delay of repair beyond process shutdown allowed if valve assembly replacement is required and other circumstances are met. | - Y |  |
| 60.483-2 | - If a process unit has 5 consecutive quarters with <2% of valves leaking at >10,000 ppm, then any individual valve which measures <100 ppm for 5 consecutive quarters may be monitored annually. | - Y |  |
| 60.485 | - Test Methods and Procedures | - Y |  |
| 60.485(a) | - Performance tests methods specified in Appendix A or 60.8(b) | - Y |  |
| 60.485(b) | - Method 21 for determining presence of leaking sources. | - Y |  |
| 60.485(d) | - Test each piece of equipment unless process unit not in VOC series. | - Y |  |
| 60.485(e) | - Light liquid service demonstrated by vapor pressure and if liquid at operating conditions. | - Y |  |
| 60.485(f) | - Samples representative of process fluid. | - Y |  |
| 60.486 | - Record keeping Requirements | - Y |  |
| 60.486(a) | - Comply with recordkeeping requirements of this section. | - Y |  |
| 60.486(b) | - Identification and tagging requirements for leaks detected as specified in 60.482-2, 60.482-3, 60.482-7, 60.482-8, and 60.483-2. | - Y |  |
| 60.486(c) | - When leak detected as specified in 60.482-2, 60.482-3, 60.482-7, 60.482-8, and 60.483-2, record in log and keep for 2 years. | - Y |  |
| 60.486(e) | - Information to be recorded for all equipment subject to requirements in 60.482-1 through 60.482-10. | - Y |  |
| 60.486(f) | Record information pertaining to all valves subject to the requirements in 60.482-7(g) and (h). | - Y |  |
| 60.486(g) | - Record information pertaining to all valves subject to the requirements in 60.483-2. | - Y |  |
| 60.486(h) | - Record design criterion required in 60.482-2(d)(5) and 60.482-3(e)(2). | - Y |  |
| 60.486(i) | - Record information in log that is readily accessible for use in determining exemption as provided in 60.480(d). | - Y |  |
| 60.486(j) | - Records to demonstrate piece of equipment not in VOC service. | - Y |  |
| 60.486(k) | - Provisions of 60.7(b) and (d) do not apply if subject to VV. | - Y |  |
| 60.487 | - Reporting Requirements | - Y |  |
| 60.487(a) | Submit semiannual reports. | Y |  |
| 60.487(c) | Information to be included in semiannual reports. | Y |  |
| 60.487(e) | Report results of all performance tests in accordance with 60.8. The provisions of 60.8(d) do not apply to affected facilities subject to VV. | Y |  |
| BAAQMD Regulation 10-52 | Incorporates by reference 40 CFR 60 Subpart VV | Y |  |

| - Table IV – DQ - Applicable Requirements - Subpart QQQ Individual drain systems |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Future Effective - Date |
| 40 Part 60 Subpart A; BAAQMD 10-1 | General Provisions |  |  |
| 60.7(a), (b), (c), (d), (e), (f), (g), and (h) | Notification and record keeping | Y |  |
| 60.8(a) through (f) | Performance tests | Y |  |
| 60.11(a) through (g) | Compliance with standards and maintenance requirements | Y |  |
| 60.12 | Circumvention | Y |  |
| 60.13(a) through (f) and (h) | Monitoring requirements | Y |  |
| 60.19(a) through (f) | General notification and reporting requirements | Y |  |
| NSPS Part 60 Subpart QQQ; BAAQMD Regulation 10-69 | Standards of Performance for VOC Emission From Petroleum Refinery Wastewater Systems (7/18/95); BAAQMD Standards of Performance for New Stationary Sources (12/20/95) |  |  |
| 60.692-1 | Standards: General | Y |  |
| 60.692-1(a) | Comply with 60.692-2 except during periods of startup, shutdown or malfunction. | Y |  |
| 60.692-1(b) | Compliance will be determined by review of records and reports, performance test results and inspection using methods and procedures specified in 60.696. | Y |  |
| 60.692-1(d) | Stormwater sewer systems, ancillary equipment, and non-contact cooling water systems are exempt if records are kept in accordance with 60.697(h), (i), and (j). | Y |  |
| 60.692-2 | Standards: Individual drain systems | Y |  |
| 60.692-2(c)(1) | Sewer lines shall be covered or enclosed with no visual gaps or cracks in joints, seals or other emission interfaces. | Y |  |
| 60.692-2(c)(2) | Inspect unburied portion of sewer lines semiannually for cracks gaps or other problems that could result in VOC emissions. | Y |  |
| 60.692-2(c)(3) | Repair sewer problems as soon as practicable but not later than 15 calendar days after identification. | Y |  |
| 60.692-6 | Delay of Repair Standards | Y |  |
| 60.697 | Recordkeeping Requirements | Y |  |
| 60.697(a) | Keep records and retain for 2 years. | Y |  |
| 60.697(b)(3) | For sewer lines, location, date, and corrective action recorded for inspections required when a problem is identified that could result in VOC emissions | Y |  |
| 60.697(e) | For delay of repair in accordance with 60.692-6, record reason, estimated date, signature of decision maker and actual date of successful repair or corrective action. | Y |  |
| 60.697(h) | For stormwater sewer systems, keep for the life of the facility in a readily accessible location, plans or specifications that demonstrate that no wastewater from any process units or equipment is directly discharged to the stormwater sewer system. | Y |  |
| 60.697(i) | For ancillary equipment, keep for the life of a facility in a readily accessible location, plans or specifications that demonstrate that the ancillary equipment does not come in contact with or store oily wastewater. | Y |  |
| 60.697(j) | For non-contact cooling water systems, keep for the life of the facility in a readily accessible location, plans or specifications which demonstrate that the cooling water does not contact hydrocarbons or oily wastewater and is not recirculated through a cooling tower. | Y |  |
| 60.698 | Reporting Requirements | Y |  |
| 60.698(b)(1) | Submit semiannually a certification that all of the required inspections have been carried out in accordance with these standards. | Y |  |
| 60.698(c) | Submit semiannually report that summarizes inspections when a water seal was dry or otherwise breached, drain cap or plug was missing or when cracks, gaps or other problems were identified , and information about repairs. | Y |  |
| BAAQMD Regulation 10-69 | Incorporates by reference 40 CFR 60 Subpart QQQ | Y |  |

| - Table IV – DR - Applicable Requirements - Subpart CC Miscellaneous Process Vents |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Future Effective - Date |
| MACT Part 63 Subpart A | General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y |  |
| 63.6(e) | Operation and Maintenance | Y |  |
| 63.7 | Performance Testing Requirements | Y |  |
| 63.8 | Monitoring Requirements | Y |  |
| 63.9 | Notification Requirements | Y |  |
| 63.10 | Recordkeeping and Reporting Requirements | Y |  |
| MACT Part 63 Subpart CC | National Emission Standards for Hazardous Air Pollutants from Petroleum Refineries |  |  |
| 63.643 | Standards for Miscellaneous Process Vents | Y |  |
| 63.643(a)(2) | Reduce emissions of organic HAPs using control device, by 98 weight percent or to concentration of 20 ppmvd, at 3% oxygen, whichever is less stringent. Demonstrate compliance by measuring either organic HAPs or TOCs using procedure in 63.645. | Y |  |
| 63.643(b) | If boiler or process heater is used to comply with 63.643(a)(2), then vent stream introduced into flame zone or location such that required reduction or concentrations achieved. Testing and monitoring is required only as specified in 63.644(a) and 63.645. | Y |  |
| 63.644 | Monitoring Provisions for Miscellaneous Process Vents | Y |  |
| 63.644(a)(3) | Any boiler or process heater with a design heat input capacity greater than or equal to 44 megawatt or any boiler or process heater in which all vent streams are introduced into the flame zone is exempt from monitoring. | Y |  |
| 63.644(c)(2) | If Group 1 miscellaneous process vent contain bypass lines, secure bypass line valve in closed position per measures in 63.644(c)(2). Closure mechanism shall be visually inspected once every month. | Y |  |
| 63.654 | Reporting and Recordkeeping Requirements | Y |  |
| 63.654(e)(3) | Records include reports described in 63.654(h). | Y |  |
| 63.654(h)(1) | Reports of startup, shutdown and malfunction. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| - Table IV – DS - Applicable Requirements - Subpart CC Equipment Leaks for Equipment in Organic HAP Service |  |  |  |
| --- | --- | --- | --- |
| - Applicable Requirement | - Regulation Title or - Description of Requirement | - Federally Enforceable (Y/N) | - Future Effective - Date |
| MACT Part 63 Subpart A | General Provisions |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y |  |
| 63.6(e) | Operation and Maintenance | Y |  |
| 63.7 | Performance Testing Requirements | Y |  |
| 63.8 | Monitoring Requirements | Y |  |
| 63.9 | Notification Requirements | Y |  |
| 63.10 | Recordkeeping and reporting requirements | Y |  |
| MACT Part 63 Subpart CC | National Emission Standards for Hazardous Air Pollutants from Petroleum Refineries |  |  |
| 63.648 | Equipment Leak Standards | Y |  |
| 63.648(a) | Existing source shall comply with the provisions of 40 CFR 60 Subpart VV and paragraph (b) of this section except as provided in paragraphs 63.648(a)(1), (a)(2), and (f) through (i). | Y |  |
| 63.648(f) | Reciprocating pumps in light liquid service are exempt from 60.482 if recasting the distance piece or reciprocating pump replacement is required. |  |  |
| 63.648(g) | Compressors in hydrogen service are exempt from the requirements of 63.648(a) if an owner operator demonstrates that a compressor is in hydrogen service. | Y |  |
| 63.648(h) | Maintain all records for a minimum of 5 years. | Y |  |
| 63.648(i) | Reciprocating compressors are exempt from seal requirements if recasting the distance piece or compressor replacement is required. | Y |  |
| 63.654 | Reporting and Recordkeeping Requirements | Y |  |
| 63.654(d)(1)(i) | Comply with recordkeeping and reporting provisions of 60.486 and 60.487 except the name of person whose decision it was that a repair could be effected without a process shutdown shall be recorded and retained for 2 years; a signature is not required. | Y |  |
| 63.654(d)(3) | For compressors that qualify for the hydrogen service exemption keep records of the demonstration required by 63.648(g) | Y |  |
| 63.654(d)(5) | Identify, either by list or location, equipment in organic HAP service less than 300 hours per year within refining process units subject to this subpart. | Y |  |
| 63.654(d)(6) | Keep a list of reciprocating pumps and compressors determined to be exempt from seal requirements as per 63.648(f) and (i). | Y |  |
| 63.654(e)(3) | Records include reports described in 63.654(h). | Y |  |
| 63.654(h)(1) | Reports of startup, shutdown and malfunction. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |
| NSPS Part 60 Subpart VV; BAAQMD Regulation 10-52 | Standards of Performance for Equipment Leaks (Fugitive Emission Sources) (8/18/95); BAAQMD Standards of Performance for New Stationary Sources (12/20/95) |  |  |
| 60.480 | Applicability and designation of affected facility | Y |  |
| 60.480(d) | An affected facility that qualifies for one or more exemption from 60.482 shall maintain records as required in 60.486(i). | Y |  |
| 60.482-1 | Standards: General | Y |  |
| 60.482-1(b) | Compliance with 60.482-1 to 60.482-10 will be determined…. | Y |  |
| 60.482-1(d) | Equipment that is in vacuum service is excluded from the requirements of 60.482-2 to 60.482-10 if it is identified as required in 60.486(e)(5). | Y |  |
| 60.482-2 | Standards: Pumps in light liquid service | Y |  |
| 60.482-2(a)(1) | Monthly monitoring of each pump, except for 60.482-2(d). | Y |  |
| 60.482-2(a)(2) | Weekly visual inspection of each pump. | Y |  |
| 60.482-2(b)(1) | Air measurement instrument reading >10,000 ppm indicates leak | Y |  |
| 60.482-2(b)(2) | Dripping liquid from pump seal indicates leak | Y |  |
| 60.482-2(c)(1) | Leak repaired within 15 calendar days, except as provided in 60.482-9. | Y |  |
| 60.482-2(c)(2) | First attempt at leak repair made within 5 calendar days. | Y |  |
| 60.482-2(d) | Pump with dual-mechanical seal system that includes barrier fluid system and meets specified requirements is exempt from 60.482-2(a). | Y |  |
| 60.482-2(g) | - Pump designated, per 60.486(f)(1), as unsafe-to-monitor pump is exempt from 60.482-2(a) and (d)(4) through (d)(6) if hazard documented and written monitoring plan is followed. | - Y |  |
| 60.482-2(h) | - Any pump located in an unmanned plant site is exempt from the requirements of 60.482-2(a)(2), (d)(4) and (d)(5) provided each pump is visually inspected as often as practicable and at least monthly. | - Y |  |
| 60.482-4 | - Standards: Pressure relief devices in gas/vapor service | - Y |  |
| 60.482-4(a) | - Except during pressure releases, pressure relief device shall be operated with no detectable emissions (< 500 ppm). | - Y |  |
| 60.482-4(b)(1) | After each pressure release, pressure release device shall be returned to a condition of no detectable emissions within 5 calendar days after pressure release, except as provided in 60.482-9. | - Y |  |
| 60.482-4(b)(2) | - No later than 5 calendar days after pressure release, the pressure relief device shall be monitored to confirm no detectable emissions. | - Y |  |
| 60.482-4(c) | - Any pressure relief device that is routed to a process or fuel gas system is exempt from 60.482-4(a) and (b). | - Y |  |
| 60.482-4(d)(1) | - Any pressure relief devise that is equipped with a rupture disk upstream of the pressure relief device is exempt from 60.482-4(a) and (b) provided complies with 60.482-4(d)(2). | - Y |  |
| 60.482-4(d)(2) | - After each pressure release, a new rupture disk shall be installed upstream of the pressure relief device as soon as practicable, but no later than 5 calendar days after each pressure release, except as provided in 60.482-9. | - Y |  |
| 60.482-5 | - Standards: Sampling connecting systems | - Y |  |
| 60.482-6 | - Standards: Open-ended valves or lines | - Y |  |
| 60.482-7 | - Standards: Valves in gas/vapor service and in light liquid service | - Y |  |
| 60.482-7(a) | - Monitor monthly to detect leaks, except as provided in 60.482-7(g) and (h) and 60.483-2. | - Y |  |
| 60.482-7(b) | - Instrument reading >10,000 ppm indicates leak. | - Y |  |
| 60.482-7(c) | - Valve that does not have a detectable leak for 2 successive months, can be monitored the first month of every quarter. | - Y |  |
| 60.482-7(d)(1) | - Leak shall be repaired within 15 calendar days, except as provided in 60.482-9. | - Y |  |
| 60.482-7(d)(2) | - First attempt at leak repair shall be made within 5 calendar days. | - Y |  |
| 60.482-7(e) | - Methods for first attempt at repair. | - Y |  |
| 60.482-7(g) | - Valve designated, per 60.486(f)(1), as unsafe-to-monitor valve is exempt from 60.482-7(a) if hazard documented and written monitoring plan is followed. | - Y |  |
| 60.482-7(h) | - Valve designated, per 60.486(f)(1), as difficult-to-monitor valve is exempt from 60.482-7(a) if hazard documented, less than 3% of facility valves are designated and written plan with is followed that requires monitoring at least once per year. | - Y |  |
| 60.482-8 | - Standards: Pumps and valves in heavy liquid service, pressure relief devices in light liquid or heavy liquid service, and flanges and other connectors. | - Y |  |
| 60.482-8(a) | - Monitor within 5 days if evidence of potential leak is found. | - Y |  |
| 60.482-8(b) | - Instrument reading >10,000 ppm indicates leak. | - Y |  |
| 60.482-8(c)(1) | - Leak shall be repaired within 15 calendar days, except as provided in 60.482-9. | - Y |  |
| 60.482-8(c)(2) | - First attempt at leak repair shall be made within 5 calendar days. | - Y |  |
| 60.482-8(d) | - Minimum requirements for first attempt at repair. | - Y |  |
| 60.482-9 | - Standards: Delay of Repair |  |  |
| 60.482-9(a) | - Delay allowed if repair is technically infeasible without a process unit shutdown and repair occurs before end of next process unit shutdown. | - Y |  |
| 60.482-9(b) | - Repair may be delayed for isolated equipment. | - Y |  |
| 60.482-9(c) | - Delay of repair for valves only allowed under certain circumstances. | - Y |  |
| 60.482-9(d)(1) | - Only dual-mechanical seal pumps qualify for delay of repair | - Y |  |
| 60.482-9(d)(2) | - Pump leaks must be repaired within 6 months. | - Y |  |
| 60.482-9(e) | - Delay of repair beyond process shutdown allowed if valve assembly replacement is required and other circumstances are met. | - Y |  |
| 60.483-2 | - If a process unit has 5 consecutive quarters with <2% of valves leaking at >10,000 ppm, then any individual valve which measures <100 ppm for 5 consecutive quarters may be monitored annually. | - Y |  |
| 60.485 | - Test Methods and Procedures | - Y |  |
| 60.485(a) | - Performance tests methods specified in Appendix A or 60.8(b) | - Y |  |
| 60.485(b) | - Method 21 for determining presence of leaking sources. | - Y |  |
| 60.485(d) | - Test each piece of equipment unless process unit not in VOC series. | - Y |  |
| 60.485(e) | - Light liquid service demonstrated by vapor pressure and if liquid at operating conditions. | - Y |  |
| 60.485(f) | - Samples representative of process fluid. | - Y |  |
| 60.486 | - Record keeping Requirements | - Y |  |
| 60.486(a) | - Comply with recordkeeping requirements of this section. | - Y |  |
| 60.486(b) | - Identification and tagging requirements for leaks detected as specified in 60.482-2, 60.482-3, 60.482-7, 60.482-8, and 60.483-2. | - Y |  |
| 60.486(c) | - When leak detected as specified in 60.482-2, 60.482-3, 60.482-7, 60.482-8, and 60.483-2, record in log and keep for 2 years. | - Y |  |
| 60.486(e) | - Information to be recorded for all equipment subject to requirements in 60.482-1 through 60.482-10. | - Y |  |
| 60.486(f) | Record information pertaining to all valves subject to the requirements in 60.482-7(g) and (h). | - Y |  |
| 60.486(g) | - Record information pertaining to all valves subject to the requirements in 60.483-2. | - Y |  |
| 60.486(h) | - Record design criterion required in 60.482-2(d)(5) and 60.482-3(e)(2). | - Y |  |
| 60.486(i) | - Record information in log that is readily accessible for use in determining exemption as provided in 60.480(d). | - Y |  |
| 60.486(j) | - Records to demonstrate piece of equipment not in VOC service. | - Y |  |
| 60.486(k) | - Provisions of 60.7(b) and (d) do not apply if subject to VV. | - Y |  |
| 60.487 | - Reporting Requirements | - Y |  |
| 60.487(a) | Submit semiannual reports. | Y |  |
| 60.487(c) | Information to be included in semiannual reports. | Y |  |
| 60.487(e) | Report results of all performance tests in accordance with 60.8. The provisions of 60.8(d) do not apply to affected facilities subject to VV. | Y |  |
| 60.487(f) | The requirements of paragraphs (a) through (c) of this section remain in force until and unless EPA, in delegating enforcement authority to a State, approves reporting requirements or an alternative means of compliance surveillance adopted by such State. | Y |  |
| BAAQMD Regulation 10-52 | Incorporates by reference 40 CFR 60 Subpart VV | Y |  |

| Table IV – DT Source-specific Applicable Requirements Subpart FF containers |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| District Regulation 11 Rule 12 | Hazardous Pollutants – National Emission Standards for Benzene Emissions from Benzene Transfer Operations and Benzene Waste Operations (7/18/90, Refer to NESHAP Subpart FF below) |  |  |
| NESHAP Part 61 Subpart A | National Emission Standards for Hazardous Air Pollutants - General Provisions (3/16/95) |  |  |
| 61.10(h), (i), and (j) | Source reporting and waiver request | Y |  |
| 61.12(a), (b) and (c) | Compliance with standards and maintenance requirements | Y |  |
| 61.13(b), (c), (d), (e), (f), (g) and (h) | Emission tests and waiver of emission tests | Y |  |
| 61.14 | Monitoring requirements | Y |  |
| 61.19 | Circumvention | Y |  |
| NESHAP Part 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.345 | Standards: Containers (3/7/90) | Y |  |
| 61.345(a)(1)(i) | Cover and all openings on each container used to handle, transfer, or store waste shall be designed, operated and maintained to < 500 ppmv above background. Instrument reading at least once per year. | Y |  |
| 61.345(a)(1)(ii) | Keep all openings on container closed, in sealed position except when necessary for waste loading, removal, inspection, or sampling. | Y |  |
| 61.345(a)(2) | Wastes transferred into a container by pumping, use submerged fill pipe with outlet extending to within 2 fill pipe diameters from the bottom. During loading, cover shall remain in place and all openings closed, sealed, except for openings required for submerged fill pipe, and openings required for venting to prevent physical damage or deformation of container or cover. | Y |  |
| 61.345(b) | Covers and openings shall be visually inspected initially and quarterly to ensure they are closed and gasketed properly. | Y |  |
| 61.345(c) | Except as provided in 61.350, when a broken seal or gasket or other problem is identified, first efforts to repair shall be made as soon as practicable, but no later than 15 calendar days after identification. | Y |  |
| 61.350 | Delay of Repair | Y |  |
| 61.355 | Test methods, procedures and compliance provisions | Y |  |
| 61.355(h) | Leak inspection procedures | Y |  |
| 61.356 | Recordkeeping Requirements | Y |  |
| 61.356(a) | Retain records for two years | Y |  |
| 61.356(g) | Maintain record for each visual inspection that identifies problem. | Y |  |
| 61.356(h) | Maintain records of tests of no detectable emissions. | Y |  |
| 61.357 | Reporting Requirements | Y |  |
| - 61.357(d)(6) | Submit quarterly certification that all required inspections have been conducted. | Y |  |
| - 61.357(d)(8) | Submit annual summary of all inspections during which detectable emissions are measured or a problem is identified, repaired or corrected. | Y |  |
| - MACT Part 63 Subpart A | National Emission Standards for Hazardous Air Pollutants for Source Categories (3/16/94, Note: For details see Table IV-N below) |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y |  |
| 63.6(e) | Operation and Maintenance | Y |  |
| 63.7 | Performance Testing Requirements | Y |  |
| 63.8 | Monitoring Requirements | Y |  |
| 63.9 | Notification Requirements | Y |  |
| 63.10 | Recordkeeping and Reporting Requirements | Y |  |
| MACT Part 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 | Wastewater Provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR 61, Subpart FF for each stream that meets the definition of 63.641. | Y |  |
| 63.647(c) | Owners/operators required under 40 CFR 61, Subpart FF to perform periodic measurement of benzene concentration in wastewater, or to monitor process or control device operating parameters shall operate consistently with the permitted concentration or operating parameter values. | Y |  |
| 63.654 | Reporting and Recordkeeping Requirements | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR 61, Subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| Table IV – DU Source-specific Applicable Requirements Subpart FF Individual Drain Systems |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| District Regulation 11 Rule 12 | Hazardous Pollutants – National Emission Standards for Benzene Emissions from Benzene Transfer Operations and Benzene Waste Operations (7/18/90, Refer to NESHAP Subpart FF below) |  |  |
| NESHAP Part 40 Subpart A | National Emission Standards for Hazardous Air Pollutants - General Provisions (3/16/95) |  |  |
| 61.10(h), (i), and (j) | Source reporting and waiver request | Y |  |
| 61.12(a), (b) and (c) | Compliance with standards and maintenance requirements | Y |  |
| 61.13(b), (c), (d), (e), (f), (g) and (h) | Emission tests and waiver of emission tests | Y |  |
| 61.14 | Monitoring requirements | Y |  |
| 61.19 | Circumvention | Y |  |
| NESHAP Part 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.346 | Standards: Individual Drain Systems | Y |  |
| 61.346(b) | Alternative to complying with 61.346(a) | Y |  |
| 61.346(b)(3) | Each sewer line shall not be open to the atmosphere and shall be covered or enclosed in a manner so as to have no visual gaps or cracks in joints, seals or other emission interfaces. | Y |  |
| 61.346(b)(4)(iv) | Unburied portion of each sewer line shall be visually inspected quarterly for indications of cracks, gaps or other problems. | Y |  |
| 61.346(b)(5) | Except as provided in 61.350, first efforts at repair shall be made as soon as practicable, but not later than 15 calendar days after identification. | Y |  |
| 61.350 | Delay of Repair | Y |  |
| 61.356 | Recordkeeping Requirements | Y |  |
| 61.356(a) | Retain records for two years. | Y |  |
| 61.356(g) | Maintain record for each visual inspection that identifies problem | Y |  |
| 61.357 | Reporting requirements | Y |  |
| - 61.357(d)(6) | Submit quarterly certification that all required inspections have been conducted. | Y |  |
| - 61.357(d)(8) | Submit annual summary of all inspections during which detectable emissions are measured or a problem is identified, repaired or corrected. | Y |  |
| - NESHAP - Part 63 - Subpart A | National Emission Standards for Hazardous Air Pollutants for Source Categories (Note: For details see Table IV-N below) | Y |  |
| 63.4 | Prohibited Activities and Circumvention | Y |  |
| 63.6(e) | Operation and Maintenance | Y |  |
| 63.7 | Performance Testing Requirements | Y |  |
| 63.8 | Monitoring Requirements | Y |  |
| 63.9 | Notification Requirements | Y |  |
| 63.10 | Recordkeeping and Reporting Requirements | Y |  |
| MACT Part 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.647 | Wastewater Provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR 61, Subpart FF for each stream that meets the definition of 63.641. | Y |  |
| 63.647(c) | Owners/operators required under 40 CFR 61, Subpart FF to perform periodic measurement of benzene concentration in wastewater, or to monitor process or control device operating parameters shall operate consistently with the permitted concentration or operating parameter values. | Y |  |
| 63.654 | Reporting and Recordkeeping Requirements | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR 61, Subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |

| Table IV – DV Source-specific Applicable Requirements Facility |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| BAAQMD Regulation 1 | General Provisions and Definitions (5/02/01) |  |  |
| 1-510 | Area Monitoring | Y |  |
| 1-530 | Area Monitoring Downtime | Y |  |
| 1-540 | Area Monitoring Date Examination | Y |  |
| 1-542 | Area Concentration Excesses | Y |  |
| 1-543 | Record Maintenance for Two Years | Y |  |
| 1-544 | Monthly Summary | Y |  |
| - BAAQMD Regulation 2, Rule 1 | - General Requirements (8/1/01) |  |  |
| - 2-1-429 | - Federal Emissions Statement | - N |  |
| - SIP - Regulation 2, - Rule 1 | - General Requirements (01/26/99) |  |  |
| - 2-1-429 | - Federal Emissions Statement | - Y |  |
| - BAAQMD · Regulation 8, - Rule 5 | - Organic Compounds, Storage of Organic Liquids (06/05/03) |  |  |
| - 8-5-117 | - Exemption, Low Vapor Pressure | - Y |  |
| - BAAQMD - Regulation 8, Rule 15 | - Emulsified and Liquid Asphalts (09/16/87) |  |  |
| - 8-15-305 | - Prohibition of Manufacturer and Sale | - Y |  |
| - 8-15-501 | - Manufacturing Records | - Y |  |
| - BAAQMD - Regulation 8, Rule 16 | - Solvent Cleaning Operations (10/16/02) |  |  |
| - 8-16-111 | - Limited Exemption, Wipe Cleaning | - N |  |
| - 8-16-115 | - Limited Exemption, Small Unheated Solvent Cleaning Equipment | - N |  |
| - 8-16-118 | - Limited Exemption, Compounds with Low Volatility | - N |  |
| 8-16-303 | Cold Cleaner Requirements | N |  |
| 8-16-303.1 | General Operating Requirements | Y |  |
| 8-16-303.1.1 | Maintain equipment in good working order. | Y |  |
| 8-16-303.1.2 | Leak Repair Requirement | Y |  |
| 8-16-303.1.3 | Solvent Storage or Disposal – Evaporation Prevention | Y |  |
| 8-16-303.1.4 | Waste Solvent Disposal | N |  |
| 8-16-303.1.4(a) | Covered Containers for Waste Solvent Awaiting Pick-up | N |  |
| 8-16-303.1.4(b) | On-site Waste Treatment | N |  |
| 8-16-303.1.5 | Solvent Evaporation Minimization Devices shall not be Removed | N |  |
| 8-16-303.1.6 | Solvent Spray Requirements | N |  |
| 8-16-303.2 | Cold Cleaner Operating Requirements | Y |  |
| 8-16-303.2.1 | Solvent shall be Drained from Cleaned Parts | Y |  |
| 8-16-303.2.2 | Solvent Agitation | Y |  |
| 8-16-303.2.3 | Solvent Cleaning of Porous or Absorbent Materials is Prohibited | Y |  |
| 8-16-303.3 | Cold Cleaner General Equipment Requirements | N |  |
| 8-16-303.3.1 | Container | Y |  |
| 8-16-303.3.2 | Solvent Evaporation Reduction for Idle Equipment | N |  |
| 8-16-303.3.3 | Used Solvent Returned to Container | N |  |
| 8-16-303.3.4 | Label Stating Operating Requirements | N |  |
| 8-16-303.4 | Control Device (one of the following) | N |  |
| 8-16-303.4.1 | Freeboard Ratio > 0.75 | N |  |
| 8-16-303.5 | VOC content < 0.42 pounds per gallon or comply with 8-16-303.4.1 and other options | N |  |
| 8-16-501 | Solvent Records | N |  |
| 8-16-501.2 | Facility-wide Monthly Solvent Usage Records | N |  |
| 8-16-501.3 | Annual Records of Type and Amount of Solvent Used for Wipe Cleaning | N |  |
| 8-16-501.5 | Records Retained | N |  |
| SIP Regulation 8, Rule 16 | Solvent Cleaning Operations (12/9/94) |  |  |
| 8-16-111 | Limited Exemption, Wipe Cleaning | Y |  |
| 8-16-115 | Limited Exemption, Small Unheated Solvent Cleaning Equipment | Y |  |
| 8-16-118 | Limited Exemption, Compounds with Low Volatility | Y |  |
| 8-16-303.1.4 | Waste Solvent Disposal | Y |  |
| 8-16-303.1.4(a) | Covered Containers for Waste Solvent Awaiting Pick-up | Y |  |
| 8-16-303.1.4(b) | On-site Waste Treatment | Y |  |
| 8-16-303.1.5 | Solvent Evaporation Minimization Devices shall not be Removed | Y |  |
| 8-16-303.1.6 | Solvent Spray Requirements | Y |  |
| 8-16-303.3 | Cold Cleaner General Equipment Requirements | Y |  |
| 8-16-303.3.2 | Solvent Evaporation Reduction for Idle Equipment | Y |  |
| 8-16-303.3.3 | Used Solvent Returned to Container | Y |  |
| 8-16-303.3.4 | Label Stating Operating Requirements | Y |  |
| 8-16-303.4 | Control Device (one of the following) | Y |  |
| 8-16-303.4.1 | Freeboard Ratio > 0.75 | Y |  |
| 8-16-501 | Solvent Records | Y |  |
| 8-16-501.2 | Facility-wide Quarterly Solvent Usage Records | Y |  |
| BAAQMD Regulation 8, Rule 40 | Aeration of Contaminated Soil and Removal of Underground Storage Tanks (04/19/2001)) |  |  |
| 8-40-116 | Exemption, Small Volume | Y |  |
| 8-40-205 | Contaminated Soil | Y |  |
| 8-40-304 | Active Storage Piles | Y |  |
| 8-40-305 | Inactive Storage Piles | Y |  |
| 8-40-306 | Contaminated Soil – Excavation and Removal | Y |  |
| 8-40-306.4 | Requirements and Schedule | Y |  |
| 8-40-601 | Contaminated Soil Sampling | Y |  |
| 8-40-601.2 | Sampling Requirements for Exemption under 8-40-116.2 | Y |  |
| 8-40-601.3 | Sampling Requirements for Projects Subject to 8-40_306.4 with 250 cubic yards or less | Y |  |
| 8-40-601.4 | Sampling Requirements for Projects Subject to 8-40_306.4 with more than 250 cubic yards | Y |  |
| 8-40-604 | Measurement of Organic Concentration | Y |  |
| BAAQMD Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (3/15/95) |  |  |
| 9-1-110 | Conditional Exemption, Area Monitoring | Y |  |
| 9-1-301 | Limitations on Ground level Concentrations | Y |  |
| 9-1-313 | Sulfur Removal Operations at Petroleum Refineries (processing more than 20,000 bbl/day of crude oil) | N |  |
| 9-1-313.2 | Operation of a sulfur removal and recovery system that removes and recovers: 95% of H2S from refinery fuel gas, 95% of H2S and ammonia from process water streams | N |  |
| 9-1-501 | Area Monitoring Requirements (Regulations 1-510, 1-530, 1-540, 1-542, 1-543, 1-544) | Y |  |
| SIP - Regulation 9, Rule 1 | Inorganic Gaseous Pollutants – Sulfur Dioxide (06/08/99) |  |  |
| - 9-1-313.2 | - operation of a sulfur removal and recovery system that removes and recovers: 95% of H2S from refinery fuel gas, 95% of H2S and ammonia from process water streams | - Y |  |
| - BAAQMD Regulation 9, Rule 2 | - Inorganic Gaseous Pollutants- Hydrogen Sulfide (10/6/99) |  |  |
| - 9-2-301 | - Limitations on Ground Level Concentrations | - N |  |
| - 9-2-501 | - Area Monitoring Requirements | - N |  |
| - BAAQMD - Regulation 11, Rule 2 | - Asbestos Demolition, Renovation and Manufacturing (10/07/98) |  |  |
| - 11-2-301 | - Prohibited Operations | - N |  |
| - 11-2-302 | - Visible Emissions | - N |  |
| - 11-2-303 | - Demolition, Renovation, and Removal | - N |  |
| - 11-2-304 | - Waste Disposal | - N |  |
| - 11-2-305 | - Waste Disposal Sites | - N |  |
| - 11-2-501 | - Temperature Records | - N |  |
| - 11-2-502 | - Waste Shipment Records | - N |  |
| - 11-2-503 | - Active Waste Disposal Records | - N |  |
| - 11-2-504 | - Conversion Operations | - N |  |
| NESHAP Part 61 Subpart A | National Emission Standards for Hazardous Air Pollutants - General Provisions (3/16/95) |  |  |
| 61.10(h), (i), and (j) | Source reporting and waiver request | Y |  |
| 61.12(a), (b) and (c) | Compliance with standards and maintenance requirements | Y |  |
| 61.13(b), (c), (d), (e), (f), (g) and (h) | Emission tests and waiver of emission tests | Y |  |
| 61.14 | Monitoring requirements | Y |  |
| 61.19 | Circumvention | Y |  |
| NESHAP Part 61 Subpart M | National Emission Standard for Asbestos (11/20/90) |  |  |
| NESHAP Part 61 Subpart FF; BAAQMD 11-12 | National Emission Standard for Benzene Waste Operations (3/7/90) |  |  |
| 61.342 | Standards: General | Y |  |
| 61.342(b) | Facilities with 10 Mg/yr or more of benzene in waste shall comply with 61.342(c) through 61.342(h) | Y |  |
| 61.342(e) | Alternative to 61.342(c) and 61.342(d) standards for facilities with 10 Mg/yr or more of benzene in waste | Y |  |
| 61.342(e)(1) | Non aqueous wastes (with a flow-weighted annual average water content of less than 10%) shall be treated in accordance with 61.342(c)(1) | Y |  |
| 61.342(e)(2) | Uncontrolled aqueous wastes (with a flow-weighted annual average water content of 10% or more by volume)shall be limited to 6 Mg/yr | Y |  |
| - 61.342(g) | Compliance with 61.342(e) is determined by review of facility records and results from tests and inspections using methods and procedures specified in 61.355. | Y |  |
| - 61.355 | Test Methods, Procedures, and Compliance Procedures | Y |  |
| - 61.355(a) | Procedures for determining total annual benzene quantity from facility waste. | Y |  |
| - 61.355(b) | For purposes of the calculation required by 61.355(a), determine the annual waste quantity at the point of waste generation, unless otherwise provided in paragraphs (b)(1), (2), (3), and (4) of this section, by one of the methods given in paragraphs (b)(5) through (7) of this section. | Y |  |
| - 61.355(c) | For the purposes of the calculation required 61.355(a), determine the flow-weighted annual average benzene concentration in a manner that meets the requirements given in paragraph (c)(1) of this section using either of the methods given in paragraphs (c)(2) and (c)(3) of this section. | Y |  |
| - 61.355(k) | Procedure for determining the benzene quantity for the purposes of the calculation required by 61.342(e)(2). | Y |  |
| - 61.356 | Recordkeeping Requirements | Y |  |
| - 61.356(a) | Retain records for two years. | Y |  |
| - 61.356(b) | Maintain records that identify each waste stream at the facility subject to this subpart, and indicate whether or not the waste stream is controlled for benzene emissions in accordance with this subpart. In addition the owner or operator shall maintain records specified in this section. | Y |  |
| - 61.357 | Reporting requirements | Y |  |
| - 61.357(a) | Specifies information to be included in annual report. | Y |  |
| - 61.357(d)(2) | Annually submit a report to the Administrator that updates the information listed in 61.357(a)(1) through (a)(3). If the information in the annual report required by 61.357(a)(1) through (a)(3) of this section is not changed in the following year, the owner or operator may submit a statement to that effect. | Y |  |
| - 61.357(d)(5) | If an owner or operator elects to comply with the alternative requirements 61.342(e), then the report required by paragraph 61.357(d)(2) shall include a table presenting the information specified in this section for each waste stream. | Y |  |
| - NESHAP - Part 63 - Subpart A | National Emission Standards for Hazardous Air Pollutants for Source Categories |  |  |
| 63.4 | Prohibited Activities and Circumvention | Y |  |
| 63.6(e) | Operation and Maintenance | Y |  |
| 63.7 | Performance Testing Requirements | Y |  |
| 63.8 | Monitoring Requirements | Y |  |
| 63.9 | Notification Requirements | Y |  |
| 63.10 | Recordkeeping and Reporting Requirements | Y |  |
| 40 CFR 63 | National Emission Standards for Hazardous Air Pollutants for Source Categories: General Provisions; and Requirements for Control Technology Determinations for Major Sources in Accordance with Clean Air Act Sections, Section 112(g) and 112(j); Final Rule |  |  |
| 63.52 | Approved process for new and existing affected sources. | Y |  |
| 63.52(a) | Sources subject to section 112(j) as of the section 112(j) deadline | Y |  |
| 63.52(a)(1) | Submit an application for Title V permit revision | Y |  |
| 63.52(e) | Permit application review | Y |  |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Combustion Turbines | Y | 12/29/03 |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Organic Liquids Distribution | Y | 12/29/03 |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Site Remediation | Y | 12/29/03 |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Process Heaters | Y | 6/27/04 |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Reciprocating Internal Combustion Engines | Y | 6/27/04 |
| 63.52(e)(1) | Submit a Part 2 MACT application meeting the requirements of 63.53(b) for Process Heaters (that burn hazardous waste) | Y | 11/12/05 |
| 63.52(h) | Enhanced monitoring | Y |  |
| 63.52(h)(i) | MACT emission limitations | Y |  |
| 63.52(h)(i)(1) | Compliance with all requirements applicable to affected sources, including compliance date for affected sources | Y |  |
| 63.53 | Application content for case-by-case MACT determination | Y |  |
| 63.53(a) | Part 1 MACT application | Y |  |
| 63.53(b) | Part 2 MACT application | Y |  |
| MACT Part 63 Subpart CC | National Emissions Standards for Hazardous Air Pollutants from Petroleum Refineries (8/18/95) |  |  |
| 63.642 | General Standards | - Y |  |
| 63.642(k) | As an alternative to compliance with 63.642(g), comply with the provisions in 63.643 through 63.647. | - Y |  |
| - 63.642(k)(1) | - Comply with applicable provisions of 63.654. | - Y |  |
| - 63.642(k)(2) | - Not required to calculate the annual emission rate specified in 63.642(g) | - Y |  |
| 63.647 | Wastewater Provisions | Y |  |
| 63.647(a) | Owners/operators of Group 1 wastewater streams shall comply with sections 61.340 to 61.355 of 40 CFR 61, Subpart FF for each stream that meets the definition of 63.641. | Y |  |
| 63.647(c) | Owners/operators required under 40 CFR 61, Subpart FF to perform periodic measurement of benzene concentration in wastewater, or to monitor process or control device operating parameters shall operate consistently with the permitted concentration or operating parameter values. | Y |  |
| 63.654 | Reporting and Recordkeeping Requirements | Y |  |
| 63.654(a) | Owner/operators subject to the wastewater provisions of 63.647 shall comply with the recordkeeping and reporting requirements in 61.356 and 61.357 of 40 CFR 61, Subpart FF, unless they comply with those specified in paragraph (o)(2)(ii) of 63.640. Recordkeeping and reporting for wastewater streams included in emission averages are specified in 63.653 and in paragraphs (f)(5) and (g)(8) of this section. | Y |  |
| 63.654(i)(4) | All information required to be reported under 63.654(a) through (h) shall be retained for 5 years. | Y |  |
| BAAQMD Condition # 12271 |  |  |  |
| Part 50 | Isom Feed storage throughput limit [basis: Cumulative Increase] | Y |  |
| Part L | Recordkeeping [basis: BACT, Offsets, Cumulative Increase] | Y |  |
| Part M | Monthly reports [basis: Offsets, Cumulative Increase] | Y |  |
| BAAQMD Condition # 18618 |  |  |  |
| Part 11 | Notification of startup or shutdown [basis: Regulation 2-1-403] | N |  |
| BAAQMD Condition #20762 |  |  |  |
| Part 1 | Verify true vapor pressure (8-5-117) | Y |  |
| Part 2 | Recordkeeping (8-5-117) | Y |  |

| Table IV – DW Source-specific Applicable Requirements Exempt Tanks S1-Tank 1, S2-Tank 2, S8-Tank 8, S10-Tank 10, S25-Tank 25, S28-Tank 28, S30-Tank 30, S31-Tank 31, S32-Tank 32, S33-Tank 33, S35-Tank 35, S36-Tank 36, S38-Tank 38, S49-Tank 49, S50-Tank 50, S53-Tank 53, S56-Tank 56, S84-Tank 84, S90-Tank 90, S109-Tank 109, S130-Tank 130, S145-Tank 145, S146-Tank 146, S147-Tank 147, S148-Tank 148 S149-Tank 149, S222-Tank 222, S258-Tank 258, S259-Tank 259, S260-Tank 260, S261-Tank 261, S262-Tank 262, S263-Tank 263, S267-Tank 267, S332-Tank 332, S334-Tank 334, S335-Tank 335, S336-Tank 336, S337-Tank 337, S343-Tank 343, S344-Tank 344, S346-Tank 346, S356-Tank 356, S368-Tank 368 S393-Tank 393, S394-Tank 394, S398-Tank 398, S400-Tank 400, S401-Tank 401, S418-Tank 418, S419-Tank 419, S422-Tank 422, S423-Tank 423, S424-Tank 424, S426-Tank 426, S427-Tank 427, S428-Tank 428, S467-Tank 467, S468-Tank 468, S475-Tank 475 S476-Tank 476, S477-Tank 477, S478-Tank 478, S485-Tank 485, S514-Tank 514, S523-Tank 523, S524-Tank 524, S525-Tank 525, S526-Tank 526, S536-Tank 536 S537-Tank 537, S597-Tank 597, S608-Tank 608, S609-Tank 609, S713-Tank 713, S714-Tank 714, S786-Tank 786, S787-Tank 787, S804-Tank 804, S806-Tank 806, S814-Tank 814, S822-Tank 822, S833-Tank 833 S834-Tank 834, S835-Tank 835, S837-Tank 837, S839-Tank 839, S850-Tank 850, S851-Tank 851, S869-Tank 869, S877-Tank 877, S879-Tank 879, S880-Tank 880, S888-Tank 888, S890-Tank 890, S902-Tank 902, S903-Tank 903, S926-Tank 926, S927-Tank 927, S928-Tank 928, S929-Tank 929, S932-Tank 932, S933-Tank 933, S934-Tank 934, S935-Tank 935, S942-Tank 942, S953-Tank 953, S954-Tank 954, S955-Tank 955, S956-Tank 956 S957-Tank 957, S958-Tank 958, S959-Tank 959, S962-Tank 962, S965-Tank 965, S966-Tank 966, S968-Tank 968, S980-Tank 980 (Butane Sphere), S981-Tank 981, S987-Tank 987, S988-Tank 988, S993-Tank 993, S1000-Tank 1000, S1001-Tank 1001, S1003-Tank 1003, S1009-Tank 1009, S1012-Tank 1012, S1013-Tank 1013, S1020-Tank 1020, S1024-Tank 1024, S1026-Tank 1026, S1027-Tank 1027 S1032-Tank 1032, S1034-Tank 1034, S1040-Tank 1040, S1047-Tank 1047 (Butane Sphere), S1049-Tank 1049, S1052-Tank 1052, S1053-Tank 1053, S1054-Tank 1054, S1055-Tank 1055, S1056-Tank 1056, S1057-Tank 1057, S1058-Tank 1058, S1059-Tank 1059, S1060-Tank 1060, S1061-Tank 1061, S1062-Tank 1062, S1066-Tank 1066, S1068-Tank 1068, S1069-Tank 1069, S1071-Tank 1071, S1073-Tank 1073, S1074-Tank 1074, S1081-Tank 1081, S1084-Tank 1084, S1112-Tank 1112, S1125-Tank 1125, S1128-Tank 1128, S1132-Tank 1132, S1142-Tank 1142, S1143-Tank 1143, S1144-Tank 1144, S1149-V 1049 Butane Sphere, S1150-V 1150 Butane Sphere, S1151-V 1151 Butane Sphere, S1152-V 1152 Propane Bullet, S1153-V 1153 Propane Bullet, S1154-V 1154 Propane Bullet, S1165-Tank 1165, S1173-Tank 1173, S1174-Tank 1174, S1175-Tank 1175, S1185-Tank 1185, S1190-Tank 1190, S1231-Tank 1231 Fresh DEA, S1347-Tank 1347, S1348-Tank 1348, S1564-Tank 1564, S1780- Tank 1338 Butane Sphere, S1781- Tank 1341 Propane Bullet, S2006-Tank 906 Caustic Service, S5123-LOG V-15115 Standby Proto Vessel, S5124-LOG V-15116 Standby Proto Vessel, S12166-Tank 12166 |  |  |  |
| --- | --- | --- | --- |
| Applicable Requirement | Regulation Title or Description of Requirement | Federally Enforceable (Y/N) | Future Effective Date |
| - BAAQMD · Regulation 8, - Rule 5 | - Organic Compounds, Storage of Organic Liquids (06/05/03) |  |  |
| - 8-5-117 | - Exemption, Low Vapor Pressure | - Y |  |

# V. SCHEDULE OF COMPLIANCE

- The permit holder shall comply with all applicable requirements cited in this permit.  The permit holder shall also comply with applicable requirements that become effective during the term of this permit on a timely basis.

# VI. PERMIT CONDITIONS

Any condition that is preceded by an asterisk is not federally enforceable.

Condition # 4041

For S1803, OPCEN Coke Corral:

- The S1803 OPCEN Coke Corral shall only be used for material handling and storage of petroleum coke materials, refractory and Stretford sulfur.  The owner/operator may also use this facility to store and/or handle materials that do not result in air emissions, or materials that are exempt from permit requirements, per Regulation 2, Rule 1.  Before storing and/or handling any other material which would require a permit, the owner/operator shall apply for, and receive approval of, a modification to S1803 to include the additional material. [basis: Cumulative Increase]

- Prior to leaving the S1803 OPCEN Coke Corral all vehicles shall have the tires and wheels washed to remove and collect fines any time that coke fines and/or sulfur fines are stockpiled. [basis: Cumulative Increase; Public Nuisances]

- Visible particulate emissions from all activity at S1803 shall not exceed Ringelmann No. 0.5 (10% opacity), and/or result in fallout on adjacent property in such quantities as to cause annoyance to any other person.  [basis: Cumulative Increase]

- The A1803 Water Spray Sprinklers for OPCEN Coke Corral shall be maintained and shall operate as necessary to meet the limitation of Condition No. 3. [basis: Cumulative Increase]

- Any stockpiles and/or containers in the S1803 OPCEN Coke Corral (Coke Recovery and Temporary Storage Facility) shall be covered at all times except during periods of when:

- material is being added or removed from the stockpile and/or containers,

- manual removal of undesirable objects (such gloves, wood) in the material is being conducted, and

- samples are being taken from the stockpile and/or container for analyses.

It may be covered with a tarp or other covering provided no head space where wind may flow under the covering occurs and provided the covering is in good condition and is secured adequately so as to minimize emissions to the atmosphere necessary to meet the limitation of Condition No. 3. [basis: Cumulative Increase]

- In no event shall stockpiles in the storage yard area of S1803 OPCEN Coke Corral be greater than 6 feet in height.  The existing six foot high concrete wall shall be increased at least 2 feet in order to provide an 8 foot high windbreak.  The 2 foot extension shall be made of solid material. [basis: Cumulative Increase]

- Condition deleted.

- Any petroleum coke-oil mixture to be temporarily stored at S1803 OPCEN Coke Corral shall be stored in containers.  The vapor pressure of the oil content of this coke-oil mixture shall not exceed 0.5 psig Reid vapor pressure (RVP). [basis: Cumulative Increase]

- Condition deleted.

10.	S1803 shall be checked for visible emissions quarterly.  The visible emissions check shall take place while the equipment is operating and during daylight hours.  If any visible emissions are detected the operator shall take corrective action within one day, and check for visible emissions after corrective action is taken.  If no visible emissions are detected, the operator shall continue to check for visible emissions at the same frequency.  [basis: Regulation 2-6-409.2]

11.	The owner/operator shall keep records of all visible emissions checks, the person performing the check, and all corrective action taken at S1803.  The records shall be retained for five years and shall be made available to District personnel upon request.  [basis: Regulation 2-6-409.2]

12.	If a coke-oil mixture is temporarily stored at S1803, the owner/operator shall sample and analyze the liquid phase of the coke-oil mixture (if present) to verify compliance with Part 8.  A representative sample of the liquid phase of the coke-oil mixure shall be obtained and sent for analysis of vapor pressure within 30 days of being placed in storage.  [basis: Regulation 2-6-409.2]

Condition # 4101

For S1523, LUBS Loading Rack Asphalt Inside T/T:

- S1523 shall be abated by A3 whenever S1523 is in operation. [basis: Cumulative Increase]

Condition # 4288

For S2001, LOG Marine Loading Berth 1

S2002, LOG Marine Loading Berth 2

S2003, LOG Marine Loading Berth 3

S2004, LOG Marine Loading Berth 4:

1.   A/C Condition deleted.

2.  	Condition deleted.

3.	The owner/operator shall do all of the following:

- Install pressure and temperature monitors and recorders on the A100 system. [basis: Regulation 8-44-301]

- The owner/operator shall reduce the baseline VOC emissions under the Emission CAP, permit #26786, for wharf activity for gasoline components, crude oils and any other material subject to Regulation 8-44, by 95%, by weight.  This baseline adjustment is effective upon expiration of the 1991 District Variance for the Marine Vapor Recovery System. [basis: BARCT-Adjustment]

- For the purposes of the tracking of the wharf emissions in the facility Emission CAP, the owner/operator shall calculate uncontrolled emissions as specified in Permit #26786 and use a 95% by weight, reduction factor to determine controlled emissions, unless the thermal oxidizer, A100 is not achieving compliance with the 95% reduction and/or is not in operation.  During these occasions the reduction shall be determined by the Air Pollution Control Officer based on the actual thermal oxidizer performance as demonstrated by District approved source test results. [basis: Regulation 8-44-301]

4.   A/C Condition deleted.

5.	The owner/operator shall not load or permit the loading of a regulated organic liquid into a marine tank vessel within the District whenever the marine vapor recovery system is not fully operational.  For the purposes of this condition, "operational" shall mean the system is achieving the reductions required by Condition No. 2 above. [basis: Regulation 8-44-301]

6.	After startup, the vapor recovery system shall be operated such that the temperature of the exhaust from the incinerator does not drop below 1400 degrees F for more than an aggregate of 15 minutes in any hour. [basis: Regulation 8-44-301]

7.	Condition deleted.

8.	A/C Condition deleted.

9.	A leak test shall be conducted on all vessels loading under positive pressure prior to loading more than 20% of the cargo.  The leak test shall include all vessel relief valves, hatch covers, butterworth plates, gauging connections, and any other potential leak points. [basis: Regulation 8-44-304]

10.	The owner/operator shall not exceed a loading pressure greater than 80% of the lowest relief valve set pressure of the vessel being loaded. [basis: Regulation 8-44-303]

11.	All maintenance records required for the vapor recovery system at this facility, which is subject to Regulation 8, Rule 44, shall be kept on site for five years and made available to the District upon request. [basis: Regulation 8-44-501]

Condition # 4298

For S1805, Tank 12038:

1.	The annual material throughput for S1805 Tank 12038 shall not exceed 563,000 bbl/year.  (Basis:  cumulative increase)

2.	Source S1805 shall be vented at all times to abatement device A1805, consisting of two 55-gallon minimum capacity activated carbon vessels, arranged in series, except when maintenance or repair of the source cannot be completed without a shutdown of the abatement device.  (basis:  40CFR 61, Section 61.343(a) and (b))

3.	The owner/operator of this source shall monitor with a photo-ionization detector (PID), flame-ionization detector (FID), or other method approved in writing by the Air Pollution Control Officer at the following locations:

a.	At the outlet of the primary carbon vessel in series.

b.	At the outlet of the secondary carbon vessel in series.

When using an FID to monitor breakthrough, readings may be taken with and without a carbon filter tip fitted on the FID probe.  Concentrations measured with the carbon filter tip in place shall be considered methane for the purpose of these permit conditions.  (basis:  40 CFR 61, Section 61.354(d))

4.	These monitor readings shall be recorded in a monitoring log at the time they are taken.  The monitoring results shall be used to estimate the frequency of carbon change-out necessary to maintain compliance with condition numbers 5 and 6, and shall be conducted on a daily basis.  The owner/operator of this source may propose for District review, based on actual measurements taken at the site during operation of the source, that the monitoring schedule be changed based on the decline in organic emissions and/or the demonstrated breakthrough rates of the carbon vessels.  Written approval by the District's Permit Services Division must be received by the owner/operator prior to a change to the monitoring schedule.  (basis:  40 CFR 61, Section 61.354(d))

5.	The primary carbon vessel shall be changed out with unspent carbon upon breakthrough, defined as the detection at its outlet of 50 ppm VOC, measured as methane (C1).  (basis:  40 CFR 61.354(d) & Equilon Consent Decree, paragraphs 55 & 59)

6.	The secondary carbon vessel shall be changed out with unspent carbon upon breakthrough, defined as the detection at its outlet of 50 ppm VOC, measured as methane (C1).  (basis:  40 CFR 61.354(d) & Equilon Consent Decree, paragraphs 55 & 59)

7.	The owner/operator of this source shall maintain the following records for operation of the source:

a.	The hours and times of operation.

b.	Each monitor reading or analysis result for the day of operation they are taken.

c.	The number of carbon beds removed from service.

All measurements, records and data required to be maintained by the owner/operator shall be retained and made available for inspection by the District for at least five years following the date the data is recorded.

(basis:  40 CFR 61, Section 61.356(a))

8.	In order to demonstrate compliance with Part 1, the owner/operator shall maintain monthly throughput records of S1805 and total the monthly throughput for each consecutive 12 month period. [basis: Cumulative Increase]

Condition # 4303

For S1804, MAINT Paint Spray Booth and Facility Coating:

- The total annual cleanup solvent usage shall not exceed 40 gallons per year. [basis: Cumulative Increase]

- The total annual coating usage shall not exceed 150 gallons per year. [basis: Cumulative Increase]

- Monthly usages of all cleanup solvents and coatings shall be recorded in a log and retained for at least five years from the date of entry.  This log shall be kept on site and made available to District staff upon request. [basis: Cumulative Increase]

- The spray booth filters shall be in place when the spray booth is in operation. [basis: Cumulative Increase; Regulation 6-301]

Condition # 4364

For S2000, OPCEN Corrosion Inhibitor Injection:

- Condition deleted.

- Not more than 90  gallons per day of Corrosion Inhibitor C-135VT  shall be  injected into the flexigas header. [basis: Cumulative Increase]

3.	Condition deleted.

4.	The owner/operator shall keep records of the composition and daily usage of corrosion inhibitor to demonstrate compliance with condition number 2.  These records shall be maintained for 5 years from date of entry and be made available for District review upon request. [basis: Cumulative Increase]

Condition # 4977

For S858, Tank 858:

1.	Condition deleted.

2.	The total material throughput at this tank shall not exceed 85,848 thousand gallons in any consecutive 12 month period. [basis: Cumulative Increase]

3.	The owner/operator of S858 shall maintain records of all the material stored and throughput at this source to confirm compliance with condition #2. [basis: Cumulative Increase]

Condition # 5077

For S1465, LOG Light Oil Products Gross Oil Separator

S1469, LOG API Separator (ETP 1)

S1779, OPCEN CPI Oil/Water Separator

S2009, LUBS Wasterwater Separator DubbsBox

For S1469:

- The LGO API Separator (ETP1) (S1469) shall not be operated over the design rated capacity of 6000 gpm or the maximum allowable capacity of the unit. [basis: Cumulative Increase]

- Condition deleted.

- Any emission leak that occurs at this source, as defined in Regulation 8-8-302.1, shall be reported to the District Enforcement Section within 15 days of the occurrence. [basis: Regulation 8-8-302.1]

For S1465:

- The Gross Oil Separator (S1465) shall not be operated over the design rated capacity of 3400 gpm or the maximum allowable capacity of the unit. [basis: Cumulative Increase]

- Condition deleted.

- Any emission leak that occurs at this source, as defined in Regulation 8-8-302.1, shall be reported to the District Enforcement Section within 15 days of the occurrence. [basis: Regulation 8-8-302.1]

For S1779:

- The CPI Oil/Water Separator (S1779) shall not be operated over the design rated capacity of 3000 gpm or the maximum allowable capacity of the unit. [basis: Cumulative Increase]

- Condition deleted.

- Any emission leak that occurs at this source, as defined in Regulation 8-8-302.1, shall be reported to the District Enforcement Section within 15 days of the occurrence. [basis: Regulation 8-8-302.1]

For S2009:

- The LUBS Wastewater Separator Dubbs Box (S2009) shall not be operated over the design rated capacity of 600 gpm or the maximum allowable capacity of the unit. [basis: Cumulative Increase]

- Condition deleted.

- Any emission leak that occurs at this source, as defined in Regulation 8-8-302.1, shall be reported to the District Enforcement Section within 15 days of the occurrence. [basis: Regulation 8-8-302.1]

Condition # 6110

For S2012, DH V-12378 Perchloroethylene Storage System  :

- The material throughput for Storage Tank S2012 shall not exceed 18,000 gallons in any consecutive 12-month period. [basis: Cumulative Increase]

- Only Perchloroethylene (PERC) shall be stored in this tank. [basis: Cumulative Increase, Toxics]

- In order to demonstrate compliance with the above conditions, the owner/operator of tank S2012 shall maintain throughput records in a District approved log.  These records shall be kept on site and made available for District inspection for a period of 5 years from the date that record was made. [basis: Cumulative Increase, Toxics]

- Condition deleted.

Condition # 6111

For S549, Tank 549:

- The throughput of material at this source shall not exceed 10,000 barrels (420,000 gallons) in any consecutive 12 month period. [basis: Cumulative Increase]

- The vapor pressure of the material stored in this tank shall not exceed 8 psia. [basis: Cumulative Increase]

- The benzene content of the material stored in this tank shall not exceed 1 mg/l. [basis: Toxics]

- To demonstrate compliance with the conditions #1, #2, and #3, the operator(s) of this source shall maintain a District approved log for this tank which indicates the composition and throughput of material to this tank. These records shall be kept on site and made available for District inspection for a period of 5 years from the date that record was made. [basis: Cumulative Increase; Toxics]

Condition # 6503

For S2013, Tank 12467:

- Total throughput for the external floating roof tank (S2013) shall not exceed 27,000,000 barrels during any consecutive 12 months period. [basis: Cumulative Increase]

- Only unleaded gasoline shall be stored in S2013. [basis: Cumulative Increase, Toxics]

- To demonstrate compliance with the above conditions, the following records shall be kept on site and made available for District inspection for a period of 5 years from the date on which a record is made.

- The material stored.

- The monthly throughput.

[basis: Cumulative Increase, Toxics]

- Condition deleted.

- Tank 12467 (S2013) shall be controlled by a liquid-mounted primary seal, and a zero-gap secondary seal. There shall be no ungasketed roof fittings.  Except for roof legs, each roof fitting shall be of the design, which yields the minimum roof fitting losses (per EPA Compilation of Air Pollution Emission Factors, AP-42, Supplement E, Section 12.3.2, Table 12.3-11).  The following list indicates the type of control required for a variety of typical roof fittings.  Control techniques for roof fittings not included in this list shall be subject to District approval, prior to installing the roof on the tank.

Fitting Type	Control Technique

Access hatch	Bolted cover, gasketed

Guide pole / Well	Solid, or Slotted with controls per API 2517 Addendum (See Note 1)

Gauge float well	Bolted cover, gasketed

Gauge hatch /Sample	Weighted mechanical well actuation, gasketed

Vacuum breaker	Weighted mechanical actuation, gasketed

Roof drain	Roof drain does not drain water into product

Roof leg	Fixed; or Adjustable, with vapor seal boot, or gasket between roof leg and leg sleeve

Rim vent	Weighted mechanical actuation, gasketed

[basis: Cumulative Increase; BACT/TBACT]

Note 1:  Slotted Guide Pole Control Configuration, per Addendum to API Publication 2517, May 1994, shall include the following components:

a.   Sliding cover;

b.   Well gasket;

c.   Pole sleeve with pole wiper approximately 6 inches above sliding cover, or District approved equivalent;

d.   Float with float wiper approximately 1 inch above sliding cover, or alternately a float with multiple wipers.

Condition # 6707

For S2445, Tank 12445

S2446, Tank 12446:

1.	Total liquid throughput at sources S2445 and S2446 shall not exceed 73,000,000 barrels during any consecutive twelve-month period. (basis:  Cumulative Increase, Toxics)

2.	Materials stored in S2445 and S2446 shall be limited to the following:

a.	For the months of November through February, Gasoline or Gasoline components not exceeding RVP 13.5; and for the remaining months only Gasoline or Gasoline components not exceeding RVP 8

b.	A liquid other than those specified above may be stored in 2.a, provided that both of the following criteria are met:

i.	POC emissions, based on the maximum throughput in Condition 1, do not exceed 10,779 pounds per year, as calculated by the EPA Tanks Program version 4.08 (or subsequent version), and

ii.	toxic emissions in lb/yr, based on the maximum throughput in Condition 1, do not exceed any risk screening trigger level specified in Table 2-1-316, except for benzene, which is limited to the following level (before abatement):

Benzene	27 lb/yr

(basis:  Cumulative Increase, Toxics)

3.	Source S2445 and S2446 shall be each equipped with a liquid-mounted primary seal and rim-mounted secondary seal, except as noted in Condition 5.  There shall be no ungasketed roof fittings.  Each roof fitting shall be of the design, which yields the minimum roof fitting losses (per EPA Compilation of Air Pollution Emission Factors, AP-42, Supplement E, Section 12.3.2, Table 12.3-11).  The following list indicates the type of control required for a variety of typical roof fittings.  Control techniques for roof fittings not included in this list shall be subject to District approval.

| Fitting Type | Control Technique |
| --- | --- |
| Access hatch | Bolted cover, gasketed |
| Slotted Guide-Pole/Sample Well | Gasketed, Sliding Cover, w. Float, Sleeve, and Wiper |
| Roof Leg or Hanger Well | Fixed |
| Vacuum Breaker (10-inch Diameter) | Weighed mechanical actuation, gasketed |
| Roof Drain (3-in Diameter) | 90% Closed |
| Ladder Well | Sliding cover, gasketed |

(Basis:  BACT/TBACT, Cumulative Increase)

4.	In order to demonstrate compliance with the condition 1, the owner/operator of  S2445 and S2446 shall either maintain the total monthly throughput of each material stored, summarized on a consecutive 12-month basis in a District approved log, or shall be able to generate these records within short notice.  These records shall be kept on site and made available for District inspection for a period of 60 months from the date that the record was made. (Basis: Cumulative Increase)

5.	The liquid mounted primary seal on S2445 shall be installed prior to any storage of gasoline or gasoline components (RVP 13.5) or by December 31, 2005, whichever is sooner. (basis: BACT/TBACT)

Condition # 7133

For S1023, Tank 1023

S1050 – Tank 1050:

- Condition deleted.

- The total material throughput at S1023 and S1050 shall not exceed 128,772 thousand gallons in any 12 consecutive month period. [basis: Cumulative Increase]

- The owner/operator of S1023 and S1050 shall maintain records of all the material stored and the throughput at these sources to confirm compliance with Condition #2. Records shall be maintained for a minimum of 5 years from the date of entry.  [basis: Cumulative Increase]

Condition # 7215

For S1114, Tank 1114 Spent Acid

S1115, Tank 1115 Spent Acid:

For S1114:

- Emissions from S1114 shall be vented to A1114 Caustic Scrubber for Alky Spent Acid Tanks and to S1471 LOP Auxiliary Flare (primary emission train) or to S1472 LOP Main Flare (secondary emission train) at all times. [basis: BACT/TBACT]

- The total liquid throughput for Storage Tanks S1114 and S1115 shall not exceed 365,000 barrels during any consecutive 12 month period. [basis: Cumulative Increase]

- In order to demonstrate compliance with Condition Number 2 the owner/operator of this source shall maintain the following records in alog.  These records shall be kept on site and be made available for District inspection for a period 5 years from the date that the record was made.

a.	The MSDS sheets for all materials stored and the dates that the materials were stored.

b.	The total throughput of each material stored, summarized on a monthly basis. [basis: Cumulative Increase]

For S1115:

- Emissions from S1115 shall be vented to A1114 Caustic Scrubber for Alky Spent Acid Tanks and to S1471 LOP Auxiliary Flare (primary emission train) or to S1472 LOP Main Flare (secondary emission train) at all times. [basis: Cumulative Increase]

- The total liquid throughput for Storage Tanks S1114 and S1115 shall not exceed 365,000 barrels during any consecutive 12-month period. [basis: Cumulative Increase]

- In order to demonstrate compliance with Condition Number 5 the owner/operator of this source shall maintain the following records in alog.  These records shall be kept on site and be made available for District inspection for a period of 5 years from the date that the record was made.

a.	The MSDS sheets for all materials stored and the dates that the materials were stored.

b.	The total throughput of each material stored, summarized on a monthly basis. [basis: Cumulative Increase]

Condition # 7382

For S1005, Tank 1005

S1072, Tank 1072

Condition deleted.

Condition deleted.

Conditions for S-1005:

1.   The material stored in this tank shall not have a vapor pressure exceeding 1.5 psia.

2.   The total material throughput at this tank shall not exceed 730,000 barrels in any consecutive 12 month period.

3.   The owner/operator of this tank shall maintain records of all the materials stored and throughput at this source to confirm compliance with conditions #1 and #2.

These Parts should not be deleted because S1005 is not an exempt source.

Conditions for S1072:

1.   This tank shall only be used for the storage of crude oil, MFO (marine fuel oil), SIFO (Shell Industrial Fuel Oil), cutter stock fuel oil and gas oils. [basis: Cumulative Increase]

2.   The total material throughput at this tank shall not  exceed 18.2 million barrels in any consecutive 12 month  period.   [basis: Cumulative Increase]

3.   The owner/operator of this tank shall maintain records of all the materials stored and throughput at this source to confirm compliance with conditions #1 and #2. Records shall be maintained for a minimum of 5 years from the date of entry. [basis: Cumulative Increase]

4.	Condition deleted.

Condition # 7618

For S14, Tank 14,

S20, Tank 20,

S483, Tank 483

S484, Tank 484

S532, Tank 532

S1067, Tank 1067

S1139, Tank 1139

S1140, Tank 1140

S1141, Tank 1141

S1420, DH Crude Unit

S1476, LUBS F-24 Atmosphric Feed

S1477, LUBS F-25 Vacuum Feed

S1478, LUBS F-26 Furfural Raff,

S1479, LUBS F-27 Furfural Extr,

S1480, LUBS F-69 Asphalt Circulation,

S1481, OPCEN F-30 DSU,

S1483, LUBS F-32 Asphalt Circulation,

S1484, LUBS F-34 LHT Charge

S1486, DH F-40 CU Feed,

S1487, DH F-41B VFU Feed,

S1488, DH  F-41A VFU Feed,

S1490, DH F-43  GOHT Feed

S1491, DH F-44  NHT Feed,

S1492, DH F-45 Primary Column Reboil,

S1493, DH F-46 Stabilizer Reboil,

S1494, DH F-47 Secondary Column Reboil

S1495, DH F-49  CRU Preheat,

S1496, DH F-50 CRU,

S1497, DH F-51 CRU,

S1498, DH F-52  CRU Reboil

S1499, DH F-53 CRU Regen

S1500, DH F-55 SGP Heat Medium

S1502, DH F-57 HCU First Stage Feed

S1503, DH F-58 HCU Second Stage Feed

S1504, DH F-59 HCU Second Stage Reboil

S1505, DH F-60 HP1 Steam Methane Reformer

S1506, CP F-61 CGP Feed,

S1507, UTIL CO Boiler .1

S1508, CP F-63 CFH Feed, S1510 - CP F-66 CCU Preheat,

S1509, UTIL CO Boiler 2

S1510, CP F-66 CCU Preheat,

S1511, CP F-67 CCU LGO Reboil

S1512, UTIL CO Boiler 3

S1515, DH F-71 HCU First Stage Reboil

S1751, Tank 1330

S1752, Tank 1331

S1753, Tank 1332 Gasoline

S1754, Tank 1333 Gasoline

S1755,  Tank 1334 Gasoline

S1756, Tank 1335 Gasoline

S1757, Tank 1336

S1758, Tank 1337

S1759, OPCEN Flexicoker (FXU)

S1760, OPCEN F-102 FXU Steam Superheater

S1761, OPCEN F-104 HP2 Steam Methane Reformer

S1763, DH F-126 CU Feed

S1764, OPCENDimersol Plant (DIMER)

S1765, OPCENSulfur Plant 3 (SRU3)

S1767, OPCEN V 1019 Coke Silo

S1768, OPCEN  V 1019 Coke Silo

S1769, OPCEN V –1020 Dry Fines Silo

S1770, OPCEN C3/C4 Splitter

S1771 – OPCEN Flexigas Flare

S1774, OPCENHydrogen Plant 2 (HP2)

S1800, UTIL F-88 Boiler

S4002, DC F-13425-A DCU

S4003, DC F-13425-B DCU

S4031, DC F-14012 HGHT Reboil

S4141, DC F-14011 HGHT Feed

S4161, DC H-101 HP3 Steam Methane Reformer

S4191, UTIL Boiler 6 Supplemental Steam Generator 1

S4193, UTIL Boiler 6 Supplemental Steam Generator 2

Condition Modifications Log:

TABLE V revised 10/94, App. No. 13814

Condition B.3.a. and Note 5 of Table IX added 2/98; App. No. 18131.

Condition B.3.a and Note 5 of Table IX amended to allow NOx CEM data for emission calculations (AN 1362, 10/01)

Table II - NOx Baseline reduced by 2 lb/day per Condition ID# 4364 Item 1, for S2000 (AN 4827)

Table II - NOx Baseline reduced by 3089 lb/day per Condition ID# 12271 Item 89, Clean Fuels Project offsets from CO Boilers (AN 8407)

Table IV – SO2 Baseline reduced by 1398 lb/day per Condition ID# 12271 Item 94, Clean Fuels Project offsets from CO Boilers (AN 8407)

- The owner/operator shall operate the units listed in Table I in such a way that any daily emission increases over the baseline profile are offset by reductions below the profile at a ratio of at least 2.0:1.  Compliance shall be demonstrated on a daily basis in the following manner:

- For each pollutant, actual daily emissions for the previous 364 days plus the day in question shall be ranked in descending order by quantity.  The calculation of actual daily emissions shall not be affected by the granting of a variance by the Hearing Board of the District unless such variance specifically includes a variance from Paragraph A (1) of these conditions.  In the event of failure or range exceedance of a monitor upon which emissions are based, emissions shall be calculated to be the maximum possible under the prevailing operating conditions in the plant during the event; emission calculations will be based upon theoretical or historical emission factors or other information which demonstrates emission levels to the satisfaction of the APCO. [basis: Regulation 2-2-302]

- The resulting profile will be compared day by day with the baseline profile. [basis: Regulation 2-2-302]

- The emissions on each day of the current profile will be subtracted from the corresponding day of the baseline profile.  Positive values are profile decreases'.  The absolute value of negative values are 'profile -increases'. [basis: Regulation 2-2-302]

- Profile increases will be totaled; profile decreases will be totaled.  Profile increases will be doubled, and the decreases will be subtracted from this total.  If the result (the profile excess) is positive, the owner/operator is out of compliance.

Profile Excess  =[(2.0) x (Profile Increases)] - (Profile Decreases)

[basis: Regulation 2-2-302]

B.   EMISSION PROFILES AND EMISSION FACTORS

- Tables II through VI list the baseline profiles for each pollutant.  The baselines reflect actual emissions during the baseline period, less reductions required by regulation of sources under the `cap', and increased by offsets and increases provided by reductions at operations not under the `cap' (i.e., tankage).

The baseline profiles were calculated as follows (this condition contains a summary of the steps used to arrive at the values in Tables II through VI, and should be updated every time the baseline profile is adjusted):

- Actual refinery emissions for the baseline period 1976 - 1978 were calculated using the usage rates contained in application 26786 and the emission factors contained in these conditions.

b.	Actual refinery baseline emissions were averaged over 365 days.

- Actual refinery baseline emissions were reduced by any amounts required by District regulations. (Reductions so far applied: SO2 emissions from sulfur recovery units; particulate emissions from CO boilers).

- The hydrocarbon baseline profile has been increased by 2300 lb/day due to abatement on tankage (tankage is not included under the cap).

- The hydrocarbon baseline profile has been decreased by 1543 lb/day due to increases in fugitive emissions from process units.

- The hydrocarbon baseline profile has been increased by 243 lb/day due to shutdown of chemical plants not under the cap (application 29376).

- The actual emissions due to shipping in the base year 1977 were added to the adjusted refinery baseline.

- The baseline daily emissions were ranked in descending order by quantity.  The results (the baseline profiles) are shown in Tables II through VI.

[basis: Regulation 2-2-302]

- Tables VII and VIII list the emission factors to be used for marine activities.  Any changes or additions will be incorporated into these conditions, and must be approved in writing by the APCO. [basis: Regulation 2-2-302]

3.	Table IX lists the emission factors to be used for combustion.  The owner/operator may summarize fuel consumption for those furnaces, which have identical emission factors.  These factors are subject to annual review and may be changed to reflect source test results and CEM data.  the owner/operator shall determine average sulfur content for each gaseous fuel on a daily basis, and for each liquid or solid fuel on a weekly basis.

a.	The owner/operator shall use P-24 CEM data and calculated stack flows to estimate NOx emissions form sources S1500, S1502, S1503, S1504, S1505 and S1515. The owner/operator shall use P-23 CEM data and calculated stack flows to estimate NOx emissions from sources S1486, S1487, S1488, S1490, S1491, S1492, S1493, S1494, S1495, S1496, S1497, S1498 and S1499.  The owner/operator shall use S1761 and S1763 CEM data and calculated stack flows to estimate NOx emissions from these two heaters.  The owner/operator shall use the NOx emission factors in Table IX to calculate NOx emissions during any period for which NOx CEM data is not available for more than 24 hours.  For periods less than 24 hours, the owner/operator shall use the average NOx CEM data for the previous 7 day period.  (added 2/98, App. No. 18131, amended 2/02, AN 1362.  Changes effective once REFEMS computer calculation program is revised)

[basis: Regulation 2-2-302]

- CCU emission factors will be reviewed annually and will be based on source test results and CEM data. [basis: Regulation 2-2-302]

- Sulfur plant emissions will be based on the actual measured sulfur emissions.  All emissions will be included in the total.  The owner/operator will operate the in-stack monitors in such a way as to provide an accurate measurement under all operating conditions. [basis: Regulation 2-2-302]

- Baseline profiles for particulates, NOx, S02, and CO may be modified in the future in the following ways:

- The owner/operator may increase the baseline profile by provision of additional offsets by abating or shutting down sources not included in the cap. The amount of offset credited shall be the actual emission reduction (as defined in the District's NSR rule at the time of offset) resulting from abatement or shutdown of the offsetting sources, reduced by the offset ratios prevailing at the time of offset.

- The owner/operator may increase the baseline profile by buying credits from the Emissions Bank or reduce the profile by making deposits in the bank.  Offset ratios shall be those prevailing at the time of offset.  The baseline profile shall be adjusted to reflect current RACT emission levels for baseline operations prior to any deposits going into the bank.

- The baseline profile will be permanently lowered to correspond to reductions required by any future regulations promulgated by the District.

- The baseline profile will be permanently lowered by the difference between baseline emissions and RACT for any source under the cap that is permanently shut down or removed from the cap. The baseline period used for determination of emissions shall be the same as was used in the evaluation of Permit 26786 (baseline operations shall be as described in  Application No. 26786), unless emissions at the affected source were increased as a result of this or other permits; in which case the baseline period shall be the two year period immediately preceding the date of application.

- If a new source is added to those under the cap, the profile will be permanently lowered by the amount of excess offsets required for onsite offsets under the prevailing NSR rule (i.e. applicable offset ratios and RACT adjustments).

- If a source already under the cap is modified with a resulting increase in emissions, the baseline profile will be permanently lowered by the amount of excess offsets required for onsite offsets under the prevailing NSR rule (i.e. applicable offset ratios and RACT adjustments).

- The baseline profile may be adjusted to reflect more accurate emission factors which  may become available, providing:

- Sufficient data are available to apply the revised emission factor to the-baseline period.

- The revised emission factor is approved by the APCO, incorporated into this permit, and is applied to all future emission calculations.

- Notwithstanding any of the above, the relaxation of any limits that increase the potential to emit may require a full PSD / NSR review of the source as though construction had not yet commenced on the source.

[basis: Regulation 2-2-302]

- The baseline profile for hydrocarbons may be modified in the future in the following ways:

- The baseline profile will be permanently lowered to correspond to reductions required by any future regulations promulgated by the District.

- The baseline profile will be permanently lowered by the difference between baseline emissions and RACT for any source under the cap that is shut down or removed from the cap.  The baseline period used for determination of emissions shall be the same as was used in the evaluation of Permit 26786 (baseline operations shall be as described in Application 26786) unless emissions at the affected source were increased as a result of this or other permits; in which case the baseline period shall be the five year period immediately preceding the date of application.

- The owner/operator may not make deposits in the emissions bank by simply lowering the hydrocarbon cap.  The hydrocarbon cap does not meet the requirement of 2-2-606.1 (b).  The calculation of baseline for the purposes of banking or new source review for any hydrocarbon source under the cap shall be determined pursuant to the provisions of Regulation 2-2-606.2 or the equivalent regulation in effect at the time of application.

- If a new source is added to those under the cap, the profile will be permanently lowered by the amount of excess offsets required for onsite offsets under the prevailing NSR rule (i.e.applicable offset ratios and RACT adjustments); the entire emission increase shall be offset by actual emission reductions from specific identifiable sources subject to enforceable permit conditions.

- If a source already under the cap is modified, the profile will be permanently lowered by the amount of excess offsets required for onsite offsets under the prevailing NSR rule (i.e. applicable offset ratios and RACT adjustments); the entire emission increase shall be offset by actual emission reductions from specific identifiable sources subject to enforceable permit conditions.

[basis: Regulation 2-2-302]

- NON-COMPLIANCE WILL RESULT IN THE FOLLOWING ACTIONS:

No. of Non-complying days

for any one pollutant

(per 365 day period)	Action

More than 0	The owner/operator shall notify the District within 15 working days of any non- complying day.  One V/N will be issued per day of noncompliance for violation of permit conditions.

More than 5	If the previous 365 days include more than 5 non-complying days, the refinery operations shall be limited in the ways listed below.  The owner/operator may submit alternative actions providing equivalent corrective control for consideration by the APCO.  Upon APCO approval, the alternative actions may replace the limitations listed below.  These limitations shall remain in effect following any day of non-compliance until the owner/operator has demonstrated compliance for three consecutive days.

- The refinery shall not process more than 124,000 barrels of crude oil per stream day. [basis: Regulation 2-2-302]

- The refinery shall not process more than 22,000 barrels of Flexicoker feed per stream day. [basis: Regulation 2-2-302]

- The refinery shall not process more than 65,000 barrels of catalytic cracker feed per stream day. [basis: Regulation 2-2-302]

- Total fuel usage at sources under the cap shall not exceed 14730 NLFE (Net Liquid Fuel Equivalent) barrels per stream day. [basis: Regulation 2-2-302]

- Total sulfur content of liquid fuels shall not exceed 1.0 ton/day.  In the event that the Flexicoker Unit is down, the total sulfur content of liquid fuels may not exceed 1.3 tons/day. [basis: Regulation 2-2-302]

- Large cargo vessels (170 MDWEIGHT or greater) shall not be offloaded while any other vessel is being offloaded.  Violation of this condition may result in the revocation of all permits. [basis: Regulation 2-2-302]

- STORAGE CONDITIONS

- For Tank 14 (S14), Tank –20 (S20), Tank –1139 (S1139), Tank –1140 (S1140), Tank –1141 (S1141), Tank 483 (S483), Tank –484 (S484), Tank –530 (S530), T-531, Tank –532(S532), Tank 1330 (S1751), Tank 1331 (S1752), Tank 1332 Gasoline (S1753), Tank 1333 Gasoline (S1754), Tank 1336 (S1757), and Tank 1337 (S1758), T-538, T-1330, T-1331, T-1332, T-1333, T-1336, T-1337:

- Liquid with a true vapor pressure of 1.5 psia or greater shall not be stored in more than 12 of the above tanks at any one time.

- The above tanks will be controlled by a vapor recovery system, or equivalent control equipment, capable of controlling hydrocarbon emissions to less than 8 lb/day.

[basis: Cumulative Increase]

- Liquid with a true vapor pressure of 1.5 psia or greater shall not be stored in Tank 1076. [basis: Cumulative Increase]

- Coke Storage Bins and Purge Silos shall be controlled by baghouses capable of reducing mass emissions of particulate matter by a minimum of 99%. [basis: BACT, Cumulative Increase]

- FUEL CONDITIONS

- Sulfur content of any liquid fuel burned in furnaces subject to the cap shall not exceed 0.5% by weight.  [basis: Regulation 9-1-304]

- While the refinery is processing more than 50 % San Joaquin Valley (SJV) crudes, the H2S concentration of Flexigas shall not exceed 80 ppmv on a daily average, nor 60 ppmv on an annual average.  At all other times, the H2S concentration of the Flexigas shall not exceed 35 ppmv.  If the owner/operator can demonstrate that the Stretford Unit cannot achieve the 35 ppmv H2S concentration on the Flexigas while processing less than 50% SJV crudes, the owner/operator may apply to the APCO for re-evaluation and possible revision of this permit condition. [basis: Cumulative Increase]

- Sulfur content of fuel oil used by tankers larger than 170 MDWEIGHT will not exceed 0.5% (by weight) while discharging crude oil at a rate greater than 35,000 Bbl/hr. [basis: Cumulative Increase]

- REPORTING

- The owner/operator shall report the following on a monthly basis:

- Daily compliance/non-compliance for each pollutant.

- Daily total emissions for each pollutant.

[basis: Cumulative Increase]

- RECORDS

1.	The owner/operator shall make available to the District, upon request, all records relating to the operation of the equipment covered by this permit. [basis: Cumulative Increase]

TABLE  I.

Units and Operations to be Included in Audit:

All refinery combustion devices (including flexigas burned in flares)

Dimersol Unit

Hydrogen Plants

Crude Units

CCU

Sulfur Plants and Tail Gas Units

Wharf Operations

Lightering and Ballasting while within District boundaries

TABLE  II.

FACILITY BASELINE PROFILE -- NOx EMISSIONS (LB/DAY)

NOx Baseline reduced 3089 lb/day per Condition ID# 12271 and 2 lb/day per Condition ID# 4364.  (AN 1362, Feb. 2002)

No. of days	Pounds per day

1	20565.4

2	19907.8

3	19427.8

4	19247.8

5	19055.8

6	19046.2

7	18983.8

8	18943.0

9	18926.2

10	18916.6

11 to 12	18873.4

13	18871.0

14	18868.6

15	18844.6

16	18818.2

17	18811.0

18	18686.2

19	18683.8

20	18633.4

21	18578.2

22	18556.6

23	18532.6

24	18518.2

25	18499.0

26	18487.0

27	18472.6

28 to 29	18448.6

30	18417.4

31	18364.6

32 to 33	18357.4

34	18340.6

35	18331.0

36	18323.8

37	18316.6

38	18292.6

39	18278.2

40	18259.0

41	18232.6

42	18223.0

43 to 44	18215.8

45	18213.4

46	18194.2

47	18153.4

48	18112.6

49	18103.0

50	18100.6

51	18086.2

52	18069.4

53	18057.4

54	18052.6

55 to 56	18035.8

57	18021.4

58	17949.4

59 to 60	17920.6

61 to 63	17870.2

64	17851.0

65 to 67	17810.2

68	17800.6

69	17793.4

70	17779.0

71	17764.6

72 to 73	17755.0

74	17747.8

75	17740.6

76	17731.0

77	17716.6

78	17704.6

79 to 80	17675.8

81	17673.4

82	17654.2

83	17639.8

84	17630.2

85	17625.4

86	17623.0

87	17599.0

88	17594.2

89	17584.6

90	17572.6

91	17570.2

92	17483.8

93 to 94	17467.0

95 to 96	17443.0

97	17368.6

98	17347.0

99	17325.4

100	17267.8

101	17258.2

102	17239.0

103	17217.4

104	17215.0

105	17212.6

106	17152.6

107	17087.8

108	17075.8

109 to 110	17061.4

111	17044.6

112	17042.2

113	17037.4

114	17020.6

115	17015.8

116	16999.0

117	16996.6

118	16987.0

119	16982.2

120	16979.8

121	16970.2

122	16963.0

123	16958.2

124	16953.4

125	16948.6

126	16934.2

127 to 128	16917.4

129	16910.2

130	16883.8

131	16881.4

132	16867.0

133	16864.6

134	16847.8

135	16845.4

136 to 137	16833.4

138 to 139	16828.6

140 to 142	16826.2

143	16809.4

144 to 145	16807.0

146	16804.6

147	16802.2

148	16799.8

149 to 150	16792.6

151	16787.8

152 to 154	16785.4

155 to 158	16783.0

159	16778.2

160	16775.8

161 to 162	16759.0

163	16756.6

164	16754.2

165	16749.4

166 to 168	16744.6

169	16737.4

170	16727.8

171	16723.0

172	16720.6

173	16718.2

174 to 176	16713.4

177	16703.8

178	16699.0

179	16696.6

180	16689.4

181 to 182	16687.0

183 to 184	16684.6

185	16682.2

186	16677.4

187 to 188	16672.6

189 to 200	16670.2

201 to 202	16648.6

203	16646.2

204 to 206	16641.4

207	16639.0

208 to 209	16636.6

210 to 213	16634.2

214 to 216	16631.8

217 to 220	16629.4

221 to 225	16627.0

226 to 228	16622.2

229 to 230	16617.4

231	16615.0

232	16610.2

233 to 234	16605.4

235 to 236	16598.2

237	16595.8

238 to 239	16593.4

240 to 241	16586.2

242	16583.8

243	16581.4

244	16579.0

245	16576.6

246 to 247	16574.2

248	16569.4

249	16559.8

250	16555.0

251 to 252	16547.8

253	16545.4

254	16540.6

255 to 259	16535.8

260	16528.6

261 to 278	16516.6

279	16502.2

280	16499.8

281 to 282	16490.2

283	16487.8

284 to 285	16485.4

286 to 290	16483.0

291 to 294	16480.6

295 to 297	16478.2

298 to 301	16475.8

302 to 304	16473.4

305 to 308	16471.0

309 to 312	16468.6

313	16463.8

314	16461.4

315	16425.4

316	16418.2

317	16406.2

318	16401.4

319 to 325	16382.2

326 to 365	16363.0

TABLE III.

FACILITY BASELINE PROFILE -- HYDROCARBON EMISSIONS (LB/DAY)

No. of days		Pounds per day

1	13842.4

2	13688.8

3	12284.8

4	12114.4

5	11761.6

6	11591.2

7	11101.6

8	11024.8

9	10614.4

10	10552.0

11	10530.4

12	10477.6

13	10446.4

14	10427.2

15	10141.6

16	9829.6

17	9673.6

18	9416.8

19	8600.8

20	8598.4

21	8548.0

22	8543.2

23	8320.0

24	8226.4

25	8185.6

26	8140.0

27	8089.6

28	7679.2

29	7638.4

30	7595.2

31	7549.6

32	7429.6

33	7391.2

34	7146.4

35	6860.8

36	6812.8

37	6752.8

38	6671.2

39	6620.8

40	6479.2

41	6385.6

42	6313.6

43	5857.6

44	5855.2

45	5831.2

46	5754.4

47	5749.6

48	5744.8

49to50	5740.0

51to52	5732.8

53to54	5730.4

55	5728.0

56to57	5723.2

58	5716.0

59	5682.4

60	5651.2

61	5598.4

62	5593.6

63	5456.8

64	5437.6

65	5423.2

66	5399.2

67	5380.0

68	5315.2

69	5260.0

70	5243.2

71	5212.0

72	4859.2

73	4844.8

74	4775.2

75	4765.6

76	4751.2

77	4657.6

78	4463.2

79	4408.0

80	4326.4

81	4268.8

82	4266.4

83	4180.0

84	4069.6

85	3949.6

86	3858.4

87	3820.0

88	3716.8

89	3690.4

90 to91	3668.8

92	3402.4

93	3325.6

94	3212.8

95	3181.6

96	3056.8

97	3025.6

98	2960.8

99	2908.0

100	2831.2

101	2826.4

102	2706.4

103	2418.4

104	2298.4

105	2048.8

106	2020.0

107	2015.2

108	2008.0

109	1969.6

110	1950.4

111	1945.6

112	1933.6

113	1928.8

114 to115	1926.4

116	1919.2

117	1912.0

118 to119	1909.6

120 to121	1907.2

122	1904.8

123	1895.2

124	1883.2

125	1880.8

126	1878.4

127	1871.2

128	1868.8

129 to131	1861.6

132 to133	1859.2

134 to135	1856.8

136 to139	1854.4

140	1852.0

141	1842.4

142 to144	1840.0

145 to146	1837.6

147 to148	1835.2

149 to150	1832.8

151	1830.4

152	1828.0

153	1823.2

154 to157	1820.8

158 to159	1818.4

160 to162	1816.0

163 to164	1813.6

165 to166	1811.2

167 to169	1808.8

170	1804.0

171 to172	1801.6

173	1787.2

174 to175	1782.4

176	1777.6

177 to179	1765.6

180	1763.2

181	1760.8

182	1753.6

183	1751.2

184	1748.8

185 to187	1746.4

188 to190	1744.0

191	1741.6

192 to195	1739.2

196 to197	1734.4

198 to208	1732.0

209 to214	1729.6

215 to225	1727.2

226 to233	1724.8

234 to245	1722.4

246 to255	1720.0

256 to267	1717.6

268 to301	1715.2

302 to311	1712.8

312 to325	1710.4

326 to365	1708.0

TABLE  IV.

FACILITY BASELINE PROFILE -- SOX EMISSIONS (LB/DAY)

SO2 baseline reduced 1398 lb/day per Condition ID# 12271. (AN 1362, Feb. 2002)

No. of days	Pounds per day

1	23023

2	23011

3	22804.6

4	22737.4

5	22658.2

6	22487.8

7	22221.4

8	22199.8

9	22125.4

10	22111

11	22065.4

12	21854.2

13	21851.8

14	21763

15	21686.2

16	21556.6

17	21472.6

18	21451

19	21424.6

20	21074.2

21	21021.4

22	20995

23	20867.8

24	20858.2

25	20723.8

26	20644.6

27	20577.4

28	20570.2

29	20512.6

30	20510.2

31	20491

32	20474.2

33	20452.6

34	20402.2

35	20327.8

36	20063.8

37	20027.8

38	19586.2

39	19571.8

40	19569.4

41	19418.2

42	19367.8

43	19137.4

44	19127.8

45	19123

46	19019.8

47	19017.4

48	18995.8

49	18902.2

50	18897.4

51	18890.2

52	18839.8

53	18681.4

54	18616.6

55	18523

56	18455.8

57	18391

58	18208.6

59	18201.4

60	18155.8

61	18136.6

62	18119.8

63	18055

64	18045.4

65	18028.6

66	18023.8

67	18011.8

68	17987.8

69	17843.8

70	17800.6

71	17793.4

72	17791

73	17788.6

74	17762.2

75	17661.4

76	17647

77	17560.6

78	17524.6

79	17493.4

80	17443

81	17387.8

82	17385.4

83	17371

84	17313.4

85	17311

86 to 87	17301.4

88	17294.2

89	17291.8

90	17267.8

91	17265.4

92	17215

93	17176.6

94	17145.4

95	17128.6

96	17126.2

97	17104.6

98	17090.2

99	17080.6

100	17054.2

101	17042.2

102	17032.6

103	17023

104	17020.6

105	16979.8

106	16946.2

107	16939

108	16929.4

109	16927

110	16883.8

111	16871.8

112	16869.4

113	16845.4

114	16831

115	16814.2

116	16797.4

117	16792.6

118	16790.2

119	16775.8

120	16749.4

121	16713.4

122	16711

123	16699

124	16689.4

125	16672.6

126	16667.8

127	16651

128	16648.6

129	16643.8

130	16615

131	16607.8

132	16595.8

133	16579

134	16567

135	16552.6

136	16538.2

137	16502.2

138	16497.4

139 to 140	16480.6

141	16468.6

142	16449.4

143	16444.6

144	16442.2

145	16437.4

146	16432.6

147	16430.2

148	16375

149	16312.6

150	16298.2

151	16271.8

152 to 153	16267

154	16264.6

155	16255

156	16252.6

157	16243

158	16221.4

159 to 160	16219

161	16214.2

162	16185.4

163	16163.8

164	16147

165	16142.2

166	16135

167	16132.6

168	16123

169	16115.8

170	16051

171	16043.8

172	16034.2

173	16007.8

174	15993.4

175	15983.8

176	15976.6

177	15971.8

178	15945.4

179	15940.6

180	15926.2

181	15919

182 to 183	15902.2

184	15895

185	15861.4

186	15854.2

187	15820.6

188 to 189	15796.6

190	15782.2

191	15775

192	15765.4

193 to 194	15763

195	15743.8

196 to 197	15736.6

198 to 199	15717.4

200	15695.8

201	15655

202	15635.8

203	15609.4

204	15547

205 to 207	15527.8

208	15518.2

209	15513.4

210	15506.2

211	15503.8

212	15501.4

213	15496.6

214	15494.2

215	15489.4

216	15484.6

217	15482.2

218	15479.8

219	15477.4

220 to 221	15475

222	15472.6

223	15467.8

224	15448.6

225	15446.2

226	15441.4

227 to 229	15439

230	15434.2

231	15429.4

232	15422.2

233 to 234	15419.8

235	15417.4

236	15410.2

237	15403

238	15395.8

239	15393.4

240	15388.6

241	15386.2

242	15367

243	15364.6

244	15362.2

245	15359.8

246	15357.4

247	15355

248 to 250	15352.6

251 to 252	15350.2

253 to 254	15345.4

255	15343

256 to 257	15340.6

258	15333.4

259	15314.2

260	15311.8

261	15307

262	15304.6

263	15302.2

264	15299.8

265	15275.8

266	15273.4

267 to 268	15268.6

269 to 270	15266.2

271	15261.4

272	15256.6

273	15254.2

274	15251.8

275	15213.4

276	15199

277	15124.6

278	15081.4

279	15047.8

280	15045.4

281	15035.8

282	14971

283 to 287	14961.4

288 to 294	14949.4

295	14702.2

296	14690.2

297 to 307	14680.6

308 to 325	14668.6

326 to 365	14659

TABLE V	(revised10/94,App.No.13814)

MODIFIED FACILITY BASELINE PROFILE—PARTICULATE EMISSIONS

No.of days		Pounds per day

1	2137.8

2	2070.6

3	2044.2

4	2034.6

5	2020.2

6	2015.4

7	2010.6

8	2008.2

9	2001.1

10	1996.2

11	1991.4

12	1989.1

13	1984.2

14	1981.8

15	1962.6

16 to17	1957.8

18 to19	1955.4

20	1953.1

21	1948.2

22	1945.8

23	1941.1

24	1938.6

25	1931.4

26	1929.1

27	1926.6

28	1912.2

29	1909.8

30	1905.1

31	1897.8

32	1871.4

33	1864.2

34	1861.8

35	1859.4

36	1857.1

37 to 38	1854.6

39	1852.2

40	1842.6

41	1837.8

42 to43	1833.1

44	1830.6

45	1821.1

46	1806.6

47	1801.8

48	1792.2

49	1780.2

50 to52	1777.8

53	1775.4

54 to55	1773.1

56 to57	1770.6

58	1761.1

59 to61	1758.6

62 to63	1756.2

64	1753.8

65	1749.1

66	1744.2

67 to 69	1737.1

70 to71	1722.6

72	1717.8

73	1715.4

74	1713.1

75	1710.6

76 to78	1708.2

79	1705.8

80	1703.4

81	1698.2

82	1696.2

83 to86	1693.8

87 to88	1691.4

89 to90	1689.1

91	1686.6

92 to94	1681.8

95 to96	1679.4

87 to99	1677.1

100	1674.6

101 to102	1672.2

103 to104	1669.8

105 to106	1665.1

107 to110	1660.2

111	1657.8

112 to116	1655.4

117 to119	1653.0

120 to121	1650.6

122	1642.2

123 to124	1645.8

125 to126	1643.4

127 to128	1641.1

129 to133	1638.6

134 to135	1636.2

136 to140	1633.8

141 to143	1631.4

144 to148	1629.1

149 to151	1626.6

152 to154	1621.8

155 to157	1619.4

158 to159	1617.1

160 to167	1612.2

168 to170	1609.8

171 to173	1607.4

174 to178	1602.6

179 to180	1600.2

181	1597.8

182 to186	1595.4

187 to188	1593.1

189	1590.6

190	1588.2

191	1585.8

192 to195	1583.4

196 to197	1581.1

198 to199	1578.6

200 to202	1576.2

203 to204	1573.8

205 to206	1571.4

207 to209	1569.1

210 to211	1566.6

212 to214	1564.2

215 to218	1561.8

219 to225	1559.4

226 to235	1557.1

236 to247	1554.6

248 to252	1552.2

253 to263	1549.8

264 to267	1547.4

268 to271	1545.1

272 to276	1542.6

277	1537.8

278 to280	1525.4

281 to83	1533.1

284 to288	1530.6

289	1525.8

290 to296	1523.4

297 to307	1521.1

308 to325	1513.8

326 to365	1506.6

TABLE VI.

FACILITY BASELINE PROFILE -- CARBON MONOXIDE EMISSIONS (LB/DAY)

No. of days	Pounds per day

1 to 365	6904.0

TABLE VII.

EMISSION FACTORS -- WHARF OPERATIONS

Material	Loading Emission Factor, lb/1000 gal

Finished Gasolines	1.4

Light Gasoline Components

Alkylate 	1.4

HT C5/100 St Run	1.4

C5/240 Cat Crk	1.4

C5/180 Hydrocrackate	1.4

Dis Light Naphtha	1.4

Heavy Gasoline Components

Reformate	1.4

240/450 Cat Crk	1.4

Dist Heavy Naphtha	1.4

Cat Reformer Feed	1.4

Shell Jet A	.05

Shell Mineral Spirits	.05

DSU Mineral Spirits	.05

Shell 1300 Solvent	.05

Crudes

Alaskan North Slope	1.7

Labuan	1.7

Heavy S.J. Valley	.06

Others	1.7

Heavy Feed Stocks

Coker Heavy Gas Oil	.016

Cat Cracker Feed 	.005

Vacuum Flashed Dist	.005

Heavy Flashed Dist	.005

Coker Feed	.016

Fuel Oils

Shell Dieseline	.005

Marine Diesel	.005

Shell Thin Fuel Oil 30	.005

Shell Thin Fuel Oil 40 	.005

Shell Thin Fuel Oil 60 	.005

Shell Thin Fuel Oil 80 	.005

Shell Thin Fuel Oil 100 	.005

Unfin Cracked Gas Oil 	.002

Shell Light Fuel Oil 	.002

Shell Steam Ship Fuel Oil	.002

Shell Marine International Grade	.002

Shell Industrial Fuel Oil	.002

Shell Thin F.U. 120	.002

Shell Thin F.O. 150	.002

Shell Thin F.U. 180	.002

Shell Thin F.U. 240	.002

Shell Thin F.U.280	.002

Shell Thin F.O. 320	.002

Shell Thin F.O. 380	.002

Shell Thin F.O. MV 240/11.4	.002

Shell Thin F.O. MV 280/11.4	.002

Shell Thin F.O. MV 320/11.4	.002

Shell Thin F.O. MV 380/11.4	.002

Miscellaneous Cutter Stock	.002

Shell Special Industrial Fuel Oil	.002

Shell UMF Grade C	.002

Shell Mect. Treating Aromatic Oil	.002

Shellflex 371	.002

Lube Oils

HVI 100 Neut TQ	.00004

HVI 250 Neut MQ	.00004

HVI 65/210	.00004

HVI 150 Brt Stk	.00004

HVI 100 Neut MQ	.00004

LVI 60 Neut MQ	.00004

LVI 100 Neut MQ	.00004

100 Base Stk 80 UR	.00004

LVI 450 Neut 	.00004

LVI 90/210 Neut	.00004

60 Spray Base	.00004

100 Spray Base 92 UR 	.00004

MVI 400 Neut 	.00004

MVI 80/210 Neut RQ	.00004

Diala A	.00004

Diala AX 	.00004

35 Base Stock	.00004

Asphalts

AR 2000 Asphalt	.00004

AR 4000 Asphalt	.00004

AR 8000 Asphalt	.00004

WOR Flux	.00004

80 Vis Blend Stock	.00004

TABLE  VIII.

SHIPPING COMBUSTION EMISSION FACTORS

(lb/1000 gallon fuel)

Organic	SO2		NOx	Part.

0.5%	0.2%		0.5%	0.2%

Steamships

Hoteling	3.10	79.77	319.08	20.90	7.03	18.99

Other	3.10	79.77	319.08	48.21	7.03	18.99

Internal Combustion Engines

Residual	32.80	78.78	315.11	339.60	35.00	35.00

Marine Diesel	32.80	70.69		367.00	20.00

Diesel (Tugs)	12.99	38.97		571.22	24.98

SHIPPING FUEL COMBUSTION RATES (gal/hr)

Cargo Discharge

Ship	Fuel		Hoteling	Rates (1000 bbl/hr)

Size		Manuevering	3	7.5	11	20	50	85

Steamships

30 MDWEIGHT	Residual	304	39	147	294

45 MDWEIGHT	Residual	417	47	184		504

60 MDWEIGHT	Residual	567	76			631

70 MDWEIGHT	Residual	666	93			650

200		Residual	1057	143				1000	1188	1656

Motorships

30 MDWEIGHT	Residual		84	147	294

Diesel	420	42	42	42

70 MDWEIGHT	Residual		84			650

Diesel	420	42			42

150		Residual		84				759	898

Diesel	588	42				42	42

120/135		Residual		84				759	898

Diesel	588	42				42	42

Tugs

Diesel	30

TABLE  IX.

Emission Factors -- Combustion

(lb/MMBTU; * = lb/MMSCF; and ** = lb/bbl)

CO BOILERS	  HC	PART	NOx	CO	SOx

Natural Gas	.0162	.015	.7	.0039	.0006

Refinery Gas	.0162	.02	.7	.0039	S(.171)*  (Note 1)

Residual Fuel	.042**	.55**	.7	.0040	S(7.2)**  (Note 2)

LSFO	.042**	.084**	.7	.0040	S(6.3)**  (Note 3)

Flexigas	.00162	.005	.05	.0039	S(.177)*  (Note 4)

ALL OTHER HEATERS (Note 5)

Natural Gas	.0162	.015	.2	.0039	.0006

Refinery Gas	.0162	.02	.2	.0039	S(.171)*  (Note 1)

Residual Fuel	.042**	.55**	.82	.0040	S(7.2)**  (Note 2)

LSFO	.042**	.084**	.23	.0040	S(6.3)**  (Note 3)

Flexigas	.00162	.005	.05	.0039	S(.177)*  (Note 4)

Notes:

(1)	S = sulfur content of Refinery Gas in ppm (60 in base case)

(2)	S = sulfur content of fuel oil in weight% (1.0 in base case)

(3)	S = sulfur content of LSFO in weight%

(4)	S = sulfur content of FXG in ppm

(5)	For sources S1486, S1487, S1488, S1490 through S1500, S1502, S1503, S1504, S1505, S1515, S1761 and S1763, The owner/operator shall use NOx CEM data to calculate NOx emissions.  If CEM data is not available, see Condition B.3.a..  (added 2/98, App. No. 18131, amended 2/02, AN 1362)

Condition # 7761

For A101 – Flare for Vine Hill Vapor Recovery System

A102 – Flare for Crude String Vapor Recovery System

A103 – Flare for Interim Gasoline Vapor Recovery System

For A103:

1.	The flare (A103) shall only be operated when the Vapor Recovery Unit (VRU), A26, is down for routine maintenance or there is an unexpected upset in the operation of the VRU that causes the VRU to be taken out of service.  The operator shall take immediate action to correct the problem in the case of an unexpected upset and the flare shall be placed in use as soon as feasible to reduce emissions. [basis: Regulation 8-5-301.]

2.	The owner/operator shall maintain records to verify compliance with Condition 1.  These records shall be maintained for a period of at least five years and be made available to the District for inspection upon request. [basis: Regulation 8-5-301.]

3.	A pilot indicator shall be installed to determine the status of the flare. [basis: Regulation 8-5-301.]

4.	The flare system shall be equipped with a flare failure alarm system. [basis: Regulation 8-5-301.]

5.	The tanks abated by this flare are subject to the applicable conditions for the tank(s) in Condition # 7618, Section D. [basis: Cumulative Increase]

For A101:

- The flare (A101) shall only be operated when the Vapor Recovery Unit (VRU), A25, is down for routine maintenance or there is an unexpected upset in the operation of the VRU that causes the VRU to be taken out of service.  The operator shall take immediate action to correct the problem in the case of an unexpected upset and the flare shall be placed in use as soon as feasible to reduce emissions. [basis: Regulation 8-5-301.]

- The owner/operator shall maintain records to verify compliance with Condition 6.  These records shall be maintained for a period of at least five years and be made available to the District for inspection upon request. [basis: Regulation 8-5-301.]

- A pilot indicator shall be installed to determine the status of the flare. [basis: Regulation 8-5-301.]

- The flare system shall be equipped with a flare failure alarm system. [basis: Regulation 8-5-301.]

- The tanks abated by this flare are subject to the applicable conditions for the tank(s) in Condition # 7618, Section D. [basis: Cumulative Increase]

For A102:

- The flare (A102) shall only be operated when the Vapor Recovery Unit (VRU), A56, is down for routine maintenance or there is an unexpected upset in the operation of the VRU that causes the VRU to be taken out of service.  The operator shall take immediate action to correct the problem in the case of an unexpected upset and the flare shall be placed in use as soon as feasible to reduce emissions. [basis: Regulation 8-5-301.]

- The owner/operator shall maintain records to verify compliance with Condition 11.  These records shall be maintained for a period of at least five years and be made available to the District for inspection upon request. [basis: Regulation 8-5-301.]

- A pilot indicator shall be installed to determine the status of the flare. [basis: Regulation 8-5-301.]

- The flare system shall be equipped with a flare failure alarm system. [basis: Regulation 8-5-301.]

- The tanks abated by this flare are subject to the applicable conditions for the tank(s) in Condition # 7618, Section D. [basis: Cumulative Increase]

Condition # 8502

For S12490, LOG Tank 12519 Wastewater ETP 1&2

S12491, LOG Tank 12520 Wastewater ETP 1&2:

1.	The wastewater storage tank shall be equipped and operated with a level-indicating device to aid the operator in minimizing tank roof movement. [basis: Cumulative Increase]

- Condition deleted.

3.	Condition deleted.

Condition # 10275

For  S1116, Tank 1116 Fresh Acid

1.	This tank shall not be used to store sulfuric acid with an acid strength greater than 100% by weight. [basis: Cumulative Increase]

Condition # 11313

For S5115, LOG Dissolved Nitrogen Flotation Unit North ETP 2 (DNF)

S5116, LOG Dissolved Nitrogen Flotation Unit South ETP 2 (DNF)

For S5115 and S5116:

- Condition deleted.

- Condition deleted.

- Condition deleted.

For S5115, S5116:

The following conditions are applicable when emissions must be vented from the sources:

1.	Emissions from S5115, S5116 shall be vented to at least two carbon vessels arranged in series. [basis: Regulation 8-8-307]

2.	The second to last carbon bed, A5115, shall be changed out with unspent carbon upon the detection of 1000 ppmv as C1 to the last carbon bed as measured with a flame ionization detector (OVA-FID), Gas Tech, or other method approved in writing by the APCO. [basis: Regulation 8-8-307]

3.	The last carbon bed, A5116, shall be changed out with unspent carbon upon the detection of 500 ppmv as C1 from the last carbon bed as measured with a flame ionization detector (OVAFID), Gas Tech, or other method approved in writing by the APCO. [basis: Regulation 8-8-307]

4.	The limits set forth in Condition # 5 and 6 shall apply to non-methane hydrocarbon emissions.  To determine the presence of  methane in  the exhaust  stream, a reading shall be  taken with  and without  a carbon  filter tip fitted on  the probe.  Concentrations measured with the carbon filter  tip in place shall be considered methane for the purpose of these permit conditions. [basis: Regulation 8-8-307]

5.	The operator  of this source shall monitor with an OVA-FID, or other method approved in writing by the APCO at the following locations:

a.	At the exhaust of second to last carbon vessel.

b.	At the exhaust of the last carbon vessel.

[basis: Regulation 8-8-504]

6.	These monitor readings shall be recorded in a monitoring log at the time they are taken.  The monitoring results shall be used to:

a.	Estimate the frequency of carbon change out necessary to maintain compliance with condition number 2.

b. 	To maintain compliance with condition number 5 the monitoring shall be  conducted  on  a daily basis for the first five days after the initial  installation  and  commissioning  of carbon vessels  and monthly  afterwards.  The operator of this source  may propose for District review, based on actual measurements taken at the site  during operation  of the source,  that   the  monitoring schedule be changed based on the decline in  organic emissions and/or the demonstrated breakthrough rates  of the carbon  vessels. Written approval by the District must be received by the applicant prior to a change to the monitoring schedule.

[basis: Regulation 8-8-307]

7.	The operator of this source shall maintain the following information in a District approved log for each month of operation of the source:

a.	The dates when venting through the carbon vessels occurs.

b.	Each monitor reading or analysis result for the day of operation they are taken.

c.   The date any carbon vessel is changed out.

[basis: Regulation 8-8-307]

8.	The operator shall maintain a file containing all measurements, records and other data that are required to be collected pursuant to the various provisions of this conditional Authority to Construct/Permit to Operate.  All measurements, records and data required to be maintained by the applicant shall be retained for at least five years following the date the data is recorded. [basis: Regulation 8-8-307]

Condition # 11504

For S5112, LOG V-15112 Main Proto Vessel with Nitrogen Blanket

S5113, LOG V-15113 Main Proto Vessel with Nitrogen Blanket

S5114, LOG V-15114  Main Proto Vessel  with Nitrogen Blanket:

- Sources S5112, 5113 and 5114 shall be controlled by a nitrogen blanket, and these sources shall be vented to A26, Vapor Recovery System #1, at all time of operation. [basis: Cumulative Increase, Regulation 8-5.]

Condition # 11806

For S3000, Portable Vacuum Distillation Unit (CCR Technologies Inc.):

- No more than one unit may operate at the facility at any one time.  The combined operating time for all units shall not exceed 8 weeks in any 12 month period. (basis: cumulative increase)

- All liquid transfers to and from the distillation unit, including waste, shall be made via hard-piped connections. (basis: cumulative increase)

- All non-condensable gases from the condenser vent shall be combusted in the heater burner. (basis: cumulative increase)

- Only lean amine solution shall be distilled. (basis: cumulative increase)

- Prior to distilling the lean amine solution, NaOH or other caustic shall be mixed with the amine solution to neutralize acid gases and react with residual H2S to form a salt. (basis: cumulative increase)

- This distillation unit shall be fired exclusively with natural gas. (basis: cumulative increase)

7.	The owner/operator shall keep a District approve log of the operating time of S3000.  This log shall be made available for District inspection and maintained for a minimum of 5 years from the date of entry. (basis: cumulative increase)

Condition # 11850

For S544, Tank 544:

- The steam coils shall not be used when the tank is storing material with an API gravity greater than 20.  The owner/operator shall keep sufficient records to demonstrate compliance with this condition.  These records shall be retained for five years from the date of entry and shall be made available to the District upon request. [basis: Cumulative Increase]

Condition # 11951

For S540, Tank 540:

- Total throughput of S540 shall not exceed 340 million gallons in any consecutive twelve-month period.  The owner/operator shall keep monthly throughput records for Tank 540 in a log.  These records shall be retained for at least five years from the date of entry, and shall be made available to the District upon request. [basis: Cumulative Increase]

- Condition deleted.

- On Tank 540 (S540), there shall be no ungasketed roof fittings.  Except for roof legs, each roof fitting shall be of the design that yields the minimum roof fitting losses, per EPA Compilation of Air Pollution Emission Factors, AP-42, Supplement E, Section 12.3.2, Table 12.3-11.  The following list indicates the type of control required for a variety of typical roof fittings.  Control techniques for roof fittings not included in this list shall be subject to District approval, prior to installing the roof on the tank.

Fitting Type	Control Technique

Access hatch	Bolted cover, gasketed

Guide pole/Well	Unslotted guide pole, gasketed sliding cover (See Note 1 below)

Gauge float well	Bolted cover, gasketed

Gauge hatch/Sample well	Weighted mechanical actuation, gasketed

Vacuum breaker	Weighted mechanical actuation, gasketed

Roof drain	Roof drain does not drain water into product

Roof leg	Adjustable, with vapor seal boot

Rim vent	Weighted mechanical actuation, gasketed

Note 1:   Tank 540 does not have a guide pole/gauge well.

[basis: BACT/TBACT]

Condition # 12174

For S545, Tank 545:

- The steam coils shall not be used when the tank is with storing material with an API gravity greater than 20. The owner/operator shall keep sufficient records to demonstrate to compliance with this condition.  These records shall be retained for five years from the date of entry and shall be made available to the District upon request. [basis: Cumulative Increase]

Condition # 12190

For S1117, Tank 1117 Skim:

- Throughput to S1117 shall not exceed 10,000 barrels per year.  Beginning with issuance of this Permit to Operate, the owner/operator shall keep quarterly throughput records to demonstrate compliance with this condition.  These records shall be retained for five years from the date of entry and shall be made available to the District upon request. [basis: Cumulative Increase]

Condition # 12271

For S13, Tank 3

S1129, Tank 1129

S1130, Tank 1130

S1131, Tank 1131

S1507, UTIL CO Boiler 1

S1509, UTIL CO BOILER 2

S1512, UTIL CO Boiler 3

S4001, DC Delayed Coking Unit (DCU)

S4002, DC -F-13425-A DCU

S4003, DC F-13425-B DCU

S4005, DC Coke Handling Facility

S4020, DC Distillate Hydrotreater (DHT)

S4021, DC F-13909 DHT Recycle

S4031, DC F-14012 HGHT Reboil

S4050, DC Catalytic Gas Depentanizer (CGDP)

S4080, DC Isomerization Unit (ISOM)

S4140, DC Heavy Cracked Gasoline Hydrotreater (HGHT)

S4141, DC F-14011 HGHT Feed

S4160, DC Hydrogen Plant –3 (HP3)

S4161, DC H-101 HP3 Steam Methane Reformer

S4170, LUBS Lube Hydrotreater 2 (LHT2)

S4171, LUBS F-13000 LHT2 Feed

S4180, OPCEN Sulfur Plant 4 (SRU4)

S4190, UTIL Boiler 6 Gas Turbine 1

S4191, UTIL Boiler 6 Supplemental Steam Generator  1

S4192, UTIL Boiler 6 Gas Turbine 2

S4193, UTIL Boiler 6 Supplemental Steam Generator 2

S4201, DC Clean Fuels Flare

S4210, Cooling Water Tower (CWT-13278)

S4211, DC V-13222 ISOM Maintenance Drop Out Vessel

S4212, DC V-13441 ISOM Maintenance Drop Out Vessel

S4307, Tank 14687 MDEA Make-up 	S4309, Tank 14687 DEA

S4310, Tank 13285 Sour Water

S4311, DC V-12555 ISOM Perchloroethylene Vessel

S4319, Tank 15096 Recovered Oil

S4322, Tank 14571 Sour Water

S4329, Tank 13260 Pentane

S4330, Tank 13261 Pentane

S4334, Tank 13276 Alkylate

S4338, LOG Pentane Loading Facility

S4347, OPCEN Pit for Sulfur Plant 4

S4349, Tank13262 Pentane

S4350, LOG Tank13187 Process Wastewater

S4356, LOG Tank13188 Process Wastewater

REVISED CLEAN FUELS PROJECT PERMIT CONDITIONS

(Amended  March 28, June 2, August 15, 1995 and Sept. 11, 1995; July 28, 1998; obsolete start-up conditions deleted 7/10/01)

Unless specified otherwise, the following permit conditions apply only to sources installed or modified as part of the Clean Fuels Project (CFP).

GENERAL PERMIT CONDITIONS

Monthly and Annual Emission Limits

A.	Emissions from the Clean Fuels Project sources described in Condition B shall not exceed the limits indicated in Table A.1 below, for any consecutive twelve-month period.  Table A.2 provides monthly trigger levels for CFP sources indicated in Condition B.  If monthly emissions exceed a trigger level, the owner/operator shall include a Compliance Plan in the next monthly report for that pollutant.  The Compliance Plan shall indicate the necessary action to avoid exceeding the annual limit for the pollutant(s) in question.

| Table A.1 12-Month Emission Limits NOx 199.4 TPY CO 252.4 TPY SO2 139.7 TPY PM/PM10 59.1 TPY POC 132.0 TPY | Table A.2 Monthly Trigger Levels NOx 20.8 Ton/Mo. CO 26.3 Ton/Mo. SO2 14.6 Ton/Mo. PM/PM10 6.2 Ton/Mo. POC 13.7 Ton/Mo. |
| --- | --- |

Emissions shall be calculated in accordance with the emission calculation procedures used in this permit application, and summarized in Appendix B of the Engineering Evaluation Report for application number 8407, including subsequent revisions.  Where CEM data is available (S4002, 4003, 4021, 4031, 4141, 4161, 4190, 4191, 4192, 4193, and A4180 and 4181) emission calculations shall be based upon the CEM data.  [Tables A1 and A2 revised per Application No.’s 14577 (3/95) and 25078 (8/95) and 18185 (7/98).]  [basis:  offsets, cumulative increase]

B.	The emission totals in Condition A include emissions from the sources in Tables B.1 through B.4 below, and emissions from process upset conditions (in accordance with Condition C) from sources in Tables B.1 through B.6.  The emission limits in Condition A do not include routine fugitive emissions.  [basis:  offsets, cumulative increase]

| Table B.1 - Combustion Sources S4002 DC F-13425-A DCU S4003 DC F-13425-B DCU– S4004 (Deleted; AN 14577) S4021 DC F-13909 DHT Recycle S4022 (Deleted; AN 14577) S4031 DC F-14012 HGHT Reboil S4141 DC F-14011 HGHT Feed S4161 DC H-101 HP3 Steam Methane Reformer S4171 LUBES F-13000 LHT2 Feed S4190 UTIL Boiler 6 Gas Turbine 1 S4191 UTIL Boiler 6 Supplemental Steam Generator 1 S4192 UTIL Boiler 6 Gas Turbine 2 S4193 UTIL Boiler 6 Supplemental Steam Generator 2 | Table B.2 - Tankage S13 .Tank 13. S1129 Tank 1129 S1130 Tank 1130 S1131 Tank 1131 S4307 Tank 14687 MDEA Make-up S4309 Tank 14517 DEA S4310 Tank 13285 Sour Water S4319 Tank 15096 Recovered Oil S4322 Tank 14571 Sour Water S4334 Tank 13276 Alkylate S4350 LOG Tank 13187 Process Wastewater S4356 LOG Tank 13188 Process Wastewater |
| --- | --- |

| Table B.3 - SRU No.4 A4180 SCOT No. 4 A4181 Thermal Oxidizer for Sulfur Plant 4S-S4347 OPCEN Pit for Sulfur Plant 4 Table B.4 - Miscellaneous (Included Emissions) S-S4210 Cooling Water Tower (CWT-13278) S-S4338 LOG Pentane Loading Facility Table B.5 - Miscellaneous (Excluded Emissions) S-S4005 DCCoke Handling Facility S-S4201 DC Clean Fuels Flare S-S4211 DC V-13222 ISOM Maintenance Drop Out Vessel S-S4212 DC V-13441 ISOM Maintenance Drop Out Vessel | Table B.6 - Process Units (Fugitives) S-S4001 DC Delayed Coking Unit (DCU) S-S4020 DC Distillate Hydrotreater (DHT) S-S4050 DC Catalytic Gas Depentanizer (CGDP) S-S4080 DC Isomerization Unit (ISOM) S-S4140 DC Heavy Cracked Gasoline Hydrotreater (HGHT)g S-S4160 DC Hydrogen Plant – 3 (HP3) S-S4170 LUBS Lube Hydrotreater –2 (LHT2) S-S4180 OPCEN Sulfur Plant –4 (SRU4) |  |
| --- | --- | --- |

C.	In the case of a process upset or other irregularity for which variance or breakdown relief is denied, the owner/operator shall calculate emissions and include those emissions in the monthly and annual emission totals, for determining compliance with Condition A.  In the case of a process upset or other irregularity for which variance or breakdown relief is granted, the owner/operator is not required to include excess emissions in the monthly and annual emission totals, for determining compliance with Condition A.  [basis:  offsets, cumulative increase]

Source Tests / Continuous Emission Monitors

For any source test or continuous emission monitor/recorder (CEM) required by any permit condition associated with the Clean Fuels Project (CFP), the following shall apply:

D.	For the purposes of determining compliance with any of the emission limits in these CFP permit conditions (including emission limits with averaging times that exceed the typical source test duration), the applicable source test methods in the District's Manual of Procedures shall be sufficient for documenting compliance and non-compliance.  All source testing and monitoring shall be done in accordance with the District Manual of Procedures.  Written source testing protocol shall be submitted to the District Source Test Section for review and approval at least 30 days prior to conducting the source test.  [basis:  Manual of Procedures, Volume IV]

E.	The District Source Test Section shall be notified in writing of the date and time of any source test, at least 2 weeks prior to conducting the source test.  [basis:  Manual of Procedures, Volume IV]

F.	Start-up condition deleted.

G.	Written source test results shall be submitted to the District Source Test Section and the District permit engineer within 60 days of completion of the source test, unless an extension is approved by the District.  In all cases, written source test results must be received by the District within 150 days of startup.  [basis: Manual of Procedures, Volume IV]

H.	Start-up condition deleted.

I.	Start-up condition deleted.

J.	Each CEM shall be installed, maintained, calibrated and operated in accordance with all applicable District regulations.  The owner/operator shall use a computer to record, store, and report a summary of the CEM data for the monthly report.  For any CEM that is used to verify compliance with a concentration limit that is averaged over a specified time period, a computer shall calculate average concentrations.  These average concentrations shall be summarized in the monthly report.  [basis:  cumulative emissions, offsets]

K.	CEM data shall be used for compliance verification.  As such, CEM data may be used by the District Enforcement Division to document a violation of District regulations or permit conditions.  [basis:  BACT, offsets]

Record Keeping & Monthly Reporting

L.	The owner/operator shall keep records of all necessary information to demonstrate compliance with all permit conditions associated with the Clean Fuels Project.  All records shall be retained for at least five years from the date of entry, and shall be made available to the District upon request.  This includes, but is not limited to, records of source test data, CEM data, fuel usage, emission calculations, fugitive component counts, product shipments, coke production, sulfur production, and rail emissions.  The owner/operator shall also keep all records required by NSPS.  [basis:  BACT, offsets, cumulative increase]

M.	Upon startup of the first process unit associated with the CFP, the owner/operator shall submit monthly reports of all information deemed necessary by the District permit engineer to determine compliance with all permit conditions required for this project.  The format of the monthly reports shall be subject to approval by the District permit engineer 60 days prior to startup, and shall include (but is not limited to) the information listed below.  Changes to the original format shall be subject to approval by both the owner/operator and the District permit engineer.

Fuel usage type and amount, for:
	Cogeneration plant;
	Hydrogen plant SMR furnace;
	Furnaces > 50 MM BTU/hr; and
	Furnaces < 50 MM BTU/hr.

Combustion emissions per source, or per group of sources with same emission factors;

CEM data and emission calculations;

CEM indicated excesses;

Fuel gas H2S and TRS concentrations;

PSA gas H2S concentrations;

SRU No.4 SO2 emissions and sulfur production rates;

Product rail shipment emissions for coke and pentane;

Storage tank emissions;

Emission calculations from flaring event:  the owner/operator is not required to specifically measure flow to the flares, but must use knowledge of process depressurization rates and duration of venting to calculate emissions;

Pressure relief valve ventings and emissions;

Breakdown requests and associated BAAQMD ID #'s.

[basis:  offsets, cumulative increase]

Miscellaneous

N.	Any process vessel depressurization shall be vented to a control device with an overall capture and destruction efficiency of 95%, on a mass basis. [basis:  BACT]

O.	Offsets for this project will be required as specified by permit conditions below.  If after completion of the Clean Fuels Project a source was not constructed, the project emissions will be adjusted and offsets (including the appropriate offset ratio) that were provided for the source, including fugitive components, will be returned. [basis:  offsets]

FUGITIVES

Conditions 1 through 14 for fugitive emissions apply only to POC gaseous, light liquid, and toxic services.

1.	Any new pump in light liquid or toxic service installed, modified or replaced as part of the Clean Fuels Project (CFP) shall be:

a.	equipped with dual mechanical seals, having a heavy liquid barrier fluid.  The barrier fluid reservoir shall be vented to a 95% efficient control device, or the barrier fluid shall be operated at a pressure higher than the process stream pressure; or

b.	a "canned" pump or magnetically driven pump; or any other sealless pump technology approved by the District.  [basis:  BACT]

2.	Condition deleted.

3.	For the purpose of these permit conditions, light liquid service shall be defined as a hydrocarbon liquid having an initial boiling point of 302oF or less.  Toxic service shall be defined as hydrocarbon service with greater than 1% benzene or 0.1% 1,3 butadiene, on a mass basis.  [basis:  BACT]

4.	Except for flow control valves and quarter turn valves, all valves that are installed, modified or replaced as part of the CFP, in gaseous hydrocarbon service, light liquid hydrocarbon service or toxic service, on 2" lines and smaller, shall be bellows valves or other "leakless" valve technology, as approved by the District, when commercially available.  Except as indicated below, no more than 2% of the 2" and smaller valves may be quarter turn valves.  S4002 &4003, Delayed Coker Unit Furnaces (F-13425 A and B), may contain up to 156 2" and smaller quarter-turn valves.  S4161, DC H-101 HP3 Steam Methane Reformer, may contain up to 140 2" and smaller quarter-turn valves.  Quarter turn valves shall use graphitic or teflon packing material, or equivalent, as approved by the District.  (revised per App. No. 15000, 6/28/95)  [basis:  BACT]

5.	Except as provided by Conditions 4 and 6, all other valves in POC gas, light liquid and toxic services installed, modified or replaced as part of the CFP shall be bellows sealed, live loaded, graphitic packed, teflon packed, or equivalent, as approved by the District.  [basis:  BACT]

6.	All flow control valves installed, modified, or replaced as part of the Clean Fuels Project, shall be equipped with live load packing systems and polished stems, or equivalent as approved by the District.  [basis:  BACT]

7.	[Condition is deleted because it is redundant to Regulation 8-18-302 and 8-18-401]

8.	 [Condition is deleted because it is redundant to Regulation 8-18-404]

9.	All flanges installed or opened in piping systems as a result of the CFP shall be equipped with graphitic-based gaskets, Teflon gaskets, or equivalent as approved by the District. [basis:  BACT][Part of condition is deleted because it is redundant to Regulation 8-18-304]

10.	All new centrifugal compressors installed, modified or replaced as part of the CFP shall be equipped with "wet" dual mechanical seals with a heavy liquid barrier fluid, or dual dry gas mechanical seals buffered with inert gas.  The barrier fluid pressure pot or inert gas system shall be vented to a control device with a 95% efficiency.  All reciprocating compressors installed, modified or replaced as part of the CFP shall be vented to a 95% efficient control device. [basis:  BACT][Part of condition is deleted because it is redundant to Regulation 8-18-303, Regulation 8-18-401 and NSPS 40CFR60 Subpart GGG]

11.	Total fugitive emissions from all equipment installed as a result of the Clean Fuels Project are 193.5 tpy precursor organic compounds and 14.7 tpy non-precursor organic compounds.  The owner/operator shall submit a count of compressors, pumps, valves, and flanges within 30 days of start-up of each unit.  If there is an increase in total emissions, the facility's cumulative emissions shall be adjusted  to reflect the difference between emissions based on predicted  versus actual component counts.  The owner/operator shall provide to the District any required offsets, at the offset ratio triggered at the time of permit issuance, but not less than 1.2:1.0, for any adjusted cumulative increase in emissions.  Additional offsets shall be provided within 60 days.  Fugitive emissions shall be calculated using the fugitive emission factors identified in the fugitive emission calculations in Appendix B of the Engineering Evaluation Report for Application Number 8407.  [basis:  offsets]

12.	No pressure relief valve in any new unit installed as part of the Clean Fuels Project in gaseous POC, light liquid, or toxic service shall vent to atmosphere.  All such relief valves shall be vented to the DC Clean Fuels Flare gas recovery system (S4201), or an equivalent control device approved by the District.  This condition does not apply to pressure relief valves on storage tanks or pressure relief valves that handle only low vapor pressure material (<0.05 psia).  If the District revises Regulation 8, Rule 28, Pressure Relief Valves at Petroleum Refineries and Chemical Plants, to increase the low vapor pressure exemption in section 8-28-111, then the vapor pressure exemption in this condition may be adjusted accordingly, not to exceed 0.5 psia.  [basis:  BACT]

13.	All process sample systems shall be the closed loop or continuous flow design.  In no event shall there be any line purging to process drains.  [basis:  BACT]

14.	All process drains installed, modified or replaced as part of the Clean Fuels Project shall be fitted with a "P" trap sealing system that prevents POC emissions from the process wastewater system from escaping through the drain.  [basis:  BACT]

FUEL GAS SYSTEM

15.	The refinery fuel gas burned in any Clean Fuels Project (CFP) combustion source and the existing CO Boilers (S1507, 1509 and 1512) shall be limited to all of the following:

a.	50 ppmv H2S averaged over any consecutive 24-hour period;

b.	0.1 grain/dscf (163 ppm) H2S averaged over 3 hours (per NSPS 40 CFR 60 subpart J); and

c.	70 ppm total reduced sulfur (hydrogen sulfide, methyl mercaptan, carbon disulfide, dimethyl sulfide, dimethyl disulfide, and carbonyl sulfide), expressed as H2S equivalent, averaged over any consecutive 12-month period.  (TRS limit amended 7/98, AN 18185)  [basis:  BACT]

16.	MRC shall install a continuous gaseous fuel monitor/recorder to determine the H2S content of the refinery fuel gas prior to combustion in any of the CFP combustion sources (except S4171 LUBS F-13000 LHT-2 Feed, which will be fired exclusively with natural gas).  MRC shall also install a continuous monitor/recorder, or equivalent as approved by the District, to measure total reduced sulfur compounds in the refinery fuel gas, expressed as H2S equivalent.  [basis:  BACT]

16A.	To verify compliance with the H2S and TRS limits of Condition No. 15 for the CO Boilers (S1507, 1509 and 1512), MRC shall install a natural gas line connection upstream of the CO Boilers in order to combine natural gas with the refinery fuel gas.  MRC shall also operate a fuel flow ratio control system, consisting of fuel flow meters, a ratio detector, and a ratio controller, as described in permit application number 18185.  This ratio controller set point shall be the same natural gas to refinery fuel gas ratio as occurs in the fuel supply system for the Clean Fuels Project combustion sources.  Compliance with the H2S and TRS limits for the CO Boilers will be inferred by directly measuring the H2S and TRS contents of the refinery fuel gas supply for the Clean Fuels Project combustion sources.  The flow meters, ratio detector and ratio controller shall be maintained and calibrated according to manufacturer specifications.  (Added 7/98 per AN 18185)  [basis:  offsets]

17.	The owner/operator shall calculate and record the: (1) 3-hour H2S content; (2) 24-hour rolling average H2S content; and (3) TRS content of the refinery fuel gas, for determining compliance with Condition No. 15, based on the previous 24 individual hourly averages.  On a monthly basis, the owner/operator shall report daily fuel consumption and the 3-hour average, 24-hour average, and maximum daily H2S content of the refinery fuel gas, for combustion sources associated with the CFP.  The owner/operator shall also report the daily, monthly, and 12-month average TRS concentrations in the refinery fuel gas.  [basis:  cumulative emissions]

COMBUSTION SOURCES

General Combustion

The following are general requirement for all new or modified combustion sources associated with the Clean Fuels Project (CFP):

18.	All combustion sources installed, modified or replaced as part of the CFP shall fire natural gas, refinery fuel gas, pressure swing absorber (PSA) gas, ultra low sulfur distillate (ULSD), liquified petroleum gases, or flexigas.  In no case shall any combustion source burn a fuel with: (1) an H2S concentration exceeding 50 ppm, 24 hour average;  or (2) a total reduced sulfur content exceeding 70 ppm, annual average.  [basis:  offsets, cumulative increase]

19.	For the purposes of these conditions ULSD is defined as a gaseous hydrocarbon mixture composed of C6 and lighter components, produced by the Straight Run Hydrotreater, Saturated Gas Plant, Cracked Gasoline Depentanizer, and Alky Depentanizer.  [basis:  offsets, cumulative increase]

20.	Every combustion source (or group of sources with the same emission factors) installed, modified or replaced as part of the CFP shall be equipped with a continuous fuel flow monitor in order to determine fuel consumption.  Fuel rates shall be recorded by a process computer or recorder.  This condition does not apply to flare pilots that are fueled with natural gas at a constant rate.  [basis:  offsets, cumulative increase]

21.	The owner/operator shall calculate and record emissions from all combustion sources (or group of sources with the same emission factors) on a monthly basis, and include those emissions in the monthly reports in accordance with conditions L and M.  [basis:  offsets, cumulative increase]

22.	For purposes of this permit, unless otherwise specified in a permit condition, startup and shutdown of combustion sources are limited to a maximum of 8 hours in duration for non-SCR controlled combustion sources, or 24 hours in duration for SCR controlled combustion sources.

a.	Upon approval by the District, the startup or shutdown period may be extended to a period not to exceed 72 hours for the following situations:

1.	the startup or shutdown has been proceeding continuously, and the owner/operator has been increasing or decreasing temperatures at a rate limited by metallurgy or other physical constraints prescribed in their startup/shutdown procedure.

2.	startup following installation or replacement of refractory lining.

3.	startup following initial catalyst pre-sulfiding following catalyst replacement or catalyst regeneration.

b.	For the Cogeneration Units (Sources S4190 through 4193), the startup period following initial construction also includes a commissioning period, consisting of an initial gas turbine roll, steam blowing, and turbine/generator synchronization.  Natural gas is the only fuel to be used during commissioning.  Emissions of NOx, SO2, CO, POC and PM10 that result during commissioning shall be included in facility’s monthly report and in the emission totals in Condition A.  (Cond. 22.b. added 9/11/95 per AN 25728).  [basis:  BACT]

23.	The firing rates for all combustion sources indicated in these permit conditions are on a higher heating value (HHV) basis.  [basis:  cumulative emissions]

Cogeneration Plant (S4190, 4191, 4192, and 4193)

Conditions 24a, 24b, 28b, 28c and 28d are requested by the California Energy Commissions, pursuant to CEC approval of the Small Power Plant Exemption for these sources.

24a.	The cogeneration power plant (S4190, 4191, 4192, and 4193) shall not use fuels other than natural gas, commercial grade propane, commercial grade butane, refinery fuel gas (RFG), flexigas (FXG), and ultra low sulfur distillate (ULSD).  The owner/operator shall install a CEM (continuous emission monitor) for SO2 before the CO catalyst for the gas turbine and HRG boiler pairs.  [basis:  offsets, cumulative increase]

24b.	During any calendar day, emissions from the cogeneration plant (S4190, 4191, 4192, and 4193) shall not exceed the daily limits indicated in the table below.  This condition does not apply to days with startup or shutdown conditions, as defined by Condition 22.

Daily Emission Limits for the Facility Cogeneration Power Plant

Daily Limit

Pollutant	(lb/day)

NOx	610.0

POC	500.0

SO2	458.0

PM10	365 +

[(610 - NOx actual)/1.9] +

[(458 - SOx actual)/2.5]  lb/day

(where NOx and SO2 actual are in lb/day)

Compliance with the daily emission caps shall be based on the records on daily fuel consumption, data from the NOx CEM, data from the O2 CEM, data from the SO2 CEM, and the emission factors established in the source test required in Condition 28b.  As indicated, the daily emission limit for PM10 is dependent on actual hourly CEM data for NOx and SO2 with a base of 365 lb/day, averaged over any single calendar day.  The NOx and SO2 emission components in the PM10 daily emission limit may not be credited to any other calendar day except the one in which they occur.  [basis: offsets, cumulative increase]

24c.	Emissions of nitrogen oxides, as NO2, from the gas turbine and HRG boiler pairs ( S4190/4191 and S4192/4193) shall be abated at all times with selective catalytic reduction systems (SCR) and catalytic oxidation systems (A4190/4191; A4192/4193), and shall not exceed 5 ppmv, dry, corrected to 15% oxygen, averaged over 3 hours.  This condition shall apply to all periods of gas turbine operations, except periods of startup and shutdown, as defined by Condition 22.  [basis:  BACT]

25.a.	Emissions of CO from the gas turbine and HRG boiler pairs (S4190/4191 and 4192/4193) shall be abated at all times with selective catalytic reduction systems (SCR) and catalytic oxidation systems (A4190/4191; A4192/4193), and shall not exceed 6.5 ppm, dry, at 15% oxygen, averaged over 8 hours; or 90% overall reduction on a mass basis.  The 90% overall reduction shall be determined by measuring inlet and outlet CO concentrations from the catalytic oxidizer.  This condition shall apply to all periods of gas turbine operations, except periods of startup and shutdown, as defined by Condition 22.  [basis:  BACT]

25.b.	Emissions of POC from the gas turbine and HRG boiler pairs (S4190/4191 and 4192/4193) shall not exceed 0.013 lb/Million BTU.  This condition shall apply to all periods of gas turbine operations, except periods of startup and shutdown, as defined by Condition 22.  (Added per AN 14577; 3/95)  [basis:  BACT]

26.	Except during period of startup and shutdown, as defined by Condition 22, ammonia emissions (ammonia slip) from the SCR / catalytic oxidation systems (A4190/4191, A4192/4193) shall not exceed 20 ppm of ammonia, dry, corrected to 15% oxygen.  [basis:  BACT]

27.	On source pairs S4190/4191 and S4192/4193, the owner/operator shall install, calibrate, maintain, and operate the following: (1) a District-approved continuous emission monitor and recorder for NOx and O2; and (2) a water to fuel ratio monitor, per 40CFR60 section 60.334.  [basis:  BACT]

28a.	Start-up condition deleted.

28b.	Start-up condition deleted.

28c.	Start-up condition deleted.

28d.	Start-up condition deleted.

Hydrogen Plant (S4160)

29.	Except during startup and shutdown periods, as defined by Condition 22, emissions of nitrogen oxides, as NO2, from the DC H-101 HP3Steam Methane Reformer furnace (S4161) shall not exceed 10 ppmv, dry, corrected to 3% oxygen, averaged over 3 hours.  [basis:  BACT]

30.	For the hydrogen plant steam methane reformer furnace (S4161), emissions of CO shall not exceed 25 ppmv, dry, corrected to 3% oxygen, averaged over 8 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

31.	S4161 shall be abated at all times by A4161, Selective Catalytic Reduction for HP3 Steam Methane Reformer.  Ammonia emissions (ammonia slip) from the SCR unit (A4161) shall not exceed 20 ppm of ammonia, dry, corrected to 3% oxygen.  [basis:  BACT]

32.	On source S4161, the owner/operator shall install, calibrate, maintain, and operate a District-approved continuous emission monitor and recorder for NOx and O2.  [basis:  BACT]

33.	Except during startup, shutdowns and emergency upsets, the HP-3 deaerator vent must be vented to S4161, DC H-101 HP3 Steam Methane Reformer.  During startups and shutdowns, the deaerator vent may be routed to atmosphere for no longer than 9 hours while feed gas is in the process.  For each bypass, the owner/operator shall keep a record of the date, duration of bypass, and reason for bypass.  These records shall be retained on site for at least five years, and shall be made available to the District upon request. (Added 2/98, AN 18235.  Original Cond. 33 – A/C start-up source test requirement deleted)  [basis:  Reg. 8-2]

34.	Combined refinery fuel gas and flexigas, on a heat input basis, shall not exceed 57.6% of the design duty (524 MM BTU/hr HHV) for source S4161, DC H-101 HP3 Steam Methane Reformer.  [basis:  offsets]

Furnaces with Selective Catalytic Reduction (S4002, 4003, 4031 and 4141)

(Conditions 35 through 39 amended per AN 14577; 3/95)

35.	For sources S4002, 4003, 4031 and 4141, emissions of nitrogen oxides, as NO2, shall not exceed 10 ppmv, dry, corrected to 3% oxygen, averaged over 3 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

36.	For sources S4002, 4003, 4031 and 4141, emissions of CO shall not exceed 50 ppm, dry, corrected to 3% oxygen averaged over 8 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

37.	Sources S4002, 4003, 4031 and 4141 shall be abated at all times by the corresponding abatement device (A4002, 4003 and 4141, Selective Catalytic Reduction Systems).  Ammonia emissions (ammonia slip) from the SCR units (A4002, 4003 and 4141) shall not exceed 20 ppm of ammonia, dry, corrected to 3% oxygen.  [basis:  BACT]

38.	On sources S4002, 4003, 4031 and 4141, the owner/operator shall install, calibrate, maintain, and operate a District-approved continuous emission monitor and recorder for NOx and O2.  [basis:  BACT]

39.	Start-up condition deleted.

Furnaces with Low NOx Burners (S4021and 4171)

(Conditions 40 through 44 amended per AN 14577; 3/95)

40.	For all natural draft furnaces (S4021and 4171), emissions of nitrogen oxides, as NO2, shall not exceed 25 ppmv, dry, corrected to 3% oxygen, averaged over 3 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

41.	In the event that any furnace (S4021and 4171) is installed as a forced draft furnace, emissions of nitrogen oxides for that furnace, as NO2, shall not exceed 20 ppmv, dry, corrected to 3% oxygen, averaged over 3 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

42.	For sources S4021 and 4171, emissions of CO shall not exceed 50 ppm, dry, corrected to 3% oxygen averaged over 8 hours, except during startup and shutdown, as defined by Condition 22.  [basis:  BACT]

43.	The owner/operator shall install low NOx burners on sources S4021and 4171, to comply with the limit in Condition No. 43.  [basis:  BACT]

44.	Start-up condition deleted.

TANKAGE

45.	Tanks S13, 4319, 4334, 4350, and 4356 shall be fixed roof tanks connected to existing vapor recovery systems.  In accordance with the NSPS requirements of 40 CFR 60, Subpart Kb, these tanks shall be maintained leak free (less than 500 ppm above background, as methane).  S4322, Tank 1471 Sour Water, shall be an internal floating roof tank with a liquid mounted primary seal and a zero-gap secondary seal.  (Amended Aug. 1995, AN 25076)  [basis:  BACT]

46.	Tanks S4311, 4329, 4330, 4349shall be pressure tanks with a minimum pressure relief valve (PRV) set pressure of 15 psig.  [basis:  BACT]

47.	Tanks S4310, , 1129, 1130, and 1131 shall be external floating roof tanks which have liquid mounted primary seals and zero-gap secondary seals.  There shall be no ungasketed roof fittings.  Except for roof legs, each roof fitting shall be of the design that yields the minimum roof fitting losses (per EPA Compilation of Air Pollution Emission Factors, AP-42, Supplement E, Section 12.3.2, Table 12.3-11).  The following list indicates the type of control required for a variety of typical roof fittings.  Control techniques for roof fittings not included in this list shall be subject to District approval, prior to installing the roof on the tank.

| Fitting Type | Control Technique |
| --- | --- |
| Access hatch | Bolted cover, gasketed |
| Guide pole / Well | Unslotted guide pole, gasketed sliding cover; or Slotted with controls per API 2517 Addendum (See Note 1) |
| Gauge float well | Bolted cover, gasketed |
| Gauge hatch / Sample well | Weighted mechanical actuation, gasketed |
| Vacuum breaker | Weighted mechanical actuation, gasketed |
| Roof drain | Roof drain does not drain water into product |
| Roof leg | Fixed; or Adjustable, with vapor seal boot, or gasket between roof leg and leg sleeve (amended 2/9/96) |
| Rim vent | Weighted mechanical actuation, gasketed |

Note 1:	Slotted Guide Pole Control Configuration, per Addendum to API Publication 2517, May 1994, shall include to following components:

a.	Sliding cover;

b.	Well gasket;

c.	Pole sleeve with pole wiper approximately 6 inches above sliding cover, or District approved equivalent;

d.	Float with float wiper approximately 1 inch above the sliding cover, or alternately a float with multiple wipers.  (Added per letter of 2/28/95)  [basis:  BACT]

49.	Start-up condition deleted.

50.	The total volume of Isom Feed stored in all storage tanks shall not exceed 100,000 barrels at any given time. the owner/operator shall keep daily records of the information listed below.  These records shall be summarized in the monthly report, per Conditions L and M.

a.	total volume of Isom Feed that is stored;

b.	the tanks, by source number, in which Isom Feed is stored; and

c.	the days on which Isom Feed is stored in other tanks.  [basis:  cumulative increase]

51.	All POC emissions from tank cleaning (degassing) or tank filling (including initial filling and filling after landing roof) shall be vented to a control device with an overall capture and destruction efficiency of 95%, on a mass basis.  For the purposes of this permit condition, the tank cleaning control device shall be operated until the concentration of organic compounds in the tank is less than 10,000 ppm, as methane.  [basis:  BACT]

WASTEWATER MAINTENANCE DROP OUT VESSELS (MDO)

52.	The DC MDO (maintenance drop out) vessels (S4211 and 4212) shall be completely enclosed and vapor tight (100 ppm, as methane), and vented to the DC Clean Fuels Flare gas recovery system (S4201) or an incinerator with a minimum destruction efficiency of 98.5 % by weight.  The owner/operator shall notify the District at least 60 days prior to startup if an abatement device other than AS4201 will be used.  Any new abatement device not authorized by this permit must first receive a separate Authority to Construct from the District.  [basis:  BACT]

53.	Each MDO vessel that is installed as part of the Clean Fuels Project (CFP), shall meet the following operating requirements:

a.	All recycled or recovered oil and oil emulsions shall be transferred through closed, leak tight systems (100 ppm, as methane).

b.	Except for wastewater flow from S4170 LUBS Lube Hydrotreater 2 (LHT2), and S4180 OPCEN Sulfur Plant 4 (SRU4), all wastewater streams from the CFP units shall be transferred to the existing Effluent Treatment Plant through closed systems.

c.	All wastewater flow from S4170 and 4180 shall be transferred to the existing Effluent Treatment Plant through covered systems that meet the requirements of Regulation 8, Rule 8.

d.	All pressure relief valves shall vent to the DC Clean Fuels Flare gas recovery system (S4201).  [basis:  BACT]

- Condition deleted.

- The owner/operator shall install a District-approved continuous hydrocarbon analyzer/recorder to determine hydrocarbon vapor concentration from the cooling water in Cooling Water Tower (CWT-13278) (S4210).  The purpose of this analyzer is to serve as an early warning/detection device to indicate a possible heat exchanger leak of process fluid into the cooling water system.  The analyzer will provide baseline data, which will be statistically evaluated to determine an  Action Level.  Any hydrocarbon reading above the Action Level will trigger an alarm.  Shell's facility's monthly report shall include a summary of the baseline data, the action level, and a description of findings and actions taken for each incident above the action level.  The actual implementation of this permit condition shall be subject to the approval of the District permit engineer upon startup of the cooling tower.  [basis:  BACT]

FLARE

56.	Start-up condition deleted.

57.	In order to prevent smoking, the first stage of the DC Clean Fuels Flare (S4201) shall be provided with steam-assist.  [basis:  BACT]

58.	Start-up condition deleted.

59.	Start-up condition deleted.

60.	All flare pilots shall be fueled continuously with natural gas or LPG.  The flare will be operated only during period of emergency upset or breakdown.  Routine venting of process gases shall go to the flare gas recovery system.  [basis:  BACT]

61.	The flare shall have a hydrocarbon destruction efficiency of at least 98.5%, on a mass basis.  [basis:  BACT]

62.	Start-up condition deleted.

SULFUR RECOVERY SYSTEM

63.	The sulfur pit for sulfur plant 4 (S4347) shall be fixed roof and vented to the Thermal Oxidizer for Sulfur Plant 4 (A4181) at all times during operation of A4181.  [basis:  BACT]

64.	The Sour Water Strippers shall remove and recover at least 95% (by weight) of the H2S and remove 95% (by weight) of the ammonia from the sour water feed to the unit; or reduce the concentration of NH3 to no more than 50 ppm and the concentration of H2S to no more than 25 ppm in the stripped water.  This condition shall not apply during periods of startup and shutdown.  The startup and shutdown exemption in this Condition does not exclude the owner/operator from complying with Regulation 9-1-312.2.  [basis:  BACT]

65.	The SCOT No. 4 and Thermal Oxidizer for Sulfur Plant 4 (A4180 and A4181) shall achieve a 99.9% weight conversion of reduced sulfur compounds to elemental sulfur.  [basis:  BACT]

66.	The concentration of total reduced sulfur (H2S, COS, and CS2) in the facility’s Claus Offgas Treatment (SCOT) unit exhaust, prior to the SCOT Thermal Oxidizer for Sulfur Plant 4 (A4181), shall not exceed 100 ppm, dry, at 0% oxygen, averaged over 8 hours.  [basis:  BACT]

67.	Total reduced sulfur compound concentrations in the SCOT tailgas shall be measured using gas chromatography to determine compliance with Condition No. 66.  Analytical data shall be accumulated by the process computer and shall be made available to the District upon request.  A summary of daily average and maximum total reduced sulfur concentrations shall be included in the monthly reports required by Conditions M and N. If the gas chromatography unit proves unable to measure CS2, the owner/operator shall conduct three separate 3-hr source tests while the SCOT unit is operating at or near maximum capacity, to determine the average concentration of CS2 in the exhaust from the SCOT unit.  If the average CS2 concentration is less than 5 ppm, dry at 0% oxygen, then the average CS2 concentration may be deducted from the 100 ppm limit in Condition No. 66, and reference to CS2 may be deleted from Conditions 66 and 67.  [basis:  BACT]

68.	The SCOT Thermal Oxidizer shall achieve a minimum of 95 %weight conversion of reduced sulfur compounds to SO2.  The concentration of H2S in the exhaust from A4181 SCOT Thermal Oxidizer for Sulfur Plant 4 shall not exceed 2.5 ppm, dry, at 0% oxygen, averaged over 24 hours.  The concentration of SO2 in the exhaust from A4181 SCOT Thermal Oxidizer for Sulfur Plant 4 shall not exceed 50 ppm, dry, at 0% oxygen, averaged over 24 hours.  The 50 ppm SO2 limit excludes the sulfur contribution from the Sulfur Pit.  Because Condition No. 63 requires the sulfur pit to be controlled by A4181, the 50 ppm SO2 limit will be adjusted to include an additional 50 ppm SO2 emissions from controlling the Sulfur Pit.  If for some reason the sulfur pit is not being vented to the SCOT Thermal Oxidizer, SO2 will be limited to 50 ppm as indicated above.  [basis:  BACT]

69.	A District-approved SO2 continuous emission monitor and recorder (CEM) shall be installed, calibrated, maintained, and operated according to District regulations, on the exhaust of the Tailgas Incinerator, to determine compliance with Condition No. 68.  In accordance with Condition G, the owner/operator shall conduct a source test for H2S following startup of source S4180, OPCEN Sulfur Plant  4 (SRU4), and annual thereafter, to determine compliance with Condition 68.  [basis:  BACT]

70.	The concentration of CO in the exhaust from the Tailgas Incinerator (A4181) shall not exceed 100 ppm, dry, at 0% oxygen, averaged over 8 hours.  [basis:  BACT, good combustion]

71.	In the event that the OPCEN Sulfur Plant 4 (SRU4) (S4180), SCOT No. 4 (A4180), and/or SCOT Tailgas Thermal Oxidizer for Sulfur Plant 4 (A4181) is shut down, all acid gas feed to the SRU4 shall be completely curtailed or reallocated to other sulfur recovery systems so that no acid gases are vented to the flare, nor any unabated SRU4 tailgas (tailgas not treated in SCOT No. 4 ) is routed to the Thermal Oxidizer for Sulfur Plant 4 (A4181).  This shall be completed within the following timeframes:

a.	Prior to any planned shutdown of OPCEN Sulfur Plant 4 (SRU4) (S4180), SCOTNo. 4 (A4180), or Thermal Oxidizer for Sulfur Plant 4 (A4181);

b.	Within 24 hours of any unplanned shutdown of OPCEN Sulfur Plant 4 (SRU4) (S4180), SCOTNo. 4 (A4180), or Thermal Oxidizer for Sulfur Plant 4 (A4181).

and the District shall be notified of all such occurrences within 48 hours.  [basis:  BACT, offsets]

The owner/operator shall calculate excess emissions resulting from any unplanned shutdown of SRU4, SCOT No. 4 or SCOT Tailgas Thermal Oxidizer.  These excess emissions shall be included for the purposes of determining compliance with Condition A.  If inclusion of these excess emissions causes the owner/operator to exceed the monthly trigger level in Condition A, Facility's Compliance Plan shall describe how the owner/operator will achieve the additional emission reductions necessary to maintain compliance with the consecutive twelve-month limit.  These reductions will be provided to the extent necessary by one or more of the following:

c.	Additional reductions in SO2 emissions from the CO Boilers (S1507, 1509, and/or 1512);

d.	the use of banked emissions reduction credits; and

e.	other means approved by the District.

For the purposes of determining compliance with the twelve-month emission limits of Condition A, an SRU4 outage will not result in immediate violation of Condition A, when the owner/operator would have otherwise complied, had the outage not occurred.  If an SRU outage occurs, the District will only include the SRU excess emissions for calculating the twelve-month totals beginning five months prior to the month in which the outage occurred.  Therefore, the owner/operator will have six months following an SRU4 outage to reduce emissions sufficiently to keep their twelve-month emission totals below the limit indicated in Condition A.  [basis:  offsets]

72.	The loading of elemental sulfur or sulfuric acid (fresh or spent) into trucks or railcars shall be controlled by a scrubber with an overall capture/removal efficiency of at least 95%, on a mass basis.  H2S emissions from the scrubber shall not exceed 5 ppm.  [basis:  BACT, odors]

73.	Start-up condition deleted.

PENTANE RAILCAR LOADING RACK

74.	Source S4338 shall only be used to load pentane, butane, or LPG into railcars under pressure, with a minimum PRV set pressure of 15 psig.  Vapors from the loading operation shall be collected and controlled by S1470, LOG LPG Loading Flare, with an overall capture and destruction of 98.5% by weight.  [basis:  BACT]

COKE HANDLING

75.	Visible emissions from any aspect of the DC Coke Handling Facility (S4005) shall not exceed Ringelmann No. 0.5, or 10% opacity, for three minutes in any consecutive 60-minute period.  [basis:  BACT]

76.	In order to reduce the potential for fugitive particulate emissions, the owner/operator shall wash the pad area surrounding the Coke Pit (where coke drops from the coker) at least once per day, when the coker is operating or when coke is being removed from the coke drums.  [basis:  BACT]

77.	Water Sprays shall be used to control particulate emissions from the Crushing operation.  The moisture content of the coke, upon discharge from the crusher, shall be maintained at or above 8 %weight.  The following method shall be used to demonstrate compliance with this condition.

a.	The moisture content of the coke shall be analyzed at least once per day;

b.	Compliance is achieved with a moisture content is greater than or equal to 8% by weight;

c.	No more frequently than once in any 30-day period, the operator has the option of re-sampling according to the procedures in subsections d through f below.  Any moisture content less than 6% by weight constitutes a violation of this condition.

d.	If the analysis of the first sample shows a moisture content greater than or equal to 6%, but less than 8% by weight, the operator shall increase the water spray rate, and re-sample within 2 hours of the first sample;

e.	If the moisture content of the second sample is greater than 7%, but less than 8 % by weight, the operator may take one final sample within 1 hour of the second sample;

f.	If the moisture content of the third sample is greater than or equal to 8% by weight, this condition is met.

The operator shall maintain records of all moisture content analyses for at least five years from the date of the analysis.  These records shall be made available to the District upon request. (Amended 12/98, AN 18589)  [basis:  BACT]

78.	All coke conveying systems downstream of the crusher shall be completely enclosed.  [basis:  BACT]

79.	The owner/operator shall use water sprays, as necessary to minimize particulate emissions from the surfaces of the coke piles within the Coke Corral.  The Coke Corral shall be enclosed and controlled by A4005, Particulate Collection Device for Delayed Coking Unit for Coke Corral , at all times during operation of the coke corral.  Particulate emissions from A4005 shall not exceed 0.01 grain/DSCF (grain per dry standard cubic foot) of exhaust.  [basis:  BACT]

80.	To minimize dust carry-out, doors to the Coke Corral shall be kept closed except for ingress or egress.  The Coke Corral shall be equipped with a vehicle wash off area, inside of the Coke Corral, to remove coke dust from the vehicle prior to exiting the Coke Corral.

If particulate emissions from the Coke Corral result in 3 or more visible emission violations within a six month period, or two public nuisance violations within a 5 year period, the owner/operator shall install additional controls, as approved by the District, which may include one or more of the following:

a.	Additional water sprays;

b.	Chemical suppressant in water spray system;

c.	Additional/improved enclosures;

d.	Wind screens;

e.	Coke silos abated by fabric filters; or

f.	Equivalent, as approved by the District.  [basis:  BACT]

81.	The Truck/Railcar Loading Hopper shall be completely enclosed and controlled by A4006, Particulate Collection Device for Delayed Coking Unit for Hopper Loading , at all times during railcar loading.  Particulate emissions from A4006 shall not exceed 0.01 grain/DSCF (grain per dry standard cubic foot) of exhaust.  (modified 8/96, AN 15942)  [basis:  BACT]

82a.	A Wind Shield shall be used to shroud the discharge point from the Truck/Railcar Loading Hopper and the opening of the truck or railcar at all times during truck or railcar loading.  The trucks and railcars shall be loaded so that the level of coke is not higher than the top of the truck trailer or railcar.  After loading onto railcars, the coke surface shall be sprayed with a surface binding agent to minimize particulate entrainment during transit.  After loading onto trucks, the coke shall be completely covered with tarpaulin or other similar material, to minimize particulate spillage and entrainment during transit.  [basis:  BACT]

82b.	Before leaving the coke loading area, the truck shall pass through a water wash system to remove coke from the truck and trailer tires, wheels and undercarriage, in order to minimize the tracking of coke onto the roadway. [basis:  BACT]

82c.	At least twice per week, the owner/operator shall use a “street sweeper” that is equipped with a vacuum device to clean the road that is used by coke haul trucks.  At a minimum, the road shall be cleaned between the Truck/Railcar Loading Hopper and the final point at which the trucks depart facility property onto the public roadway.  Both directions of the roadway shall be cleaned. [basis:  BACT]

82d.	With the exception of maintenance activities, no vehicle shall be allowed to enter from any unpaved area onto the roadway that is used by coke haul trucks (coke haul road).  Following a maintenance activity for which it is necessary for a vehicle to drive onto the coke haul road from an unpaved area, the owner/operator shall use a “street sweeper” to sweep the road from the point of entry of the maintenance vehicle to the final point at which the maintenance vehicle departed the coke haul road. [basis:  BACT]

82e.	Start-up condition deleted..

82f.	Start-up condition deleted.

83.	The owner/operator shall not load out more than a combined total of 750,000 tons of coke onto trucks or railcars in any consecutive twelve month period.  The owner/operator shall keep monthly records and twelve-month averages of coke load out.  (modified 8/96, AN 15942)  [basis:  offsets, cumulative increase]

84.	To reduce potential odors from the coke handling operations, water from any aspect of the coke handling operations, including water from the hydraulic drill, crusher water sprays, and coke storage pile water sprays, shall be collected and hard-piped to tankage.  All storage tanks, oil/water separators, or other equipment with the potential to receive water from coke handling operations shall be constructed in such a way as to facilitate retrofit with roofs or other enclosures, and control by an abatement device.  If odors from the coke discharge water storage or handling result in or contribute to a public nuisance, the owner/operator shall enclose all storage tanks, oil/water separators and other equipment that receives water from coke handling operations, and install vapor recovery/control on the storage tanks, oil/water separators, and other equipment to reduce hydrocarbon and total reduced sulfur compounds by at least 95%, on a mass basis.  [basis:  odors]

NOx OFFSETS

85.	Total emissions of nitrogen oxides, as NO2, from the three existing CO Boilers (S1507, 1509, and 1512) shall not exceed 6770  lb/day [baseline date of 8/8/92: (1799.2 - 563.7 TPY) x (2000 lb/ton) / (365 day/yr)], averaged over any consecutive 365-day period.

(Amended 3/95 per AN 14577; baseline reduced by 5.9% on June 2, 1995)  [basis:  offsets]

86.	The owner/operator shall continue to operate the NOx continuous emission monitors on the CO Boilers in accordance with the District's Manual of Procedures, to verify compliance with Condition No. 85.  The owner/operator shall use the CEM data to calculate NOx emissions on a daily basis.  If a NOx CEM on a CO Boiler is not functioning for more than one day, then NOx emissions from that CO Boiler will be assumed to be 2257 lb/day (6770 / 3) for each day that the monitor is not functioning, if the owner/operator can demonstrate that urea injection is occurring at a normal rate.  If the owner/operator cannot demonstrate that urea injection is occurring at a normal rate, then NOx emissions from that CO Boiler will be assumed to be the uncontrolled emission rate of 3286 lb/day (9858 / 3) for each day that the monitor is not functioning.  [basis:  offsets]

(Amended 3/95 per AN 14577; baseline reduced by 5.9% on June 2, 1995)

87.	The owner/operator shall calculate the 365-day average NOx emissions to determine compliance with the emission limit in Condition No. 85.  Emission calculations will be based on the CEM data, and exhaust flow from fuel combustion calculations.  [basis:  offsets]

88.	The owner/operator shall report actual daily NOx emissions and the 365-day average NOx emissions in the monthly reports required by Conditions L and M.  [basis:  offsets]

89.	Start-up condition deleted.

SO2 OFFSETS

90.	Total emissions of SO2 from the three existing CO Boilers (S1507, 1509, and 1512) shall not exceed 6805 lb/day [1991 baseline: (1427.2 - 185.2 TPY) x (2000 lb/ton) / (365 day/yr)], averaged over any consecutive 365-day period.  (Amended 3/95 per AN 14577; 7/98 per AN 18185)  [basis:  offsets]

91.	The owner/operator shall continue to operate the SO2 continuous emission monitor/recorders on CO Boilers 1 and 2 in accordance with the District's Manual of Procedures.  The owner/operator shall install and operate a new SO2 continuous emission monitor/recorder on CO Boiler 3 (S1512).  The owner/operator shall use the CEM data to verify compliance with Condition No. 90.  The owner/operator shall use the CEM data to calculate SO2 emissions on a daily basis.  If a SO2 CEM on a CO Boiler is not functioning for more than one day, then SO2 emissions from that CO Boiler will be assumed to be 2262 lb/day (6805 / 3) for each day that the monitor is not functioning.  (Amended 3/95 per AN 14577; 7/98 per AN 18185)  [basis:  offsets]

92.	The owner/operator shall calculate the 365-day average SO2 emissions to determine compliance with the emission limit in Condition No. 90.  Emission calculations will be based on the CEM data, and exhaust flow from fuel combustion calculations.  [basis:  offsets]

93.	The owner/operator shall report actual daily SO2 emissions and the 365-day average SO2 emissions in the monthly reports required by Conditions L and M.  [basis:  offsets]

94.	Start-up condition deleted.

95.	Start-up condition deleted.

RAIL EMISSIONS

96.	The owner/operator shall calculate rail emissions for coke and pentane shipments from the CFP on a monthly basis, in accordance with the rail emission calculation procedure in Appendix B of the Engineering Evaluation Report for Application No. 8407.  [basis:  offsets, cumulative increase]

97.	Monthly rail emissions for coke and pentane shipments from the CFP shall be included in the monthly report required by Conditions L and M.  [basis:  offsets, cumulative increase]

TOXICS

98.	The carcinogenic risk from any source in the Clean Fuels Project shall not exceed the risk attributed to that source in the Project Health Risk Assessment (HRA).  [basis:  toxics]

99.	Start-up condition deleted.

100.	Start-up condition deleted.

101.	Start-up condition deleted.

102.	Start-up condition deleted.

103.	Start-up condition deleted.

SOURCE TESTING

- Effective April 1, 2004, for S4021 and S4171, the owner/operator shall conduct a District approved source test annually for S4171 and semi-annually for S4021 to determine the nitrogen oxide emissions from each source to verify compliance with Part 40 and 41, respectively. For S4021, the interval between source tests shall not exceed eight months.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced from annully to once every five years or from semi-annually to twice every five years if three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to annually or semi-annually, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced if another three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4002, S4003, S4031, and S4141, the owner/operator shall conduct a District approved source test semi-annually to determine the carbon monoxide emissions from each source to verify compliance with Part 36. The interval between source tests shall not exceed eight months.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced from annully to once every five years or from semi-annually to twice every five years if three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to annually or semi-annually, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced if another three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4021 and S4171, the owner/operator shall conduct a District approved source test annually for S4171 and semi-annually for S4021 to determine the carbon monoxide emissions from each source to verify compliance with Part 42. For S4021, the interval between source tests shall not exceed eight months.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced from annully to once every five years or from semi-annually to twice every five years if three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to annually or semi-annually, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced if another three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4161, the owner/operator shall conduct a District approved source test semi-annually to determine the carbon monoxide emissions from each source to verify compliance with Part 30. The interval between source tests shall not exceed eight months.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced from annully to once every five years or from semi-annually to twice every five years if three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to annually or semi-annually, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced if another three consecutive annual or four consecutive semi-annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4190, S4191, S4192, and S4193, the owner/operator shall conduct a District approved source test annually to determine the carbon monoxide emissions from each source to verify compliance with Part 25a.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4180, the owner/operator shall conduct a District approved source test annually to determine the carbon monoxide emissions from the source to verify compliance with Part 70.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4180, the owner/operator shall conduct a District approved source test annually to determine the efficiency and hydrogen sulfide (H2S) emissions to verify compliance with Part 72.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4002, S4003, S4031, and S4141, the owner/operator shall conduct a District approved source test annually to determine the ammonia (NH3) emissions to verify compliance with Part 37.  The results of the source test shall be made available to the District within 60 days of the source test and kept for minimum of 5 years from the date of the report. [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4161, the owner/operator shall conduct a District approved source test annually to determine the ammonia (NH3) emissions to verify compliance with Part 31.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report. [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4190, S4191, S4192, and S4193, the owner/operator shall conduct a District approved source test annually to determine the ammonia (NH3) emissions to verify compliance with Part 26.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report. [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for S4190, S4191, S4192, and S4193, the owner/operator shall conduct a District approved source test annually to determine the VOC emissions to verify compliance with Part 25b.  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard.  [basis: Regulation 2-6-409.2]

115.Effective April 1, 2004, the owner/operator will demonstrate compliance with Regulation 9-1-313.2 by performing the following for S4180:

- Perform monitoring of the facility fuel gas for H2S using continuous online H2S analyzers.  Refinery fuel gas will be monitored at both refinery fuel gas distribution systems (Clean Fuels and Main Refinery systems).

- Perform monitoring of the Sour Water Stripper Bottoms Water for NH3 concentration in facility's Sour Water Strippers twice per week.

The owner/operator shall summarize the results of these analyses in a District approved log.  The owner/operator shall include in the log an annual report summarizing the facility's compliance with the requirements of Regulation 9-1-313.2.  The owner/operator shall retain each analysis, annual report, and District approved log on site and available for inspection upon District request for a minimum of 5 years from the date of record.  (Basis: Regulation 9-1-313.2)

Condition # 12911

For S1426, CP Catalytic Cracking Unit (CCU):

- The Additive Catalyst Storage and Injection System associated with the CP Catalytic Cracking Unit (CCU) (S1426) shall be abated by the Catalyst Additive Storage and Injection System for CCU (A1427) Baghouse (A1427).  . [basis: BACT]

- A visible emission that is darker than No. 0.5 on the Ringelmann Chart, or of such opacity as to obscure an observer's view to an equivalent or greater degree, shall not be emitted from the Baghouse (which is integral to the Catalyst Storage and Injection System) for a period or periods aggregating more than three minutes in any hour. [basis: BACT]

3.	The exhaust from S1426 shall be vented to S1507, S1509, and/or S1512, unless allowed per Condition 18407.  [basis: Regulation 2-6-409.2]

4.	The water seal of the CCU (S-1426) dump stack shall be maintained such that a water seal exists.  If a breakthrough of the water seal at the CCU (S-1426) dump stack is detected, the owner/operator shall conduct a visible emission inspection and initiate corrective action to restore the water seal.  The BAAQMD shall be notified within 24 hours if breakthrough of the water seal is detected.  [basis: Regulation 6-301]

5.	A continuous level monitor shall be installed on the water seal compartment of the CCU (S-1426) dump stack.  Water level records shall be maintained for a period of at least 5 years from the date of entry and shall be made available to the APCO upon request. Any occurrence of visible emissions detected during water seal breakthrough shall also be recorded.  [basis: Regulation 6-301]

Condition # 14098

For S1598, MAINT Gasoline Dispensing Facility:

1.	Pursuant to BAAQMD Toxic Section Policy, this facility's annual gasoline throughput shall not exceed 940,000 gallons in any consecutive 12 month period. [basis: Cumulative Increase, Toxics]

2.	To determine compliance with the above condition, the owner/operator shall maintain monthly throughput records.  All records shall be recorded in a log.  All records shall be retained on-site for five years, from the date of entry, and  made available for inspection by District staff upon request.  These recordkeeping requirements shall not replace the recordkeeping requirements contained in any applicable District Regulations. [basis: Cumulative Increase, Toxics]

Condition # 16688

For S1476, LUBS F-24 Atmospheric Feed

S1477, LUBS F-25 Vacuum Feed

S1478, LUBS F-26 Fufural Raff

S1479, LUBS F-27 Furfural Extr

S1480, LUBS F-69 Asphalt Circulation

S1481, OPCEN F-30 DSU

S1483, LUBS F-32 Asphalt Circulation

S1484, LUBS F-34 LHT Charge

S1486, DH  F-40 CU Feed

S1487, DH F-41B VFU Feed

S1488, DH  F-41A VFU Feed

S1490, DH F-43  GOHT Feed

S1491, DH F-44  NHT Feed

S1492, DH F-45 Primary Column Reboil

S1493, DH F-46 Stabilizier Reboil

S1494, DH F-47 Secondary Column Reboil

S1495, DH F-49  CRU Preheat

S1496, DH F-50  CRU

S1497, DH F-51  CRU

S1498, DH F-52  CRU Reboil

S1499, DH F-53 CRU Regen

S1500, DH F-55 SGP Heat Medium

S1502, DH F-57 HCU First Stage Feed

S1503, DH F-58 HCU Second Stage Feed

S1504, DH F-59 HCU Second Stage Reboil

S1505, DH F-60 HP1 Steam Methane Reformer

S1506, CPF-61 CGP Feed

S1508, CP F-63 CFH Feed

S1510, CP F-66 CCU Preheat

S1511, CP F-67 CCU LGO Reboil

S1515, DH F-71 HCU First Stage Reboil

S1760, OPCEN F-102 FXU Steam Superheater

S1761, OPCENF-104 HP2 Steam Methane Reformer

S1762, DH F-128 CRU Interheater

S1763, DH F-126 CU Feed

S1800, UTIL F-88 Boiler 5

S4002, DC F-13425-A DCU

S4003, DC F-13425-B DCU

S4021, DC F-13909 DHT Recycle

S4031, DC F-14012 HGHT Reboil

S4141, DC F-14011 HGHT Feed

S4161, DC H-101 HP3 Steam Methane Reformer

S4171, LUBS F-13000 LHT2 Feed:

- Each combustion source listed below shall not exceed its indicated maximum firing rate (higher heating value), expressed in the units of million BTU per day (MM BTU/day).  These firing rates are sustainable maximum firing rates.  The sustainable hourly firing rates, used for billing purposes, are established by dividing the maximum daily firing rates by 24 hours.

Enforceable	Used for

Limit	Fees

Source Description	MM BTU/day	MM BTU/hr

S1476  LUBS F-24 Atmospheric Feed	2040	85

S1477  LUBS F-25 Vacuum Feed	1152	48

S1478  LUBS F-26 Furfural Raff	312	13

S1479  LUBS F-27 Furfural Extr	720	30

S1480  LUBS F-69 Asphalt Circulation	600	25

S1481  OPCEN F-30 DSU	600	25

S1483  LUBS F-32 Asphalt Circulation	480	20

S1484  LUBS F-34 LHT Charge	600	25

S1486  DH F-40 CU Feed	8976	374

S1487  DH F-41B VFU Feed	3600	150

S1488  DH F-41A VFU Feed	3600	150

S1490  DH F-43 GOHT Feed	792	33

S1491  DH F-44 NHT Feed	1248	52

S1492  DH F-45 Primary Reboil	2496	104

S1493  DH F-46 Stabilizer Reboil	1320	55

S1494  DH F-47 Secondary Column Reboil	1104	46

S1495  DH F-49 CRU Preheat	4560	190

S1496  DH F-50 CRU	5400	225

S1497  DH F-51 CRU	2544	106

S1498  DH F-52 CRU Reboil	936	39

S1499  DH F-53 CRU Regen	744	31

S1500  DH F-55 SGP Heat Medium	4008	167

S1502  DH F-57 HCU First Stage Feed	1464	61

S1503  DH F-58 HCU Second Stage Feed	1548	66

S1504  DH F-59 HCU Second Stage Reboil	4224	176

S1505  DH F-60 HP1 Steam Methane Reformer	13200	550

S1506  CP F-61 CGP Feed	600	25

S1508  CP F-63 CFH Feed	4440	185

S1510  CP F-66 CCU Preheat	4800	200

S1511  CP F-67 CCU LGO Reboil	1440	60

S1514  UTIL F-70 Boiler 4	9816	409

S1515  DH F-71 HCU First Stage Reboil	2640	110

S1760  OPCEN F-102 FXU Steam Superheater	3336	139

S1761  OPCEN F-104 HP2 Steam Methane Reformer	11424	476

S1762  DH F-128 CRU Interheater	4800	200

S1763  DH F-126 CU Feed	5280	220

S1800  UTIL F-88 Boiler 5	5376	224

S4002  DC F-13425A DCU	2520	105

S4003  DC F-13425B DCU	2520	105

S4021  DC F-13909 DHT Recycle	1176	49

S4031  DC F-14012 HGHT Reboil	1176	49

S4141  DC F-14011 HGHT Feed	768	32

S4161  DC H-101 HP3 Steam Methane Reformer	21840	910

S4171  LUBS F-13000 LHT2 Feed	504	21

[basis: Regulation 2-1-234]

Condition # 17532

For S1514, UTIL F-70 Boiler 4

1.	Only gaseous fuel shall be burned in S-1514.  (Basis:  Reg. 1-520.1)

2.	Startup Condition Deleted.

3.	The owner/operator shall operate S1514 to not exceed 0.05 lb NOx/MMBTU (HHV) based on a rolling hourly 8760-hour average heat input.  The annual average heat input rate used to calculate the allowable (potential to emit) NOx emissions shall be the source's maximum permitted daily heat input rate of 9816 MMBTU (HHV)/day expressed on a  24-hour basis as 409 MMBTU (HHV)/hr. [basis: Shell-EPA Consent Decree]

Condition # 17533

For S1507, UTIL CO Boiler 1

S1509, UTIL CO Boiler 2

S1512, UTIL CO Boiler 3:

- Condition deleted.

2. In no event shall the ammonia (NH3) exhaust outlet concentration exceed 50 ppmv, at 3% O2, averaged over 3 hours. [basis: Toxics]

3. The owner/operator of this source shall install a continuous emission monitor (CEM) and recording system for NH3 in each CO Boiler stack if there are 3 separate violations of condition number 2 in any consecutive 12 month period. The NH3 CEM and recording system shall be installed within 12 months after the first time three violations, in a consecutive 12 month period, of condition number 2 occurs. The type of NH3 CEM and recording instrumentation installed is subject to prior approval from the District Source Test Section. The owner/operator, upon installation of the CEM, shall conduct a Field Accuracy Test that meets the approval of the District Source Test Section. [basis: Toxics]

4. Condition deleted.

5. The Urea injection system shall be automatically controlled during all periods of its use. [basis: Offsets]

6. (A/C startup condition deleted)

7. (A/C startup condition deleted)

8. 	In order to demonstrate compliance with condition number 2, the owner/operator shall conduct a source test for ammonia emissions during operation of the urea injection at either S1507, S1509 or S1512 at least once every year. The source test procedure and reporting of results to the District is subject to approval by the District Source Test Section. Test results shall be submitted to the District within 60 days of the date of the source test. If an ammonia CEM is installed, the yearly source tests can be discontinued except when necessary to demonstrate monitor performance. (Revised 9/6/95)  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard.  [basis:Toxics]

9. 	The owner/operator of this source shall maintain daily records of Urea usage, temperature, and stack NOx (in ppmv corrected to 3% oxygen averaged over 1 hour and in pounds per day). The record keeping format is subject to the approval of the District Source Test Section. [basis: Offsets]

10. A temperature indicator and recorder shall be installed in the CO Boiler to measure the temperature at the area of Urea injection. [basis: Offsets]

11. In the event the APCO determines that the stack opacity is in excess of District Regulation 6, Visible emissions, the owner/operator shall immediately curtail use of the Urea injection system to the extent required to abate the excessive emissions. [basis: Regulation 6-301

12. The owner/operator shall retain and make available upon request by the District any additional source test results performed to evaluate the Urea injection system. [basis: Offsets, Toxics]

13. Each CO Boiler exhaust point shall be equipped with a District approved NOx and O2 continuous emission monitor and recording system. [basis: Offsets]

14. The owner/operator of the NOx monitor and O2 monitor shall conduct a Field Accuracy Test in accordance with the BAAQMD Manual of Procedures. Following the BAAQMD review and approval of the test results, the CEM monitors shall be used by the owner/operator for tracking daily NOx emissions under the Condition # 7618. [basis: Offsets]

15. In the event the CEM system is rendered inoperable for greater than 24 hours, or fails an accuracy test, the NOx emission factors for the CO Boilers that appear in Table ix of Permit #26876 shall be used to calculate the NOx emissions from the CO Boiler until such time as the situation is remedied to the satisfaction of the APCO. [basis: Offsets]

Condition # 17648

For S1006, Tank 1006

S2013, Tank 12467

S2445, Tank 12445

S2446, Tank 12446

S4322, Tank 14571 Sour Water:

1.	For purposes of the Storage Tank Emission Reduction Partnership Program, each opening through the deck of the floating roof for a slotted guidepole shall be equipped at a minimum with a deck cover, a pole wiper and a pole float.  The deck cover shall also be equipped with a gasket between the cover and the deck.  The wiper or seal of the pole float shall be at or above the height of the pole wiper. (basis: STERPP)

2.	The sliding cover shall be in place over the slotted-guidepole opening through the floating roof at all times except when the sliding cover must be removed for access.  The guidepole float shall be floating within the guidepole at all times except when it must be removed for access to the stored liquid or when the tank is empty. (basis: STERPP)

3.	The Permit Holder shall visually inspect the deck fitting for the slotted guidepole at least once every 10 years and each time the vessel is emptied and degassed.  If the slotted guidepole deck fitting or control devices have defects, or if a gap of more than 0.32 centimeters (1/8 inch) exists between any gasket required for control of the slotted guidepole deck fitting and any surface that it is intended to seal, such items shall be repaired before filling or refilling the storage vessel with regulated material. (basis: STERPP)

4.	Tanks taken out of hydrocarbon service, for any reason, do not have to have any controls in place during the time they are out of service. (basis: STERPP)

Condition # 18153

For S1070, Tank 1070

1.	Total liquid throughput at source S1070 shall not exceed 175,200 thousand gallons during any consecutive twelve-month period. [basis:  Cumulative Increase]

2.	Materials stored in S1070 shall be limited to the following:

a.	Hydrocarbon Blanket Layer (RVP 3);

b.	Sour Water

c.	A liquid other than those specified above may be stored in S1070, provided that both of the following criteria are met:

i.	POC emissions, based on the maximum throughput in Condition 1, do not exceed 2,501 pounds per year; and

ii.	toxic emissions in lb/yr, based on the maximum throughput in Condition 1, do not exceed any risk screening trigger level.

[basis:  Cumulative Increase, Toxics]

3.	Source S1070 shall be equipped with a mechanical shoe primary seal and a rim-mounted secondary seal.  There shall be no ungasketed roof fittings.  Except for roof legs, each roof fitting shall be of the design that yields the minimum roof fitting losses (per EPA Compilation of Air Pollution Emission Factors, AP-42, Supplement E, Section 12.3.2, Table 12.3-11).  The following list indicates the type of control required for a variety of typical roof fittings.  Control techniques for roof fittings not included in this list shall be subject to District approval, prior to installing the roof on the tank.

| Fitting Type | Control Technique |
| --- | --- |
| Access hatch | Bolted cover, gasketed |
| Column Well (24-inch dia.) | Pipe column-sliding cover, gasketed |
| Vacuum Breaker | Weighed mechanical actuation, gasketed |
| Ladder Well | Sliding cover, gasketed |
| Roof Leg (3-inch Diameter) Sample Pipe or Well (24-inch dia.) | Adjustable Slotted Pipe-Sliding Cover, gasketed |
| Automatic Gauge Float Well | Bolted cover, gasketed |

[Basis:  BACT, Cumulative Increase]

4.	In order to demonstrate compliance with the condition 1, the owner/operator of tank S1070 shall either maintain the total monthly throughput of each material stored, summarized on a consecutive 12-month basis in a District approved log, or shall be able to generate these records on short notice.  These records shall be kept on site and made available for District inspection for a period of 5 years from the date that the record was made. [Basis:  Cumulative Increase]

Condition # 18265

For S1476, LUBS F-24 Atmospheric Feed

S1477, LUBS F-25 Vacuum Feed

S1478, LUBS F-26 Furfural Raff

S1479, LUBS F-27 Furfural Extr

S1480, LUBS F-69 Asphalt Circulation

S1481, OPCEN F-30 DSU

S1483, LUBS F-32 Asphalt Circulation

S1484, LUBS F-34 LHT Charge

S1486, DH F-40 CU Feed

S1487, DH F-41B VFU Feed

S1488, DH  F-41A VFU Feed

S1490, DH F-43  GOHT Feed

S1491, DH F-44  NHT Feed

S1492, DH F-45 Primary Column Reboil

S1493, DH F-46 Stabilizer Reboil

S1494, DH F-47 Secondary Column Reboil

S1495, DH F-49  CRU Preheat

S1496, DH F-50 CRU

S1497, DH F-51  CRU

S1498, DH F-52  CRU Reboil

S1499, DH F-53 CRU Regen

S1500, DH F-55 SGP Heat Medium

S1502, DH F-55 SGP Heat Medium

S1503, DH F-58 HCU Second Stage Feed

S1504, DH F-59 HCU Second Stage Reboil

S1505, DH F-60 HP1 Steam Methane Reformer

S1506, CP F-61 CGP Feed

S1508, CP F-63 CFH Feed

S1510, CP F-66 CCU Preheat

S1511, CP F-67 CCU LGO Reboil

S1514, UTIL F-70 Boiler 4

S1515, DH F-71 HCU First Stage Reboil

S1760, OPCEN F-102 FXU Steam Superheater

S1761, OPCEN F-104 HP2 Steam Methane Reformer

S1762, DH F-128  CRU Interheater

S1763, DH F-126 CU Feed Heater

S1800, UTIL F-88 Boiler 5

S4002, DC F-13425-A DCU

S4003, DC F-13425-B DCU

S4021, DC -F-13909 DHT Recycle

S4031, DC F-14012 HGHT Reboil

S4141, DC F-14011 HGHT Feed

S4161, DC H-101 HP3 Steam Methane Reformer

S4171, LUBS F-13000 LHT2 Feed

Regulation 9-10 Refinery-Wide Compliance

*1.	Effective June 1, 2004, the following sources are subject to the refinery-wide NOx emission rate and CO concentration limits in Regulation 9-10: (basis: Regulation 9-10-301 & 305)

S#	Description	CEM  (Y/N)

S1476	LUBS F-24 Atmospheric Feed	N

S1477	LUBS F-25 Vacuum Feed	N

S1478	LUBS F-26 Furfural Raff	N

S1479	LUBS F-27 Furfural Extr	N

S1480	LUBS F-69 Asphalt Circulation	N

S1481	OPCEN F-30 DSU	N

S1483	LUBS F-32 Asphalt Circulation	N

S1484	LUBS F-34 LHT Charge	N

S1486	DH F-40 CU Feed	Y

S1487	DH F-41B VFU Feed	Y

S1488	DH  F-41A VFU Feed	Y

S1490	DH F-43  GOHT Feed	Y

S1491	DH F-44  NHT Feed	Y

S1492	DH F-45 Primary Column Reboil	Y

S1493	DH F-46 Stabilizer Reboil	Y

S1494	DH F-47 Secondary Column Reboil	Y

S1495	DH F-49  CRU Preheat	Y

S1496	DH F-50 CRU	Y

S1497	DH F-51  CRU	Y

S1498	DH F-52  CRU Reboil	Y

S1499	DH F-53 CRU Regen	Y

S1500	DH F-55 SGP Heat Medium	Y

S1502	DH F-57 HCU 1st Y	Y

S1503	DH F-58 HCU Second Stage Feed	Y

S1504	DH F-59 HCU Second Stage Reboil	Y

S1505	DH F-60 HP1 Steam Methane Reformer	Y

S1506	CP F-61 CGP Feed	N

S1508	CP F-63 CFH Feed	Y

S1510	CP F-66 CCU Preheat	Y

S1511	CP F-67 CCU LGO Reboil	Y

S1514	UTIL F-70 Boiler 4	Y

S1515	DH F-71 HCU First Stage Reboil	Y

S1760	OPCEN F-102 FXU Steam Superheater	N

S1761	OPCEN F-104 HP2 Steam Methane Reformer	Y

S1762	DH F-128  CRU Interheater	Y

S1763	DH F-126 CU Feed Heater 	Y

S1800	UTIL F-88 Boiler 5	Y

S4002	DC F-13425-A DCU	Y

S4003	DC F-13425-B DCU	Y

S4021	DC -F-13909 DHT Recycle 	N

S4031	DC F-14012 HGHT Reboil	Y

S4141	DC F-14011 HGHT Feed	Y

S4161	DC H-101 HP3 Steam Methane Reformer	Y

S4171	LUBS F-13000 LHT2 Feed	N

*2.	The owner/operator of each source listed in Part 1 shall properly install, properly maintain, and properly operate an O2 monitor and recorder. This Part shall be effective September 1, 2004. (basis: Regulation 9-10-502)

*3.	Effective June 1, 2004, the owner/operator shall operate each source listed in Part 1, which does not have a NOx CEM within specified ranges of operating conditions (firing rate and oxygen content) as detailed in Part 5.  The ranges shall be established by utilizing data from district-approved source tests. (Reg. 9-10-502)

  - The NOx Box for units with a maximum firing rate of 25 MMBH or more shall be established using the procedures in Part 4.

  - The NOx Box for units with a maximum firing rate less than 25MMBH shall be established as follows: High-fire shall be the maximum rated capacity. Low-fire shall be 20% of the maximum rated capacity. There shall be no maximum or minimum O2.

*4.	The owner/operator shall establish the initial NOx box for each source subject to Part 3 by June 1, 2004.  The NOx Box may consist of two operating ranges in order to allow for operating flexibility and to encourage emission minimization during standard operation. (Reg. 9-10-502)The procedure for establishing the NOx box is

- A.	Conduct district approved source tests for NOx and CO, while varying the oxygen concentration and firing rate over the desired operating ranges for the furnace;

- B.	Determine the minimum and maximum oxygen concentrations and firing rates for the desired operating ranges (Note that the minimum O2 at low-fire may be different than the minimum O2 at high-fire.  The same is true for the maximum O2). The owner/operator shall also verify the accuracy of the O2 monitor on an annual basis.

- C.	Determine the highest NOx emission factor (lb/Mmbtu) over the preferred operating ranges while maintaining CO concentration below 200 ppm; the owner/operator may choose to use a higher NOx emission factor than tested.

- Plot the points representing the desired operating ranges on a graph.  The resulting polygon(s) are the NOx Box, which represents the allowable operating range(s) for the furnace under which the NOx emission factor from part 5a is deemed to be valid.

    - The NOx Box can represent/utilize either one or two emission factors.

    - The NOx Box for each emission factor can be represented either as a 4- or 5-sided polygon The NOx box is the area within the 4- or 5-sided polygon formed by connecting the source test parameters that lie about the perimeter of successful approved source tests. The source test parameters forming the corners of the NOx box are listed in Part 5.

- Upon establishment of each NOx Box, the owner/operator shall prepare a graphical representation of the box. The representation shall be made available on-site for APCO review upon request.  The box shall also be submitted to the BAAQMD with permit amendments.

(basis: Regulation 9-10-502)

*5.	Except as provided in part 5B & C, the owner/operator shall operate each source within the NOx Box ranges listed below at all times of operation. This part shall not apply to any source that has a properly operated and properly installed NOx CEM. (Reg. 9-10-502)

A.  	NOx Box ranges

| Source No. | Emission Factor (lb/MMBtu) | Min O2 at Low Firing (O2% , MMBtu/hr) | Max O2 at Low Firing (O2% , MMBtu/hr) | Min O2 at High Firing (O2% , MMBtu/hr) | Mid O2 at Mid/High Firing (polygon) (O2% , MMBtu/hr) | Max O2 at High Firing (O2% , MMBtu/hr) |
| --- | --- | --- | --- | --- | --- | --- |
| S1476 | 0.18 | 3.9, 35.7 | 12.7,35.7 | 3.9, 72.2 | n/a | 12.7, 72.2 |
| S1477 | 0.16 | 4.1, 10.6 | 14.2, 10.6 | 4.1, 26.6 | n/a | 14.2, 26.6 |
| S1478 | 0.16 | 3.5, 2.3 | 12.4, 2.3 | 3.5, 11.5 | n/a | 12.4, 11.5 |
| S1479 | 0.16 | 3.7, 10.3 | 12.0, 10.3 | 3.7, 19.3 | n/a | 12.0, 19.3 |
| S1480 | 0.20 | n/a | n/a | n/a | n/a | n/a |
| S1481 | 0.16 | n/a | n/a | n/a | n/a | n/a |
| S1483 | 0.16 | n/a | n/a | n/a | n/a | n/a |
| S1484 | 0.16 | n/a | n/a | n/a | n/a | n/a |
| S1506 | 0.2 | n/a | n/a | n/a | n/a | n/a |
| S1760 | 0.5 | 1.0, 17 | 7.5, 17 | 1.0, 100 | n/a | 7.5, 100 |
| S4021 | 0.029 | 0.7, 11.9 | 5.6, 11.9 | 0.7, 33.7 | n/a | 5.6, 33.7 |
| S4171 | 0.029 | n/a | n/a | n/a | n/a | n/a |

The limits listed above are based on a calendar day averaging period for both firing rate and O2%.

  - Part 5A. does not apply to low firing rate conditions (i.e., firing rate less than or equal to 20% of the unit’s rated capacity) during startup or shutdown periods or periods of curtailed operation (ex. during heater idling, refractory dryout, etc.) lasting 5 days or less.  During these conditions the means for determining compliance with the refinery wide limit shall be accomplished using the method described in 9-10-301.2 (i.e. units out of service & 30-day averaging data).

  - Part 5A.  does not apply during any source test required or permitted by this condition. (Reg. 9-10-502). See Part 7 for the consequences of source test results that exceed the emission factors in Part 5.

(basis: Regulation 9-10-502)

*6.	NOx Box Deviations [Effective June 1, 2004] (Reg. 9-10-502)

##### A.	The owner/operator may deviate from the NOx Box (either the firing rate or oxygen limit) provided that the owner/operator conducts a district approved source test which replicates the past operation outside of the established ranges.  The source test representing the new conditions shall be conducted no later than the next regularly scheduled source test period, or within eight months, whichever is sooner.  The source test results will establish whether the source was operating outside of the emission factor utilized for the source. The source test results shall be submitted to the district source test manager within 45 days of the test. As necessary, a permit amendment shall be submitted.

- Source Test <= Emission Factor

If the results of this source test do not exceed the higher NOx emission factor in Part 5, or the CO limit in Part 9, the unit will not be considered to be in violation during this period for operating out of the "box."

  - The facility may submit an accelerated permit program permit application to request an administrative change  of the permit condition to adjust the NOx  Box operating range(s), based on the new test data.

- Source Test > Emission Factor

If the results of this source test exceed the permitted emission concentrations or emission rates then the actions described below must be followed:

  - Utilizing measured emission concentration or rate, the owner/operator shall perform an assessment, retroactive to the date of the previous source test, of compliance with Section 9-10-301.  The unit will be considered to have been in violation of 9-10-301 for each day the facility was operated in excess of the refinery wide limit.

  - The facility may submit a permit application to request an alteration of the permit condition to change the NOx emission factor and/or adjust the operating range, based on the new test data.

##### Reporting -  The owner/operator must report conditions outside of box within 96 hours of occurrence.

(basis: Regulation 9-10-502)

*7.	Effective June 1, 2004, for each source subject to Part 3, the owner/operator shall conduct source tests at the schedule listed below.  The source tests are performed in order to measure NOx, CO, and O2 at the as-found firing rate, or at conditions reasonably specified by the APCO.  The source test results shall be submitted to the district source test manager within 45 days of the test. (Reg.9-10-502)

##### Source Testing Schedule

- Heater  < 25 MMBtu/hr

One source test per consecutive 12 month period.  The time interval between source tests shall not exceed 16 months.

- Heaters ≥ 25 MMBtu/hr

Two source tests per consecutive 12 month period.  The time interval between source tests shall not exceed 8 months and not be less than 5 months apart. The source test results shall be submitted to the district source test manager within 45 days of the test. (Reg.9-10-502)

##### B.	Source Test Results > NOx Box Emission Factor

If the results of any source test under this part exceed the permitted concentrations or emission rates the owner/operator shall follow the requirements of Part 6A2 If the owner/operator chooses not to submit an application to revise the emission factor, the owner/operator shall conduct another Part 7 source test, at the same conditions, within 90 days of the initial test.

*8.	Effective June 1, 2004, for each source listed in Part 1 with a NOx CEM installed, the owner/operator shall conduct semi-annual district approved CO source tests at as-found conditions.  The time interval between source tests shall not exceed 8 months.  District conducted CO emission tests associated with District-conducted NOx CEM field accuracy tests may be substituted for the CO semi-annual source tests.  (basis: Regulation 9-10-502, 1-522)

*9.	Effective June 1, 2004, for any source listed in Part 1 for which any two source test results over any consecutive five year period are greater than or equal to 200 ppmv CO at 3% O2, the owner/operator shall properly install, properly maintain, and properly operate a CEM to continuously measure CO and O2.  The owner/operator shall install the CEM within the time period allowed in the District's Manual of Procedures. (basis: Regulation 9-10-502, 1-522)

*10.Effective June 1, 2004 , in addition to records required by 9-10-504, the facility must maintain records of all source tests conducted to demonstrate compliance with Parts number 1 and 5.   These records shall be kept on site for at least five years from the date of entry in a District approved log and be made available to District staff upon request. (basis: recordkeeping, Regulation 9-10-504)

*11.	Until June 1, 2004, the owner/operator shall operate a continuous emission monitor (CEM) to measure the NOx and O2 concentrations from the following sources that are subject to this Alternative Compliance Plan.  In the case where two or more sources exhaust through a common stack, a single NOx and O2 CEM may be used to measure the combined concentrations from all sources that exhaust through the stack.

Phase I (effective July 1, 2000)

S1494	DH F-47 Secondary Column Reboil

S1500	DH F-55 SGP Heat Medium

S1502	DH F-55 SGP Heat Medium

S1503	DH F-58 HCU Second Stage Feed

S1504	F-59  HCU 2nd Stage Reboiler

S1505	DH F-60 HP1 Steam Methane Reformer

S1515	DH F-71 HCU First Stage Reboil

S1761	OPCEN F-104 HP2 Steam Methane Reformer

S1763	DH F-126 CU Feed Heater

S1800	UTIL F-88 Boiler 5

S4002	DC F-13425-A DCU

S4003	DC F-13425-B DCU

S4031	DC F-14012 HGHT Reboil

S4141	DC F-14011 HGHT Feed

S4161	DC H-101 HP3 Steam Methane Reformer

Phase II (effective July 1, 2002)

S1486	DH F-40 CU Feed

S1487	DH F-41B VFU Feed

S1488	DH  F-41A VFU Feed

S1490	DH F-43  GOHT Feed

S1491	DH F-44  NHT Feed

S1492	DH F-45 Primary Column Reboil

S1493	DH F-46 Stabilizer Reboil

S1495	DH F-49  CRU Preheat

S1496	DH F-50 CRU

S1497	DH F-51  CRU

S1498	DH F-52  CRU Reboil

S1499	DH F-53 CRU Regen

S1510	CP F-66 CCU Preheat

S1511	CP F-67 CCU LGO Reboil

S1514	UTIL F-70 Boiler 4

S1762	DH F-128  CRU Interheater

*12.	Until June 1, 2004, for the following sources for which there is not a CEM, NOx emissions shall not exceed the emission rates indicated below, in the units of pounds of NOx per million BTU of fuel fired (lb NOx/MM BTU).  These emission rates shall be used to calculate daily NOx emissions.  These emission rates may be adjusted, subject to District approval, based on source testing results.  The Permit Holder may elect to use a more conservative (higher) emission factor than would be require solely based on source testing results.  If the owner/operator chooses to install NOx and O2 CEM’s on any source listed below, the maximum emission rate listed below will no longer be in effect, and daily NOx emissions from that source will be calculated using the CEM NOx concentration.

Phase I (effective July 1, 2000)	          lb/MM BTU

S4021	DC -F-13909 DHT Recycle	0.029

S4171	LUBS F-13000 LHT2 Feed	0.029

##### Phase II (effective July 1, 2002)	          lb/MM BTU

S1476	LUBS F-24 Atmospheric Feed	0.180

S1477	LUBS F-25 Vacuum Feed	0.160

S1478	LUBS F-26 Furfural Raff	0.160

S1479	LUBS F-27 Furfural Extr	0.160

S1480	LUBS F-69 Asphalt Circulation	0.20

S1481	OPCEN F-30 DSU	0.160

S1483	LUBS F-32 Asphalt Circulation	0.160

S1484	LUBS F-34 LHT Charge	0.160

S1506	CP F-61 CGP Feed	0.20

S1508	CP F-63 CFH Feed	0.20

S1760	OPCEN F-102 FXU Steam Superheater	0.18

*13.	Until June 1, 2004, on a daily basis, the owner/operator shall determine the amount of IERC’s necessary for compliance with Regulation 9, Rule 10.  IERC’s shall be calculated using a District-approved spreadsheet that calculates actual daily NOx emissions and allowable NOx emissions based on the Reg. 9-10 limit of 0.033 lb/MM BTU.  (Table 2 of the Engineering Evaluation Report for AN 498 is an example of a District-approved daily spreadsheet.)

*14.	Until June 1, 2004, for each source (or group of sources with a common stack), the owner/operator shall summarize the daily actual emissions, daily allowable emissions, and actual NOx emission rate (lb/MM BTU), on a monthly basis.  This monthly summary shall done using a District-approved spreadsheet. (Table 3 of the Engineering Evaluation Report for AN 498 is an example of a District-approved monthly source-by-source summary spreadsheet.)

*15.	Until June 1, 2004, the owner/operator shall summarize the daily IERC’s on a monthly basis, using a District-approved spreadsheet.  (Table 4 of the Engineering Evaluation Report for AN 498 is an example of a District-approved monthly IERC summary spreadsheet.)

*16.	Until June 1, 2004, the initial source testing that is required by Reg. 9-10-501 has not been performed on source S1800 because this source has not been operating.  If the owner/operator uses S1800 in the future, the owner/operator shall conduct a District-approved source test for NOx and CO emissions within 30 days of the initial operation of S1800.

*17.	Until June 1, 2004, all sources covered by this Alternative Compliance Plan are subject to the District’s policy for “NOx, CO and O2 Monitoring Compliance with Regulation 9, Rule 10”, dated June 23, 2000 (and any subsequent revisions).

*18.	Until June 1, 2004, in accordance with Section II C of the monitoring policy in condition 7, the District will limit the operating range for S1476, S1477, S1478, S1479, S1508, S1760 and S4021, as indicated in Condition 9.

*19.	Until June 1, 2004, except during startup and shutdown, the allowable operating ranges for S1476, S1477, S1478, S1479, S1508, S1760 and S4021 are as follows:

Source (F-#)	O2-min	O2-max	Firing-min	Firing-max

1476 (F-24)	3.9	12.7	35.7	72.2

1477 (F-25)	4.1	14.2	10.6	26.6

1478 (F-26)	3.5	12.4	2.3	11.5

1479 (F-27)	3.7	12.0	10.3	25.7

1508 (F-63)	2.1	8.5	32.2	94.0

1760 (F-102)	1.8	4.4	54.9	90.5

4021 (F-13909)	0.7	5.6	11.9	33.7

The O2 concentrations are expressed as percent, and the firing rates are in million BTU/hr, higher heating value.

In the event that the owner/operator operates any source outside of its associated operating range above, the owner/operator shall perform a source test to determine NOx emissions, in accordance with the monitoring policy in Condition 7.  If NOx emissions are less than or equal to the emission rate indicated in Condition 2, the source testing results may be used to expand the allowable operating range above.  If NOx emissions exceed the emission rate indicated in Condition 2, this source is in violation for each day it operated outside of the defined operating range.

*20.Until June 1, 2004, 	the owner/operator shall operate and maintain a fuel flow meter for each fuel line on each source included in this ACP.

*21.Until June 1, 2004, t	he owner/operator shall maintain permanent hourly continuous emission monitoring or equivalent system parameter records and fuel-rate records, in a form suitable for inspection for a period of at least five (5) years.  Such records shall be retained for a minimum of 5 years from date of entry and made available to the APCO upon request.  These records shall include, but are not limited to:

a.	The continuous emission monitoring measurements or equivalent system parameters for NOx, CO, and O2 in ppmv and lb/hour;

b.	The type, quantity (BTU/hr), and higher heating value of fuel burned and the injection rate for any reactant chemicals used by the emission control system(s) on a daily basis.

c.	The date, time, and duration of any start-up, shutdown or malfunction in the operation of any unit, emission control equipment, or emission monitoring equipment; and

d.	The results of performance testing, evaluations, calibrations, checks, adjustments, and maintenance of any continuous emission monitors that have been installed pursuant to Section 9-10-502 of this Rule.

Condition # 18407

For S1426, CP Catalytic Cracking Unit (CCU)

1.	During periods when the spent catalyst hopper is blinded off from the CO Boilers, all catalyst loading shall be abated by A1426, Baghousefor CCU Spent Catalyst Hopper (Portable) for CCU Spent Catalyst Hopper.  (basis:  cumulative increase)

2.	During periods when the spent catalyst hopper is blinded off from the CO Boilers, the amount of catalyst loaded into the spent catalyst hopper shall not exceed 2000 tons in any consecutive 12-month period.  (basis:  cumulative increase)

3.	To demonstrate compliance with item 2 above, the permit holder shall keep daily records of the amount of catalyst loaded.  Daily  records shall be totaled on a monthly basis.  Monthly records shall be totaled on a consecutive 12-month basis.  The records shall be retained for 5 years following the date the record is made, and shall be made available to the District upon request.  (basis:  cumulative increase)

Condition # 18618

General Throughput Conditions and other miscellaneous monitoring requirements for Title V:

1.	The following throughput limits are based upon District records at the time of MFR permit issuance. Exceedance of those limits for which Regulation 2-1-234.4 was the identified basis are not a violation of the permit if the operator can, within 60 days, provide documentation demonstrating the throughput limit should be higher, established in accordance with 2-1-234.3, and the excess throughput complies with the new limit. Exceedance of those limits which have other permit conditions or application information as the basis are a violation of Regulation 2-1-307 immediately upon exceedance of the limit. (basis: Regulation 2-1-234.3, Regulation 2-1-307)

| S-# | Description | Daily Limit | Annual Limit |
| --- | --- | --- | --- |
| 3 | Tank 3 |  | S3+S4+S967+S1076 < 130,971 bbl/day x 365 |
| 4 | Tank 4 |  | S3+S4+S967+S1076 < 130,971 bbl/day x 365 |
| 13 | Tank 13 |  | 36,000 bbl/day x 365 |
| 14 | Tank 14 |  | 143,657 bbl/day x 365 |
| 20 | Tank 20 |  | 13,131 bbl/day x 365 |
| 21 | Tank 21 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+ S1160 < 42,000 bbl/day x 365 |
| 22 | Tank 22 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 23 | Tank 23 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 24 | Tank 24 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+1160 < 42,000 bbl/day x 365 |
| 26 | Tank 26 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 129 | Tank 129 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 257 | Tank 257 |  | 10,526 bbl/day x 365 |
| 483 | Tank 483 |  | S483+S484+S530 + S532+S545 < 217,097 bbl/day x 365 |
| 484 | Tank 484 |  | S483+S484+S530 + S532+S545 < 217,097 bbl/day x 365 |
| 497 | Tank 497 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 530 | Tank 530 |  | S483+S484+S530 + S532+S545 < 217,097 bbl/day x 365 |
| 532 | Tank 532 |  | S483+S484+S530 + S532+S545 < 217,097 bbl/day x 365 |
| 541 | Tank 541 |  | 248,743 bbl/day x 365 |
| 545 | Tank 545 |  | S483+S484+S530 + S532+S545 < 217,097 bbl/day x 365 |
| 548 | Tank 548 |  | S548+S549+S1006+S1235+S1236 < 5,412 bbl/day x 365 |
| 552 | Tank 552 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 553 | Tank 553 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 554 | Tank 554 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 555 | Tank 555 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 556 | Tank 556 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 557 | Tank 557 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 558 | Tank 558 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+ S559 +S567 + S568 < 10,650 bbl/day x 365 |
| 559 | Tank 559 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 560 | Tank 560 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 561 | Tank 561 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 567 | Tank 567 Asphalt Storage |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 568 | Tank 568 |  | S552+S553+S554+S555+S556+S557+S558+S559 +S567 + S568< 10,650 bbl/day x 365 |
| 572 | Tank 572 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 573 | Tank 573 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 598 | Tank 598 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 610 | Tank 610 |  | S610+S1133 < 48,000 bbl/day x 365 |
| 611 | Tank 611 |  | 82,217 bbl/day x 365 |
| 612 | Tank 612 |  | S612+S613 < 210,686 bbl/day x 365 |
| 613 | Tank 613 |  | S612+S613 < 210,686 bbl/day x 365 |
| 815 | Tank 815 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+ S1160 < 42,000 bbl/day x 365 |
| 860 | Tank 860 |  | S860+S861+S1004 < 546 bbl/day x 365 |
| 861 | Tank 861 |  | S860+S861+S1004 < 546 bbl/day x 365 |
| 967 | Tank 967 |  | S3+S4+S967+S1076 < 130,971 bbl/day x 365 |
| 985 | Tank 985 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 1004 | Tank 1004 |  | S860+S861+S1004 < 546 bbl/day x 365 |
| 1006 | Tank 1006 |  | S548+S549+S1006+S1235+S1236 < 5,412 bbl/day x 365 |
| 1031 | Tank 1031 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1043 | Tank 1043 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 1044 | Tank 1044 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 1045 | Tank 1045 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+S1160 < 42,000 bbl/day x 365 |
| 1046 | Tank 1046 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1051 | Tank 1051 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1076 | Tank 1076 |  | S3+S4+S967+S1076 < 130,971 bbl/day x 365 |
| 1129 | Tank 1129 |  | 120,000 bbl/day x 365 |
| 1130 | Tank 1130 |  | S1130+S1131 < 47,314 bbl/day x365 |
| 1131 | Tank 1131 |  | S1130+S1131 < 47,314 bbl/day x 365 |
| 1133 | Tank 1133 |  | S610+S1133 < 48,000 bbl/day x 365 |
| 1134 | Tank 1134 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1140 | Tank 1140 |  | 27,840 bbl/day x365 |
| 1146 | Tank 1146 |  | S1146+S1147 < 14,640 bbl/day x 365 |
| 1147 | Tank 1147 |  | S1146+S1147 < 14,640 bbl/day x 365 |
| 1159 | Tank 1159 |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1160 | Tank 1160 Asphalt Storage |  | S21+S22+S23+S24+S26+S497+S560+S561+S572+S573+S598+S815+S985+S1043+S1044+S1045+ S1160 < 42,000 bbl/day x 365 |
| 1161 | Tank 1161 |  | 240,000 bbl/day x 365 |
| 1235 | Tank 739 Chem Storage |  | S548+S549+S1006+S1235+S1236 < 5,412 bbl/day x 365 |
| 1236 | Tank 740 Chem Storage |  | S548+S549+S1006+S1235+S1236 < 5,412 bbl/day x 365 |
| 1409 | LUBS Sulfonation Plant (SULF) | 2,805 bbl/day | 803,000 bbl/yr |
| 1411 | LUBS Atmospheric Distillation LDU | 18,801 bbl/day | 6,643,000 bbl/yr |
| 1415 | LUBS Furfural Plant (SEP3) | 6,700 bbl/day | 1,752,000 bbl/yr |
| 1416 | LUBS Lube Hydrotreater 1 (LHT1) | 6,500 bbl/day | 1,934,500 bbl/yr |
| 1417 | OPCEN Distillate Saturation Unit (DSU) | 26,000 bbl/day | 365 x Daily Limit |
| 1420 | DH Crude Unit (CU) | 160,000 bbl/day | 52,925,000 bbl/yr |
| 1423 | DH Gas Oil Straightrun Hydrotreater (GOHT) | 28,000 bbl/day | 365 x Daily Limit |
| 1424 | DH Naphtha Straightrun Hydrotreater (NHT) | 28,500 bbl/day | 9,599,500 bbl/yr |
| 1425 | DH Catalytic Reformer Unit (CRU) | 32,000 bbl/day | 365 x Daily Limit |
| 1426 | CP Catalytic Cracking Unit (CCU) | 79,500 bbl/day | 365 x Daily Limit |
| 1428 | CP Catalytic Feed Hydrotreater (CFH) | 60,000 bbl/day | 19,856,000 bbl/yr |
| 1429 | CP Catalytic Gasoline Hydrotreater (CGH) | 27,500 bbl/day | 365 x Daily Limit |
| 1430 | CP Alkylation Plant (ALKY) | 14,000 bbl/day alkylate produced | 365 x Daily Limit |
| 1431 | CP Sulfur Plant 1 (SRU1) | S1431+S1432 < 331 Equivalent Long Tons/Day | 365 x Daily Limit |
| 1432 | CP Sulfur Plant 2 (SRU2) | S1431+S1432 < 331 Equivalent Long Tons/Day | 365 x Daily Limit |
| 1445 | DH Hydrogen Plant 1 (HP1) | 75,000,000 scf/day H2 | 24,710,500,000 scf/yr H2 |
| 1449 | DH Hydrocracking Unit (HCU) | 46,000 bbl/day | 365 x Daily Limit |
| 1507 | UTIL CO Boiler 1 | 5568 MMBTU/day | 365 x Daily Limit |
| 1509 | UTIL CO Boiler 2 | 5568 MMBTU/day | 365 x Daily Limit |
| 1512 | UTIL CO Boiler 3 | 5568 MMBTU/day | 365 x Daily Limit |
| 1751 | Tank 1330 |  | S1751+S1752 < 45,953 bbl/day x 365 |
| 1752 | Tank 1331 |  | S1751+S1752 < 45,953 bbl/day x 365 |
| 1753 | Tank 1332 Gasoline |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1754 | Tank 1333 Gasoline |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1755 | Tank 1334 Gasoline |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1756 | Tank 1335 Gasoline |  | S129+ S1031+S1046+S1051+S1134+S1159+S1753+S1754+S1755+S1756 < 508,114 bbl/day x 365 |
| 1757 | Tank 1336 | S1757+S1758+S4334<125,829 bbl/day | 365 x Daily Limit |
| 1758 | Tank 1337 | S1757+S1758+S4334<125,829 bbl/day | 365 x Daily Limit |
| 1759 | OPCEN Flexicoker (FXU) | 48,300 bbl/day | 16,245,500 bbl/yr |
| 1764 | OPCEN Dimersol Plant (DIMER) | 3,200 bbl/day dimate produced | 365 x Daily Limit |
| 1765 | OPCEN Sulfur Plant 3 (SRU3) | 73 equivalent long ton/day | 365 x Daily Limit |
| 1774 | OPCEN Hydrogen Plant 2 (HP2) | 43,500,000 scf/day H2 | 14,600,000,000 scf/yr H2 |
| 1900 | MAINT_Machine Shop Parts Cleaner |  | S1900 + S1903 < 192 gal/yr solvent |
| 1903 | MAINT Paint Shop Solvent Tub |  | S1900 + S1903 < 192 gal/yr solvent |
| 3000 | Portable Vacuum Distillation Unit (CCR Technologies Inc.) | 168 MMBTU/day | 365 x Daily Limit |
| 4001 | DC Delayed Coking Unit (DCU) | 65,000 bbl/day | 365 x Daily Limit |
| 4020 | DC Distillate Hydrotreater (DHT) | 60,000 bbl/day | 365 x Daily Limit |
| 4080 | DC Isomerization Unit (ISOM) | 15,100 bbl/day | 365 x Daily Limit |
| 4140 | DC Heavy Cracked Gasoline Hydrotreater (HGHT) | 23,200 bbl/day | 365 x Daily Limit |
| 4160 | DC Hydrogen Plant –3 (HP3) | 90,000,000 scf/day | 365 x Daily Limit |
| 4170 | LUBS Lube Hydrotreater 2 (LHT2) | 7,200 bbl/day | 365 x Daily Limit |
| 4180 | OPCEN Sulfur Plant 4 (SRU4) | 140 long tons/day | 365 x Daily Limit |
| 4190 | UTIL Boiler 6 Gas Turbine 1 | 13,152 MMBTU/day | 365 x Daily Limit |
| 4191 | UTIL Boiler 6 Supplemental Steam Generator 1 | 6,192 MMBTU/day | 365 x Daily Limit |
| 4192 | UTIL Boiler 6 Gas Turbine 2 | 13,152 MMBTU/day | 365 x Daily Limit |
| 4193 | UTIL Boiler 6 Supplemental Steam Generator 2 | 6,192 MMBTU/day | 365 x Daily Limit |
| 4334 | Tank13276 Alkylate | S1757+S1758+S4334<125,829 bbl/day | 365 x Daily Limit |

2. 	Effective April 1, 2004, the facility shall maintain daily throughput records for all sources listed in the Table in Part 1 of this condition that are not tanks summarized on a consecutive 12-month basis in a District approved log, or shall be able to generate these records within 5 working days.  The facility shall maintain annual throughput records for all storage tanks. These records shall be kept on site and made available for District inspection for a period of 60 months from the date that the record was made. (basis: Regulation 2-1-234.3)

3.	Effective April 1, 2004, for S1476, S1477, S1486, S1487, S1488, S1491, S1492, S1493, S1495, S1496, S1497, S1498, S1500, S1504, S1508, S1510, S1511, S1763, S4002, S4003, S4190, S4191, S4192, S4193, the owner/operator shall conduct a visible emissions inspection at each source after every 1 million gallon of liquid fuel combusted, to be counted cumulatively over a 5 year period.  If a visible emissions inspection documents opacity, a method 9 evaluation shall be completed within 3 working days, or during the next scheduled operating period if the unit ceases firing on diesel fuel within the 3 working day time frame. (basis: Regulation 2-6-409.2).

4.	Effective April 1, 2004, for S1476, S1477, S1486, S1487, S1488, S1491, S1492, S1493, S1495, S1496, S1497, S1498, S1500, S1504, S1507, S1508, S1509, S1510, S1511, S1512, S1763, S4002, S4003, S4190, S4191, S4192, S4193, the owner/operator shall sample and analyze the liquid fuel to determine its sulfur content after every 1 million gallon of liquid fuel is combusted, to be counted cumulatively over a 5 year period, or at least once every 5 years, whatever comes first.  Such quantity ot liquid fuel combusted and any resulting sampling and analysis shall be recorded monthly in a District approved log.  The log and all supporting documentation (such as analytical results) shall also be kept for a period of 5 years from the date of entry and made available for inspection upon request. (basis: Regulation 2-6-409.2).

5.	Effective April 1, 2004, S1650, S1767, S1768, and S1769 shall be checked for visible emissions quarterly.  The visible emissions check shall take place while the equipment is operating and during daylight hours.  If any visible emissions are detected, the operator shall take corrective action within one day, and check for visible emissions after corrective action is taken.  If no visible emissions are detected, the operator shall continue to check for visible emissions at the same frequency.  [basis: Regulation 2-6-409.2]

6.	Effective April 1, 2004, during exterior tube cleaning of heaters or boilers (S1476, S1477, S1478, S1479, S1480, S1481,  S1483, S1484, S1486, S1487, S1488, S1491, S1492, S1493, S1495, S1496, S1497, S1498, S1500, S1504, S1506, S1508, S1510, S1511, S1760, S1763, S1490, S1499, S1494, S1502, S1503, S1504, S1505, S1515, S1761, S1762, S1800, S4002, S4003, S4021, S4031, S4141, S4161, S4171, S4191, and S4193), the owner/operator shall check for visible emissions  The visible emissions check shall take place while the tube is being cleaned and during daylight hours.  If anyvisible emissions are detected, the operator shall take corrective action within one day, and check for visible emissions after the corrective action is taken.  If no visible emissions are detected, the operator shall continue to check for visible emissions on an hourly basis until the tube cleaning activity is completed. [basis: Regulation 2-6-409.2]

7.	Effective April 1, 2004, the operator shall keep records of all visible emissions checks per Parts 3, 5, and 6 of this condition, the person performing the check, and all corrective action taken.  The records shall be retained for five years and shall be made available to District personnel upon request.  [basis: Regulation 2-6-409.2]

8.	Effective April 1, 2004, for the sources subject to Regulation 6-330 (S1431, S1432, S1765, and S4180) the owner/operator shall conduct a District approved source test annually to determine the concentration of SO3 or H2SO4, or both, expressed as 100% H2SO4 .  The results of the source test shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard. [basis: Regulation 2-6-409.2]

- Effective April 1, 2004, for the CO Boilers (S1507, S1509, and S1512), the owner/operator shall conduct a District approved source test annually on each source to determine its grain loading rate.  The results of the source tests shall be made available to the District within 60 days of the source test and kept for a minimum of 5 years from the date of the report.  The frequency of source testing required under this condition shall be reduced to once every five years if three consecutive annual source tests document that emissions are less than 50 percent of the standard.  The frequency of source testing shall revert back to once per year, if a source test documents that emissions are 50 percent of the standard or more.  The source testing frequency can again be reduced to once every five years if another three consecutive annual source tests document that emissions are less than 50 percent of the standard. [basis: Regulation 2-6-409.2]

10.	Effective April 1, 2004, the owner/operator will demonstrate compliance with Regulation 9-1-313.2 by performing the following:

- Perform monitoring of the facility fuel gas for H2S using continuous online H2S analyzers.  Refinery fuel gas will be monitored at both refinery fuel gas distribution systems (Clean Fuels and Main Refinery systems).

- Perform monitoring of the Sour Water Stripper Bottoms Water for NH3 concentration in facility's Sour Water Strippers twice per week.

The owner/operator shall summarize the results of these analyses in a District approved log.  The owner/operator shall include in the log an annual report summarizing the facility's compliance with the requirements of Regulation 9-1-313.2.  The owner/operator shall retain each analysis, annual report, and District approved log on site and available for inspection upon District request for a minimum of 5 years from the date of record.  (Basis: Regulation 9-1-313.2)

(Basis: Regulation 9-1-313.2)

11.	The owner/operator shall notify the District in writing by fax or email no less than three calendar days in advance of any scheduled startup or shutdown of any process unit and as soon as feasible for any unscheduled startup or shutdown of a process unit, but no later than 48 hours after the unscheduled startup/shutdown or within the next normal business day. The notification shall be sent in writing by fax or email to the Director of Enforcement and Compliance. The requirement is not federally enforceable.

[basis: Regulation 2-1-403]

- Effective June 1, 2004, the owner/operator shall not flare more than the following limits of vent gas, as defined in Regulation 12-11-210, at following sources :

- S1471 LOP Auxiliary Flare 	630,000 lbs/hr

- S1472 LOP Main Flare	 0 lb/hr

- S1771 OPCEN Flexigas Flare	750,000 lbs/hr

- S1772 OPCEN HC Flare	510,000 lbs/hr

- S4201 Clean Fuels Flare            		2,000,000 lbs/hr

(basis: Regulation 8-1-110.3; 2-1-403)

- Effective June 1, 2004, in order to demonstrate compliance with Part 1 of this condition, the owner/operator shall record on an hourly basis the pounds of vent gas flared at S1470, S1771, S1772, and S4201 Flares.  The owner/operator shall maintain these records for a period of five years from the date of entry and make sure records are available for the APCO upon request.  (basis: Regulation 8-1-110.3; 2-6-409.2; 2-6-501)

- Effective June 1, 2004, for the purposes of these conditions, a flaring event is defined as a flow rate of vent gas flared in any consecutive 15 minutes period that continuously exceeds 330 standard cubic feet per minute (scfm).  If during a flaring event, the vent gas flow rate drops below 330 scfm and then increases above 330 scfm within 30 minutes, that shall still be considered a single flaring event, rather than two separate events.  For each flaring event during daylight hours (between sunrise and sunset), the owner/operator shall inspect the flare within 15 minutes of determining the flaring event, and within 30 minutes of the last inspection thereafter, using video monitoring or visible inspection following the procedure described in Part 2 of this condition.  (basis: Regulation 2-6-409.2)

- Effective June 1, 2004, the owner/operator shall use the following procedure for the initial inspection and each 30-minute inspection of a flaring event.

- If the owner/operator can determine that there are no visible emissions using video monitoring, then no further monitoring is necessary for that particular inspection.

- If the owner/operator cannot determine that there are no visible emissions using video monitoring, the owner/operator shall conduct a visual inspection outdoors using either:

- EPA Reference Method 9; or

- Survey the flare by selecting a position that enables a clear view of the flare at least 15 feet, but not more than 0.25 miles, from the emission source, where the sun is not directly in the observer’s eyes.

- If a visible emission is observed, the owner/operator shall continue to monitor the flare for at least 3 minutes, or until there are no visible emissions, whichever is shorter.

- The owner/operator shall repeat the inspection procedure for the duration of the flaring event, or until a violation is documented in accordance with Part 5.  After a violation is documented, no further inspections are required until the beginning of a new calendar day.

(basis: Regulation 6-301, 2-1-403)

- Effective June 1, 2004, the owner/operator shall comply with one of the following requirements if visual inspection is used:

- If EPA Method 9 is used, the owner/operator shall comply with Regulation 6-301 when operating the flare.

- If the procedure of 4.b.ii is used, the owner/operator shall not operate a flare that has visible emissions for three consecutive minutes.

- (basis: Regulation 2-6-403)

- Effective June 1, 2004, the owner/operator shall keep records of all flaring events, as defined in Part 3.  The owner/operator shall include in the records the name of the person performing the visible emissions check, whether video monitoring or visual inspection (EPA Method 9 or visual inspection procedure of Part 4 of this condition) was used, the results of each inspection, and whether any violation of this condition (using visual inspection procedure in Part 4 of this condition) or Regulation 6-301 occurred (using EPA Method 9). (basis: Regulation 2-6-501; 2-6-409.2)

18. For those flares that exclusively burn flexicoker gas with or without supplemental natural gas the owner/operator shall conduct a visual emission inspection weekly following the protocol in Part 5 a, b and c and shall comply with Parts 6 and 7 of this condition. If no visible emissions are observed after 52 weekly inspections, then this condition no longer applies to the flare. (basis: Regulation 2-6-501; 2-6-409.2)

- Effective June 1, 2004, the owner/operator shall operate S1471, S1472, S4201, A101, A102, and A103 Flares to burn only process upset gases as defined by 60.101(e) or fuel gas as defined by 60.101(d) that is released to it  as a result of relief valve leakage or other emergency malfunctions. (basis: 60.104(a)(1); Regulation 2-1-403)

Condition # 18643

For S1417, OPCEN Distillate Saturation Unit (DSU),

S1426, CP Catalytic Cracking Unit (CCU),

S1429, CP Catalytic Gasoline Hydrotreater (CGH),

S1430, CP Alkylation Plant (ALKY), S1449, DH Hydrocracking Unit (HCU),

S1764, OPCEN Dimersol Plant (DIMER), S4050, DC Catalytic Gas Depentanizer (CGDP), S4080, DC Isomerization Unit (ISOM),

S4140, DC Heavy Cracked Gasoline Hydrotreater (HGHT):

1. 	Condition deleted.

2.	Any new pump in light liquid service installed as a new or as a replacement, as part of the MTBE Removal Project shall be:

a.	equipped with dual mechanical seals, having a  heavy liquid barrier fluid.  The barrier fluid reservoir shall be vented to a 95% efficient control device, or the barrier fluid shall be operated at a pressure higher than the process  stream pressure; or

b.  a "canned" pump or magnetically driven; or any other sealless pump technology approved by the District.

(basis: BACT/TBACT, Cumulative Increase)

- All valves in POC gas and light liquid services installed as new or as a replacement, as part of the MTBE Removal Project shall be bellows sealed, live loaded, graphitic packed, teflon packed, or equivalent, as approved by the District. (basis: BACT/TBACT, Cumulative Increase)

- All flanges installed as a new, as a replacement, or opened in piping systems as a result of the MTBE Removal Project shall be equipped with graphitic-based gaskets, Teflon gaskets, or equivalent as approved by the District. (basis: BACT/TBACT, Cumulative Increase)

- All new centrifugal compressors installed as a new or as a replacement, as part of the MTBE Removal Project shall be equipped with "wet" dual mechanical seals with a heavy liquid barrier fluid, or dual dry gas mechanical seals buffered with inert gas.  The barrier fluid pressure pot or inert gas system shall be vented to a control device with a 95% efficiency.  All reciprocating compressors installed as part of the MTBE Removal Project shall be vented to a 95% efficient control device.  In accordance with the NSPS of 40 CFR 60, Subpart GGG, any new compressor in hydrocarbon service (< 50% hydrogen) shall be equipped with an automatic leak indicator. (basis: BACT/TBACT, Cumulative Increase)

- No new pressure relief valve installed as a new or as a replacement as part of the MTBE Removal Project in gaseous POC, light liquid, or toxic service shall vent to atmosphere. This condition does not apply to pressure relief valves on storage tanks or pressure relief valves that handle only low vapor pressure material (<0.05 psia). (basis: BACT/TBACT, Cumulative Increase)

- All process sample systems in the DSU (S1417) and CGH (S4050) shall be the closed loop or continuous flow design.  In no event shall there be any line purging to process drains.  (basis: BACT/TBACT, Cumulative Increase)

8.   Condition deleted.

9.   Condition deleted.

Condition # 18646

For S19, Tank 19

S1139, Tank 1139:

1.	Total liquid throughput at sources S19 and S1139 shall not exceed 7,300,000 barrels during any consecutive twelve-month period. (basis:  Cumulative Increase, Toxics)

2.	Materials stored in S19 and S1139 shall be limited to the following:

a.	Gasoline or gasoline components (not to exceed RVP 12);

b.	A liquid other than those specified above in 2.a may be stored in S19 or

S1139 provided that both of the following criteria are met:

i.	POC emissions, based on the maximum throughput in Condition 1, do not exceed 20,731 pounds per year, as calculated by the EPA Tanks Program version 4.08 with a 99.5% reduction for vapor recovery; and

ii.	toxic emissions in lb/yr, based on the maximum throughput in Condition 1, do not exceed any risk screening trigger level specified in Table 2-1-316.

(basis:  Cumulative Increase, Toxics)

3.	Sources S19 and S1139 shall be vented to A26 whenever they contain organic liquids. (basis: BACT/TBACT)

4.	In order to demonstrate compliance with the condition 1, the owner/operator of S19 and S1139 shall either maintain the total monthly throughput of each material stored, summarized on a consecutive 12-month basis in a District approved log, or shall be able to generate these records on short notice.  These records shall be kept on site and made available for District inspection for a period of 60 months from the date that the record was made. (Basis: Cumulative Increase)

Condition # 19097

For S5140 Diesel Engine,

S6051 through S6060 Diesel Engines:

1.	Hours of Operation: The emergency standby generators, S-6051, S-6052, S-6053, S-6054, S-6055, S-6056, S-6057, S-6058, S-6059, and S-6060, shall only be operated for emergency use or for reliability-related activities. Operation for reliability-related activities shall not exceed    100 hours per calendar year for S-6051, S-6052, S-6053, S-6055, S-6056, S-6057, and S-6060. No time limit is imposed on the operation for reliability-related activities for S-6054, S-6058, and S-6059.  Operation for emergency use is unlimited. [Basis: 9-8-330]

2.	Emergency use is defined as the use of an emergency standby engine during any of the following: [Basis: 9-8-231]

1 In the event of loss of regular natural gas supply;

2 In the event of failure of regular electric power supply;

3 Flood mitigation;

4 Sewage overflow mitigation;

5 Fire;

6 Failure of a primary motor, but only for such time as needed to repair or replace the primary motor.

3.	Reliability-related activities is defined as the use of an emergency standby engine during any of the following: [Basis: 9-8-232]

1  Operation of an emergency standby engine to test its ability to perform for an emergency use;

2  Operation of an emergency standby engine during maintenance of a primary motor.

4.	Monitoring: Each emergency standby engine shall be equipped with either: [Basis: 9-8-530]

1. A non-resettable totalizing meter that measures and records hours of operation.

2. A non-resettable fuel usage meter

5.	Recordkeeping: All records shall be kept for at least two years, and shall be available for inspection by District staff upon request. The operator shall keep a monthly log of usage that shall indicate the following: [Basis: 9-8-530, 1-441]

1.	Hours of operation (total)

- Hours of operation (emergency) and the nature  of the emergency condition.

6.	The sulfur content of the fuel oil shall be certified by the fuel oil vendor . [basis: Regulation 2-6-409.2]

7.	The engines shall be checked for shall be checked for visible emissions after combustion of 1000 gallons of fuel oil.  The visible emissions check shall take place during daylight hours, while the equipment is operating.  If any visible emissions are detected, the operator shall take corrective action within one week, and check for visible emissions after corrective action is taken.  If no visible emissions are detected, the operator shall continue to check for visible emissions at the same frequency.  All incidents of visible emissions monitoring and any resulting corrective actions shall be recorded in a District approved log and kept for a 5 years from the date of entry. [basis: Regulation 2-6-409.2]

Condition # 20042

For S517095, Tank 17095

1.	The owner/operator shall not exceed a combined, total throughput at tank 17095 (S-17095) of 27,000,000 barrels in any consecutive 12-month period.  [Cumulative Increase]

2.	The owner/operator shall comply with the following design requirements, in addition to any others required by Regulation 8, Rule 5, NSPS Subpart Kb or NESHAP Subpart CC:

a.	adjustable roof legs, if used, must be equipped with

vapor boot seals, or with an equivalent vapor

loss control device approved by the District

b. 	slotted guidepole sampling wells must be equipped with gasketed sliding covers and with float, sleeve and wiper

[BACT, Cumulative Increase]

3.	The owner/operator shall maintain monthly records of the type and net amount of materials stored at S-17095 in a District-approved log for at least 5 years.  The owner/operator shall make available such records to the District upon request.  [Cumulative Increase]

Condition # 20398

For S534, Tank 534

S1141, Tank 1141:

1.	The owner/operator shall not exceed a total throughput at tank 534 (S534) and Tank 1141 (S1141) of 4,755,000 barrels in any consecutive 12-month period.	[Cumulative Increase]

2.	The owner/operator shall not store material with an RVP greater than 11 psia. [Cumulative Increase]

3.	Sources S534 and S1141 shall be abated by the Vapor Recovery System (A26 and A103) whenever it is used to store organic materials. [BACT; Cumulative Increase]

4.	The owner/operator shall maintain monthly records of the type and net amount of materials stored at S534 and S1141 in a District-approved log for at least 5 years.  The owner/operator shall make available such records to the District upon request.  [Cumulative Increase]

Condtion # 20747:

For S4201, DC Clean Fuels Flare,

A101, Flare for Vine Hill Vapor Recovery System,

A102, Flare for Crude String Vapor Recovery System, and

A103, Flare for Interim Gasoline Vapor Recovery System

1.  The owner/operator of flare (S4201, A101, A102, and A103) shall burn only process upset gases or fuel gas that is released to it as a result of relief valve leakage or other emergency malfunctions. (basis: 40 CFR 60.104(a)(1))

2.	The owner/operator shall record in a District-approved log every flaring event.  This log shall be made available to the District upon request and keep for a period of 5 years from the date of record. (basis: 40 CFR 60.104(a)(1))

Condition 20762

For Refinery:

This condition applies to tanks that are exempt from Regulation 8, Rule 5, Storage of Organic Liquids, due to the exemption in Regulation 8-5-117 for storage of organic liquids with a true vapor pressure of less than or equal to 25.8 mm Hg (0.5 psia).

1.	Effective April 1, 2004, whenever the type of organic liquid in the tank is changed, the owner/operator shall verify that the true vapor pressure at the storage temperature is less than or equal to 25.8 mm Hg (0.5 psia).  The owner/operator shall use Lab Method 28 from Volume III of the District’s Manual of Procedures, Determination of the Vapor Pressure of Organic Liquids from Storage Tanks or other approved District method.  For materials listed in Table 1 of Regulation 8 Rule 5, the owner/operator may use Table 1 to determine vapor pressure, rather than Lab Method 28.  If the results are above 25.8 mm Hg (0.5 psia), the owner/operator shall report non-compliance in accordance with Standard Condition I.F and shall submit an application to the District for a new permit to operate for the tank as quickly as possible.  (Basis:  Regulation 8-5-117)

2.	Effective April 1, 2004, the results of the testing shall be maintained in a District-approved log for at least five years from the date of the record, and shall be made available to District staff upon request.  (Basis: 8-5-117)

# VII.	APPLICABLE LIMITS & COMPLIANCE MONITORING REQUIREMENTS

This section has been included to summarize the applicable emission limits contained in Section IV, Source-Specific Applicable Requirements, of this permit.  The following tables show the relationship between each emission limit and the associated compliance monitoring provisions, if any.  The monitoring frequency column indicates whether periodic (P) or continuous (C) monitoring is required.  For periodic monitoring, the frequency of the monitoring has also been shown using the following codes: annual (A), quarterly (Q), monthly (M), weekly (W), daily (D), or on an event basis (E).  No monitoring (N) has been required if the current applicable rule or regulation does not require monitoring, and the operation is unlikely to deviate from the applicable emission limit based upon the nature of the operation.

| Table VII – A Applicable Limits and Compliance Monitoring Requirements S3 - Tank 3, S4 - Tank 4, S13 - Tank 13, S14 - Tank 14, S20 - Tank 20, S21 - Tank 21 Asphalt Storage, S22 - Tank 22 Asphalt Storage, S23 - Tank 23 Asphalt Storage, S24 - Tank 24 Asphalt Storage, S26 - Tank 26 Asphalt Storage, S129 - Tank 129, S257 - Tank 257, S483 - Tank 483, S484 - Tank 484, S497 - Tank 497 Asphalt Storage, S530 - Tank 530, S532 - Tank 532, S541 - Tank 541, S545 - Tank 545, S548 - Tank 548, S552 - Tank 552 Asphalt Storage, S553 - Tank 553 Asphalt Storage, S554 - Tank 554 Asphalt Storage, S555 - Tank 555 Asphalt Storage, S556 - Tank 556 Asphalt Storage, S557 - Tank 557 Asphalt Storage, S558 - Tank 558 Asphalt Storage, S559 - Tank 559 Asphalt Storage, S560 - Tank 560 Asphalt Storage, S561 - Tank 561 Asphalt Storage, S567 - Tank 567 Asphalt Storage, S568 - Tank 568, S572 - Tank 572 Asphalt Storage, S573 - Tank 573 Asphalt Storage, S598 - Tank 598 Asphalt Storage, S611 - Tank 611, S612 - Tank 612, S613 - Tank 613, S815 - Tank 815 Asphalt Storage, S860 - Tank 860, S861 - Tank 861, S967 - Tank 967, S985 - Tank 985 Asphalt Storage, S1004 - Tank 1004, S1006 - Tank 1006, S1031 - Tank 1031, S1043 - Tank 1043 Asphalt Storage, S1044 - Tank 1044 Asphalt Storage, S1045 - Tank 1045 Asphalt Storage, S1046 - Tank 1046, S1051 - Tank 1051, S1076 - Tank 1076, S1129 - Tank 1129, S1130 - Tank 1130, S1131 - Tank 1131, S1134 - Tank 1134, S1140 - Tank 1140, S1146 - Tank 1146, S1147 - Tank 1147, S1159 - Tank 1159, S1160 - Tank 1160 Asphalt Storage, S1161 - Tank 1161, S1235 - Tank 739 Chem Storage, S1236 - Tank 740 Chem Storage, S1409 - LUBS Sulfonation Plant (SULF), S1411 - LUBS Atmospheric Distillation LDU, S1415 - LUBS Furfural Plant (SEP3), S1416 - LUBS Lube Hydrotreater 1 (LHT1),, S1420 - DH Crude Unit (CU), S1423 - DH Gas Oil Straightrun Hydrotreater (GOHT), S1424 - DH Naphtha Straightrun Hydrotreater (NHT), S1428 - CP Catalytic Feed Hydrotreater (CFH), S1431 - CP Sulfur Plant 1 (SRU1), S1432 - CP Sulfur Plant 2 (SRU2), S1445 - DH Hydrogen Plant 1 (HP1), S1509 - UTIL CO Boiler 2, S1512 - UTIL CO Boiler 3, S1751 - Tank 1330, S1752 - Tank 1331, S1753 - Tank 1332 Gasoline, S1754 - Tank 1333 Gasoline, S1755 - Tank 1334 Gasoline, S1756 - Tank 1335 Gasoline, S1759 - OPCEN Flexicoker (FXU), S1765 - OPCEN Sulfur Plant 3 (SRU3), S1774 - OPCEN Hydrogen Plant 2 (HP2), S1900 - MAINT_Machine Shop Parts Cleaner, S1903 - MAINT Paint Shop Solvent Tub , S3000 - Portable Vacuum Distillation Unit (CCR Technologies Inc.), S4001 - DC Delayed Coking Unit (DCU), S4020 - DC Distillate Hydrotreater (DHT), S4160 - DC Hydrogen Plant –3 (HP3), S4170 - LUBS Lube Hydrotreater 2 (LHT2), S4180 - OPCEN Sulfur Plant 4 (SRU4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Through-put | BAAQMD Condition #18618, Part 1 | N |  | Annual throughput limits | BAAQMD Condition #18618, Part 2 | P/A | Records |

| Table VII – B Applicable Limits and Compliance Monitoring Requirements S4307 – Tank 14687 MDEA Make-up , S4309 – Tank 14517 DEA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502, and 8-5-603.2 | P/A | Source Test |

| Table VII – C Applicable Limits and Compliance Monitoring Requirements S13 Tank 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| See Table VII – X for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition #12271, Part 45 | Y |  | Tank shall be kept leak free, less than 500 ppm above background as methane | NESHAP Subpart FF 61.350, 61.356(k) and 61.357(d)(8) and NESHAP Subpart CC 63.642(e), 63.642(f) and 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |

| Table VII – Da Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S14 – Tank 14, S19 - Tank 19, S20 – Tank 20, S483 – Tank 483, S484 – Tank 484, S534 – Tank 534, S530 – Tank 530, S610 – Tank 610, S611 – Tank 611, S612 – Tank 612, S613 – Tank 613, S1070 – Tank 1070, S1133 – Tank 1133, S1134 – Tank 1134, S1139 – Tank 1139, S1140 – Tank 1140, S1141 – Tank 1141 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-306 | Y |  | Control device standards; includes 95% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.1 | P/E | Records of liquids stored. Emissions of organic compounds measured using MOP, Vol. IV, ST-34 |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |

| Table VII – Db Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S534 – Tank 534, S1141 – Tank 1141 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD Condition # 20398 Part 1 | Y |  | Throughput of S543+S1141 < 4,755,000 bbl/yr | BAAQMD Condition # 20398 Part 4 | PM | Records |
|  | BAAQMD Condition # 20398 Part 2 | Y |  | RVP < 11 psia | BAAQMD Condition # 20398 Part 4 | PM | Records |

| Table VII – Dc Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S1751 – Tank 1330, S1752 – Tank 1331, S1753 – Tank 1332 Gasoline, S1754 – Tank 1333 Gasoline, S1757 – Tank 1336, S1758 – Tank 1337 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-306 | Y |  | Control device standards; includes 95% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.1 | P/E | Records of liquids stored. Emissions of organic compounds measured using MOP, Vol. IV, ST-34 |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Ka 60.112a(a)(3) | Y |  | Control device standards; at least 95% efficient | 40 CFR 63.640(d)(5) | N | None |

| Table VII – E Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S14 – Tank 14, S20 – Tank 20, S483 – Tank 483, S484 – Tank 484, S530 – Tank 530, S532 – Tank 532, S1139 – Tank 1139, S1140 – Tank 1140, S1141 – Tank 1141 S1751 – Tank 1330, S1752 – Tank 1331, S1753 –Tank 1332 Gasoline, S1754 – Tank 1333 Gasoline, S1757 –Tank 1336, S1758 – Tank 1337 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – Da, Db and L for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition #7618, Part D.1.a | Y |  | No more than 12 tanks shall be used to store a liquid with a true vapor pressure of 1.5 psig or greater. [basis: Cumulative Increase] | BAAQMD Condition #7618, Part G | P/A | Reporting |

| Table VII – F Applicable Limits and Compliance Monitoring Requirements S19 - Tank 19, S1139 – Tank 1139 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit |  |  | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Through-put |  | BAAQMD Condition #18646, Part 1 | Y |  |  | Combined throughput shall not exceed 7,300,000 barrels in any 12-month period and combined gasoline throughput shall not exceed 1,650,000 barrels in any 12-month period | BAAQMD Condition #18646, Part 4 | P/M | Records |
| VOC |  | BAAQMD Condition #18646, Part 2a |  |  |  | Gasoline or Gasoline components not exceeding RVP 12 | BAAQMD Condition #18646, Part 4 | P/M | Records |
| VOC |  | BAAQMD Condition #18646, Part 2bi | Y |  |  | POC emissions, based on the maximum throughput in Part 1, do not exceed 20,731 pounds per year, as calculated by the EPA Tanks Program version 4.08 (or subsequent version) with a 99.5% reduction for vapor recovery | BAAQMD Condition #18646, Part 4 | P/M | Records |
| HAP |  | BAAQMD Condition #18646, Part 2bii | N |  |  | Toxic emissions in lb/yr, based on the maximum throughput in Condition 1, do not exceed any risk screening trigger level specified in Table 2-1-316 | BAAQMD Condition #18646, Part 4 | P/M | Records |

| Table VII - G Applicable Limits and Compliance Monitoring Requirements S21 –Tank 21 Asphalt Storage, S22 –Tank 22 Asphalt Storage, S23 –Tank 23 Asphalt Storage, S24 –Tank 24 Asphalt Storage, S26 –Tank 26 Asphalt Storage, S497 –Tank 497 Asphalt Storage, S552 –Tank 552 Asphalt Storage, S553 –Tank 553 Asphalt Storage, S554 –Tank 554 Asphalt Storage, S555 –Tank 555 Asphalt Storage, S556 –Tank 556 Asphalt Storage, S557 –Tank 557 Asphalt Storage, S558 –Tank 558 Asphalt Storage, S559 –Tank 559 Asphalt Storage, S560 –Tank 560 Asphalt Storage, S561 –Tank 561 Asphalt Storage, S567 –Tank 567 Asphalt Storage, S571 –Tank 571 Asphalt Storage, S572 –Tank 572 Asphalt Storage, S573 –Tank 573 Asphalt Storage, S598 –Tank 598 Asphalt Storage, S815 –Tank 815 Asphalt Storage, S867 –Tank 867 Asphalt Storage, S868 –Tank 868 Asphalt Storage, S876 –Tank 876 Asphalt Storage, S961 –Tank 961 Asphalt Storage, S985 –Tank 985 Asphalt Storage, S1017 –Tank 1017 Asphalt Storage, S1018 –Tank 1018 Asphalt Storage, S1041 –Tank 1041 Asphalt Storage, S1043 –Tank 1043 Asphalt Storage, S1044 –Tank 1044 Asphalt Storage, S1045 –Tank 1045 Asphalt Storage, S1048 –Tank 1048 Asphalt Storage, S1075 - Tank 1075 Asphalt Storage, S1160 –Tank 1160 Asphalt Storage, S1408 – LUBS Asphalt Blending and Shipping, S1523 – LUBS Loading Rack Asphalt Inside T/T, S1524 – LUBS Loading Rack Asphalt outside T/T and T/C, S1525 – LUBS Loading Rack Asphalt paving, S1539 – LR-25 Loading Rack Acid Sludge and Sulfonation T/T |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |

| Table VII – H Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S129 – Tank 129, S540 – Tank 540, S541 – Tank 541, S544 – Tank 544, S545 – Tank 545, S1031 – Tank 1031, S1046 – Tank 1046, S1051 – Tank 1051, S1072 – Tank 1072, S1129 –Tank 1129, S1159 – Tank 1159, S1161 - Tank 1161 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| HAP | NESHAP Subpart CC 63.646(a), 63.120(b)(3) and 63.120(b)(5) | Y |  | Primary rim-seal standards; includes gap criteria | NESHAP Subpart CC 63.646(a), 63.120 (b)(1) and 63.120(b)(2) | P/5 yr intervals | Measurement and visual inspection |
| HAP | NESHAP Subpart CC 63.646(a) 63.120(b)(4) and 63.120(b)(6) | Y |  | Secondary rim-seal standards; includes gap criteria | NESHAP Subpart CC 63.646(a), 63.120 (b)(1) and 63.120(b)(2) | P/A | Measurement and visual inspection |

| Table VII – I Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S992 – Tank 992, S1076 – Tank 1076, S1130 –Tank 1130, S1131 –Tank 1131 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501.1 | P/E | Records |
| VOC |  |  |  |  | BAAQMD 8-5-501.2 | P/Semi-Annual | Records |
| VOC | BAAQMD 8-5-328.1.2 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |

| Table VII – J Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S1755 –Tank 1334 Gasoline, S1756 – Tank 1335 Gasoline, S1191 –Tank -1256 Crude Oil Storage, S1192 –Tank –1257 Crude Oil Storage |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Ka 60.112a(a)(1) | Y |  | External Floating Roof requirements, including primary and secondary seal requirements and gap limits | NSPS Subpart Ka 60.113a(a)(1) | P/5 year interval for primary seals; P/A for secondary seals | Measurement and visual inspection |
| HAP | NESHAP Subpart CC 63.646(a), 63.120(b)(3) and 63.120(b)(5) | Y |  | Primary rim-seal standards; includes gap criteria | NESHAP Subpart CC 63.646(a), 63.120 (b)(1) and 63.120(b)(2) | P/5 yr intervals | Measurement and visual inspection |
| HAP | NESHAP Subpart CC 63.646(a) 63.120(b)(4) and 63.120(b)(6) | Y |  | Secondary rim-seal standards; includes gap criteria | NESHAP Subpart CC 63.646(a), 63.120 (b)(1) and 63.120(b)(2) | P/A | Measurement and visual inspection |

| Table VII – K Applicable Limits and Compliance Monitoring Requirements S1076 – Tank 1076 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – I for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition #7618, Part D.2 | Y |  | Liquid vapor pressure limit of less than 1.5 psia | BAAQMD Condition #7618, Part G | P/A | Records |

| Table VII – L Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S532 – Tank 532 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| HAP | NESHAP Subpart FF 61.343(a)(1) (i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.343(c), 61.350, 61.355(h), 61.356(d), 61.356(h) and 61.357(d)(8) NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| HAP | NESHAP Subpart FF 61.349(a)(1)(i)) | Y |  | Closed vent system leak tightness standards (< 500 ppmw - unless maintained under negative pressure) | NESHAP Subpart FF 61.355(h) and 61.356(h) and Subpart CC 63.640(d)(5) ) | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs) |
| HAP | NESHAP Subpart FF 63.649(a)(2)(ii) | Y |  | Abatement efficiency of at least 95% by weight | None | N | None |

| Table VII – M Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S540 – Tank 540 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – H for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #11951, Part 1 | Y |  | Throughput shall not exceed 340 million gallons in any 12-month period | BAAQMD Condition #11951, Part 1 | P/M | Records |

| Table VII – N Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S544 – Tank 544 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – H for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition #11850, Part 1 |  |  | Steam coils shall not be used when the stored material has an API greater than 20 | BAAQMD Condition #11850, Part 1 | P/E | Records |

| Table VII – O Applicable Limits and Compliance Monitoring Requirements S549 - Tank 549 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – A for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #6111, Part 1 | Y |  | Throughput shall not exceed 420,000 gallons in any 12-month period | BAAQMD Condition #6111, Part 4 | P/M | Records |
| VOC | BAAQMD Condition #6111, Part 2 | Y |  | Vapor pressure of material stored in this tank shall not exceed 8 psia | BAAQMD Condition #6111, Part 4 | P/E | Records |
| HAP | BAAQMD Condition #6111, Part 3 | N |  | Benzene content of material stored in this tank shall not exceed 1 mg/l | BAAQMD Condition #6111, Part 4 | P/E | Records |

| Table VII – P Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S858 – Tank 858, S1023 – Tank 1023, S1050 – Tank 1050, S2445 – Tank 12445, S2446 – Tank 12446, S4322 – Tank 14571 Sour Water |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501.1 | P/E | Records |
| VOC |  |  |  |  | BAAQMD 8-5-501.2 | P/Semi-Annual | Records |
| VOC | BAAQMD 8-5-328.1.2 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Kb 60.116b(c) | Y |  | True vapor pressure determination | NSPS Subpart Kb 60.116b(e) | P/E (Upon change of service) | Calculate |

| Table VII – Q Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S858 – Tank 858 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – P for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #4977, Part 2 | Y |  | Throughput shall not exceed 85,848 thousand gallons in any 12-month period | BAAQMD Condition #4977, Part 3 | P/M | Records |

| Table VII – R Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S1006 – Tank 1006, S2013 – Tank 12467, S4310 – Tank 13285 Sour Water, S17095 – Tank 17095 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | N |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Kb 60.113b (b)(4)(i) | Y |  | Primary rim-seal standards; Accumulated area of gaps shall not exceed 21.2 cm2/m of tank diameter and 3.81 cm at any point | NSPS Subpart Kb 60.113b(b)(1), 60.113b(b)(2) and 60.113b(b)(3) | P/(5 yr intervals) | Measurement and visual inspection |
| VOC | NSPS Subpart Kb 60.113b (b)(4)(ii) | Y |  | Secondary rim-seal standards; Accumulated area of gaps shall not exceed 21.2 cm2/m of tank diameter and 3.81 cm at any point | NSPS Subpart Kb 60.113b(b)(1), 60.113b(b)(2) and 60.113b(b)(3) | P/A | Measurement and visual inspection |
| VOC | NSPS Subpart Kb 60.116b(c) | Y |  | True vapor pressure determination | NSPS Subpart Kb 60.116b(e) | P/E (Upon change of service) | Measurement and visual inspection |

| Table VII – S Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S1023 – Tank 1023, S1050 – Tank 1050 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – P for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #7133, Part 2 | Y |  | Combined throughput shall not exceed 128,772 thousand gallons in any 12-month period | BAAQMD Condition #7133, Part 3 | P/M | Records |

| Table VII – T Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tank S1063 – Tank 1063 ETP 1, S1067 – Tank 1067 ETP 1, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| HAP | NESHAP Subpart FF 61.351(a)(2) NSPS and Subpart Kb 60.112b(a)(2) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.350, 61.356(k) and 61.357(f) and NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |

| Table VII – U Applicable Limits and Compliance Monitoring Requirements S1070 – Tank 1070 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD Condition # 18153 Part 1 | Y |  | Total liquid throughput shall not exceed 175,200 thousand gallons during any consecutive twelve-month period | BAAQMD Condition # 18153 Part | P/M | Records |
|  | BAAQMD Condition # 18153 Part 2 | Y |  | Materials stored in S-1070 shall be limited to the following: a. Hydrocarbon Blanket Layer (RVP 3); b. Sour Water | BAAQMD Condition # 18153 Part 2ci and 2cii | P/E | Records |

| Table VII – V Applicable Limits and Compliance Monitoring Requirements S1072 – Tank 1072 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – I & J for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition # 7382 Part 4 and 5 | Y |  | Total liquid throughput shall not exceed 18.2 million barrels during any consecutive twelve-month period | BAAQMD Condition # 7382 Part 6 | P/M | Records |

| Table VII – W Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks, S1077 – Tank 1411, S12490 – LOG Tank 12519 Wastewater ETP 1&2 , S12491 – LOG Tank 12520 Wastewater ETP 1&2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Kb 60.113b (b)(4)(i) | Y |  | Primary rim-seal standards; includes gap criteria | NSPS Subpart Kb 60.113b(b)(1), 60.113b(b)(2) and 60.113b(b)(3) | P/(5 yr intervals) | Measurement and visual inspection |
| VOC | NSPS Subpart Kb 60.113b (b)(4)(ii) | Y |  | Primary rim-seal standards; Accumulated area of gaps shall not exceed 21.2 cm2/m of tank diameter and 3.81 cm at any point | NSPS Subpart Kb 60.113b(b)(1), 60.113b(b)(2) and 60.113b(b)(3) | P/A | Measurement and visual inspection |
| VOC | NSPS Subpart Kb 60.116b(c) | Y |  | Primary rim-seal standards; Accumulated area of gaps shall not exceed 21.2 cm2/m of tank diameter and 1.27 cm at any point | NSPS Subpart Kb 60.116b(e) | P/E (Upon change of service) | Measurement and visual inspection |
| HAP | NESHAP Subpart FF 61.351(a)(2) and NSPS Subpart Kb 60.112b(a)(2) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.350, 61.356(k), and 61.357(f) NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |

| Table VII – X Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S13 Tank 13, S1114 –Tank 1114 Spent Acid, S1115 –Tank 1115 Spent Acid, S4334 – Tank 13276 Alkylate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Kb 60.112b (a)(3)(i) | Y |  | Closed vent system leak tightness standards, VOC concentrations shall not exceed 500 ppmv above background. | 40 CFR 63.640(d)(5) | N | None |
| VOC | NSPS Subpart Kb 60.112b (a)(3)(ii) | Y |  | Control device standards; includes 95% efficiency requirement | 40 CFR 63.640(d)(5) | N | None |

| Table VII – Y Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S1805 Tank 12038 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-306 | Y |  | Control device standards; includes 95% efficiency requirement | BAAQMD 8-5-501 and 8-5-603.1 | P/E | Records of liquids stored. Emissions of organic compounds measured using MOP, Vol. IV, ST-34 |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.2 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-404, 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| HAP | NESHAP Subpart FF 61.343(a)(1)(i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.343(c), 61.350, 61.355(h), 61.356(d), 61.356(h) and 61.357(d)(8) and NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| HAP | NESHAP Subpart FF 63.649(a)(2)(ii) | Y |  | Abatement efficiency of at least 95% by weight | None | N | None |
| HAP | NESHAP Subpart FF 61.349(a)(1)(i)) | Y |  | Closed vent system leak tightness standards (< 500 ppmw - unless maintained under negative pressure) | NESHAP Subpart FF 61.355(h) and 61.356(h) | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs) |

| Table VII – Z Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S1114 –Tank 1114 Spent Acid, S1115 –Tank #1115 Spent Acid, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – Y for additional requirements. |  |  |  |  |  |  |  |
| Through-put S1114 and S1115 | BAAQMD Condition #7215, Part 2 | Y |  | Combined throughput shall not exceed 365,000 barrels in any 12-month period | BAAQMD Condition #7215, Part 3 | P/M | Records |

| Table VII – AA Applicable Limits and Compliance Monitoring Requirements S1116 – Tank 1116 Fresh Acid |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |

| Table VII – AB Applicable Limits and Compliance Monitoring Requirements Tanks Subject To Submerged Fill S1117 – Tank 1117 Skim |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |  |
| VOC | BAAQMD Condition #12190, Part 1 | Y |  | Throughput shall not exceed 10,000 barrels per year | BAAQMD Condition #12190, Part 1 | P/Q | Records |  |

| Table VII – AC Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S1129 –Tank 1129, S1130 - Tank 1130, S1131 - Tank 1131, S4310 - Tank 13285 Sour Water |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – I & J for additional requirements. |  |  |  |  |  |  |  |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502, and 8-5-603.2 | P/A | Source Test |

| Table VII – AD Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S1146 – Tank 1146, S1147 – Tank 1147 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| HAP | NESHAP Subpart FF 61.351(a)(2) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.350, 61.356(k) and 61.357(f) and NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |

| Table VII – AE Applicable Limits and Compliance Monitoring Requirements S1409 – LUBS Sulfonation Plant (SULF), S1411 – LUBS Atmospheric Distillation LDU, S1412 – LUBS Vacuum Distillation LDU, S1415 – LUBS Furfural Plant (SEP3), S1416 – LUBS -Lube Hydrotreater 1 (LHT1), S1417 – OPCEN Distillate Saturation Unit (DSU), S1420 – DH Crude Unit (CU), S1421 – DH Vacuum Flasher Unit (VFU), S1422 – DH Marine Fuel Oil Blender, S1423 – DH Gas Oil Straightrun Hydrotreater (GOHT), S1424 – DH Naphtha Straightrun Hydrotreater (NHT), S1425 – DH Catalytic Reformer Unit (CRU), S1427 – CP Catalytic Gas Plant (CGP), S1428 – CP Catalytic Feed Hydrotreater (CFH), S1429 – CP Catalytic Gasoline Hydrotreater (CGH), S1430 – CP Alkylation Plant (ALKY), S1433 – CP Light CC Gasoline Treater, S1434 – CP Unsaturated C3/C4 Treater, S1435 – CP Fuel Gas Treater 1, S1436 – CP Fuel Gas Treater 2, S1445 – DH Hydrogen Plant 1 (HP1), S1446 – DH Saturates Gas Plant (SGP), S1447 – DH Saturates Dry Gas Treater, S1448 – DH Saturates Gas Plant C3/C4 Treater, S1449 – DH Hydrocracking Unit (HCU), S1462 – LOG Distillate Blender (Jet & Diesel Fuel), S1463 – LOG Gasoline Blender, S1464 – LOG Thin Fuel Blender (Wharf), S1759 – OPCEN Flexicoker (FXU), S1764 – OPCEN Dimersol Plant (DIMER), S1770 – OPCEN C3/C4 Splitter, S1774 – OPCEN Hydrogen Plant 2 (HP2), S4001 – DC Delayed Coking Unit (DCU), S4020 – DC Distillate Hydrotreater (DHT), S4050 – DC Catalytic Gas Depentanizer (CGDP), S4080 – DC Isomerization Unit (ISOM), S4140 – DC Heavy Cracked Gasoline Hydrotreater (HGHT), S4160 – DC Hydrogen Plant 3 (HP3), S4170 – LUBS Lube Hydrotreater 2 (LHT2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-10-301 | Y |  | Abatement of emissions from process vessel depressurization is required until pressure is reduced to less than 1000 mm Hg | BAAQMD 8-10-401 | P/E | Records |
| VOC | BAAQMD 8-18-301 | Y |  | Equipment that leaks > 100 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-302 | Y |  | Valves that leak > 100 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-404, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-303 | Y |  | Pumps and compressors that leak > 500 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-403, 8-18-501, 8-18-502 and 8-18-602 | P/D – visual inspection and Q - monitoring | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-304 | Y |  | Connections that leak > 100 ppm shall not be used unless the leak is discovered by an operator and minimized within 24 hours and repaired within 7 days or if the leak is discovered by an APCO and repaired within 24 hours | BAAQMD 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-305 | Y |  | Pressure release devices that leak > 500 ppm shall not be used unless the leak is discovered by an operator and minimized within 24 hours and repaired within 15 days or if the leak is discovered by an APCO and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-306 | Y |  | Limitations on non-repairable equipment | BAAQMD 8-18-501, 8-18-502 and 8-18-602 | P/5 years or next turnaround | Records And Sampling |
| VOC | BAAQMD 8-18-307 and 8-18-211 | Y |  | Equipment that leaks liquid > 3 drops per minute and > 100 ppm VOC shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD Regulation 8-18-306, 8-18-401, 8-18-402, 8-18-501, 8-18-502, and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-28-303 | N |  | Pressure relief devices shall be vented to vapor recovery or disposal system with a control efficiency of 95% by weight | BAAQMD 8-28-403, 8-28-404, 8-28-405 and 8-28-602 | C | Records and testing with approved methods |
| VOC | BAAQMD 8-28-304 | N |  | If one reportable Release Event from a pressure relief device in any consecutive 5 year period, shall meet specified conditions | BAAQMD 8-28-401, 8-28-402, 8-28-403, 8-28-404 and 8-28-405 | P/E | Reporting and prescribed measures. |

| Table VII – AF Applicable Limits and Compliance Monitoring Requirements S1420 – DH Crude Unit (CU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.a | Y |  | If profile has more than 5 non-complying days, the refinery shall not process more than 124,000 barrels of crude oil per stream day | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – AFa Applicable Limits and Compliance Monitoring Requirements S1425 – DH Catalytic Reformer Unit (CRU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table AE for additional requirements. |  |  |  |  |  |  |  |
| HCl | MACT Subpart UUU 63.1567(a)(1) | Y | 4/11/05 | Reduce HCl emissions by 97% (wt) or HCl emissions of 10 ppmv at 3%O2 | MACT Subpart UUU 63.1567(b) 63.1567(c) 63.1570(c) 63.1571(b) 63.1572(c) 63.1572(d) 63.1574(a)(2) 63.1574(a)(3)(i) 63.1574(a)(3)(ii) 63.1574(d) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(d) 63.1575(f)(1) 63.1576(a) 63.1576(b) 63.1576(d) 63.1576(f) 63.1576(g) 63.1576(h) | P/E (Initial compliance demonstration, performance test, CPM installation and performance evaluation, establish operating limits, submit initial notifications and NOCS), P/Semi-Annual ( compliance report), and C (parameter monitoring, maintain records) | Continuous pH and water/gas flow monitors, Performance test, Records, and reports |

| Table VII - AG Applicable Limits and Compliance Monitoring Requirements S1426 – CP Catalytic Cracking Unit (CCU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Emission Limit Citation | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | NSPS Subpart J 60.105(a)(1), 60.105(e)(1) BAAQMD 6-501, 6-502 and 1-522 | C | Opacity monitor and Records |
| Opacity | BAAQMD Condition # 12911 Part 4 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 12911 Part 5 | C | Water seal monitoring and Records |
| Opacity | BAAQMD 6-302 | Y |  | 20% opacity for no more than 3 minutes/hour | NSPS Subpart J 60.105(a)(1), 60.105(e)(1) BAAQMD 6-501, 6-502 and 1-522 | C | Opacity monitor and Records |
| Opacity | NSPS Subpart J 60.102(a) (2) | Y |  | 30 % opacity, except for one 6 minute average opacity reading in 1 hour | NSPS Subpart J 60.105(a)(1), 60.105(e)(1)) | C | Opacity monitor and records |
| Opacity | BAAQMD Condition #12911 Part 2 | Y |  | Ringelmann No. 0.5 for no more than 3 minutes/hour for Catalyst Storage and Injection System | BAAQMD Condition # 12911 Part 3 and Part 4 | P/E | Visible emissions inspection |
| Opacity (Surrogate for Metal HAPs) | MACT Subpart UUU 63.1564(a)(1)(i) | Y | 4/11/05 | Opacity of emissions must not exceed 30%, except for one 6-minute opacity reading in any 1-hour period | 63.1564(b)(1) 63.1564(c)(1) 63.1570(c) 63.1572(b) 63.1572(d) 63.1574(a)(3)(I) 63.1574(d) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(d) 63.1576(a) 63.1576(b) 63.1576(c) 63.1576(d) 63.1576(f) 63.1576(g) 63.1576(h) 63.1576(i) | P/E (Initial Compliance demonstration, COMS installation and performance test, submit initial notifications and NOCS), P/D (operational records), P/Semi-Annual (compliance report), and C (opacity monitoring, maintain records) | Records, calculations, Performance Tests, and Reports |
| FP | Regulation 6-310 | Y |  | 0.15 grain per dscf | BAAQMD Condition # 18618 Part 9 & Condition 12911 Part 3 | P/A | Source Test |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | BAAQMD Condition # 18618 Part 9 & Condition 12911 Part 3 | P/A | Source Test |
| PM | NSPS Subpart J 60.102(a)(1) and 60.102(b) | Y |  | PM emissions shall not exceed 1.0 lb per 1000 lb of coke burn-off plus incremental emissions of 0.10 lb/MMBtu for auxiliary fuel | NSPS Subpart J 60.105(c), 60.105(d)) | P/D | Source test, calculations and records |
| PM | BAAQMD Condition # 18407 Part 2 | Y |  | During periods when the spent catalyst hopper is blinded off from the CO Boilers, the amount of catalyst loaded into the spent catalyst hopper shall not exceed 2000 tons in any consecutive 12-month period | BAAQMD Condition # 18407 Part 3 | P/D | Records |
| PM (Surrogate for Metal HAPs) | MACT Subpart UUU 63.1564(a)(1)(i) | Y | 4/11/05 | PM emissions must not exceed 1.0 lb per 1000 lb of coke burn-off plus incremental emissions of 0.10 lb/MMBtu for auxiliary fuel | Subpart UUU 63.1564(b)(2) 63.1564(b)(4)( i) 63.1564(b)(5) 63.1564(c)(1) 63.1570(c) 63.1571(a)(1) 63.1571(b) 63.1574(a)(3)(ii) 63.1574(d) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(d) 63.1575(f) 63.1576(a) 63.1576(d) 63.1576(f) 63.1576(g) 63.1576(h) 63.1576(i) | P/E (Initial Compliance demonstration: performance test, emission calculation, submit initial notifications and NOCS); P/D (operational records); P/Semi-Annual (compliance report); and C (maintain records) | Records, calculations, Performance Test, and Reports |
| SO2 | BAAQMD 9-1-310.1 | Y |  | SO2 emissions shall not exceed 1000 ppmv | BAAQMD 9-1-502 and 1-522 | C | SO2 CEM and records |
| SO2 | NSPS Subpart J 60.104(b)(2) | Y |  | Without add-on control device, maintain SO2 emissions to atmosphere at less than or equal to 9.8 kg of SO2 per 1000 kg of coke burn-off | NSPS Subpart J 60.104(c), 60.105(c), 60.106(i)(12), 60.108(d) and 60.108(e) | P | Records |
| VOC | BAAQMD 8-10-301 | Y |  | Abatement of emissions from process vessel depressurization is required until pressure is reduced to less than 1000 mm Hg | BAAQMD 8-10-401 | P/E | Records |
| VOC | BAAQMD 8-18-301 | Y |  | Equipment that leaks > 100 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-302 | Y |  | Valves that leak > 100 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-404, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-303 | Y |  | Pumps and compressors that leak > 500 ppm shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-403, 8-18-501, 8-18-502 and 8-18-602 | P/D – visual inspection and Q - monitoring | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-304 | Y |  | Connections that leak > 100 ppm shall not be used unless the leak is discovered by an operator and minimized within 24 hours and repaired within 7 days or if the leak is discovered by an APCO and repaired within 24 hours | BAAQMD 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/Q | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-305 | Y |  | Pressure release devices that leak > 500 ppm shall not be used unless the leak is discovered by an operator and minimized within 24 hours and repaired within 15 days or if the leak is discovered by an APCO and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-18-306 | Y |  | Limitations on non-repairable equipment | BAAQMD 8-18-501, 8-18-502 and 8-18-602 | P/5 years or next turnaround | Records And Sampling |
| VOC | BAAQMD 8-18-307 and 8-18-211 | Y |  | Equipment that leaks liquid > 3 drops per minute and > 100 ppm VOC shall not be used unless the leak is discovered and minimized within 24 hours and repaired within 7 days | BAAQMD 8-18-306, 8-18-401, 8-18-402, 8-18-501, 8-18-502 and 8-18-602 | P/Q or A | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
| VOC | BAAQMD 8-28-303 | N |  | Pressure relief devices shall be vented to vapor recovery or disposal system with a control efficiency of 95% by weight | BAAQMD 8-28-403, 8-28-404, 8-28-405, and 8-28-602 | C | Records and testing with approved methods |
| VOC | BAAQMD 8-28-304 | N |  | If one reportable Release Event from a pressure relief device in any consecutive 5 year period, shall meet specified conditions | BAAQMD 8-28-401, 8-28-402, 8-28-403, 8-28-404 and 8-28-405 | P/E | Reporting and prescribed measures. |
| CO | NSPS Subpart J 60.103(a) | Y |  | CO emissions shall not exceed 500 ppmv | NSPS Subpart J 60.105(a)(2), 60.105(c), | C and P/D | CO CEM and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.c | Y |  | If profile has more than 5 non-complying days, the refinery shall not process more than 65,000 barrels of catalytic cracker feed per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| CO (Surrogate for HAP VOC) | MACT Subpart UUU 63.1565(a)(1) | Y | 4/11/05 | CO emissions shall not exceed 500 ppmv | MACT Subpart UUU 63.1565(b)(1) 63.1565(b)(2) 63.1565(b)(4) 63.1565(b)(6) 63.1565(c) 63.1570(c) 63.1571(a)(1) 63.1571(b) 63.1572(a) 63.1572(d) 63.1574(a)(ii) 63.1574(d) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(e) 63.1575(f)(1) 63.1576(a) 63.1576(b) 63.1576(d) 63.1576(f) 63.1576(g) 63.1576(h) | P/E (Initial Compliance Demonstration, performance test, CEMS installation and performance evaluation, submit initial notifications and NOCS), P/Semi-Annual (compliance report), and C (emissions monitoring, maintain records) | Records, Performance Test, and CO CEM |

| Table VII – AH Applicable Limits and Compliance Monitoring Requirements S1431 –CP Sulfur Plant 1 (SRU1), S1432 – CP Sulfur Plant 2 (SRU2), S1765 – OPCEN Sulfur PLant 3(SRU3), S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| SO2 | NSPS Subpart J 60.104 (a)(2)(i) | Y |  | 250 ppmv SO2 at 0% O2 | NSPS Subpart J 60.105(a)(5), 60.106(f)(3) | C | SO2 and O2 CEM |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | None | N | None |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | None | N | None |
| SO3/ H2SO4 | BAAQMD 6-330 | Y |  | 0.08 grain/dscf exhaust concentration of SO3 and H2SO4, expressed as 100% H2SO4 | BAAQMD Condition # 18618, Part 8 | P/A | Source Test |
| H2S/NH3 | BAAQMD 9-1-313.2 | N |  | Operation of a sulfur removal and recovery system that removes and recovers: 95% of H2S from refinery fuel gas, 95% of H2S and ammonia from process water streams | BAAQMD Condition # 18618 Part 10 | P/A | Analysis and Recordkeeping |

| Table VII – AHa Applicable Limits and Compliance Monitoring Requirements S1431 –CP Sulfur Plant 1 (SRU1), S1432 – CP Sulfur Plant 2 (SRU2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table AH for additional requirements. |  |  |  |  |  |  |  |
| SO2 | MACT Subpart UUU 63.1568(a)(1)(i) | Y | 4/11/05 | SO2 emissions shall not exceed 250 ppmv at 0% excess O2 | MACT Subpart UUU 63.1568(b)(1) 63.1568(b)(2) 63.1568(b)(5) 63.1568(c)(1) 63.1570(c) 63.1571(b) 63.1572(a) 63.1572(d) 63.1574(a)(2) 63.1574(a)(3) 63.1574(d) 63.1574(f)(1) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(e) 63.1575(f)(1) 63.1576(a) 63.1576(b) 63.1576(d) 63.1576(f) 63.1576(g) 63.1576(h) 63.1576(i) | P/E (Initial compliance demonstration, performance test, CEMs installation and performance evaluation, submit initial notifications and NOCS), P/D (records) P/Semi-Annual (compliance report) and C (emissions monitoring, maintain records) | Records, SO2 and O2 CEM, Performance Test, Reports |

| Table VII – AHb Applicable Limits and Compliance Monitoring Requirements S1765 – OPCEN Sulfur PLant 3(SRU3), S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table AH for additional requirements. |  |  |  |  |  |  |  |
| SO2 | MACT Subpart UUU 63.1568(a)(1) | Y | 4/11/05 | 250 ppmv SO2 at 0% excess O2 | MACT Subpart UUU 63.1568(b)(1) 63.1568(b)(5) 63.1568(b)(7) 63.1568(c)(2) 63.1570(c) 63.1572(a)(1) 63.1572(a)(3) 63.1572(a)(4) 63.1572(d) 63.1574(a) 63.1574(d) 63.1575(a) 63.1575(b) 63.1575(c) 63.1575(e) 63.1575(g) 63.1575(h) 63.1576(a) 63.1576(b) 63.1576(d) 63.1576(e) 63.1576(f) 63.1576(g) 63.1576(h) 63.1576(i) | P/E (Initial Compliance Demonstration, Submit initial notifications and NOCS), P/D (records), P/Semi-Annual (compliance report), and C (emissions monitoring, maintain records) | Records, SO2 and O2 CEM, Reports |

| Table VII – AI Applicable Limits and Compliance Monitoring Requirements S1765 – OPCEN Sulfur Plant t3 (SRU3) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type of Limit |  | Citation of Limit |  | FE Y/N |  | Future Effective Date |  | Limit |  | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AH for additional requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOx, SO2, CO and PM |  | BAAQMD Condition #7618, Part A |  | Y |  |  |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 |  | BAAQMD Condition #7618, Part B, F and G |  | P/D | Calculation, reporting and records |

| Table VII – AJ Applicable Limits and Compliance Monitoring Requirements S1457 – Cooling Water Tower (CWT-32), S-1778 – Cooling Water Tower (CWT-50), and S4210 – Cooling Water Tower (CWT-13278) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | None | N | None |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | None | N | None |
| VOC | BAAQMD 8-2-301 | Y |  | < 300 ppmv C1 | None | N | None |

| Table VII – AK Applicable Limits and Compliance Monitoring Requirements S1465 – LOG Light Oil Products Gross Oil Separator, S1779 OPCEN CPI Oil/Water Separator, S2009 LUBSWastewater Separator Dubbs Box |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-8-302.4 | Y |  | VOC concentrations shall not exceed 1,000 ppm (expressed as methane) above background | BAAQMD 8-8-302.4, 8-8-503, 8-8-603 and BAAQMD Condition #5077, Part 6, 9, 12 | P/S | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |

| Table VII – AL Applicable Limits and Compliance Monitoring Requirements S1465 – LOG Light Oil Products Gross Oil Separator, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AK for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #5077, Part 4 | Y |  | Throughput shall not exceed 3400 gpm or the maximum allowable capacity of the unit | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AM Applicable Limits and Compliance Monitoring Requirements S1469 – LOG API Separator (ETP 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-8-302.4 | Y |  | VOC concentrations shall not exceed 1,000 ppm (expressed as methane) above background | BAAQMD 8-8-302.4, 8-8-503, 8-8-603 and BAAQMD Condition #5077, Part 3 | P/S | Visual inspections, portable HC detector (EPA Method 21) and records of inspections and repairs |
|  | NESHAP Subpart FF 61.347(a)(1)(i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.347(b), 61.347(c), 61.350, 61.355(h), 61.356(d), 61.356(h), and 61.357(d)(8) and NESHAP Subpart CC 63.354(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| HAP | NESHAP Subpart FF 61.349(a)(1)(i)) | Y |  | Closed vent system leak tightness standards (< 500 ppmw - unless maintained under negative pressure) | NESHAP Subpart FF 61.355(h) and 61.356(h) | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs) |
| HAP | NESHAP Subpart FF 63.649(a)(2)(ii) | Y |  | Abatement efficiency of at least 95% by weight | None | N | None |
| Through-put | BAAQMD Condition #5077, Part 1 | Y |  | Throughput shall not exceed 6000 gpm or the maximum allowable capacity of the unit | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AN Applicable Limits and Compliance Monitoring Requirements S1470 – LOG LPG Loading Flare |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) except for gas burned as a result of process upset or gas burned at flares from relief valve leaks or other emergency malfunctions | NSPS Subpart A 60.13(i) | C | Monitoring and records |
| VOC | BAAQMD Condition #12271, Part 74 | Y |  | Emissions from loading operation shall be controlled with an overall capture and destruction efficiency at least 98.5% by weight | None | N | None |

| Table VII – AO Applicable Limits and Compliance Monitoring Requirements A101 – Flare for Vine Hill Vapor Recovery System, A102 – Flare for Crude String Vapor Recovery System, A103 – Flare for Interim Gasoline Vapor Recovery System |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| SO2 | 60.104(a)(1) | Y |  |  | Condition # 20747 Part 2 | P/E | Records |
| VOC, HAP |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-501 & 12-11-505 | P/E | Flow Rate |
|  |  | N | 9/4/03 |  | BAAQMD Regulation 12-11-502.1 & 12-11-505 | P/E | Composition |
|  |  | N | 3/4/04 |  | BAAQMD Regulation 12-11-502.3 & 12-11-505 | P/E | Composition |
|  |  | N |  |  | BAAQMD Regulation 12-11-503 & 12-11-505 | P/C | Flame Detector |
|  |  | N |  |  | BAAQMD Regulation 12-11-504 & 12-11-505 | P/C | Purge Gas Flow Rate |
|  |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-507 | P/ | 1 frame per minute image video recording |

| Table VII – AO Applicable Limits and Compliance Monitoring Requirements S1471 – LOP Auxiliary Flare, S1472 – LOP Main Flare |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| VOC, HAP |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-501 & 12-11-505 | P/E | Flow Rate |
|  |  | N | 9/4/03 |  | BAAQMD Regulation 12-11-502.1 & 12-11-505 | P/E | Composition |
|  |  | N | 3/4/04 |  | BAAQMD Regulation 12-11-502.3 & 12-11-505 | P/E | Composition |
|  |  | N |  |  | BAAQMD Regulation 12-11-503 & 12-11-505 | P/C | Flame Detector |
|  |  | N |  |  | BAAQMD Regulation 12-11-504 & 12-11-505 | P/C | Purge Gas Flow Rate |
|  |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-507 | P/ | 1 frame per minute image video recording |

| Table VII – AP Applicable Limits and Compliance Monitoring Requirements S1476 – LUBS F-24 Atmospheric Feed, S1477 – LUBS F-25 Vacuum Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 3 and Part 7 | P/106 gallon of fuel combusted | Visible Inspection |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and Part 7 | P/E (during tube cleaning) | Visible emission inspection & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 3 and Part 7 | P/106 gallon of fuel combusted | Visible Inspection |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampline and Analysis |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-301 | N | 06/01/04 | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 11 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| NOx | BAAQMD Condition # 18265 Part 2 | Y |  | NOx emission rate shall not exceed 0.18 lb/MMBTU/hr for S1476 and 0.16 for S1477; unless monitored by a CEM | BAAQMD 9-10-502; Condition # 18265 Part 19 | C | Monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part C.e | Y |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E.1 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampline and Analysis |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AQ Applicable Limits and Compliance Monitoring Requirements S1478 – LUBS F-26 Furfural Raff, S1479 – LUBS F-27 Furfural Extr, S1480 – LUBS F-69 Asphalt Circulation, S1481 – OPCEN F-30 DSU, S1483 – LUBS F-32 Asphalt Circulation, S1484 – LUBS F-34 LHT Charge, S1506 – CP F-61 CGP Feed, S1760 – OPCEN F-102 FXU Steam Superheater |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and Part 7 | P/E (during tube cleaning) | Visible emission inspection & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-301 | N | 06/01/04 | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AQa Applicable Limits and Compliance Monitoring Requirements S1480 – LUBS F-69 Asphalt Circulation, S1481 – OPCEN F-30 DSU, S1484 – LUBS F-34 LHT Charge, S1506 – CP F-61 CGP Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.1 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) for refinery fuel gas | C | H2S analyzer |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.1 gr/dscf (163 ppm) | 40 CFR 60.13(i) for Sulfonation Unit SO2 Adsorber vent gas for S1480 and Lubricants Hydrotreater #1 Vacuum Flash Dryer vent gas for S1484 | P | Monitoring, records, and reporting |

| Table VII – AR Applicable Limits and Compliance Monitoring Requirements S1486 – DH F-40 CU Feed, S1487 – DH F-41B VFU Feed, S1488 – DH F-41A VFU Feed, S1491 – DH F-44 NHT Feed, S1492 – DH F-45 Primary Column Reboil, S1493 – DH F-46 Stabilizer Reboil, S1495 – DH F-49 CRU Preheat, S1496 – DH F-50 CRU, S1497 – DH F-51 CRU, S1498 – DH F-52 CRU Reboil, S1508 – CP F-63 CFH Feed, 1510 – CP F-66 CCU Preheat, S1511 – CP F-67 CCU LGO Reboil |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible Inspection |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible Inspection |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible Inspection |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-301 | N | 06/01/04 | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part C.e | Y |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E1 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AS Applicable Limits and Compliance Monitoring Requirements S1486 – DH F-40 CU Feed, S1487 – DH F-41B VFU Feed, S1488 – DH F-41A VFU Feed, S1495 – DH F-49 CRU Preheat, S1496 – DH F-50 CRU, S1497 – DH F-51 CRU, S1508 – CP F-63 CFH Feed, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AQ for additional requirements. |  |  |  |  |  |  |  |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – AT Applicable Limits and Compliance Monitoring Requirements S1490 – DH F-43 GOHT Feed, S1499 – DH F-53 CRU Regen, S1762 – DH F-128 CRU Interheater |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible emissions monitoring & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Part 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Part 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 S1762 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AU Applicable Limits and Compliance Monitoring Requirements S1494 – DH F-47 Secondary Column Reboil, S1502 – DH F-57 HCU First Stage Feed, S1503 – DH F-58 HCU Second Stage Feed, S1505 – DH F-60 HP1 Steam Methane Reformer, S1515 – DH F-71 HCU First Stage Reboil, S1761 – OPCEN F-104 HP2 Steam Methane Reformer, S4031 – DC F-14012 HGHT Reboil, S4141 – DC F-14011 HGHT Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible emissions monitoring & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AV Applicable Limits and Compliance Monitoring Requirements S1494 – DH F-47 Secondary Column Reboil, S1502 – DH F-57 HCU First Stage Feed, S1503 – DH F-58 HCU Second Stage Feed, S1505 – DH F-60 HP1 Steam Methane Reformer, S1515 – DH F-71 HCU First Stage Reboil, S1761 – OPCEN F-104 HP2 Steam Methane Reformer, |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit |  | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AU for additional requirements. |  |  |  |  |  |  |  |  |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A |  | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d |  | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – AW Applicable Limits and Compliance Monitoring Requirements S1494 – DH F-47 Secondary Column Reboil S1505 – DH F-60 HP1 Steam Methane Reformer, S1515 – DH F-71 HCU First Stage Reboil, S1761 – OPCEN F-104 HP2 Steam Methane Reformer |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type of Limit |  | Citation of Limit |  | FE Y/N |  | Future Effective Date |  | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| For S1494, See Table VII – AU & AV for additional requirements. For S1505, S1515 & S1761, See Table VII – AV for additional requirements. |  |  |  |  |  |  |  |  |  |  |  |  |
| SO2 |  | BAAQMD Condition #7618, Part E.2 |  | Y |  |  |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) |  | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – AX Applicable Limits and Compliance Monitoring Requirements S1500 – DH F-55 SGP Heat Medium, S1504 – DH F-59 HCU Second Stage Reboil, S1763 – DH F-126 CU Feed, S4002 – DC F-13425-A DCU, S4003 – DC F-13425-B DCU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible Inspection |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible emissions monitoring & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible Inspection |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted P/M | Sampling and Analysis |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N | N | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – AY Applicable Limits and Compliance Monitoring Requirements S1500 – DH F-55 SGP Heat Medium, S1504 – DH F-59 HCU Second Stage Reboil, S1763 – DH F-126 CU Feed |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit |  | Citation of Limit |  | FE Y/N |  | Future Effective Date |  | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AX for additional requirements. |  |  |  |  |  |  |  |  |  |  |  |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A |  | Y |  |  |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 |  | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d |  | Y |  |  |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day |  | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part C.e |  | Y |  |  |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) |  | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E.1 |  | Y |  |  |  | Sulfur content of liquid fuel limited to 0.5% by weight |  | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |

| Table VII – AZ Applicable Limits and Compliance Monitoring Requirements S1504 – DH F-59 HCU Second Stage Reboil, S1763 – DH F-126 CU Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AY for additional requirements. |  |  |  |  |  |  |  |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – BA Applicable Limits and Compliance Monitoring Requirements S1507 – UTIL CO Boiler 1, S1509 – UTIL CO Boiler 2, S1512 – UTIL CO Boiler 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | NSPS Subpart J 60.105(a)(1), 60.105(e)(1), 60.106(a) and 60.106(b)(4) BAAQMD 6-501, 6-502 and 1-522 | C | Opacity monitor and Records |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | NSPS Subpart J 60.105(a)(1), 60.105(e)(1), 60.106(a) and 60.106(b)(4) BAAQMD 6-501, 6-502 and 1-522 | C | Opacity monitor and Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 9 | P/A | Source Test |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| NOx | BAAQMD 9-10-303.1 | Y |  | NOx emission rate shall not exceed 300 ppm dry, at 3% O2, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-304 | N |  | NOx emissions shall not exceed 150 ppm dry at 3% O2, operating-day average or must be controlled by at least 50% | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | NOx and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | CO and O2 CEM, monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part C.e | Y |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E.1 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampline and Analysis |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #12271, Part 15 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 85 | Y |  | Combined NOx emissions shall not exceed 6,770 lb/day, averaged over any consecutive 365-day period | BAAQMD Condition #12271, Part 86, 87, 88, L and M | C | NOx and O2 CEM, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 90 | Y |  | Combined SO2 emissions shall not exceed 6,805 lb/day, averaged over any consecutive 365-day period | BAAQMD Condition #12271, Part 91, 92, 93, L & M | C | SO2 and O2 CEM, records and reporting |
| NH3 | BAAQMD Condition #17533, Part 2 | N |  | NH3 emissions shall not exceed 50 ppmv at 3% O2, 3-hour average | BAAQMD Condition #17533, Part 8 (Part 3 only if Part 2 is violated 3 times in any 12-month period) | P/A (C) | Source test (NH3 CEM) |

| Table VII – BB Applicable Limits and Compliance Monitoring Requirements S1514 – UTIL F-70 Boiler 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and Part 7 | P/E (during tube cleaning) | Visible emissions Inspection & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible emissions Inspection & Records |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504,2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-301 | N | 06/01/04 | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504,2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | NOx and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 11 through 21 | C | CO and O2 CEM, monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523; Condition # 18265 Parts 1 through 10 | C | CO and O2 CEM, monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition 7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx | BAAQMD Condition # 17532 Part 3 | Y |  | NOx < 0.05 lb/MMBTU (HHV) based on a rolling hourly 8760-hour average heat input. | BAAQMD Condition # 17532 Part 3 | P/D | Records |
| SO2 | Condition #7618, Part C.e | Y |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) | BAAQMD Condition 7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E.1 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampline and Analysis |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII - BC Applicable Limits and Compliance Monitoring Requirements S1540 – LUBS LR-26 Oleum Unloading |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | None | N | None |

| Table VII – BD Applicable Limits and Compliance Monitoring Requirements S1598 – MAINT Gasoline Dispensing Facility |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| HAP | BAAQMD Condition #14098, Part 1 | N |  | Annual gasoline throughput shall not exceed 940,000 gallons in any 12-month period | BAAQMD Condition #14098, Part 2 and Condition #18618, Part 2 | P/M | Records |

| Table VII – BE Applicable Limits and Compliance Monitoring Requirements S1650 – MAINT Sandblasting Sand hopper |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition #18618, Part 5 and 7 | PA | Visible emissions inspection and Records |
| FP | BAAQMD 6-311 | Y |  | 4.10 P0.67 lb/hr particulate, where P is process weight rate in ton/hr | BAAQMD Condition #18618, Part 5 and 7 | P/A | Visible emissions inspection and Records |

| Table VII – BF Applicable Limits and Compliance Monitoring Requirements S1759 – OPCEN Flexicoker (FXU) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AE & AF for additional requirements. |  |  |  |  |  |  |  |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.b | Y |  | If profile has more than 5 non-complying days, the refinery shall not process more than 22,000 barrels of Flexicoker feed per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part C.e | Y |  | If profile has more than 5 non-complying days, the sulfur content of liquid fuel shall be limited to 1.0 ton/day (or 1.3 ton/day if flexicoker is down) | BAAQMD Condition #7618, Part G | P/E | Records |

| Table VII – BG Applicable Limits and Compliance Monitoring Requirements S1767 – OPCENV 1019 Coke Silo, S1768 – OPCEN V1020 Coke Silo, S1769 – OPCEN V 1021 Dry Fines Silo |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 5 and 7 | P/A | Visible emissions inspection & Records |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | BAAQMD Condition # 18618 Part 5 and 7 | P/A | Visible emissions inspection & Records |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | BAAQMD Condition # 18618 Part 5 and 7 | P/A | Visible emissions inspection & Records |
| PM | BAAQMD Condition #7618 Part D.3 | Y |  | 99% control efficiency of baghouse | BAAQMD Condition # 18618 Part 5 and 7 | P/A | Visible emissions inspection & Records |

|  |  |  |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) except for gas burned as a result of process upset or gas burned at flares from relief valve leaks or other emergency malfunctions |  |  | H2S analyzer |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Table VII – BI Applicable Limits and Compliance Monitoring Requirements S1771 – OPCEN Flexigas Flare |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| VOC, HAP |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-501 & 12-11-505 | P/E | Flow Rate |
|  |  | N | 9/4/03 |  | BAAQMD Regulation 12-11-502.1 & 12-11-505 | P/E | Composition |
|  |  | N | 3/4/04 |  | BAAQMD Regulation 12-11-502.3 & 12-11-505 | P/E | Composition |
|  |  | N |  |  | BAAQMD Regulation 12-11-503 & 12-11-505 | P/C | Flame Detector |
|  |  | N |  |  | BAAQMD Regulation 12-11-504 & 12-11-505 | P/C | Purge Gas Flow Rate |
|  |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-507 | P/ | 1 frame per minute image video recording |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition #7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) except for gas burned as a result of process upset or gas burned at flares from relief valve leaks or other emergency malfunctions | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| Vent Gas Limit | BAAQMD Condition # 18618, Part 12 | Y | 06/01/04 | 750 lb/hr | BAAQMD Condition # 18618, Part 13 | P/E | Recordkeeping |

| Table VII – BJ Applicable Limits and Compliance Monitoring Requirements S1772 – OPCEN Hydrocarbon Flare |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible Emissions Check |
| VOC, HAP |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-501 & 12-11-505 | P/E | Flow Rate |
|  |  | N | 9/4/03 |  | BAAQMD Regulation 12-11-502.1 & 12-11-505 | P/E | Composition |
|  |  | N | 3/4/04 |  | BAAQMD Regulation 12-11-502.3 & 12-11-505 | P/E | Composition |
|  |  | N |  |  | BAAQMD Regulation 12-11-503 & 12-11-505 | P/C | Flame Detector |
|  |  | N |  |  | BAAQMD Regulation 12-11-504 & 12-11-505 | P/C | Purge Gas Flow Rate |
|  |  | N | 12/4/03 |  | BAAQMD Regulation 12-11-507 | P/ | 1 frame per minute image video recording |
| Vent Gas Limit | BAAQMD Condition # 18618, Part 12 | Y | 06/01/04 | 510 lb/hr | BAAQMD Condition # 18618, Part 13 | P/E | Recordkeeping |

| Table VII – BK Applicable Limits and Compliance Monitoring Requirements S1779 – OPCEN CPI Oil/Water Separator, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AJ for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #5077, Part 7 | Y |  | Throughput shall not exceed 3000 gpm or the maximum allowable capacity of the unit | BAAQMD Condition # 18618 Part 2 | P/D and A | Records |

| Table VII – BL Applicable Limits and Compliance Monitoring Requirements S1800 – UTIL F-88 Boiler 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and Part 7 | P/E (during tube cleaning) | Visible emissions Inspection & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-301 | N |  | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Part 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-301 | N | 06/01/04 | Refinery-wide NOx emission rate shall not exceed 0.033 lb/MMBtu, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523 | P | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part A | Y |  | Daily emission increases over baseline profile shall be offset by reductions below profile at a ratio of at least 2.0:1 | BAAQMD Condition 7618, Part B, F and G | P/D | Calculation, reporting and records |
| NOx, SO2, CO and PM | BAAQMD Condition #7618, Part C.d | Y |  | If profile has more than 5 non-complying days, the fuel usage at sources under the cap shall not exceed 14,730 NLFE (net liquid fuel equivalent) barrels per stream day | BAAQMD Condition #7618, Part G | P/E | Records |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| Through-put | BAAQMD Condition #16688 | N |  | 5376 MMBtu/day maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – BM Applicable Limits and Compliance Monitoring Requirements S1803 – OPCEN Coke Corral |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 4041 Part 10 and 11 | P/Q | Visible emissions inspection |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | BAAQMD Condition # 4041 Part 10 and 11 | P/Q | Visible emissions inspection |
| Opacity | BAAQMD Condition #4041, Part 3 | Y |  | Ringelmann No. 0.5 and 10% opacity | BAAQMD Condition # 4041 Part 10 and 11 | P/E | Visible emissions inspection |
| VOC | BAAQMD Condition #4041, Part 8 | Y |  | RVP of oil content of coke-oil mixture in temporary storage shall not exceed 0.5 psig | BAAQMD Condition # 4041 Part 12 | P/E | Sampling & Analysis |

| Table VII – BN Applicable Limits and Compliance Monitoring Requirements S1804 – MAINT Paint Spray Booth AND FACILITY COATING |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | None | N | None |
| VOC | BAAQMD 8-4-302 | N |  | Emissions < 4,533 Kg (5 tons)/yr or 85% mass reduction of emissions with abatement device (90% if oxidized to CO2) or VOC content < 420 g/l (3.5 lb/gal) as applied, excluding water | BAAQMD 8-4-501 | P/E – maintain current list of coatings and P/A – record quantities; P/M for 8-4-302.3 compliance | Records |
| VOC | BAAQMD 8-19-302 | Y |  | VOC content limits for coating of metal parts unless abatement device with 85% efficiency is used | BAAQMD 8-19-134 and 8-19-501 | P/W or D if abatement device is used or M if usage is less than 20 gal/yr; P/M for cleanup and surface preparation solvent usage | Records |
| VOC | BAAQMD 8-19-312 | Y |  | VOC content limits of specialty coating of metal parts | BAAQMD 8-19-501 | P/W (or D if abatement device is used) | Records |
| VOC | BAAQMD 8-31-123 | Y |  | Exemption from spray application equipment (8-31-310) provided coating of plastic parts < 50 gal/yr | BAAQMD 8-4-501 | P/A | Records |
| VOC | BAAQMD 8-31-302 | Y |  | VOC content < 340 g/L (2.8 lb/gal) coating as applied to plastic parts excluding water, unless abatement device with 85% efficiency is used | BAAQMD 8-31-124 and 8-31-501 | P/W or D if abatement device is used or M if total coating usage is less than 50 gal/yr; P/M for cleanup and surface preparation solvent usage | Records |
| VOC | BAAQMD 8-31-306 | Y |  | VOC content limits of flexible coating for plastic parts | BAAQMD 8-31-124 and 8-31-501 | P/W or D if abatement device is used or M if total coating usage is less than 50 gal/yr | Records |
| VOC | BAAQMD 8-31-309 | Y |  | VOC content limits and usage limits for specialty coatings for plastic parts | BAAQMD 8-31-124 and 8-31-501 | P/W or D if abatement device is used or M if total coating usage is less than 50 gal/yr | Records |
| VOC | BAAQMD 8-32-111 | Y |  | VOC content limits in coatings for wood products do not apply if facility coating usage < 20 gal/yr | BAAQMD 8-4-501 | P/A | Records |
| VOC | BAAQMD 8-49-301 | N |  | VOC content limits in aerosol paint products | None | N | None |
| VOC | SIP 8-49-301 | Y |  | VOC content limits in aerosol paint products | None | N | None |
| VOC | BAAQMD 8-51-114 | Y |  | VOC content limits in coatings for adhesive product substrate do not apply if facility adhesive usage < 20 gal/yr | BAAQMD 8-4-501 and 8-51-504 | P/A | Records |
| VOC | BAAQMD 8-51-115 | Y |  | Requirements of 8-51 do not apply if VOC content of adhesive or sealant product is less than 20 grams/l VOC | BAAQMD 8-4-501 and 8-51-504 | P/A | Records |
| VOC | BAAQMD 8-51-125 | Y |  | Requirements of 8-51-301.4 do not apply if facility contact bond adhesive usage < 55 gal/yr | BAAQMD 8-4-501 and 8-51-504 | P/A | Records |
| VOC | BAAQMD 8-51-126 | Y |  | Requirements of 8-51-301.4 do not apply if 80% of facility’s annual contact bond adhesive use meets definition in 8-21-249 | BAAQMD 8-4-501 and 8-51-504 | P/A | Records |
| VOC | BAAQMD Condition #4303, Part 1 | Y |  | Annual cleanup solvent usage shall not exceed 40 gallons/yr | BAAQMD Condition #4303, Part 3 | P/M | Records |
| VOC | BAAQMD Condition #4303, Part 2 | Y |  | Annual coating usage shall not exceed 150 gallons/yr | BAAQMD Condition #4303, Part 3 | P/M | Records |

| Table VII – BO Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S1805 Tank 12038 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – Y for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #4298, Part 1 | Y |  | Annual throughput shall not exceed 563,000 bbl/yr | BAAQMD Condition #4298, Part 8 | P/A | Records |

| Table VII – BP Applicable Limits and Compliance Monitoring Requirements S1900 – MAINT Machine Shop Parts Cleaner, S1902 – MAINT Seal Room Parts Cleaner, S1903 – MAINT Paint Shop Solvent Tub |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N |  | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-16-303.4.1 | N |  |  | Cold cleaner shall not operate without a freeboard ratio > 0.75 | None | P/E | Visual Inspection |
| VOC | BAAQMD 8-16-303.5 | N |  |  | VOC content < 0.42 pounds per gallon | BAAQMD 8-16-501.2 and 8-16-.501.5 | P/A | Records |
| VOC | SIP 8-16-303.4.1 | Y |  |  | Cold cleaner shall not operate without a freeboard ratio > 0.75 | None | P/E | Visual Inspection |

| - Table VII - BQ - Applicable Limits and Compliance Monitoring Requirements - S2000 – OPCEN Corrosion Inhibitor Injection |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| - NOx | - BAAQMD Condition #4364, Part 2 | - Y |  | - Corrosion inhibitor injection into the flexigas header shall not exceed 90 gallons per day | - BAAQMD Condition #4364, Part 4 | - P/D | - Records |

| - Table VII - BR - Applicable Limits and Compliance Monitoring Requirements - S2001, 2002, 2003 and 2004 – LOG Marine Loading Berths 1, 2, 3 and 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| - VOC | - BAAQMD 8-44-301 | - Y |  | - VOC emission shall not exceed 2 lb/1000 barrels loaded or reduced by at least 95% by weight | - BAAQMD 8-44-501 | - P/E | - Records |
| - VOC | - BAAQMD Condition #4288, Part 6 | - Y |  | - Vapor recovery system exhaust temperature shall not drop below 1400 degrees F for more than 15 minutes per hour | - BAAQMD Condition #4288, Part 3a | - P/E | - Monitoring |
| - VOC | - BAAQMD Condition #4288, Part 10 | - Y |  | - Loading pressures shall not exceed 80% of the lowest relief valve set pressure of the vessel being loaded | - BAAQMD Condition #4288, Part 3a | - P/E | - Monitoring |

| Table VII – BS Applicable Limits and Compliance Monitoring Requirements S2007 LOG Dissolved Nitrogen Floatation Unit North ETP 1 (DNF), S2008 LOG Dissolved Nitrogen Floatation Unit South ETP 1 (DNF), S5115 – LOG Dissolved Nitrogen Floatation Unit North ETP 2 (DNF), S5116 – LOG Dissolved Nitrogen Floatation Unit South ETP 2 (DNF) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | 8-8-307.1 |  |  | Cover standards for air flotation units. No cracks or gaps greater than 0.125 inches occur in the roof or in the roof or walls. | BAAQMD 8-8-503 | P/S | Visual inspection and records of inspections and repairs |
| HAP | NESHAP Subpart FF 61.343(a)(1)(i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.343(c), 61.350, 61.355(h), 61.356(d), 61.356(h) and 61.357(d)(8) and NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| HAP | NESHAP Subpart FF 63.649(a)(2)(ii) | Y |  | Abatement efficiency of at least 95% by weight | None | N | None |
| HAP | NESHAP Subpart FF 61.349(a)(1)(i)) | Y |  | Closed vent system leak tightness standards (< 500 ppmw - unless maintained under negative pressure) | NESHAP Subpart FF 61.355(h) and 61.356(h) | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs) |

| Table VII – BT Applicable Limits and Compliance Monitoring Requirements S2009 LUBS Wastewater Separator Dubbs Box |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AJ for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #5077, Part 10 | Y |  | Throughput shall not exceed 600 gpm or the maximum allowable capacity of the unit | BAAQMD Condition # 18618 Part 2 | P/D and A | Records |

| Table VII – BU Applicable Limits and Compliance Monitoring Requirements S2012 – DH V-12378 Perchloroethylene Storage System |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – A for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #6110, Part 1 | Y |  | Throughput shall not exceed 18,000 gallons in any 12-month period | BAAQMD Condition #6110, Part 3 | P/M | Records |

| Table VII – BV Applicable Limits and Compliance Monitoring Requirements External Floating-Roof Tanks S2013 – Tank 12467 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Tables VII – R & S for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #6503, Part 1 | Y |  | Throughput shall not exceed 27,000,000 barrels in any 12-month period | BAAQMD Condition #6503, Part 3 | P/M | Records |

| Table VII – BW Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S2445 – Tank 12445, S2446 – Tank 12446 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – P for additional requirements. |  |  |  |  |  |  |  |
| Through-put | BAAQMD Condition #6707, Part 1 | Y |  | Combined throughput shall not exceed 73,000,000 barrels in any 12-month period and combined gasoline throughput shall not exceed 1,650,000 barrels in any 12-month period | BAAQMD Condition #6707, Part 4 | P/M | Records |
| VOC | BAAQMD Condition #6707, Part 2a |  |  | For the months of November through February, Gasoline or Gasoline components not exceeding RVP 13.5; and for the remaining months only Gasoline or Gasoline components not exceeding RVP 8 | BAAQMD Condition #6707, Part 4 | P/M | Records |
| VOC | BAAQMD Condition #6707, Part 2bi | Y |  | POC emissions, based on the maximum throughput in Condition 1, do not exceed 10,779 pounds per year, as calculated by the EPA Tanks Program version 4.08 (or subsequent version) | BAAQMD Condition #6707, Part 4 | P/M | Records |
|  | BAAQMD Condition #6707, Part 2bii | N |  | Toxic emissions in lb/yr, based on the maximum throughput in Condition 1, do not exceed any risk screening trigger level specified in Table 2-1-316, except for benzene < 27 lb/yr | BAAQMD Condition #6707, Part 4 | P/M | Records |

| Table VII – BX Applicable Limits and Compliance Monitoring Requirements S3000 – Portable Vacuum Distillation Unit (CCR Technologies Inc.) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| Through-put | BAAQMD Condition #11806, Part 1 | Y |  | No more than one unit shall be operated at a time and combined operating time for all units shall not exceed 8 weeks in any 12-month period | BAAQMD Condition #11806, Part 7 | P/E | Records |

| Table VII – BY Applicable Limits and Compliance Monitoring Requirements S4002 – DC F-13425-A DCU, S4003 – DC 13425-B DCU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AW for additional requirements. |  |  |  |  |  |  |  |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) for refinery fuel gas and flexigas | C | H2S analyzer |
| SO2 | NSPS Subpart J 60.104 (a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | 40 CFR 60.13(i) for CR-2 vent gas | P | Monitoring, records, and reporting |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 35 | Y |  | NOx emissions shall not exceed 10 ppmv dry at 3% O2, 3-hour average | BAAQMD Condition #12271 Part 38, L and M | C | NOx and O2 CEM, records and reporting |
| CO | BAAQMD Condition #12271, Part 36 | Y |  | CO emissions shall not exceed 50 ppmv dry at 3% O2, 8-hour average | BAAQMD Condition # 12271 Part 105 | P/A | Source Test |
| NH3 | BAAQMD Condition #12271, Part 37 | N |  | NH3 emissions shall not exceed 20 ppmv dry at 3% O2 | BAAQMD Condition # 12271 Part 111 | P/A | Source Test |

| Table VII – BZ Applicable Limits and Compliance Monitoring Requirements S4005 – DC Coke Handling Facility |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 12271 Part 77, L and M | P/D | Water content monitoring & Records |
|  | BAAQMD Condition # 12271 Part 75 | Y |  | Ringelmann No. 0.5 and 10% opacity for no more than 3 minutes/hour | BAAQMD Condition # 12271 Part 77, L and M | P/D | Water content monitoring & Records |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain per dscf | BAAQMD Condition # 12271 Part 77, L and M | P/D | Water content monitoring & Records |
| FP | BAAQMD 6-311 | Y |  | 4.10 P 0.67 lb/hr particulate, where P is process weight rate in ton/hr | BAAQMD Condition # 12271 Part 77, L and M | P/D | Water content monitoring & Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| Opacity | BAAQMD Condition #12271, Part 77 | Y |  | Moisture content of coke, upon discharge from the crusher, shall be maintained at 8% by weight or more | BAAQMD Condition # 12271, Part 77, L and M | P/D | Water content monitoring & Records |
| PM | BAAQMD Condition #12271, Part 79 and 81 | Y |  | Coke Corral and Truck/Railcar Loading Hopper PM emissions shall not exceed 0.01 grain/dscf | BAAQMD Condition #12271, Part 75, L and M | P/D | Sampling, records and reporting |
| Through-put | BAAQMD Condition #12271, Part 83 | Y |  | Load out of coke into trucks and railcars shall be limited to 750,000 tons in any 12-month period | BAAQMD Condition #12271, Part 83, L and M | P/M | Records and reporting |

| Table VII – CA Applicable Limits and Compliance Monitoring Requirements S4001 – DC Delayed Coking Unit (DCU), S4020 – DC Distillate Hydrotreater (DHT), S4050 – DC Catalytic Gas Depentanizer (CGDP), S4080 – DC Isomerization Unit (ISOM), S4140 – DC Heavy Cracked Gasoline Hydrotreater(HGHT), S4160 – DC Hydrogen Plant 3 (HP3), S4170 – LUBS Lube Hydrotreater 2 (LHT2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AE & AF for additional requirements. |  |  |  |  |  |  |  |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAMQD Condition # 12271 Part N | Y |  | Any process depressurization shall be vented to a control device with an overall capture and destruction efficiency of 95% capture on a mass basis | BAAQMD 8-10-401 | P/E | Records |
| VOC | BAAQMD Condition #12271, Part 11 | Y |  | Fugitive emissions from Clean Fuels Project sources shall not exceed emission 19.3 tpy VOC and 14.7 NPOC | BAAQMD Condition #12271, Part 11, L and M | P/E | Calculation, records and reporting |

| Table VII – CB Applicable Limits and Compliance Monitoring Requirements S4021 – DC F-13909 DHT Recycle, S4171 – LUBS F-13000 LHT2 Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible emissions monitoring & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) | C | H2S analyzer |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 40 and 41 | Y |  | NOx emissions shall not exceed 25 ppmv dry at 3% O2, 3-hour average (for natural draft furnaces) and 20 ppmv dry at 3% O2, 3-hour average (for forced draft furnaces) | BAAQMD Condition # 12271 Part 104 | P/Semi-A for S4021; P/A for S4171 | Source Test |
| CO | BAAQMD Condition #12271, Part 42 | Y |  | CO emissions shall not exceed 50 ppmv dry at 3% O2, 8-hour average | BAAQMD Condition # 12271 Part 106 | P/A | Source Test |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – CC Applicable Limits and Compliance Monitoring Requirements S4171 – LUBS F-13000 LHT2 Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) for flexigas | C | H2S analyzer |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | 40 CFR 60.13(i) for Lubricants Hydrotreater #2 Vacuum Flash Dryer vent gas | P | Monitoring, records, and reporting |

| Table VII – CD Applicable Limits and Compliance Monitoring Requirements S4031 – DC F-14012 HGHT Reboil, S4141 – DC F-14011 HGHT Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AU for additional requirements. |  |  |  |  |  |  |  |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 35 | Y |  | NOx emissions shall not exceed 10 ppmv dry at 3% O2, 3-hour average | BAAQMD Condition #12271 Part 38, L and M | C | NOx and O2 CEM, records and reporting |
| CO | BAAQMD Condition #12271, Part 36 | Y |  | CO emissions shall not exceed 50 ppmv dry at 3% O2, 8-hour average | BAAQMD Condition # 12271 Part 105 | P/A | Source Test |
| NH3 | BAAQMD Condition #12271, Part 37 | N |  | NH3 emissions shall not exceed 20 ppmv dry at 3% O2 | BAAQMD Condition # 12271 Part 111 | P/A | Source Test |

| Table VII – CDa Applicable Limits and Compliance Monitoring Requirements S4141 – DC F-14011 HGHT Feed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AU & CD for additional requirements. |  |  |  |  |  |  |  |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) and 60.105(e)(3) for refinery fuel gas and flexigas | C | H2S analyzer |
| SO2 | NSPS Subpart J 60.104 (a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | 40 CFR 60.13(i) for CR-2 vent gas | P | Monitoring, records, and reporting |

| Table VII – CE Applicable Limits and Compliance Monitoring Requirements S4161 - DC H-101 HP3 Steam Methane Reformer |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | None | N | None |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and 7 | P/E (during tube cleaning) | Visible emissions monitoring & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | None | N | None |
| NOx | BAAQMD 9-10-303 | Y |  | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 11 through 21 | C | Monitoring, records, and reporting |
| NOx | BAAQMD 9-10-303 | Y | 06/01/04 | NOx emission rate shall not exceed 0.2 lb/MMBtu, operating-day average (facility-wide) | BAAQMD 9-10-502, 9-10-504.2, 9-10-505 and 1-523; Condition # 18265 Parts 1 through 10 | C | Monitoring, records, and reporting |
| CO | BAAQMD 9-10-305 | N |  | CO emissions shall not exceed 400 ppmv dry at 3% O2, operating-day average | BAAQMD 9-10-502, 9-10-504.2, 9-10-505, 1-522 and 1-523 | C | Monitoring, records, and reporting |
| SO2 | 40 CFR 60.104 (a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | 40 CFR 60.105(a)(4) & 60.105(e)(3) for refinery fuel gas and flexigas | C | Monitoring, records, and reporting |
| SO2 | 40 CFR 60.104 (a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | 40 CFR 60.13(i) for process swing absorption gas | P | Monitoring, records, and reporting |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 29 | Y |  | NOx emissions shall not exceed 10 ppmv dry at 3% O2, 3-hour average | BAAQMD Condition #12271, Part 32, L and M | C | NOx and O2 CEM, records and reporting |
| CO | BAAQMD Condition #12271, Part 30 | Y |  | CO emissions shall not exceed 25 ppmv dry at 3% O2, 8-hour average | BAAQMD Condition # 12271 Part 107 | P/A | Source Test |
| NH3 | BAAQMD Condition # 12271, Part 31 | N |  | Cogeneration Plant NH3 emissions shall not exceed 20 ppmv dry at 15% O2 | BAAQMD Condition # 12271 Part 112 | P/A | Source Test |
| Through-put | BAAQMD Condition #12271, Part 34 | Y |  | Combined refinery fuel gas and flexigas heat input shall not exceed 57.6% of the design duty (524 MMBtu/Hr) | BAAQMD Condition #12271, Part 20, L and M | C | Fuel flow meter, records and reporting |
| Through-put | BAAQMD Condition #16688 | N |  | Maximum firing rate (higher heating value) | BAAQMD Condition #18618, Part 2 | P/D and A | Records |

| Table VII – CF Applicable Limits and Compliance Monitoring Requirements S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AI for additional requirements. |  |  |  |  |  |  |  |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 65 | Y |  | Sulfur Plant 4 and SCOT unit shall achieve 99.9% weight conversion of reduced S compounds to elemental S | BAAQMD Condition #12271, Part 67, L and M | C | SO2 CEM & Records |
| SO2 | BAAQMD Condition #12271, Part 66 | Y |  | Concentration of total reduced S in the SCOT unit exhaust prior to the SCOT thermal Oxidizer shall not exceed 100 ppmv dry at 0 % O2, 8-hour average | BAAQMD Condition #12271, Part 67, L and M | P/D and M | Monitoring, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 68 | Y |  | (1) SCOT thermal oxidizer shall achieve a minimum conversion of 95% by weight of reduced S to SO2; (2) H2S emissions in SCOT thermal oxidizer exhaust shall not exceed 2.5 ppmv dry at 0% O2, 24-hour average; (3) SO2 emissions in SCOT thermal oxidizer exhaust shall not exceed 50 ppmv dry at 0% O2, 24-hour average (excluding contribution from the sulfur pit) | BAAQMD Condition #12271, Part 69, L and M | C for SO2 and P/A for H2S | SO2 CEM, annual H2S source test, records and reporting |
| CO | BAAQMD Condition #12271, Part 70 | Y |  | CO emissions in SCOT thermal oxidizer exhaust shall not exceed 100 ppmv dry at 0% O2, 8-hour average; | BAAQMD Condition # 12271 Part 109 | P/A | Source Test |
| NH3, H2S | BAAQMD Condition #12271, Part 64 | N |  | Sour water strippers to remove and recover at least 95% (by weight) of ammonia from the sour water feed to the unit; or reduce the concentration of ammonia in the stripped water to no more than 50 ppm | BAAQMD Condition # 12271 Part 115 | P/A | Records |
| H2S | BAAQMD Condition #12271, Part 72 | Y |  | Loading of elemental sulfur or sulfuric acid shall be controlled by scrubber with overall capture/removal efficiency of at least 95% by weight and H2S emissions from the scrubber shall not exceed 5 ppm. | BAAQMD Condition # 12271 Part 110 | P/A | Source Test |

| Table VII – CG Applicable Limits and Compliance Monitoring Requirements - S4190 – UTIL Boiler 6 Gas Turbine 1, S4192 – UTIL Boiler 6 Gas Turbine 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 3 and Part 7 | P/106 gallon of fuel combusted | Visible emissions inspection |
| FP | BAAQMD 6-310 | Y |  | 0.15 grain/dscf | BAAQMD Condition # 18618 Part 3 and Part 7 | P/106 gallon of fuel combusted | Visible emissions inspection |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| SO2 | BAAQMD 9-1-304.3 | Y |  | As an alternative to the fuel sulfur content limit in BAAQMD 9-1-304, SO2 emissions shall not exceed 300 ppm dry when burning liquid fuels | BAAQMD 9-1-502 and 1-522 | C | SO2 CEM and records |
| NOx | BAAQMD 9-9-301.3 | Y |  | 9 ppmv dry at 15% O2 for gaseous fuel firing | BAAQMD 9-9-501 and 1-522 | C | NOx and O2 CEM and records |
| NOx | BAAQMD 9-9-301.3 | Y |  | 25 ppmv dry at 15% O2 for non-gaseous fuel firing during natural gas curtailment or short testing periods | BAAQMD 9-9-501 and 1-522 | C | NOx and O2 CEM and records |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) or 60.105(a)(3) and 60.105(e)(3) | C | H2S analyzer or SO2 CEM |
| SO2 | NSPS Subpart GG 60.333(b) | Y |  | 0.8 % sulfur in fuel by weight | NSPS Subpart GG 60.334(b) | P | Fuel analysis |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 24b | Y |  | Cogeneration Plant emissions shall not exceed 610.0 lb/day NOx, 500.0 lb/day VOC, 458.0 lb/day SO2, and 365+ [(610-NOx actual)/1.9]+[(458- SO2 actual)/2.5] lb/day PM10, where NOx and SO2 are in lb/day | BAAQMD Condition #12271, Part 24a, 24b, 27, L and M | P/D | NOx, SO2 and O2 CEM, fuel usage, emission factors, records and reporting |
| NOx | BAAQMD Condition #12271, Part 24c | Y |  | Cogeneration Plant NOx emissions shall not exceed 5 ppmv dry at 15% O2, 3-hour average | BAAQMD Condition #12271, Part 27, L and M | C | NOx and O2 CEM, records and reporting |
| CO | BAAQMD Condition #12271, Part 25a | Y |  | Cogeneration Plant CO emissions shall not exceed 6.5 ppmv dry at 15% O2, 8-hour average or 90% reduction on a mass basis | BAAQMD Condition # 12271 Part 108 | P/A | Source Test |
| VOC | BAAQMD Condition #12271, Part 25b | Y |  | Cogeneration Plant VOC emissions shall not exceed 0.013 lb/MMBtu | BAAQMD Condition # 12271 Part 114 | P/A | Source Test |
| NH3 | BAAQMD Condition #12271, Part 26 | N |  | Cogeneration Plant NH3 emissions shall not exceed 20 ppmv dry at 15% O2 | BAAQMD Condition #12271, Part 113 | P/A | Source Test |

| Table VII – CH Applicable Limits and Compliance Monitoring Requirements S4191 – UTIL Boiler 6 Supplemental Steam Generator 1, S4193 – UTIL Boiler 6 Supplemental Steam Generator 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible emissions inspection |
| Opacity | BAAQMD 6-304 | Y |  | Ringelmann No. 2 for no more than 3 minutes/hour during tube cleaning | BAAQMD Condition # 18618 Part 6 and Part 7 | P/E (during tube cleaning) | Visible emission inspection & Records |
| FP | BAAQMD 6-310.3 | Y |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 3 and 7 | P/106 gallon of fuel combusted | Visible emissions inspection |
| SO2 | BAAQMD 9-1-304 | Y |  | Sulfur content of liquid fuel limited to 0.5% by weight | BAAQMD Condition # 18618 Part 4 | P/106 gallon of fuel combusted | Sampling and Analysis |
| SO2 | BAAQMD 9-1-304.3 | Y |  | As an alternative to the fuel sulfur content limit in BAAQMD 9-1-304, SO2 emissions shall not exceed 300 ppm dry when burning liquid fuels | BAAQMD 9-1-502 and 1-522 | C | SO2 CEM and records |
| Opacity | NSPS Subpart Db 60.43b(f) | Y |  | Oil-fired 20 percent opacity and 27 percent opacity for 6 minutes, except during start-up, shutdown or malfunction. | NSPS Subpart Db 60.48b Opacity CEM if firing on fuel oil and 60.49b(f), (h), (o) and (v) Records | C | CEM |
| NOx | NSPS Subpart Db 60.44b(a)(4) | Y |  | Natural gas and distillate oil: 0.20 lb NOx/MMBtu and Residual oil: 0.40 lb NOx/MMBtu | BAAQMD 1-520.1 and NSPS Subpart Db 60.49b(d), (g), (o) (v) and (w) | C | CEM |
| SO2 | NSPS Subpart J 60.104(a)(1) | Y |  | Fuel gas H2S limited to 0.10 gr/dscf (163 ppm) | NSPS Subpart J 60.105(a)(4) or 60.105(a)(3) and 60.105(e)(3) | C | H2S analyzer or SO2 CEM |
| SO2 | BAAQMD Condition #7618, Part E.2 | Y |  | Flexigas H2S shall not exceed 35 ppmv except when the refinery is processing more than 50% SJV crudes, flexigas H2S shall not exceed 80 ppmv (daily average) and 60 ppmv (annual average) | BAAQMD Condition #7618, Part G | P/E | Records |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| SO2 | BAAQMD Condition #12271, Part 15 and 18 | Y |  | Refinery fuel gas limited to 50 ppmv H2S, 24-hour average; 0.1 grain/dscf (163 ppm) H2S, 3-hour average; and 70 ppm TRS expressed as H2S, 12-month average | BAAQMD Condition #12271 Part 16, 16A, 17, L and M | C | H2S and TRS analyzer records and reporting |
| NOx | BAAQMD Condition #12271, Part 24b | Y |  | Cogeneration Plant emissions shall not exceed 610.0 lb/day NOx, 500.0 lb/day VOC, 458.0 lb/day SO2, and 365+ [(610-NOx actual)/1.9]+[(458- SO2 actual)/2.5] lb/day PM10, where NOx and SO2 are in lb/day | BAAQMD Condition #12271, Part 24a, 24b, 27, L and M | P/D | NOx, SO2 and O2 CEM, fuel usage, emission factors, records and reporting |
| NOx | BAAQMD Condition #12271, Part 24c | Y |  | Cogeneration Plant NOx emissions shall not exceed 5 ppmv dry at 15% O2, 3-hour average | BAAQMD Condition #12271, Part 27, L and M | C | NOx and O2 CEM, records and reporting |
| CO | BAAQMD Condition #12271, Part 25a | Y |  | Cogeneration Plant CO emissions shall not exceed 6.5 ppmv dry at 15% O2, 8-hour average or 90% reduction on a mass basis | BAAQMD Condition # 12271 Part 108 | P/A | Source Test |
| VOC | BAAQMD Condition #12271, Part 25b | Y |  | Cogeneration Plant VOC emissions shall not exceed 0.013 lb/MMBtu | BAAQMD Condition # 12271 Part 114 | P/A | Source Test |
| NH3 | BAAQMD Condition #12271, Part 26 | N |  | Cogeneration Plant NH3 emissions shall not exceed 20 ppmv dry at 15% O2 | BAAQMD Condition # 12271 Part 113 | P/A | Source Test |

| Table VII – CI Applicable Limits and Compliance Monitoring Requirements S4201 – DC Clean Fuels Flare |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N |  |  | Future Effective Date | Limit |  | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – AN for additional requirements. |  |  |  |  |  |  |  |  |  |  |
| Opacity | BAAQMD 6-301 |  | Y |  |  |  | Ringelmann No. 1 for no more than 3 minutes/hour | BAAQMD Condition # 18618 Part 15 | P/E | Visible emissions inspection |
| FP | BAAQMD 6-310 |  | Y |  |  |  | 0.15 grain per dscf at 6% O2 | BAAQMD Condition # 18618 Part 15 | P/E | Visible emissions inspection |
| VOC, HAP |  |  | N | 12/4/03 |  |  |  | BAAQMD Regulation 12-11-501 & 12-11-505 | P/E | Flow Rate |
|  |  |  | N | 9/4/03 |  |  |  | BAAQMD Regulation 12-11-502.1 & 12-11-505 | P/E | Composition |
|  |  |  | N | 3/4/04 |  |  |  | BAAQMD Regulation 12-11-502.3 & 12-11-505 | P/E | Composition |
|  |  |  | N |  |  |  |  | BAAQMD Regulation 12-11-503 & 12-11-505 | P/C | Flame Detector |
|  |  |  | N |  |  |  |  | BAAQMD Regulation 12-11-504 & 12-11-505 | P/C | Purge Gas Flow Rate |
|  |  |  | N | 12/4/03 |  |  |  | BAAQMD Regulation 12-11-507 | P/ | 1 frame per minute image video recording |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A |  | Y |  |  |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 61 |  | Y |  |  |  | Flare shall have a VOC destruction efficiency of at least 98.5% by weight | None | N | None |
| Vent Gas Limit | BAAQMD Condition # 18618, Part 12 |  | Y | 06/01/04 |  |  | 2,000 lb/hr | BAAQMD Condition # 18618, Part 13 | P/E | Recordkeeping |

| Table VII – CJ Applicable Limits and Compliance Monitoring Requirements S4210 – Cooling Water Tower (CWT-13278) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition 12271, Part 55 | Y |  | Hydrocarbon reading above defined Action Level will trigger an alarm. | BAAQMD Condition # 12271 Part L, M and55 | P/C | District-approved continuous hydrocarbon analyzer /recorder to determine hydrocarbon vapor concentra-tion fromcooling water |

| Table VII – CK Applicable Limits and Compliance Monitoring Requirements S4211 – DC V-13222 ISOM Maintenance Drop Out Vessel, S4212 – DC V-13441 ISOM Maintenance Drop Out Vessel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAMQ 8-10-301 | Y |  | Abatement of emissions from process vessel depressurization is required until pressure is reduced to less than 1000 mm Hg | BAAQMD 8-10-401 | P/E | Records |
| HAP | NESHAP Subpart FF 61.343(a)(1)(i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentration shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.343(c), 61.350, 61.355(h) 61.356(d), 61.356(h) and 61.357(d)(8) NESHAP Subpart CC 63.642(e), 63.642(f) and 63.642(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAMQD Condition # 12271 Part N | Y |  | Any process depressurization shall be vented to a control device with an overall capture and destruction efficiency of 95% capture on a mass basis | BAAQMD 8-10-401 | P/E | Records |
| VOC | BAAQMD Condition #12271, Part 52 | Y |  | MDO vessels shall be vapor tight (100 ppm, as methane) and vented to FGR or an incinerator with at least 98.5 % destruction efficiency by weight | None | N | None |

| Table VII – CL Applicable Limits and Compliance Monitoring Requirements S1802 - LOGV-1533 Odorant Storage Tank S4311 – DC V-12555 ISOM Perchloroethylene Vessel , S4329 – Tank 13260 Pentane, S4330 – Tank 13261 Pentane, S4349 - Tank 13262 Pentane |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-10-301 | Y |  | Abatement of emissions from process vessel depressurization is required until pressure is reduced to less than 1000 mm Hg | BAAQMD 8-10-401 | P/E | Records |

| Table VII – CM Applicable Limits and Compliance Monitoring Requirements 4311 – DC V-12555 ISOM Perchloroethylene Vessel , S4329 – Tank 13260 Pentane, S4330 – Tank 13261 Pentane, S4349 - Tank 13262 Pentane |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAMQD Condition # 12271 Part N | Y |  | Any process depressurization shall be vented to a control device with an overall capture and destruction efficiency of 95% capture on a mass basis | BAAQMD 8-10-401 | P/E | Records |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502 and 8-5-603.2 | P/A | Source Test |

| Table VII – CO Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S4319 – Tank 15096 Recovered Oil, S4350 – LOG Tank 13187 Process Wastewater, S4356 – LOG Tank 13188 Process Wastewater |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |
| VOC | NSPS Subpart Kb 60.112b (a)(3)(i) | Y |  | Closed vent system leak tightness standards, VOC concentrations shall not exceed 500 ppmv above background. | 40 CFR 63.640(d)(5)) | N | None |
| VOC | NSPS Subpart Kb 60.112b (a)(3)(ii) | Y |  | Control device standards; includes 95% efficiency requirement | 40 CFR 63.640(d)(5) | N | None |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 45 | Y |  | Tank shall be kept leak free, less than 500 ppm above background as methane | NESHAP Subpart FF 61.350, 61.356(k) and 61.357(d)(8) and NESHAP Subpart CC 63.642(e), 63.642(f) and 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502, and 8-5-603.2 | P/A | Source Test |
| HAP | NESHAP Subpart FF 61.343(a)(1) (i)(A) and NESHAP Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background | NESHAP Subpart FF 61.343(c), 61.350, 61.355(h), 61.356(d), 61.356(h) and 61.357(d)(8) and NESHAP Subpart CC 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| HAP | NESHAP Subpart FF 61.349(a)(1)(i)) | Y |  | Closed vent system leak tightness standards (< 500 ppmw - unless maintained under negative pressure) | NESHAP Subpart FF 61.355(h) and 61.356(h) and Subpart CC 63.640(d)(5) ) | P/A | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs) |
| HAP | NESHAP Subpart FF 63.649(a)(2)(ii) | Y |  | Abatement efficiency of at least 95% by weight | None | N | None |

| Table VII – CQ Applicable Limits and Compliance Monitoring Requirements Internal Floating-Roof Tanks S4322 – Tank 14571 Sour Water |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – P for additional requirements. |  |  |  |  |  |  |  |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502, and 8-5-603.2 | P/A | Source Test |

| Table VII – CR Applicable Limits and Compliance Monitoring Requirements Closed Vent Systems & Control Devices S4334 – Tank 13276 Alkylate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – X for additional requirements. |  |  |  |  |  |  |  |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 45 | Y |  | Tank shall be kept leak free, less than 500 ppm above background as methane | NESHAP Subpart FF 61.350, 61.356(k) and 61.357(d)(8) and NESHAP Subpart CC 63.642(e), 63.642(f) and 63.654(i)(4) | P/Q-visual and A-measurements and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |
| VOC | BAAQMD Condition #12271, Part 51 | Y |  | Tank cleaning VOC emissions shall be vented to a control device with 95% capture and destruction efficiency on a mass basis until concentration of organic compounds in tank is less than 10,000 ppm as methane | BAAQMD Regulation 8-5-404, 8-5-502, and 8-5-603.2 | P/A | Source Test |

| Table VII – CS Applicable Limits and Compliance Monitoring Requirements S4338- LOG Pentane Loading Facility |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| NOx, CO, SO2, PM and VOC | BAAQMD Condition #12271, Part A | Y |  | Emissions from Clean Fuels Project sources shall not exceed 199.4 tpy NOx, 252.4 tpy CO, 139.7 tpy SO2, 59.1 tpy PM, 132.0 tpy VOC, 20.8 ton/mon NOx, 26.3 ton/mon CO, 14.6 ton/mon SO2, 6.2 ton/mon PM and 13.7 ton/mon VOC | BAAQMD Condition #12271, Part A, B, C, L and M | P/M and A | Calculation, records and reporting |
| VOC | BAAQMD Condition #12271, Part 74 | Y |  | Emissions from loading operation shall be controlled with an overall capture and destruction efficiency at least 98.5% by weight | None | N | None |

| Table VII – CT Applicable Limits and Compliance Monitoring Requirements S5112 – LOG V-15112 Main Proto Vessel with Nitrogen Banket, S5113 – LOG V-15113 Main Proto Vesselwith Nitrogen Blanket, S5114 – LOG V-15114 Main Proto Vessel with Nitrogen Blanket, S5125 – LOG V-15117 Standby Proto Vessel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control by liquid balanceing in which the resulting organic liquid has a TVP is less than 0.5 psia | BAAQMD 8-5-501 | P/E | Records |
| VOC | BAAQMD 8-5-328.1 | Y |  | Tank cleaning control device standards; includes 90% efficiency requirement | BAAQMD 8-5-502 and 8-5-603.2 | P/A | Annual source test using MOP, Vol. IV, ST-7 |

| Table VII - CTa S5140 – Diesel Engine, S6051 through S6060 Diesel Engines |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| Opacity | BAAQMD 6-301 | Y |  | Ringelmann 1.0 for < 3 minutes in any hour | Condition # 19097 Part 7 | P/1000 gal fuel oil | Visible Emissions Check |
| FP | BAAQMD 6-310 | Y |  | 0.15 gr/dscf | Condition # 19097 Part 7 | P/1000 gal fuel oil | Visible Emissions Check |
| SO2 | BAAQMD 9-1-301 | Y |  | GLC1 of 0.5 ppm for 3 min or 0.25 ppm for 60 min or 0.05 ppm for 24 hours |  | N |  |
|  | BAAQMD 9-1-302 | Y |  | SO2 shall not exceed 300 ppm (dry) |  | N |  |
|  | BAAQMD 9-1-304 | Y |  | Sulfur Content of Fuel Oil < 0.5 wt% | Condition # 19097 Part 6 | P/E | Fuel Oil Certification |
| Emer-gency | BAAQMD 9-8-330.1 & Condition # 19097 Part 1 | N |  |  | BAAMQD 9-8-530 & Condition # 19097 Part 5 | P/M | Records |
| Reli-ability Related Activities | BAAQMD 9-8-330.2 & Condition # 19097 Part 1 | N |  | Hours of Reliability Related Activities < 100/yr | BAAMQD 9-8-530 & Condition # 19097 Part 5 | P/M | Records |

| Table VII – CTb Applicable Limits and Compliance Monitoring Requirements S17095 – Tank 17095 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| See Table VII – R for additional requirements. |  |  |  |  |  |  |  |
| VOC | BAAQMD Condition # 20042 Part 1 | Y |  | Throughput < 27,000,000 bbl/yr | BAAQMD Condition # 20042 Part 3 | P/M | Records |

| Table VII – CU Applicable Limits and Compliance Monitoring Requirements - SUBPART GGG Equipment and Components |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| - VOC | - NSPS Subpart VV 60.482-2 (b)(1) | - Y |  | - Light liquid service pump leak < 10,000 ppm | - NSPS Subpart VV 60.482-2 (a)(1), (c), 60.482-9(a), (b), (d) 60.485(a), (b) 60.486(a), (b), (c), (e) and 60.487(a) and (c) | - P/M | - Measure for leaks and repair |
| - VOC | - NSPS Subpart VV 60.482-3 - (e)(2) and (f) | - Y |  | - Compressor sensor shall detect failure of seal system, barrier fluid system, or both based on criterion established in 60.482-3(e)(2). | - NSPS Subpart VV 60.482-3 (e)(1), (g), 60.482-9(a), (b), 60.486(a), (b), (c), (e) (h), and 60.487(a) and (c) | - P/C or P/D | - Sensor with audible alarm or checked daily. Repair system. |
| - VOC | - NSPS Subpart VV 60.482-4(a) | - Y |  | - Except during pressure release, pressure relief device (gas/vapor service) must operate at no detectable emissions (<500 ppm) | - NSPS Subpart VV 60.482-4 (b)(2), 60.482-9(a), (b), 60.485(a), (b), 60.486(a), (e) and 60.487(a) and (c) | - P/E | - Measure for leaks within 5 days after release using Method 21 |
| - VOC | - NSPS Subpart VV 60.482-4 - (b)(1) | - Y |  | - After each pressure release, pressure release device shall be returned to a condition of no detectable emissions (<500 ppm) within 5 calendar days after pressure release | - NSPS Subpart VV 60.482-4 (b)(2), 60.482-9(a), (b), 60.485(a), (b), 60.486(a), (e) and 60.487(a) and (c) | - P/E | - Measure for leaks within 5 days after release using Method 21 |
| - VOC | - NSPS Subpart VV 60.482-7(b) | - Y |  | - Valve leak > 10,000 ppm | - NSPS Subpart VV 60.482-7(a), (c), (d), (e), 60.482-9(a), (b), (c), (e), 60.483-2, 60.485 (a),(b), 60.486 (a), (b), (c), (e), (f) and 60.487(a) and (c) | - P/M or Q | - Measure for leaks and repair |
| - VOC | - NSPS Subpart VV 60.482-8(b) | - Y |  | - Pump and valves in heavy liquid service, pressure relief devices, flanges and other connector leak > 10,000 ppm | - NSPS Subpart VV 60.482-8(a), (c), (d), 60.482-9(a), (b), (c), (d), (e), 60.483-2, 60.485(a), (b), 60.486(a), (b), (c), (e) and 60.487(a) and (c) | - P/5 days if evidence of potential leak | - Visual, audible, olfactory Inspection; Measure for leaks |
| - VOC | - NSPS Subpart VV 60.483-2(a) | - Y |  | - Alternate standards for valves - allowable percentage of valves leaking is <2% or revert to standards in 60.482-7. After 2 consecutive quarterly leak detection periods within limits, allowed to skip 1 of quarterly detection period monitoring. After 5 consecutive quarters with <2% valves leaking may monitor annually. | - NSPS Subpart VV 60.483-2(b), 60.485(a), (b), (d), (e), (f), - 60.486 (a), (b), (c), (g), (j) and 60.487(a) and (c) | - P/Q and A | - Measure for leaks |

| Table VII – CV Applicable Limits and Compliance Monitoring Requirements - SUBPART CC Miscellaneous Process Vents |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| - HAP | - MACT Subpart CC 63.643(a)(2) | - Y |  | - 98% destruction efficiency of control device or 20ppmv at 3% O2 | - MACT Subpart CC 63.644(a)(3) | - N | - N |

| Table VII – CW Applicable Limits and Compliance Monitoring Requirements - SUBPART CC Equipment Leaks for equipment in Organic HAP Service |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| - VOC | - NSPS Subpart VV 60.482-2 (b)(1) | - Y |  | - Light liquid service pump leak < 10,000 ppm | - NSPS Subpart VV 60.482-2 (a)(1), (c), 60.482-9(a), (b), (d) 60.485(a), (b) 60.486(a), (b), (c), (e) and 60.487(a) and (c) MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/M | - Measure for leaks and repair |
| - VOC | - NSPS Subpart VV 60.482-4(a) | - Y |  | - Except during pressure release, pressure relief device (gas/vapor service) must operate at no detectable emissions (<500 ppm) | - NSPS Subpart VV 60.482-4 (b)(2), 60.482-9(a), (b), 60.485(a), (b), 60.486(a), (e) and 60.487(a) and (c) MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/E | - Measure for leaks within 5 days after release using Method 21 |
| - VOC | - NSPS Subpart VV 60.482-4 - (b)(1) | - Y |  | - After each pressure release, pressure release device shall be returned to a condition of no detectable emissions (<500 ppm) within 5 calendar days after pressure release | - NSPS Subpart VV 60.482-4 (b)(2), 60.482-9(a), (b), 60.485(a), (b), 60.486(a), (e) and 60.487(a) and (c) MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/E | - Measure for leaks within 5 days after release using Method 21 |
| - VOC | - NSPS Subpart VV 60.482-7(b) | - Y |  | - Valve leak > 10,000 ppm | - NSPS Subpart VV 60.482-7(a), (c), (d), (e), 60.482-9(a), (b), (c), (e), 60.483-2, 60.485 (a), (b), - 60.486 (a), (b), (c), (e), (f) and 60.487(a) and (c) MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/M or Q | - Measure for leaks and repair |
| - VOC | - NSPS Subpart VV 60.482-8(b) | - Y |  | - Pump and valves in heavy liquid service, pressure relief devices, flanges and other connector leak > 10,000 ppm | - NSPS Subpart VV 60.482-8(a), (c), (d), 60.482-9(a), (b), (c), (d), (e), 60.483-2, 60.485(a), (b), 60.486(a), (b), (c), (e) and 60.487(a) and (c) MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/5 days if evidence of potential leak | - Visual, audible, olfactory Inspection; Measure for leaks |
| - VOC | - NSPS Subpart VV 60.483-2(a) | - Y |  | - Alternate standards for valves - allowable percentage of valves leaking is <2% or revert to standards in 60.482-7. After 2 consecutive quarterly leak detection periods within limits, allowed to skip 1 of quarterly detection period monitoring. After 5 consecutive quarters with <2% valves leaking may monitor annually. | - NSPS Subpart VV 60.483-2(b), 60.485(a), (b), (d), (e), (f), - 60.486(a), (b), (c), (g), (j) and 60.487(a) and (c) - MACT Subpart CC 63.648(h), 63.654(d)(1) - (i), 63.654(e)(3), 63.654(h)(1) and 63.654(i)(4) | - P/Q and A | - Measure for leaks |

| Table VII – CX Applicable Limits and Compliance Monitoring Requirements Subpart FF containers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| HAP | NESHAP Subpart FF 61.345(a)(1)(i) and MACT Subpart CC 63.647(a) | Y |  | VOC concentrations shall not exceed 500 ppmv above background from containers used to handle, transfer or store waste. | NESHAP Subpart FF 61.345(a)(1)(i), 61.345(c), 61.350, 61.355(h), 61.356(a), (h), and 61.357(d)(6) and (8) and MACT Subpart CC 63.654(a) and (i)(4) | P/A-measure-ments and reports | Visual inspections, portable HC detector (EPA Method 21) and records of detectable emissions, inspections and repairs |

| Table VII – CY Applicable Limits and Compliance Monitoring Requirements Facility |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of Limit | Citation of Limit | FE Y/N | Future Effective Date | Limit | Monitoring Requirement Citation | Monitoring Frequency (P/C/N) | Monitoring Type |
| SO2 | BAAQMD 9-1-301 | Y |  | Ground level concentrations < 0.5 ppm continuously for 3 consecutive minutes or < 0.25 ppm averaged over 60 minutes or < 0.05 ppm averaged over 24 hours | BAAQMD 9-1-501, 1-510, 1-530, 1-540, 1-542 1-543 and 1-544 | C | Area Monitoring |
| H2S/NH3 | BAAQMD 9-1-313.2 | N |  | Operation of a sulfur removal and recovery system that removes and recovers: 95% of H2S from refinery fuel gas, 95% of H2S and ammonia from process water streams | BAAQMD Condition # 12271 Part 115 | P/A | Analysis and Recordkeeping |
| H2S | BAAQMD 9-2-301 | N |  | Ground level concentrations < 0.06 ppm averaged over 3 consecutive minutes or < 0.03 ppm averaged over any 60 consecutive minutes | BAAQMD 9-2-501, 1-510, 1-530 1-540, 1-542, 1-543 and 1-544 | C | Area Monitoring |
| VOC | BAAQMD 8-15-305 | Y |  | Manufacture and sale of liquid asphalt or emulsified asphalt that meet standard specifications | BAAQMD 8-15-501 | P/E | ASTM distillation methods D402 (liquid) or D244 (emulsified) and records |
| VOC | BAAQMD 8-16-118 | N |  | Limited exemption for solvent cleaners using compounds with low volatility (IBP>128 C and storage temperature and operating T > IBP + 100C) | BAAQMD 8-16-501.2 and 8-16-501.5 | P/A | Records |
| VOC | BAAQMD 8-16-303.4.1 | N |  | Freeboard ratio > 0.75 | None | P/E | Visual Inspection |
| VOC | BAAQMD 8-16-303.5 | N |  | VOC content < 0.42 pounds per gallon | BAAQMD 8-16-501.2 and 8-16-.501.5 | P/A | Records |
| VOC | SIP 8-16-303.4.1 | Y |  | Freeboard ratio > 0.75 | None | P/E | Visual Inspection |
| VOC | BAAQMD 8-40-116.1 | Y |  | Exempt from requirements of 8-40 if total volume of contaminated soil (50 ppmv) < 1 cubic yard | BAAQMD 8-40-604 | P/E | Sampling and analysis with EPA Method 21, Section 3 |
| VOC | BAAQMD 8-40-116.2 | Y |  | Exempt from requirements of 8-40 if total volume of contaminated soil (50 ppmv) < 8 cubic yards and < 500 ppmw (provided exemption is not used more than once per 3 months) | BAAQMD 8-40-601.2 and 8-40-604 | P/E | Sampling and analysis with EPA Method 21, Section 3 |
| VOC | BAAQMD 8-40-306.4 | Y |  | Within 45 days of excavation or within 90 days for organic content < 500 ppmw, cover, remove or treat soil | BAAQMD 8-40-601.3 or 8-40-601.4 and 8-40-604 | P/E | Sampling and analysis with EPA Method 21, Section 3 |
| Through-put | BAAQMD Condition #12271, Part 50 | Y |  | Total volume of isom feed in all Clean Fuels Project storage tanks shall not exceed 100,000 barrels at any given time | BAAQMD Condition #12271, Part 50, L and M | P/D and M | Records and reporting |
| HAP | NESHAP Subpart FF 61.342(e)(2) and MACT Subpart CC 63.654(a) | Y |  | National Emission Standard for Benzene Waste Operations, Manage and treat “aqueous” wastes such the “aqueous” wastes managed in uncontrolled systems do not exceed 6 Mg/hr (6.6 tons/yr) of benzene | NESHAP Subpart FF 61.355(a), (b), (c), (k), 61.356(a), and (b), 61.357(a), (d)(2) and 61.357(d)(5) and MACT Subpart CC 63.654(i)(4) | P/A | Records and results from tests and inspections using methods and procedures specified in 61.355(k) |
| True Vapor Pressure |  |  |  |  | BAAQMD Condition # 20762 Part 1 | P/E | Sampling and analysis with EPA Method 28 or Table 1 of of Regulation 8-5 |

# Viii.	TEST METHODS

The test methods associated with the emission limit of a District regulation are generally referenced in Section 600 et seq. of the regulation.  The following table indicates only the test methods associated with the emission limits referenced in Section VII, Applicable Emission Limits & Compliance Monitoring Requirements, of this permit.

| Table VIII Test Methods |  |  |
| --- | --- | --- |
| Applicable Requirement | Description of Requirement | Acceptable Test Methods |
| 6-301 | Ringelmann No. 1 Limitation | Manual of Procedures, Volume I, Evaluation of Visible Emissions; EPA Method 9 |
| 6-304 | Tube Cleaning | Manual of Procedures, Volume I, Evaluation of Visible Emissions |
| 6-310 | Particulate Weight Limitation | Manual of Procedures, Volume IV, ST-15 Particulate Sampling; or USEPA Method 5, Determination of Particulate Matter Emissions from Stationary Sources |
| BAAQMD 8-5-601 | Reid Vapor Pressure | Manual of Procedures, Volumne III, Lab Method 13, Determination of the Reid Vapor Pressure of Petroleum Products |
| BAAQMD 8-5-602 | True Vapor Pressure | Manual of Procedures, Volumne III, Lab Method 28, Determination of Vapor Pressure of Organic Liquids from Storage Tanks |
| BAAQMD 8-7-301 | Phase I Vapor Recovery Requirements | Manual of Procedures, Volume IV, ST-30, Gasoline Vapor Recovery Leak Test Procedure; and ST-36, Gasoline Dispensing Facility Phase I Volumetric Efficiency |
| BAAQMD 8-7-302 | Phase II Vapor Recovery Requirements | Manual of Procedures, Volume IV, ST-30, Vapor Tightness; ST-37, Liquid Removal; and ST-41, Liquid Retain and Spitting from Nozzles |
| BAAQMD Regulation 8-3-302 | VOC Limits | Manual of Procedures, Volume III, Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings or Manual of Procedures, Volume III, Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| BAAQMD Regulation 8-3-304 | VOC Limits | Manual of Procedures, Volume III, Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings or Manual of Procedures, Volume III, Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| BAAQMD 8-4-302 | Solvent and Surface Coating Requirements, VOC Emissions | Manual of Procedures, Volume IV, ST-7, Organic Compounds; or EPA Method 25, Determination of Total Gaseous Nonmethane Organic Emissions as Carbon; or EPA Method 25A, Determination of Total Gaseous Nonmethane Organic Emissions Using a Flame Ionization Analyzer |
| BAAQMD 8-4-302.3 | Surface Coating, VOC Content | Manual of Procedures, Volume III; Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings; or Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| SIP 8-4-302 | Solvent and Surface Coating Requirements, VOC Emissions | Manual of Procedures, Volume IV, ST-7, Organic Compounds; or EPA Method 25, Determination of Total Gaseous Nonmethane Organic Emissions as Carbon; or EPA Method 25A, Determination of Total Gaseous Nonmethane Organic Emissions Using a Flame Ionization Analyzer |
| 8-8-504 | Portable Hydrocarbon Detector | A gas detector that meets the specifications and performance criteria of and has been calibrated in accordance with EPA Reference Method 21 (40 CFR 60, Appendix A) |
| 8-8-601 | Wastewater Analysis for Critical OCs | Samples of wastewater shall be taken at the influent stream for each unit and analyzed for the concentration of dissolved critical organic compounds as prescribed in the District's Manual of Procedures, Volume III, Lab Method 33. |
| 8-8-602 | Determination of Emissions | Emissions of POCs, as specified in Sections 8-8-301.3, 8-8-302.3, 8-8-304, 8-8-305.2, 8-8-306.2, and 8-8-307.2 shall be measured by as prescribed by any of the following methods: 1). BAAQMD MOP, Volume IV, ST-7 or; 2). EPA Method 25 or 25(A). |
| 8-8-603 | Inspection Procedures | For the purposes of 8-8-301, 302, 303, and 304, leaks shall be measured using a portable gas detector as prescribed in EPA Reference Method 21 (40 CFR 60, Appendix A) |
| BAAQMD 8-19-302, 312 | Surface Coating, VOC Content | Manual of Procedures, Volume III; Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings; or Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| BAAQMD 8-19-302, 312, 313 | Determination of VOC Emissions | Manual of Procedures, Volume IV, ST-7, Organic Compounds; or EPA Method 25, Determination of Total Gaseous Nonmethane Organic Emissions as Carbon; or EPA Method 25A, Determination of Total Gaseous Nonmethane Organic Emissions Using a Flame Ionization Analyzer |
| BAAQMD 8-31-302, 306, 309 | Surface Coating, VOC Content | Manual of Procedures, Volume III; Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings; or Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| BAAQMD 8-31-302, 306, 309, 310 | Determination of VOC Emissions | Manual of Procedures, Volume IV, ST-7, Organic Compounds; or EPA Method 25, Determination of Total Gaseous Nonmethane Organic Emissions as Carbon; or EPA Method 25A, Determination of Total Gaseous Nonmethane Organic Emissions Using a Flame Ionization Analyzer |
| BAAQMD 8-32-302 | Surface Coating, VOC Content | Manual of Procedures, Volume III; Method 21, Determination of Compliance of Volatile Organic Compounds for Water Reducible Coatings; or Method 22, Determination of Compliance of Volatile Organic Compounds for Solvent Based Coatings |
| BAAQMD 8-32-302 | Determination of VOC Emissions | Manual of Procedures, Volume IV, ST-7, Organic Compounds; or EPA Method 25, Determination of Total Gaseous Nonmethane Organic Emissions as Carbon; or EPA Method 25A, Determination of Total Gaseous Nonmethane Organic Emissions Using a Flame Ionization Analyzer |
| BAAQMD 8-44-301 | POC emission rate limitation during vessel loading | ST-34, Bulk and Marine Loading Terminals, Vapor Recovery Units |
| BAAQMD 8-44-304.1 | Tank vessel is leak free and gas tight | EPA Method 21 |
| 9-1-304 | Fuel Burning (Liquid and Solid Fuels) | Manual of Procedures, Volume III, Method 10, Determination of Sulfur in Fuel Oils. |
| 9-1-301, 9-2-301 | Ground Level Monitoring | Manual of Procedures, Volume VI, Section 1, Area Monitoring |
| 9-1-501, 9-1-502, 9-2-501 | Continuous Monitoring | Manual of Procedures, Volume 5, Continuous Monitoring |
| BAAQMD 9-8-301.1 | NOx Limits for Rich Burn Engines Burning Exclusively Fossil Fuel Derived Fuel Gas | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-8-301.2 | NOx Limits for Lean Burn Engines Burning Exclusively Fossil Fuel Derived Fuel Gas | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-8-301.3 | CO Limits for Engines Burning Exclusively Fossil Fuel Derived Fuel Gas | Manual of Procedures, Volume IV, ST-6, Carbon Monoxide, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-9-301.1 | Emission Limits- Turbines Rated < 10 MW | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-9-301.2 | Emission Limits- Turbines Rated > 10 MW without SCR | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-9-301.3 | Emission Limits- Turbines Rated > 10 MW with SCR | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| BAAQMD 9-9-503.2 | Deadline for Demonstration of Compliance with §9-9-301 | Manual of Procedures, Volume IV, ST-13A, Oxides of Nitrogen, Continuous Sampling and ST-14, Oxygen, Continuous Sampling |
| 40 CFR 60, Appendix A | Inspection Procedures | EPA Reference Method 21 |
| 40 CFR Subpart A 60.18(c)(1) | Visibile emission monitoring | EPA Method 22: Visible Emissions |
| 40 CFR Subpart J 60.106(e) | H2S concentration monitoring | EPA Method 11: H2S |
| 40 CFR Subpart J 60.106(f)(3) | H2S concentration monitoring | EPA Method 3: O2 |
| 40 CFR Subpart J 60.106(f)(1) | SO2 concentration monitoring | EPA Method 6: SO2 |
| 40 CFR Subpart J 60.106(f)(2) | TRS concentration monitoring | EPA Method 15: Total Reduced Sulfur |
| 40 CFR, Subpart CC | Test methods, procedures | EPA reference method 21 (40 CFR 60, Appendix A), Determination of Volatile Organic Compound Leaks |
| 40 CFR Subpart GG 60.333 (a) | SO2 Volumetric Emission Limit | EPA Method 20, Determination of Nitrogen Oxides, Sulfur Dioxide, and Diluent Emissions from Stationary Gas Turbines |
| - 40 CFR - Subpart FF - 61.349 (a)(1)(i) | - Leak inspection procedures | - 61 Subpart FF, 61.355(h): - EPA reference method 21 (40 CFR 60, Appendix A), Determination of Volatile Organic Compound Leaks |
| - 40 CFR - Subpart FF - 61.354 (f) | - Visual Inspection | - 61 Subpart FF, 61.354(f) |
| 40 CFR Subpart GG 60.332 (a)(1) | Performance Standard, NOx | EPA Method 20, Determination of Nitrogen Oxides, Sulfur Dioxide, and Diluent Emissions from Stationary Gas Turbines |
| 40 CFR Subpart GG 60.332 (a)(2) | Performance Standard, NOx | EPA Method 20, Determination of Nitrogen Oxides, Sulfur Dioxide, and Diluent Emissions from Stationary Gas Turbines |
| 40 CFR Subpart GG 60.333 (b) | Fuel Sulfur Limit (gaseous fuel) | ASTM D 1072-80, Standard Method for Total Sulfur in Fuel Gases ASTM D 3031-81, Standard Test Method for Total Sulfur in Natural Gas by Hydrogenation ASTM D 4084-82, Standard Method for Analysis of Hydrogen Sulfide in Gaseous Fuels (Lead Acetate Reaction Rate Method), ASTM D 3246-81, Standard Method for Sulfur in Petroleum Gas by Oxidative Microcoulometry |
| 40 CFR Subpart GG 60.333 (b) | Fuel Sulfur Limit (liquid fuel) | ASTM D 2880-71, Standard Specification for Gas Turbine Fuel Oils |
| 40 CFR, Subpart VV, 63.1046 | Test methods, procedures | Method 21of 40 CFR part 60, appendix A. Acceptable floating roof seal gap criteria included. |
| 40 CFR, Subpart QQQ, 60.696 | Performance test methods and procedures and compliance provisions | Sources equipped with a closed-vent system and control device shall use EPA Method 21 to measure the emission concentrations, using 500 ppm as the no detectable emission limit. Acceptable seal gap criteria also included. |
| - 40 CFR, Subpart QQQ | - Leak inspection procedures | - 60 Subpart QQQ, 60.696: - EPA reference method 21 (40 CFR 60, Appendix A), Determination of Volatile Organic Compound Leaks |

# Ix.	PERMIT SHIELD

## Non-applicable Requirements

Pursuant to District Regulations 2-6-233 and 2-6-409.12, the federally enforceable regulations and/or standards cited in the following table[s] do not apply to the source or group of sources identified at the top of the table[s].  Enforcement actions and litigation may not be initiated against the source or group of sources covered by this shield based on the regulatory and/or statutory provisions cited, as long as the reasons listed below remain valid for the source or group of sources covered by this shield.

| Table IX A - 1 Permit Shield for Non-applicable Requirements S1431 –CP Sulfur Plant 1 (SRU1) S1432 – CP Sulfur Plant 2 (SRU2) S1765 –OPCEN Sulfur Plant 3 (SRU3) S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 9-1-304 | Inorganic Gaseous Pollutants – Sulfur Dioxide, Fuel Burning (Liquid and Solid Fuel) (Exempt from standard per Regulation 9-1-304.1) |

| Table IX A – 1a Permit Shield for Non-applicable Requirements S1765 –OPCEN Sulfur Plant 3 (SRU3) S4180 – OPCEN Sulfur Plant 4 (SRU4) |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 63 Subpart UUU 63.1568(b)(2) and 63.1571 | Initial performance test for Subpart UUU initial compliance demonstration (Exempt because sources are subject to NSPS and have already completed initial performance tests). |

| Table IX A - 2 Permit Shield for Non-applicable Requirements S1470 – LOP LPG Loading Flare A101 – Flare for Vine Hill Vapor Recovery System A102 – Flare for Crude String Vapor Recovery System A103 – Flare for Interim Gasoline Vapor Recovery System |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 12-11-110 | Miscellaneous Standards of Performance – Flare Monitoring at Petroleum Refineries (Exempt from standard per Regulation 12-11-110) |

| Table IX A - 4 Permit Shield for Non-applicable Requirements A1501 – F-56 Backup Thermal Oxidizer for Sulfur Plants 1 and 2 A1517 - F-77 Primary Thermal Oxidizer for Sulfur Plants 1 and 2 S3000 – Portable Vacuum Distillation Unit (CCR Technologies Inc.) |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 60, Subpart J | Standards of Performance for Petroleum Refineries (sources only combust natural gas) |

| Table IX A - 5 Permit Shield for Non-applicable Requirements S1759 – OPCENFlexicoker (FXU) S4001 – DC Delayed Coking Unit (DCU) |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 1-520.6 | General Provisions and Definitions, Continuous Emission Monitoring for SO2 and opacity from fluid cokers (Not applicable because sources are not fluid cokers) |
| BAAQMD 1-522 | General Provisions and Definitions, Continuous Emission Monitoring and Recordkeeping Procedures (Not applicable because sources are not fluid cokers) |
| BAAQMD 9-1-310 | Inorganic Gaseous Pollutants – Sulfur Dioxide, Emission Limitations for Fluid Catalytic Cracking Units, Fluid Cokers, and Coke Calcining Kilns (Not applicable because no fluid catalytic cracking units, fluid cokers, and coke calcining kilns) |

| Table IX A – 6 Permit Shield for Non-applicable Requirements - S2001, 2002, 2003 and 2004 - LOG Marine Loading Berths 1, 2, 3 and 4 |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD Regulation 8-46 | Organic Compounds – Marine Tank Vessel to Marine Tank Vessel Loading (Marine tank vessel to marine tank vessel loading is not performed) |

| Table IX A - 7 Permit Shield for Non-applicable Requirements S4191 – UTIL Boiler 6 Supplemental Steam Generator 1 S4193 – UTIL Boiler 6 Supplemental Steam Generator 2 |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 9, Rule 9 | Inorganic Gaseous Pollutants – Nitrogen Oxides from Stationary Gas Turbines (Not applicable because steam generators are not gas turbines) |

| Table IX A - 8 Permit Shield for Non-applicable Requirements Secondary Wastewater Treatment Processes and Stormwater Sewer Systems Exempt Per Regulation 8-8-114 |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 8-8-301 | Organic Compounds - Wastewater (Oil-Water) Separators, Wastewater Separators Greater than 760 Liters per Day and Smaller than 18.9 Liters per Second (Exempt per Regulation 8-8-114) |
| BAAQMD 8-8-302 | Organic Compounds - Wastewater (Oil-Water) Separators, Wastewater Separators Larger than or Equal to 18.9 Liters per Second (Exempt per Regulation 8-8-114) |
| BAAQMD 8-8-306 | Organic Compounds - Wastewater (Oil-Water) Separators, Oil-Water Separator Effluent Channel, Pond, Trench, or Basin (Exempt per Regulation 8-8-114) |
| BAAQMD 8-8-308 | Organic Compounds - Wastewater (Oil-Water) Separators, Junction Boxes (Exempt per Regulation 8-8-114) |

| Table IX A - 9 Permit Shield for Non-applicable Requirements Process Drains |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD Regulation 8, Rule 8 | Organic Compounds - Wastewater (Oil-Water) Separators (No requirements exist for Process Drains) |

| Table IX A - 10 Permit Shield for Non-applicable Requirements FACILITY |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 9-1-302 | Inorganic Gaseous Pollutants – Sulfur Dioxide, General Emission Limitation (Exempt from standard per Regulation 9-1-110) |
| BAAQMD 9-1-303 | Inorganic Gaseous Pollutants – Sulfur Dioxide, Emissions from Ships (Not applicable because no ships) |
| BAAQMD 9-1-309 | Inorganic Gaseous Pollutants – Sulfur Dioxide, Emissions from Sulfuric Acid Plants (Not applicable because no sulfuric acid plants) |
| BAAQMD 11, Rule 7 | Hazardous Pollutants –Benzene (refinery does not operate any “benzene service”) |
| BAAQMD 11, Rule 11 | Hazardous Pollutants – National Emission Standards for Benzene Emissions from Coke By-Product Recovery Plants and Benzene Storage Vessels (refinery does not store or transfer benzene) |
| 40 CFR 60 Subpart D | Standards of Performance for Fossil-Fuel-Fired Steam Generators for Which Construction is Commenced after August 17, 1971 (refinery does not operate any steam generators that are subject to this subpart) |
| 40 CFR 60 Subpart Da | Standards of Performance for Electric Utility Steam Generating Units for Which Construction is Commenced After September 18, 1978 (refinery does not operate any steam generating units that are subject to this subpart) |
| 40 CFR 60 Subpart Dc | Standards of Performance for Small Industrial-Commercial-Institutional Steam Generating Units (refinery does not operate any steam generators that are subject to this subpart) |
| 40 CFR 60 Subpart XX | Standards of Performance for Bulk Gasoline Terminals (refinery does not own or operate bulk gasoline terminal) |
| 40 CFR 60 Subpart III | Standards of Performance for Volatile Organic Compound (VOC) Emissions From the Synthetic Organic Chemical Manufacturing Industry (SOCMI) Air Oxidation Unit Processes (refinery does not operate any SOCMI operations) |
| 40 CFR 60 Subpart NNN | Standards of Performance for Volatile Organic Compound (VOC) Emissions From the Synthetic Organic Chemical Manufacturing Industry (SOCMI) Distillation Operations (refinery does not operate any SOCMI operations) |
| 40 CFR 60 Subpart RRR | Standards of Performance for Volatile Organic Compound Emissions From Synthetic Organic Chemical (refinery does not operate any SOCMI operations) |
| 40 CFR 61 Subpart J | National Emission Standard for Equipment Leaks (Fugitive Emission Sources) of Benzene (refinery does not operate any equipment in “benzene service”) |
| 40 CFR 61 Subpart V | Subpart V – National Emission Standard for Equipment Leaks (Fugitive Emission Sources) (refinery does not operate any equipment in “benzene service”) |
| 40 CFR 61 Subpart Y | National Emission Standards for Benzene Emissions from Benzene Storage Vessels (refinery does not store or transfer benzene) |
| 40 CFR 61 Subpart BB | National Emission Standards for Benzene Emissions from Benzene Transfer Operations (refinery does not store or transfer benzene) |
| 40 CFR 63 Subpart F | National Emission Standards for Hazardous Air Pollutants from the Synthetic Organic Chemical Manufacturing Industry(refinery does not operate any SOCMI operations) |
| 40 CFR 63 Subpart G | National Emission Standards for Organic Hazardous Air Pollutants From the Synthetic Organic Chemical Manufacturing Industry for Process Vents, Storage Vessels, Transfer Operations, and Wastewater (refinery does not operate any SOCMI operations) |
| 40 CFR 63 Subpart H | National Emission for Organic Hazardous Air Pollutants for Equipment Leaks (refinery does not operate any SOCMI operations) |
| 40 CFR 63 Subpart Q | National Emission Standards for Hazardous Air Pollutants for Industrial Process Cooling Towers (refinery does not use chromium based water treatment chemicals) |
| 40 CFR 63 Subpart R | National Emission Standards for Hazardous Air Pollutants for Gasoline Distribution Facilities (refinery does not own or operate a bulk gasoline terminal or pipeline breakout station) |
| 40 CFR 63 Subpart SS | National Emission Standards for Hazardous Air Pollutants for Closed Vent Systems, Control Devices, Recovery Devices and Routing to a Fuel Gas System or a Process (refinery is not subject to any subpart that references the use of this subpart for air emission control) |
| 40 CFR 63 Subpart EEE | National Emission Standards for Hazardous Air Pollutants for Hazardous Waste Incinerators (refinery does not own or operate hazardous waste incinerator, cement kiln, or aggregate kiln) |

| Table IX A - 11 Permit Shield for Non-applicable Requirements S4161 – DC H-101 HP3 Steam Methane Reformer |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 60 Subpart J 60.105 | Standards of Performance for Petroleum Refineries for Monitoring of Emissions and Operations (Not applicable when source uses alternative monitoring in accordance with 60.13(i) for process swing absorption gas.) |

| Table IX A - 12 Permit Shield for Non-applicable Requirements S1471 – LOP Auxiliary Flare S1472 – LOP Main Flare S1772 – OPCEN HC Flare S4201 – DC Clean Fuels Flare A101 – Flare for Vine Hill Vapor Recovery System A102 – Flare for Crude String Vapor Recovery System A103 – Flare for Interim Gasoline Vapor Recovery System |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 60 Subpart J 60.104(a)(1) | Standards of Performance for Petroleum Refineries for Standards for Sulfur Oxides (Not applicable because these flares combust only process upset gases or fuel gas that is released to the flare as a result of relief valve leakage or other emergency malfunction and is therfore exempt from the standard in accordance with 60.104(a)(1).) |
| 40 CFR 60 Subpart J 60.105 | Standards of Performance for Petroleum Refineries for Monitoring of Emissions and Operations (Not applicable because sources are not subject to the standard for sulfur oxides in 60.104(a)(1).) |

| Table IX A - 13 Permit Shield for Non-applicable Requirements S1771 – Flexigas Flare |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| BAAQMD 12-11-401.2 | Flare Monitoring at Petroleum Refineries; exempt per BAAQMD 12-11-114. |
| BAAQMD 12-11-401.3 | Flare Monitoring at Petroleum Refineries; exempt per BAAQMD 12-11-114. |
| BAAQMD 12-11-401.5 | Flare Monitoring at Petroleum Refineries; exempt per BAAQMD 12-11-114. |
| BAAQMD 12-11-502.2 | Flare Monitoring at Petroleum Refineries; exempt per BAAQMD 12-11-114. |
| BAAQMD 12-11-502.3 | Flare Monitoring at Petroleum Refineries; exempt per BAAQMD 12-11-114. |

| Table IX A - 13 Permit Shield for Non-applicable Requirements S1771 – OPCEN Flexigas Flare S1772 – OPCEN Hydrocarbon Flare |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 60 Subpart J 60.104(a)(1) | Standards of Performance for Petroleum Refineries for Standards for Sulfur Oxides (Not applicable only when these flares combust only process upset gases or fuel gas that is released to the flare as a result of relief valve leakage or other emergency malfunction that is exempt from the standard in accordance with 60.104(a)(1).) |
| 40 CFR 60 Subpart J 60.105 | Standards of Performance for Petroleum Refineries for Monitoring of Emissions and Operations (Not applicable only when these flares combust only process upset gases or fuel gas that is released to the flare as a result of relief valve leakage or other emergency malfunction that is exempt from the standard in accordance with 60.104(a)(1).) |

| Table IX A – 14 Permit Shield for Non-applicable Requirements S1425 – DH Catalytic Reformer Unit (CRU) |  |
| --- | --- |
| Citation | Title or Description (Reason not applicable) |
| 40 CFR 63 Subpart UUU 63.1566 | Organic HAP emission limitations from Catalytic Reforming Units (exempt per 40 CFR 1562(f)(5); gaseous stream routed to fuel gas system). |

## Subsumed Requirements

Pursuant to District Regulations 2-6-233.2 and 2-6-409.12, as of the date this permit is issued, the federally enforceable monitoring, recordkeeping, and reporting requirements cited in the following table for the source or group of sources identified at the top of the table[s] are subsumed by the monitoring, recordkeeping, and reporting for more stringent requirements or by a “hybrid” monitoring scheme.  The District has determined that compliance with the requirements listed below and elsewhere in this permit will assure compliance with the substantive requirements of the subsumed monitoring requirements.  Enforcement actions and litigation may not be initiated against the source or group of sources covered by this shield based on the subsumed monitoring requirements cited.

| Table IX B – 1 Permit Shield for Subsumed Requirements S129 - Tank 129, S540 - Tank 540, S541 – Tank 541, S544 - Tank 544, S545 - Tank 545, S992 – Tank 992, S1031 - Tank 1031, S1046 - Tank 1046, S1051 - Tank 1051, S1063 - Tank 1063 ETP 1, S1067 - Tank 1067 ETP 1, S1072 - Tank 1072, S1076 – Tank 1076, S1129 - Tank 1129, S1146 - Tank 1146, S1147 - Tank 1147, S1161 - Tank 1161, S1159 - Tank 1159, S1755 - Tank 1334 Gasoline, S1756 - Tank 1335 Gasoline |  |  |  |
| --- | --- | --- | --- |
| Subsumed Requirement Citation | Title or Description | Streamlined Requirements | Title or Description |
| BAAQMD Reg 8 Rule 5 | Organic Compounds – Storage of Organic Liquids |  |  |
| 8-5-501 | Records: Subsumed into the Refinery MACT recordkeeping requirements. | Section 63.654 | Notification of Compliance Status report |

| Table IX B – 2 Permit Shield for Subsumed Requirements S858 - Tank 858, S1023 - Tank 1023, S1050 Tank 1050, S2445 - Tank 12445, S2446 – Tank 12446, S4322 - Tank 14571 Sour Water |  |  |  |
| --- | --- | --- | --- |
| Subsumed Requirement Citation | Title or Description | Streamlined Requirements | Title or Description |
| BAAQMD Reg 8 Rule 5 | Organic Compounds – Storage of Organic Liquids |  |  |
| 8-5-501 | Records: Subsumed into the Refinery MACT recordkeeping requirements. | Section 63.654 | Notification of Compliance Status report |
| 40 CFR Part 60 Subpart Kb | Standards of Performance for Volatile Organic Liquid Storage Vessels (Including Petroleum Liquid Storage Vessels) for Which Construction, Reconstruction, or Modification Commenced After July 23, 1984 |  |  |
| 60.115b(b) | Reporting and Recordkeeping for EFRTs. Subsumed into the Refinery MACT requirements. | Section 63.654 | Notification of Compliance Status report |
| 60.116b (a)-(c) | Additional Recordkeeping. Subsumed into the Refinery MACT requirements. | Section 63.654 | Notification of Compliance Status report |

| Table IX B – 3 Permit Shield for Subsumed Requirements S1006 - Tank 1006, S1077 - Tank 1411, S1130 - Tank 1130, S1131 - Tank 1131, S1191 - Tank -1256 Crude Oil Storage, S1192 - Tank -1257 Crude Oil Storage, S2013 - Tank 12467, S4310 - Tank 13285 Sour Water, S12490 - LOG Tank 12519 Wastewater ETP 1&2, S12491 - LOG Tank 12520 Wastewater ETP 1&2, S17095 – Tank 17095 |  |  |  |
| --- | --- | --- | --- |
| Subsumed Requirement Citation | Title or Description | Streamlined Requirements | Title or Description |
| BAAQMD Regulation 8, Rule 5 | Organic Compounds – Storage of Organic Liquids |  |  |
| 8-5-501 | Records: Subsumed into the Refinery MACT recordkeeping requirements. | Section 63.654 | Notification of Compliance Status report |
| 40 CFR Part 60 Subpart Kb | Standards of Performance for Volatile Organic Liquid Storage Vessels (Including Petroleum Liquid Storage Vessels) for Which Construction, Reconstruction, or Modification Commenced After July 23, 1984 |  |  |
| 60.115b(b) | Reporting and Recordkeeping for EFRTs. Subsumed into the Refinery MACT requirements. | Section 63.654 | Notification of Compliance Status report |
| 60.116b (a)-(c) | Additional Recordkeeping. Subsumed into the Refinery MACT requirements. | Section 63.654 | Notification of Compliance Status report |

# Glossary

- ACT

- Federal Clean Air Act

- APCO

- Air Pollution Control Officer

- API

- American Petroleum Institute

- ARB

- Air Resources Board

- BAAQMD

- Bay Area Air Quality Management District

- BACT

- Best Available Control Technology

- BARCT

- Best Available Retrofit Control Technology

Basis

The underlying authority that allows the District to impose requirements.

- C5

- An Organic chemical compound with five carbon atoms

- C6

- An Organic chemical compound with six carbon atoms

- CAA

- The federal Clean Air Act

- CAAQS

- California Ambient Air Quality Standards

- CCR-2

- Canadian Chemical Reclaimer heater.

- CEC

- California Energy Commission

CEM

A "continuous emission monitor" is a monitoring device that provides a continuous direct measurement of some pollutant (e.g. NOx concentration) in an exhaust stream.

- CEQA

- California Environmental Quality Act

CFP

Clean Fuels Project

- CFR

The Code of Federal Regulations.  40 CFR contains the implementing regulations for federal environmental statutes such as the Clean Air Act.  Parts 50-99 of 40 CFR contain the requirements for air pollution programs.

- CO

- Carbon Monoxide

- CO2

- Carbon Dioxide

- Cumulative Increase

The sum of permitted emissions from each new or modified source since a specified date pursuant to BAAQMD Rule 2-1-403, Permit Conditions (as amended by the District Board on 7/17/91) and SIP Rule 2-1-403, Permit Conditions (as approved by EPA on 6/23/95).  Used to determine whether threshold-based requirements are triggered.

- DAF

- A "dissolved air flotation" unit is a process vessel where air bubbles injected at the bottom of the vessel are used to carry solids in the liquid into a froth on the liquid surface, where it is removed.

- DWT

- Dead Weight Ton

- District

- The Bay Area Air Quality Management District

- DNF

- Dissolved Nitrogen Flotation  (See DAF)

dscf

Dry Standard Cubic Feet

dscm

Dry Standard Cubic Meter

E 6, E 9, E 12

Very large or very small number values are commonly expressed in a form called scientific notation, which consists of a decimal part multiplied by 10 raised to some power.  For example, 4.53 E 6 equals (4.53) x (106) = (4.53) x (10 x 10 x 10 x 10 x 10 x 10) = 4,530,000.  Scientific notation is used to express large or small numbers without writing out long strings of zeros.

EFRT

An "external floating roof tank" minimizes VOC emissions with a roof with floats on the surface of the liquid, thus preventing the formation of a VOC-rich vapor space above the liquid surface as the level in the tank drops.  If such a vapor space were allowed to form, it would be expelled when the tank was re-filled.  On an EFRT, the floating roof is not enclosed by a second, fixed tank roof, and is thus described as an "external" roof.

EMP

Environmental Management Plan

- EPA

- The federal Environmental Protection Agency.

ESP

Electrostatic Precipitator

- ETP

- Effluent Treatment Plant

- Excluded

- Not subject to any District Regulations.

- CCU

- (Fluidized) Catalytic Cracking Unit

- Federally Enforceable, FE

All limitations and conditions which are enforceable by the Administrator of the EPA including those requirements developed pursuant to 40 CFR Part 51, subpart I (NSR), Part 52.21 (PSD), Part 60 (NSPS), Part 61 (NESHAPs), Part 63 (HAP), and Part 72 (Permits Regulation, Acid Rain), and also including limitations and conditions contained in operating permits issued under an EPA-approved program that has been incorporated into the SIP.

- FP

Filterable Particulate as measured by BAAQMD Method ST-15, Particulate.

- Furfural Raff/Furfural Extr

- These sources are heaters that contain furnaces within them.  The heater is the overall unit and the combustion box is the furnace.

FRT

Floating Roof Tank (See EFRT and IFRT)

- GDF

- Gasoline Dispensing Facility

- GLM

- Ground Level Monitor

- grains

- 1/7000 of a pound

- Graphitic

- Made of graphite.

GRU

Gas Recovery Unit

- HAP

Hazardous Air Pollutant.  Any pollutant listed pursuant to Section 112(b) of the Act.  Also refers to the program mandated by Title I, Section 112, of the Act and implemented by 40 CFR Part 63.

H2S

Hydrogen Sulfide

H2SO4

Sulfuric Acid

HC

Hydrocarbon

Hg

Mercury

HHV

Higher Heating Value.  The quantity of heat evolved as determined by a calorimeter where the combustion products are cooled to 60F and all water vapor is condensed to liquid.

IERC

Interchangeable Emission Reduction Credits.

IFRT

An "internal floating roof tank" minimizes VOC emissions with a roof with floats on the surface of the liquid, thus preventing the formation of a VOC-rich vapor space above the liquid surface as the level in the tank drops.  If such a vapor space were allowed to form, it would be expelled when the tank was re-filled.  On an IFRT, the floating roof is enclosed by a second, fixed tank roof, and thus is described as an "internal" roof.

ISOM

Isomerization plant

LHV

Lower Heating Value.  Similar to the higher heating value (see HHV) except that the water produced by the combustion is not condensed but retained as vapor at 60F.

- Lighter

- "Lightering" is a transfer operation during which liquid is pumped from an ocean-going tanker vessel to a smaller vessel such as a barge.  Like any liquid transfer operation, lightering of organic liquids produces organic vapor emissions.

Long ton

2200 pounds

LPG

Liquid Petroleum Gas

LFSO

Low sulfur fuel oil

- Major Facility

A facility with potential emissions of: (1) at least 100 tons per year of regulated air pollutants, (2) at least 10 tons per year of any single hazardous air pollutant, and/or (3) at least 25 tons per year of any combination of hazardous air pollutants, or such lesser quantity of hazardous air pollutants as determined by the EPA administrator.

- MDEA

- Methyl Diethanolamine

- MFR

Major Facility Review.  The District's term for the federal operating permit program mandated by Title V of the Act and implemented by District Regulation 2, Rule 6.

MM

Million

Mo Gas

Motor gasoline

- MOP

- The District's Manual of Procedures

- MOSC

- Mobil Oil Sludge Conversion (licensed technology)

- MSDS

- Material Safety Data Sheet

- MTBE

- methyl tertiary-butyl ether

- NA

- Not Applicable

- NAAQS

- National Ambient Air Quality Standards

- NESHAPS

- National Emission Standards for Hazardous Air Pollutants.  See in 40 CFR Parts 61 and 63.

- NMHC

- Non-methane Hydrocarbons

- NMOC

Non-methane Organic Compounds (Same as NMHC)

- NOx

- Oxides of nitrogen.

- NSPS

Standards of Performance for New Stationary Sources.  Federal standards for emissions from new stationary sources.  Mandated by Title I, Section 111 of the Act, and implemented by 40 CFR Part 60 and District Regulation 10.

- NSR

New Source Review.  A federal program for pre-construction review and permitting of new and modified sources of air pollutants for which the District is classified "non-attainment".  Mandated by Title I of theFederal Clean Air Act and implemented by 40 CFR Parts 51 and 52 as well as District Regulation 2, Rule 2.  (Note:  There are additional NSR requirements mandated by the California Clean Air Act.)

- O2

- The chemical name for naturally-occurring oxygen gas.

- Offset Requirement

A New Source Review requirement to provide federally enforceable emission offsets at a specified ratio for the emissions from a new or modified source and any pre-existing cumulative increase minus any onsite contemporaneous emission reduction credits.  Applies to emissions of POC, NOx, PM10, and SO2.

- Phase II Acid Rain Facility

A facility that generates electricity for sale through fossil-fuel combustion and is not exempted by 40 CFR 72 from Titles IV and V of the Clean Air Act.

- POC

- Precursor Organic Compounds

- PM

- Total Particulate Matter

- PM10

- Particulate matter with aerodynamic equivalent diameter of less than or equal to 10 microns

- PSD

Prevention of Significant Deterioration.  A federal program for permitting new and modified sources of air pollutants for which the District is classified "attainment" of the National Air Ambient Quality Standards.  Mandated by Title I of the Act and implemented by both 40 CFR Part 52 and District Regulation 2, Rule 2.

Process Unit

For the purpose of start-up and shutdown a unit is defined as found in 40 CFR Part 60 Subpart GGG, which states:

Process Unit means components assembled to produce intermediate or final products from petroleum, unfinished petroleum derivates, or other intermediates; a process unit can operate independently if supplied with sufficient feed or raw materials and sufficient storage facilities for the product.

RACT

Reasonably Available Control Technology

Regulated Organic Liquid

"Regulated organic liquids" are those liquids which require permits, or which are subject to some regulation, when processed at a liquid-handling operation.  For example, for refinery marine terminals, regulated organic liquids are defined as "organic liquids" in Regulation 8, Rule 44.

RFG

Refinery Fuel Gas

RMG

Refinery Make Gas

- SCR

- A "selective catalytic reduction" unit is an abatement device that reduces NOx concentrations in the exhaust stream of a combustion device.  SCRs utilize a catalyst, which operates at a specific temperature range, and injected ammonia to promote the conversion of NOx compounds to nitrogen gas.

- SIP

State Implementation Plan.  State and District programs and regulations approved by EPA and developed in order to attain the National Air Ambient Quality Standards.  Mandated by Title I of the Act.

- SO2

- Sulfur dioxide

- SO2 Bubble

- An SO2 bubble is an overall cap on the SO2 emissions from a defined group of sources, or from an entire facility.  SO2 bubbles are sometimes used at refineries because combustion sources are typically fired entirely or in part by "refinery fuel gas" (RFG), a waste gas product from refining operations.  Thus, total SO2 emissions may be conveniently quantified by monitoring the total amount of RFG that is consumed, and the concentration of H2S and other sulfur compounds in the RFG.

- SO3

- Sulfur trioxide

SRU

Sulfur Recovery Unit

ST-7

Source Test Method #7:  Non-Methane Organic Carbon Sampling

Start-up

For reporting purposes only, a start-up shall be defined as any of the following; the removal of boundary blinds, first fire to a furnace, or the introduction of process feed to a unit.  A start-up only occurs following a shutdown unless it involves a newly constructed process unit.

Shutdown

For reporting purposes only, a shutdown shall be defined as any of the following; there is no process feed to a unit, no furnace fires, or the boundary blinds are installed.

THC

Total Hydrocarbons (NMHC + Methane)

therm

100,000 British Thermal Unit

- Title V

Title V of the federal Clean Air Act.  Requires a federally enforceable operating permit program for major and certain other facilities.

TOC

Total Organic Compounds (NMOC + Methane, Same as THC)

TPH

Total Petroleum Hydrocarbons

TRMP

Toxic Risk Management Plan

- TRS

- "Total reduced sulfur" is a measure of the amount of sulfur-containing compounds in a gas stream, typically a fuel gas stream, including, but not limited to, hydrogen sulfide.  The TRS content of a fuel gas determines the concentration of SO2 that will be present in the combusted fuel gas, since sulfur compounds are converted to SO2 by the combustion process.

- TSP

- Total Suspended Particulate

- TVP

- True Vapor Pressure

VGO

Vacuum Gas Oil

VR

Vapor Recovery

- VOC

- Volatile Organic Compounds

- Units of Measure:

bbl	=	barrel of liquid (42 gallons)

bhp	=	brake-horsepower

btu	=	British Thermal Unit

C	= 	degrees Celsius

F	=	degrees Farenheit

f3	=	cubic feet

g	=		grams

gal	=	gallon

gpm	=	gallons per minute

hp	=	horsepower

hr	=	hour

lb	=	pound

in	=	inches

M	= 	thousand

m2	=	square meter

max	=	maximum

min	=	minute

Mg	=	mega-gram, one thousand grams

mm	=	millimeter

g	=	micro-gram, one millionth of a gram

MM	=	million

mm	=	millimeter

MMbtu	=	million btu

mm Hg	=	millimeters of Mercury (pressure)

MW	=	megawatts

ppmv	=	parts per million, by volume

ppmw	=	parts per million, by weight

psia	=	pounds per square inch, absolute

psig	=	pounds per square inch, gauge

scfm	=	standard cubic feet per minute

yr	=	year

- Symbols:

<		=	less than

>		=	greater than

<		=	less than or equal to

>		=	greater than or equal to

# APPLICABLE STATE IMPLEMENTATION PLAN

The Bay Area Air Quality Management District’s portion of the State Implementation Plan can be found at EPA Region 9’s website.  The address is:

http://yosemite1.epa.gov/r9/r9sips.nsf/California?ReadForm&Start=1&Count=30&Expand=3.1